| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G355 |
| --- | --- |

| Title: | Joint coding of splitting flag and inter modes in CAVLC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wen Zhang Ping Wu | Tel: Email: | +86-13528803560 zhang.wen4@zte.com.cn |
| Source: | ZTE Corporation |  |  |

_____________________________

- Abstract

This contribution was successor to JCTVC-F418 and was focused on CAVLC only. It had resolved parsing dependency problem in JCTVC-F418 meanwhile maintained coding gain. It provided an alternative joint coding scheme of splitting flag and inter modes in CAVLC rather than what adopted in HM 4.0. According test results, -0.1%/-0.4%/-0.6%/-0.6% BD-rate gain was achieved comparing to HM 4.0 for AI-LC/RA-LC/LB-LC/LP-LC configurations.

Introduction

In HM 4.0, the joint coding scheme of splitting flag and inter modes in CAVLC was characterized as depth-dependent (JCTVC-D370) and counter adaptive (JCTVC-E143). In this proposal, the representing method of CU splitting flag and inter modes was however context-dependent and slice adaptive.

Firstly, the coding CU splitting flag and inter modes was classified into 4 types according to contexts of current CU. It’s specifically classified according to CU splitting and prediction mode of left and above CUs, along with current CU-depth. Please refer to Table-1 for detailed coding types of CU splitting flag and inter modes.

Table-1 4 coding types (contexts) defined for CU splitting flag and inter modes

| CU-depth | Prediction mode-top CU | Prediction mode-left CU | Coding Type |
| --- | --- | --- | --- |
| CU size > 8x8 (min) | SKIP | SKIP | 0 |
|  | INTER | SKIP |  |
|  | SKIP | INTER |  |
|  | INTRA | SKIP |  |
|  | SKIP | INTRA |  |
|  | CU Splitting | SKIP |  |
|  | SKIP | CU Splitting |  |
|  | INTER | INTER |  |
|  | INTRA | INTER | 1 |
|  | INTER | INTRA |  |
|  | CU Splitting | INTER |  |
|  | INTER | CU Splitting |  |
|  | INTRA | INTRA |  |
|  | CU Splitting | INTRA |  |
|  | INTRA | CU Splitting |  |
|  | CU Splitting | CU Splitting |  |
| CU size==8x8(min) | SKIP | SKIP | 2 |
|  | INTER | SKIP |  |
|  | SKIP | INTER |  |
|  | INTRA | SKIP |  |
|  | SKIP | INTRA |  |
|  | INTER | INTER |  |
|  | INTRA | INTER | 3 |
|  | INTER | INTRA |  |
|  | INTRA | INTRA |  |

Secondly, regarding to AMP had been added in HM 4.0 meanwhile inter 4x4 was disabled, CU splitting flag and inter modes supported in HM 4.0 were revaluated and summarized in below table-2. A new syntax element split_mode was thus defined to represent CU splitting flag and inter modes.

From table-2, the value of split_mode was ranged from 0(SKIP) to 7(AMP). When split_mode equals to INTER_AMP, it included four PartMode of 2NxnU, 2NxnD, nLx2N, nRx2N which were further signaled by two flags. If split_mode is INTER_2NxN or INTER_Nx2N, two additional flags were used to signal merge information of two PU included in current CU.

Table-2 split_mode defined for CU splitting flag and inter modes

| PredMode and CU splitting | PartMode | CU size > 8x8 (min) split_mode | CU size == 8x8(min) split_mode |
| --- | --- | --- | --- |
| MODE_SKIP | PART_2Nx2N | 0 (SKIP_2Nx2N) | 0 (SKIP_2Nx2N) |
| MODE_MRG | PART_2Nx2N | 5 (MRG_2Nx2N) | 5 (MRG_2Nx2N) |
| MODE_INTER | PART_2Nx2N | 1(INTER_2Nx2N) | 1 (INTER_2Nx2N) |
|  | PART_2NxN | 3 (INTER_2NxN) | 3 (INTER_2NxN) |
|  | PART_Nx2N | 4 (INTER_Nx2N) | 4 (INTER_Nx2N) |
|  | PART_2NxnU | 7 (INTER_AMP) | -- |
|  | PART_2NxnD |  |  |
|  | PART_nLx2N |  |  |
|  | PART_nRx2N |  |  |
|  | PART_NxN | -- | 7 (INTER_NxN) disabled |
| MODE_INTRA | PART_2Nx2N | 2 (INTRA_2Nx2N) | 2(INTRA_2Nx2N) |
|  | PART_NxN | -- | 6(INTRA_NxN) |
| SPLIT |  | 6(SPLIT) | -- |
| PCM | PART_2Nx2N | disabled | Disabled |

Thirdly, 31 codeword depending on distribution of each coding types for split_mode were defined and explicitly signaled per slice. Please refer to below g_auiCtxDepthInterSplitMode for detailed codeword information.

#define ENCTYPE(a,b,c,d,e,f,g,h) (((h)<<21)+((g)<<18)+((f)<<15)+((e)<<12)+((d)<<9)+((c)<<6)+ ((b)<<3)+((a)<<0))

#define CODEWORD_LEN                31

Const UInt g_auiCtxDepthInterSplitMode[4][CODEWORD_LEN] = {

{ENCTYPE(0,6,7,1,4,5,3,2),ENCTYPE(0,6,1,7,4,3,2,5),ENCTYPE(0,6,5,7,1,2,4,3),ENCTYPE(6,0,7,5,1,2,4,3),ENCTYPE(0,6,7,4,1,5,3,2),ENCTYPE(0,6,4,7,1,3,5,2),ENCTYPE(0,6,7,5,1,4,3,2),ENCTYPE(0,6,1,4,7,3,2,5),ENCTYPE(6,7,0,5,1,2,4,3),ENCTYPE(0,6,4,1,7,3,5,2),ENCTYPE(0,5,6,1,7,4,3,2),ENCTYPE(0,1,6,2,7,3,4,5),ENCTYPE(5,6,0,7,2,1,3,4),ENCTYPE(7,6,5,0,1,2,4,3),ENCTYPE(6,5,0,7,1,2,4,3),ENCTYPE(6,5,7,0,1,3,4,2),ENCTYPE(6,7,5,1,4,3,2,0),ENCTYPE(6,1,0,7,5,2,3,4),ENCTYPE(7,5,6,0,1,2,4,3),ENCTYPE(7,0,6,5,1,2,4,3),ENCTYPE(5,0,6,7,2,1,3,4),ENCTYPE(5,2,6,7,0,1,3,4),ENCTYPE(5,7,6,0,2,1,3,4),ENCTYPE(2,5,6,0,1,7,3,4),ENCTYPE(1,5,6,0,7,2,3,4),ENCTYPE(0,2,6,1,5,7,3,4),ENCTYPE(0,3,6,1,5,7,4,2),ENCTYPE(0,4,6,1,5,7,3,2),ENCTYPE(0,7,6,1,5,4,3,2),ENCTYPE(0,6,2,7,1,4,3,5),ENCTYPE(0,6,3,7,1,4,2,5)},

{ENCTYPE(6,0,7,1,4,3,5,2),ENCTYPE(6,7,0,4,1,5,3,2),ENCTYPE(0,6,1,7,4,3,5,2),ENCTYPE(6,0,1,7,4,3,2,5),ENCTYPE(6,2,7,5,0,1,4,3),ENCTYPE(6,5,7,2,0,1,4,3),ENCTYPE(6,0,4,7,1,5,2,3),ENCTYPE(6,7,5,0,2,4,1,3),ENCTYPE(2,6,0,1,5,7,3,4),ENCTYPE(6,7,2,0,5,4,1,3),ENCTYPE(6,7,4,0,5,2,1,3),ENCTYPE(6,2,5,7,0,1,4,3),ENCTYPE(0,1,6,4,3,7,2,5),ENCTYPE(0,6,7,1,4,3,5,2),ENCTYPE(6,0,5,7,1,4,2,3),ENCTYPE(1,0,6,4,3,2,5,7),ENCTYPE(2,0,6,1,5,7,3,4),ENCTYPE(0,6,4,1,7,3,5,2),ENCTYPE(0,2,6,1,4,3,7,5),ENCTYPE(0,3,6,1,4,2,7,5),ENCTYPE(0,4,6,1,3,7,2,5),ENCTYPE(0,7,6,1,4,3,2,5),ENCTYPE(6,1,0,7,2,5,4,3),ENCTYPE(6,4,1,0,7,2,5,3),ENCTYPE(6,7,1,0,5,2,4,3),ENCTYPE(6,7,3,1,0,5,2,4),ENCTYPE(6,0,2,7,1,4,5,3),ENCTYPE(6,2,1,7,5,0,4,3),ENCTYPE(6,2,0,7,5,1,4,3),ENCTYPE(6,5,0,7,2,1,4,3),ENCTYPE(6,5,2,7,0,1,4,3)},  {ENCTYPE(0,1,4,2,3,5,6,7),ENCTYPE(0,5,1,4,2,3,6,7),ENCTYPE(0,4,1,3,5,2,6,7),ENCTYPE(0,1,2,4,3,5,6,7),ENCTYPE(5,0,1,4,2,3,6,7),ENCTYPE(0,4,3,1,5,2,6,7),ENCTYPE(0,5,4,1,2,3,6,7),ENCTYPE(0,5,2,1,4,3,6,7),ENCTYPE(0,2,1,5,6,4,3,7),ENCTYPE(0,1,3,4,2,5,6,7),ENCTYPE(0,1,5,4,2,3,6,7),ENCTYPE(0,2,5,1,6,4,3,7),ENCTYPE(0,4,5,1,3,2,6,7),ENCTYPE(0,3,4,1,5,2,6,7),ENCTYPE(5,1,0,4,3,2,6,7),ENCTYPE(5,0,4,1,2,3,6,7),ENCTYPE(0,6,1,5,4,2,3,7),ENCTYPE(0,3,1,4,5,2,6,7),ENCTYPE(0,3,5,4,1,2,6,7),ENCTYPE(0,2,4,1,5,6,3,7),ENCTYPE(0,2,6,1,5,4,3,7),ENCTYPE(0,4,2,1,3,5,6,7),ENCTYPE(1,5,4,0,2,3,6,7),ENCTYPE(2,0,6,5,1,4,3,7),ENCTYPE(4,0,1,5,3,2,6,7),ENCTYPE(0,5,3,1,4,2,6,7),ENCTYPE(5,2,0,1,4,3,6,7),ENCTYPE(5,3,0,1,4,2,6,7),ENCTYPE(5,4,0,1,3,2,6,7),ENCTYPE(5,6,0,1,4,2,3,7),ENCTYPE(5,0,2,1,4,6,3,7)},  {ENCTYPE(2,6,0,5,1,4,3,7),ENCTYPE(0,1,2,4,3,5,6,7),ENCTYPE(2,0,6,5,1,4,3,7),ENCTYPE(0,2,1,4,3,5,6,7),ENCTYPE(2,5,6,0,1,4,3,7),ENCTYPE(5,2,0,1,4,6,3,7),ENCTYPE(6,2,5,1,4,0,3,7),ENCTYPE(2,1,6,0,5,4,3,7),ENCTYPE(1,0,2,4,6,5,3,7),ENCTYPE(0,4,1,2,3,5,6,7),ENCTYPE(5,0,2,1,4,6,3,7),ENCTYPE(1,2,0,4,6,5,3,7),ENCTYPE(3,0,4,1,5,2,6,7),ENCTYPE(0,3,1,2,4,5,6,7),ENCTYPE(4,0,1,3,5,6,2,7),ENCTYPE(0,6,1,2,4,5,3,7),ENCTYPE(0,5,1,2,4,3,6,7),ENCTYPE(1,4,0,2,6,5,3,7),ENCTYPE(1,5,0,2,4,6,3,7),ENCTYPE(1,6,0,2,4,5,3,7),ENCTYPE(3,4,0,1,5,2,6,7),ENCTYPE(4,1,0,3,5,6,2,7),ENCTYPE(4,3,0,1,5,6,2,7),ENCTYPE(4,5,0,1,3,6,2,7),ENCTYPE(5,1,2,0,4,6,3,7),ENCTYPE(5,3,2,0,1,4,6,7),ENCTYPE(5,4,2,0,1,6,3,7),ENCTYPE(5,6,2,0,1,4,3,7),ENCTYPE(6,1,2,5,4,0,3,7),ENCTYPE(6,4,2,5,1,0,3,7),ENCTYPE(6,5,2,1,4,0,3,7)}

};

In each inter slice, when the first time it has one coding type (bFirstCodingTypeInSlice), which ranged from 0 to 3 before coding split_mode. A codeword_index which ranged from 0 to CODEWORD_LEN-1 to represent codeword for the coding type was signaled and transmitted using UVLC. For each codeword, it represented the coding order of unary flags in split_mode. And, the codeword was highly context dependent and basically slice adaptive.

Since codeword was explicitly signaled and transmitted, the decoder didn't need to gather statistics of previous slices like in JCTVC-F418. Thus there is no parsing dependency problem anymore. But such information was still helpful and could be used at encoder side in RDO progress.

The fourth, except for above context dependent and slice adaptive coding method, swapping adaption process in HM 4.0 was also incorporated as complementary part to explicit codeword signaling.

Finally, in order to further remove redundancy existed in signaling of CU splitting flag both in inter and intra slice, A new syntax element pred_split_flag was defined while CU-depth equals to 0 or 1. If this flag equals to 1, it means not only CU of current depth with split_flag equals to 1 but also all CUs here of current depth plus one with split_flags equal to 1. The coding gain of pred_split_flag was roughly 0.1% BD-rate for AI-LC and RA-LC configurations.

Meanwhile, the presence of pred_split_flag was adaptively controlled at two levels. At slice level, a controlling flag (pred_split_coding_flag) was added into slice header, it could switch on/off the presence of pred_split_flag in current slice. At CU level, it was decided by bHasPredSplitFlag to code pred_split_flag in current CU or not according to above and left CU-depth information. The detailed constrained conditions were defined as below to deduce bHasPredSplitFlag of current CU.

If ((getSlice()->isIntra() && (uiDepth == 0)) ||

((getSlice()->isIntra() ||  (uiDepth == 0)) && (getSlice()->getPredSplitCodingFlag())))

{

bHasPredSplitFlag = above A CUs with smaller CU-depth OR left B CUs with smaller CU-depth;

}

Else if (!getSlice()->isIntra() && (uiDepth > 0) && getSlice()->getPredSplitCodingFlag())

{

bHasPredSplitFlag = above A CUs with smaller CU-depth AND left B CUs with smaller CU-depth;

}

Else

{

bHasPredSplitFlag = false;

}

WD Text

The change to current WD draft in slice_header() was shown in table-3.

Table-3 Add pred_split_coding_flag to slice_header()

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| if (!entropy_coding_mode_flag) |  |
| pred_split_coding_flag | u(1) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| } |  |
| } |  |

The change to current WD draft in coding_tree() was shown in table-4.

Table-4 Add pred_split_flag and codeword_index/split_mode in coding_tree()

| coding_tree( x0, y0, log2CUSize, cuDepth ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CUSize ) <= PicWidthInSamplesL && y0 + ( 1 << log2CUSize ) <= PicHeightInSamplesL && cuAddress( x0, y0 ) >= SliceAddress ) { |  |
| if (!entropy_coding_mode_flag) { |  |
| if (bHasPredSplitFlag) |  |
| pred_split_flag | u(1) |
| if (!pred_split_flag) { |  |
| if (slice_type == I) |  |
| split_coding_unit_flag[x0][y0] | u(1) |
| else { |  |
| if (bFirstCodingTypeInSlice) |  |
| codeword_index | ue(v) |
| split_mode[x0][y0] | ce(v) |
| } |  |
| } |  |
| } |  |
| else if ( log2CUSize > Log2MinCUSize ) |  |
| split_coding_unit_flag[x0][y0] | u(1)¦ae(v) |
| } |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) { |  |
| if( cuDepth <= alf_cu_control_max_depth ) |  |
| if( cuDepth == alf_cu_control_max_depth ¦¦ |  |
| split_coding_unit_flag[ x0 ][ y0 ] == 0 ) |  |
| AlfCuFlagIdx++ |  |
| } |  |
| if( cu_qp_delta_enabled_flag && log2CUSize >= log2MinCUDQPSize ) |  |
| IsCuQpDeltaCoded = 0 |  |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if( cuAddress( x1, y0 ) > SliceAddress ) |  |
| moreDataFlag = coding_tree( x0, y0, log2CUSize – 1, cuDepth + 1 ) |  |
| if( cuAddress( x0, y1 ) > SliceAddress && moreDataFlag && x1 < PicWidthInSamplesL ) |  |
| moreDataFlag = coding_tree( x1, y0, log2CUSize − 1, cuDepth + 1 ) |  |
| if( cuAddress( x1, y1 ) > SliceAddress && moreDataFlag && y1 < PicHeightInSamplesL ) |  |
| moreDataFlag = coding_tree( x0, y1, log2CUSize − 1, cuDepth + 1 ) |  |
| if( moreDataFlag && x1 < PicWidthInSamplesL && y1 < PicHeightInSamplesL ) |  |
| moreDataFlag = coding_tree( x1, y1, log2CUSize − 1, cuDepth + 1 ) |  |
| } else { |  |
| if(adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlag[ x0 ][ y0 ] = alf_cu_flag[ AlfCuFlagIdx ] |  |
| coding_unit( x0, y0, log2CUSize ) |  |
| if( !entropy_coding_mode_flag ) |  |
| moreDataFlag = more_rbsp_data( ) |  |
| else { |  |
| if( granularity_block_boundary( x0, y0, log2CUSize ) ) { |  |
| end_of_slice_flag | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |
| } else |  |
| moreDataFlag = 1 |  |
| } |  |
| } |  |
| return moreDataFlag |  |
| } |  |

The change to current WD draft in coding_unit() was shown in Table-5.

Table-5 Delete intra_part_mode in coding_unit()

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| else { |  |
| if( !entropy_coding_mode_flag ) { |  |
| } else if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) |  |
| pred_type | u(v) ¦ ae(v) |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxnU ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y2 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxnD ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y3 , log2CUSize ) |  |
| } else if( PartMode == PART_nLx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x2, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_nRx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x3, y0 , log2CUSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| prediction_unit( x1, y1 , log2CUSize ) |  |
| } |  |
| } |  |
| if( !pcm_flag ) { |  |
| transform_tree( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 2 ) |  |
| } |  |
| } |  |

Simulation Results

The proposed algorithm was implemented based on HM 4.0 and tested on NT server 2003 32 bits OS. So, please be noted that the anchor used was a little bit different to BBC’s as a known problem of ticket #203 (slideshow LDPLC QP==32). For cross check results, please refer to JCTVC-G741 by Samsung and JCTVC-G905 by Qualcomm.

Table-6 Simulation Results with CODEWORD_LEN equals to 31.

|  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V |  |
| Class A | 0.0% | 0.0% | 0.0% |  |
| Class B | 0.0% | -0.1% | 0.0% |  |
| Class C | -0.1% | -0.1% | -0.1% |  |
| Class D | -0.1% | -0.1% | -0.1% |  |
| Class E | 0.0% | 0.0% | 0.0% |  |
| Class F | -0.1% | -0.1% | -0.1% |  |
| Overall | -0.1% | -0.1% | -0.1% |  |
|  | -0.1% | -0.1% | -0.1% |  |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Random Access LC |  |  |  |  |
|  | Y | U |  | V |  |
| Class A | -0.2% | 0.0% |  | 0.1% |  |
| Class B | -0.3% | -0.2% |  | -0.2% |  |
| Class C | -0.7% | -0.7% |  | -0.6% |  |
| Class D | -0.6% | -0.7% |  | -0.6% |  |
| Class E |  |  |  |  |  |
| Class F | -0.3% | -0.4% |  | -0.4% |  |
| Overall | -0.4% | -0.4% |  | -0.3% |  |
|  | -0.4% | -0.4% |  | -0.3% |  |
| Enc Time[%] | 99% |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |
|  | Low delay B LC |  |  |  |
|  | Y | U | V |  |
| Class A |  |  |  |  |
| Class B | -0.3% | -0.2% | -0.2% |  |
| Class C | -0.7% | -0.6% | -0.9% |  |
| Class D | -0.8% | -1.1% | -1.0% |  |
| Class E | -1.2% | -0.7% | -1.3% |  |
| Class F | -0.3% | -0.6% | -0.5% |  |
| Overall | -0.6% | -0.6% | -0.7% |  |
|  | -0.6% | -0.6% | -0.8% |  |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Low delay P LC |  |  |  |
|  | Y | U | V |  |
| Class A |  |  |  |  |
| Class B | -0.3% | -0.1% | -0.3% |  |
| Class C | -0.7% | -0.7% | -0.7% |  |
| Class D | -0.8% | -1.0% | -1.3% |  |
| Class E | -1.0% | -0.9% | -0.8% |  |
| Class F | -0.4% | -0.5% | -0.4% |  |
| Overall | -0.6% | -0.6% | -0.7% |  |
|  | -0.6% | -0.6% | -0.7% |  |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

Additional tests with shorten CODEWORD_LEN equals to 15 and 7 was shown in table-7.

Table-7 Coding gain comparison of different CODEWORD_LEN.

- References

JCTVC-F418 Joint Coding of CU splitting flag and inter modes based on HM 3.1 [W. Zhang (ZTE)].

F. Bossen, “Common test conditions and software reference configurations”, Document no JVCVC-F900, Torino, July 2011.

Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan, Thomas Wiegand, “WD4: Working Draft 4 of High-Efficiency Video Coding”, Document no JVCVC-F803, Torino, July 2011.

Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).