| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11-20 July 2012 | Document: JCTVC-J0063_r2 |
| --- | --- |

| Title: | Syntax for NAL Packet Priority |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Eun-Seok Ryu Yan Ye Yuwen He Yong He 9710 Scranton Rd, Suite 250 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4808 eun.ryu@interdigital.com yan.ye@interdigital.co,m yuwen.he@interdigital.com yong.he@interdigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

Pictures in the same temporal level in hierarchical-B structure can have different influence on error propagation and on decoded video quality. Currently in HEVC working draft 7, NAL unit header does not indicate packet priority within the same temporal layer. This contribution proposes two syntax options to indicate such priority of a NAL unit.

# Problem Statement

In video compression and transmission, packet prioritization is of utmost importance for the role it plays in UEP (Unequal Error Protection), packet dropping for bandwidth adaptation, as well as QP (Quantization Parameter) control for enhanced video quality, to name a few. As such, packet priority is essential for optimal QoS handling in applications such as video streaming. However, the current HEVC draft specification [1] does not provide sufficient information for prioritization. For example, priority ID is not specified. And although temporal ID (temp_id) in the NAL header is provided, video packets with the same temporal ID could have different priorities.

## Uniform Prioritization

Fig. 1 denotes the current uniform prioritization in hierarchical-B structure [2] with four dyadic stages in temporal domain. Although pictures in lower temporal levels have higher priority, in the current HEVC working draft, however, there are no additional methods to assign different priorities to pictures at the same temporal level.

Fig. 1. The current uniform prioritization in hierarchical-B structure with four temporal levels.

Fig. 1 shows the random access setting in the common test condition [4]. In Fig. 1, POC 2 and POC 6 have the same priority. However, uniform prioritization at the same temporal level presents a problem because the importance of two POCs could be different according to both the RPS (Reference Picture Set) and the size of the reference picture list. The following sections explain why these two pictures have different importance.

## The Features of Picture Referencing

To compare the importance of two POCs located in the same temporal level, we define Position A as the pictures with POC equal to 2 + N × GOP (Group of Picture), and Position B as the pictures with POC equal to 6 + N × GOP, where GOP is 8 and N represents the number of GOP(s). Then, POC 2, 10, 18, and 26 belong to Position A, and POC 6, 14, 22, and 30 belong to Position B.

In the RA setting, Position A and Position B appear in the reference picture lists (L0 and L1) with different frequencies during each intra period. Table 1 shows the reference picture lists for each POC and the total number of times Position A and Position B were referenced, using intra period equal to 32 as an example. In the table, POCs in Position A (blue) were referenced 12 times and POCs in Position B (red) 16 times during the intra period (= 32 POCs). Thus, compared to Position A, Position B is referenced more, indicating the pictures in Position B are more likely to cause severe error propagation if they are dropped during transmission.

Table 1. The amount of appearance in reference picture list (RA setting, GOP 8, IntraPeriod 32).

| Picture | QP Offset | Reference Buffer size (L0 and L1) | Temporal ID | Reference Picture Set (RPS) |  |  |  | Reference Picture Lists |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | L0 |  |  |  | L1 |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 1 | 4 | 0 | -8 | -10 | -12 | -16 | 0 |  |  |  | 0 |  |  |  |
| 4 | 2 | 2 | 0 | -4 | -6 | 4 |  | 0 | 8 |  |  | 8 | 0 |  |  |
| 2 | 3 | 2 | 0 | -2 | -4 | 2 | 6 | 0 | 4 |  |  | 4 | 8 |  |  |
| 1 | 4 | 2 | 0 | -1 | 1 | 3 | 7 | 0 | 2 |  |  | 2 | 4 |  |  |
| 3 | 4 | 2 | 0 | -1 | -3 | 1 | 5 | 2 | 0 |  |  | 4 | 8 |  |  |
| 6 | 3 | 2 | 0 | -2 | -4 | -6 | 2 | 4 | 2 |  |  | 8 | 4 |  |  |
| 5 | 4 | 2 | 0 | -1 | -5 | 1 | 3 | 4 | 0 |  |  | 6 | 8 |  |  |
| 7 | 4 | 2 | 0 | -1 | -3 | -7 | 1 | 6 | 4 |  |  | 8 | 6 |  |  |
| 16 | 1 | 4 | 0 | -8 | -10 | -12 | -16 | 8 | 6 | 4 | 0 | 8 | 6 | 4 | 0 |
| 12 | 2 | 2 | 0 | -4 | -6 | 4 |  | 8 | 6 |  |  | 16 | 8 |  |  |
| 10 | 3 | 2 | 0 | -2 | -4 | 2 | 6 | 8 | 6 |  |  | 12 | 16 |  |  |
| 9 | 4 | 2 | 0 | -1 | 1 | 3 | 7 | 8 | 10 |  |  | 10 | 12 |  |  |
| 11 | 4 | 2 | 0 | -1 | -3 | 1 | 5 | 10 | 8 |  |  | 12 | 16 |  |  |
| 14 | 3 | 2 | 0 | -2 | -4 | -6 | 2 | 12 | 10 |  |  | 16 | 12 |  |  |
| 13 | 4 | 2 | 0 | -1 | -5 | 1 | 3 | 12 | 8 |  |  | 14 | 16 |  |  |
| 15 | 4 | 2 | 0 | -1 | -3 | -7 | 1 | 14 | 12 |  |  | 16 | 14 |  |  |
| 24 | 1 | 4 | 0 | -8 | -10 | -12 | -16 | 16 | 14 | 12 | 8 | 16 | 14 | 12 | 8 |
| 20 | 2 | 2 | 0 | -4 | -6 | 4 |  | 16 | 14 |  |  | 24 | 16 |  |  |
| 18 | 3 | 2 | 0 | -2 | -4 | 2 | 6 | 16 | 14 |  |  | 20 | 24 |  |  |
| 17 | 4 | 2 | 0 | -1 | 1 | 3 | 7 | 16 | 18 |  |  | 18 | 20 |  |  |
| 19 | 4 | 2 | 0 | -1 | -3 | 1 | 5 | 18 | 16 |  |  | 20 | 24 |  |  |
| 22 | 3 | 2 | 0 | -2 | -4 | -6 | 2 | 20 | 18 |  |  | 24 | 20 |  |  |
| 21 | 4 | 2 | 0 | -1 | -5 | 1 | 3 | 20 | 16 |  |  | 22 | 24 |  |  |
| 23 | 4 | 2 | 0 | -1 | -3 | -7 | 1 | 22 | 20 |  |  | 24 | 22 |  |  |
| 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | 2 | 2 | 0 | -4 | -6 | 4 |  | 24 | 22 |  |  | 32 | 24 |  |  |
| 26 | 3 | 2 | 0 | -2 | -4 | 2 | 6 | 24 | 22 |  |  | 28 | 32 |  |  |
| 25 | 4 | 2 | 0 | -1 | 1 | 3 | 7 | 24 | 26 |  |  | 26 | 28 |  |  |
| 27 | 4 | 2 | 0 | -1 | -3 | 1 | 5 | 26 | 24 |  |  | 28 | 32 |  |  |
| 30 | 3 | 2 | 0 | -2 | -4 | -6 | 2 | 28 | 26 |  |  | 32 | 28 |  |  |
| 29 | 4 | 2 | 0 | -1 | -5 | 1 | 3 | 28 | 24 |  |  | 30 | 32 |  |  |
| 31 | 4 | 2 | 0 | -1 | -3 | -7 | 1 | 30 | 28 |  |  | 32 | 30 |  |  |
| The number of appearances in reference picture list (L0 and L1) *counted once if the ref. POC. number is in both L0 and L1 |  |  |  |  |  |  |  | Position A |  |  |  | Position B |  |  |  |
|  |  |  |  |  |  |  |  | 12 |  |  |  | 16 |  |  |  |

## The effect of Error Propagation

In order to quantify video quality degradation, we created packet dropping tests with encoded bitstreams; i.e., we dropped a picture per every IntraPeriod in Position A for 1st test and a picture per every IntraPeriod in Position B for 2nd test. To measure the difference of objective video quality in two tests, the HM video decoder decodes the damaged bitstream using frame copy.

The testing was conducted on Windows 64-bit operating system, and used HM 6.1 decoder [3] and three test sequences (Traffic, PeopleOnStreet, and ParkScene) from HEVC common test conditions [4]. The PSNR degradation caused by dropping pictures in Position A was less than the PSNR degradation caused by dropping pictures in Position B. In the following rate-distortion curves (Fig. 2), we see the same results, meaning the picture in Position B (red-colored curve) is more important than the picture in Position A (blue-colored curve).

Table 2. Comparison of Position A drop with Position B drop (RA setting).

|  |  | Drop Position B |  |  |  |  | Drop Position A |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence | QP | kbps | Y psnr | U psnr | V psnr |  | kbps | Y psnr | U psnr | V psnr |
| Traffic | 22 | 12752.16 | 36.19 | 40.66 | 42.78 |  | 12751.01 | 38.18 | 40.94 | 43.39 |
|  | 27 | 5231.44 | 34.96 | 39.27 | 41.42 |  | 5230.93 | 36.72 | 39.36 | 41.69 |
|  | 32 | 2531.52 | 33.42 | 38.01 | 40.18 |  | 2531.58 | 34.91 | 38.06 | 40.30 |
|  | 37 | 1320.95 | 31.86 | 37.16 | 39.34 |  | 1321.07 | 32.77 | 37.19 | 39.44 |
| PeopleOnStreet | 22 | 31596.15 | 33.25 | 43.64 | 43.72 |  | 31614.23 | 36.04 | 44.04 | 44.01 |
|  | 27 | 15247.32 | 31.55 | 42.09 | 42.59 |  | 15257.37 | 34.15 | 42.43 | 42.88 |
|  | 32 | 8049.88 | 29.97 | 40.79 | 41.54 |  | 8054.42 | 32.04 | 40.98 | 41.68 |
|  | 37 | 4514.95 | 28.27 | 39.71 | 40.60 |  | 4516.58 | 29.88 | 39.88 | 40.74 |
| ParkScene | 22 | 7440.76 | 37.13 | 41.46 | 43.01 |  | 7522.99 | 38.10 | 41.75 | 43.25 |
|  | 27 | 3273.72 | 35.52 | 39.97 | 41.19 |  | 3304.31 | 36.15 | 40.05 | 41.21 |
|  | 32 | 1526.84 | 33.33 | 38.49 | 39.73 |  | 1538.67 | 33.88 | 38.59 | 39.82 |
|  | 37 | 717.57 | 31.36 | 37.44 | 38.95 |  | 722.82 | 31.64 | 37.50 | 39.00 |

(a)

(b)                                               (c)

Fig. 2. The RD curves by Position A drop and Position B drop ((a),(b),(c): Drop 1 picture per IntraPeriod)

# Proposed Syntax for NAL Packet Priority

This section proposes three syntax options to indicate the priority of a NAL unit.

- Option 1: This method changes nal_ref_flag to nal_priority_flag, and uses it to indicate the packet priority in the same temporal level. This method adds some priority indication using 1-bit flag (nal_priority_flag) without major modification of current NAL header syntax. For example, in Fig. 1, if picture with POC 6 is more important than picture with POC 2 in the same temporal level, this method could set nal_priority_flag to  1 (higher importance) to POC 6 and set nal_priority_flag to 0 (lower importance) to POC 2. The location of nal_priority_flag could be moved to another location in NAL header. As an example, it could be located next to temporal_id.

Table 3. Option 1: (7.3.1) NAL unit syntax.

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_priority_flag // indicates NAL priority in same temporal level | u(1) |
| nal_unit_type | u(6) |
| temporal_id | u(3) |
| reserved_one_5bits | u(5) |
| …… |  |
| } |  |

- Option 2: Provide the priority information in AU (Access Unit) delimiter by modifying the current syntax (Table 4).

Table 4. Option 2: (7.3.2.8) Access unit delimiter RBSP syntax

| access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| pic_type | u(3) |
| priority_id | u(4) |
| rbsp_trailing_bits( ) |  |
| } |  |

# References

B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, High efficiency video coding (HEVC) text specification draft 7, Document no. JCTVC-I1103, May 2012.

H. Schwarz, D. Marpe, T. Wiegand, Analysis of hierarchical B pictures and MCTF, IEEE International Conference on Multimedia and Expo, pp. 1929-1932, 2006.

HEVC Reference Software ver. 6.1 (HM 6.1), url: http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-6.1?rev=2385

F. Bossen, Common test conditions and software reference configurations, Document no. JCTVC-H1100, Feb. 2012.

# Patent rights declaration(s)

InterDigital Communications may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: DIS text

Option 1:

Section 7.4.1: nal_ref_flag equal to 1 specifies that the content of the NAL unit contains a sequence parameter set, a picture parameter set, an adaptation parameter set or a slice of a reference picture. nal_ref_flag equal to 0 for a NAL unit containing a slice indicates that the slice is part of a non-reference picture. nal_ref_flag shall be equal to 1 for video parameter set, sequence parameter set, picture parameter set or adaptation parameter set NAL units. When nal_ref_flag is equal to 0 for one VCL NAL unit of a particular picture, it shall be equal to 0 for all VCL NAL units of the particular picture. nal_ref_flag shall be equal to 1 for NAL units with nal_unit_type equal to 4, 5, 6, 7, or 8. nal_ref_flag shall be equal to 0 for all NAL units having nal_unit_type equal to 29, 30, or 31.

nal_priority_flag equal to 1 specifies that the NAL unit has a higher priority than the other NAL units in the same temporal level. nal_priority_flag equal to 0 specifies that the NAL unit has a normal priority in the same temporal level. NOTE 2 - When the pictures have different temporal levels (temporal_id), the pictures in lower temporal level always have higher priorities than the pictures in higher temporal level. However, the pictures in same temporal level also need to be distinguished with each other. The nal_priority_flag indicates whether the picture is more important than the other pictures in the same temporal level.

Option 2:

Section 7.4.2.8: priority_id specifies a priority identifier for the following NAL unit(s) until the next access unit delimiter is present. The priority identifier indicates the priority of NAL unit(s) in the same temporal level.