| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0442 |
| --- | --- |

| Title: | Inter-Prediction Residual DPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or | Matteo Naccari Andrea Gabriellini Marta Mrak (BBC) Saverio Blasi Ebroul Izquierdo (QMUL) |  | matteo.naccari@bbc.co.uk |
| Source: | BBC Research & Development and Queen Mary, University of London (QMUL) |  |  |

_____________________________

# Abstract

In this contribution, a DPCM scheme for inter-prediction of residuals is proposed and implemented in HM 10.0-RExt-2.0 for HEVC lossless coding. The scheme follows the residual DPCM scheme proposed for intra-prediction in JCTVC-M0079. Same set of DPCM modes as in intra case (no DPCM, vertical and horizontal DPCM) is used, where each mode is available for coding of inter residual prediction block. The choice is signalled in the bit-stream. To enable lower complexity of DPCM processing at the decoder side for larger blocks, the residual samples are divided into chunks of a fixed length of eight rows or columns reducing the maximal number of operations per sample from 63 to 7. It is reported that the overall bit-rate saving of applying intra and inter DPCM in random access and low-delay configurations is -9.7 % and -10.6 %, respectively, for screen content sequences.

# Introduction

Residual DPCM has been proposed for intra-prediction in HEVC lossless coding. As a part of the Core experiment RCE2 the method was evaluated in terms of both coding efficiency and complexity [2]. In this contribution a DPCM scheme for inter-prediction residuals is proposed and evaluated. DPCM can be used to reduce the bitrates of lossless coding due to possible redundancy that is left in the residual block after motion compensation. The proposed inter-DPCM modes are selected and applied to the residual samples independently at a PU level, to allow joint optimization of motion estimation and DPCM mode selection.

As proposed in [1], residual DPCM at the decoder assumes dependency on i – 1 previous samples in the current row or column when inverse residual DPCM is performed on a sample at position i. In order to limit the prediction dependencies at the decoder side, in this proposal the residual samples are subdivided in chunks of a predetermined length R in the direction of prediction, so that no residual requires more than R - 1 additions to be reconstructed. In our test R is limited to 8 which reduces the maximum number of operations per sample from 63 (= 64 – 1) to 7 (= 8 – 1), where 64 is the maximum number of pixels in a row or column of a PU. Various test results are provided in this contribution to show the effectiveness of the proposed inter-DPCM scheme.

# Algorithm Description

## Inter-DPCM at PU level with chunking

The scheme proposed in this contribution is applied at a PU level, when lossless coding is enabled and the CU is inter-predicted. The proposed scheme is applied on each residual PU block independently. A choice of two inter-DPCM modes (vertical and horizontal) is available.

The vertical inter-DPCM mode is defined as follows. Let r(i,j) be an element of an M x N residual PU block of luma or chroma samples. The samples in the first row are left unchanged, whereas the other residual samples are predicted from the samples immediately above in the same column. The residual difference between each sample and its predictor is signalled in the bitstream instead of the original residual sample. After the prediction of (R-1) consecutive rows, the DPCM prediction chain is halted: samples in the Rth row are not predicted from the previous samples but instead they are directly signalled in the bitstream. The Rth row is the start of the second chunk of residue samples. This subdivision of the residue block is introduced to limit the number of samples required to fully reconstruct a sample at the decoder side. Formally the modified residual samples resulting from the application of the vertical inter-DPCM mode are defined as:

r'veri,j= ri,j                                     if j=0, R,2R,…ri,j-ri-1,j            otherwise   .

R

Samples in these rows are left unchanged.

Prediction direction

Figure 1 - Chunking for Vertical Inter-DPCM.

The horizontal inter-DPCM mode is defined in a similar way. Each residual sample is predicted from the sample immediately to the left in the same row. The subdivision of the residual samples in chunks is introduced at a fixed period of R columns. The modified samples resulting from the application of the horizontal inter-DPCM mode are defined as:

r'hori,j= ri,j                                       if   i=0, R, 2R…ri,j-ri,j-1,             otherwise           .

Under the proposed scheme three distinct coding modes are available: no DPCM (i.e., the conventional system where the inter-prediction residuals are not predicted but they are directly entropy-encoded); vertical DPCM; and horizontal DPCM. The choice of coding mode is signalled with other PU-specific information, after the merge flag and before motion information. If the CU is skipped the coding mode is not signalled. When either the vertical or horizontal inter-DPCM mode is selected, the modified residual samples r'i,j are signalled to the decoder instead of the original samples.

At the decoder side and during the reconstruction process, the inverse scheme is applied as follows. The residual samples in a CU are partitioned in PU blocks. Inverse inter-DPCM is then performed on each residual PU block according to the decoded coding mode. When using vertical inter-DPCM, the inverse residuals are obtained as:

ri,j= k=iRRir'k,j,

where the ∙ operator returns the largest integer value that is smaller than or equal to its argument.

Similarly when using horizontal inter-DPCM the inverse residuals are obtained as:

ri,j= k=jRRjr'i,k .

R

Samples in these rows are reconstructed in the same way:

r(i, j) = r'(i , j) + r'(i - 1, j) + r'(i - 2, j) + r'(i - 3, j)

Prediction direction

Figure 2 - Effect of Chunking on Data Dependency on Decoder-Side

## Implementation

The scheme proposed in this contribution is applied at PU level, when lossless coding is enabled and the CU is inter-predicted. At the encoder side the method is applied after the inter-prediction algorithm returns the optimal prediction for each PU and the residual samples have been computed for the entire CU. The samples are then partitioned into residual PU blocks, following the same partitioning performed on the original and prediction samples when testing the current inter-prediction mode. The proposed scheme is applied on each residual PU block independently.

The three coding modes proposed in the previous section (no DPCM; vertical DPCM; horizontal DPCM) are tested in each residual PU block. . When either the vertical or horizontal DPCM mode is tested, the modified residual PU block resulting from the application of such mode is computed. A distortion measure is also computed for each coding mode as the sum of absolute values of the residual samples, or:

Dmode=i=0M-1j=0N-1r'modei,j

Eventually the best mode is selected as the solution at minimum distortion for each residual PU block.

Notice that the effect of subdividing the differential residue block in chunks is to make each residual sample only depend on up to (R-1) previous samples. Also notice that the proposed inter-DPCM modes and their corresponding inverse modes can be fully parallelized at both encoder and decoder.

# Tests Results

Various test results are provided here to evaluate the impact of the proposed inter-DPCM scheme with respect to conventional HEVC lossless coding. The scheme was implemented on top of HEVC range extension software HM10.0_RExt2.0.

First set or results, Table 1, shows bit-rate savings when only inter residual DPCM is applied on top of HM10.0_RExt2.0, i.e. without intra residual DPCM. Second set or results, Table 2, shows bit-rate savings when both intra and nter residual DPCM are applied on top of HM10.0_RExt2.0. In both cases all DPCM methods use chunking, i.e. DPCM is not performed on every 8th sample of each row (for horizontal prediction) and of each column (for vertical prediction).

Table 1 – Results for inter DPCM with chunking

| Config. Class | Random Access | Low Delay B |
| --- | --- | --- |
|  | Bit-rate saving | Bit-rate saving |
| Class F | -4.3% | -6.4% |
| Class B | -0.7% | -0.8% |
| SC (GBR) | -3.5% | -6.9% |
| RangeExt | - | - |
| Overall (w/o SC) | - | - |
| Overall (w/ SC) | - | - |
| Enc Time[%] | - | - |
| Dec Time[%] | - | - |

Table 2 – Results for intra and inter DPCM with chunking

| Config. Class | All Intra | Random Access | Low Delay B |
| --- | --- | --- | --- |
|  | Bit-rate saving | Bit-rate saving | Bit-rate saving |
| Class F | -9.1% | -8.5% | -9.0% |
| Class B | -4.5% | -1.3% | -1.1% |
| SC (GBR) | -11.0% | -9.7% | -10.6% |
| RangeExt | -2.8% | - | - |
| Overall (w/o SC) | -5.7% | - | - |
| Overall (w/ SC) | -8.4% | - | - |
| Enc Time[%] | 100% | - | - |
| Dec Time[%] | 99% | - | - |

# Proposed text changes

The proposed text changes are based on the document JCTVC-L1005-v4-JCTVC-L1003_v34.doc.

The proposed additions and modifications are highlighted in yellow. It is proposed to modify “7.3.8.6 Prediction unit syntax” and “7.4.9.6 Prediction unit semantics”. It is also proposed to modify “8.5.1 General decoding process for coding units coded in inter prediction mode” and to introduce “8.5.5. Inter residual transquant-bypass decoding process”.

It is also proposed to add “8.6.6 Intra residual transform-bypass decoding process” to extend the proposed scheme to intra-coded blocks.

Finally, it is proposed to modify table 9.4, add table 9.17, modify table 9.32 and add “9.3.3.7 Binarization process for inter_dpcm_idc”.

| prediction_unit( x0, y0, nPbW, nPbH ) { | Descriptor |
| --- | --- |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_transquant_bypass_flag = = 0 ) { |  |
| for( cIdx = 0; cIdx < ( ChromaArrayType > 0 ? 3 : 1 ); cIdx++ ) |  |
| inter_dpcm_idc[ x0 ][ y0 ][ cIdx ] | ae(v) |
| } |  |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( x0, y0, 1 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

7.4.9.6	Prediction unit semantics

...

merge_idx[ x0 ][ y0 ] specifies the merging candidate index of the merging candidate list where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

When merge_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

inter_dpcm_idc[ x0 ][ y0 ] [ cIdx ] specifies whether to invoke the inter-residual transform-bypass decoding process as specified in 8.5.5 for the prediction block whose top-left luma sample is at location x0, y0 relative to the top-left luma sample of the picture and for component cIdx. Table 7-11 specifies the list of inter_dpcm_idc possible values and associated interpretation. The process for the Cb and Cr components is applied in the same way as for the luma component.

Table 7-11 – inter_dpcm_idc interpretation

| inter_dpcm_idc | Interpretation |
| --- | --- |
| 0 | no inter-residual transform-bypass decoding |
| 1 | inter-residual transform-bypass decoding with horizontal mode |
| 2 | inter-residual transform-bypass decoding with vertical mode |

inter_pred_idc[ x0 ][ y0 ] specifies whether list0, list1, or bi-prediction is used for the current prediction unit according to Table 7-11. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

...

### 8.5.1 General decoding process for coding units coded in inter prediction mode

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current coding block.

Output of this process is a modified reconstructed picture before deblocking filtering.

The derivation process for quantization parameters as specified in subclause 8.6.1 is invoked with the luma location ( xCb, yCb ) as input.

The variable nCbSL is set equal to 1  <<  log2CbSize. When ChromaArrayType is not equal to 0, the variable nCbSwC is set equal to 1  <<  ( log2CbSize ) / SubWidthC and the variable nCbShC is set equal to ( 1  <<  log2CbSize ) / SubHeightC.

The decoding process for coding units coded in inter prediction mode consists of following ordered steps:

- The inter prediction process as specified in subclause 8.5.2 is invoked with the luma location ( xCb, yCb ) and the luma coding block size log2CbSize as inputs, and the output is the array predSamplesL and when ChromaArrayType is not equal to 0, the arrays predSamplesCb, and predSamplesCr.

- The decoding process for the residual signal of coding units coded in inter prediction mode specified in subclause 8.5.4 is invoked with the luma location ( xCb, yCb ) and the luma coding block size log2CbSize as inputs, and the output is the array resSamplesL and when ChromaArrayType is not equal to 0, the arrays resSamplesCb, and resSamplesCr.

- If cu_transquant_bypass_flag is equal to 1 the inter residual transform-bypass decoding process specified in subclause 8.5.5 is invoked with the luma location ( xCb, yCb ), the luma coding block size log2CbSize, the array resSamplesL and inter_dpcm_idc[ xCb ][ yCb ][ 0 ] and when ChromaArrayType is not equal to 0, the arrays resSamplesCb, inter_dpcm_flag[ xCb ][ yCb ][ 1 ], resSamplesCr and inter_dpcm_idc[ xCb ][ yCb ][ 2 ] as inputs, and the output is the array resSamplesL and when ChromaArrayType is not equal to 0, the arrays resSamplesCb, and resSamplesCr.

- The reconstructed samples of the current coding unit are derived as follows:

–	The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause 8.6.5 is invoked with the luma coding block location ( xCb, yCb ), the variable nCurrSw set equal to nCbSwL, the variable nCurrSh set equal to nCbShL, the variable cIdx set equal to 0, the (nCbSL)x(nCbSL) array predSamples set equal to predSamplesL, and the (nCbSL)x(nCbSL) array resSamples set equal to resSamplesL as inputs.

–	When ChromaArrayType is not equal to 0, the picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause 8.6.5 is invoked with the chroma coding block location ( xCb / SubWidthC, yCb / SubHeightC ), the variable nCurrSw set equal to nCbSwC, the variable nCurrSh set equal to nCbShC, the variable cIdx set equal to 1, the (nCbSwC)x(nCbShC) array predSamples set equal to predSamplesCb, and the (nCbSwC)x(nCbShC) array resSamples set equal to resSamplesCb as inputs.

–	When ChromaArrayType is not equal to 0, the picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause 8.6.5 is invoked with the chroma coding block location ( xCb / SubWidthC, yCb / SubHeightC ), the variable nCurrSw set equal to nCbSwC, the variable nCurrSh set equal to nCbShC, the variable cIdx set equal to 2, the (nCbSwC)x(nCbShC) array predSamples set equal to predSamplesCr, and the (nCbSwC)x(nCbShC) array resSamples set equal to resSamplesCr as inputs.

8.5.5 Inter residual transquant-bypass decoding process

8.5.5.1 General

This process is invoked when CuPredMode[ xCb ][ yCb ] is not equal to MODE_INTRA and cu_transquant_bypass_flag is equal to 1.

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block,

-	the luma coding block size log2CbSize,

-	the array resSamplesL,

-	when ChromaArrayType is not equal to 0, the arrays resSamplesCb,

-	when ChromaArrayType is not equal to 0, the array resSamplesCr.

Outputs of this process are:

–	an (nCbSL)x(nCbSL) array resSamplesL of luma residual samples, where nCbSL is derived as specified below,

–	when ChromaArrayType is not equal to 0, an (nCbSwC)x(nCbShC) array resSamplesCb of chroma residual samples for the component Cb, where nCbSwC and nCbShC are derived as specified below,

–	when ChromaArrayType is not equal to 0, an (nCbSwC)x(nCbShC) array resSamplesCr of chroma residual samples for the component Cr, where nCbSwC and nCbShC are derived as specified below.

The variable nCbSL is set equal to 1  <<  log2CbSize. When ChromaArrayType is not equal to 0, the variable nCbSwC is set equal to nCbSL / SubWidthC and the variable nCbShC is set equal to nCbSL / SubHeightC.

The variable nCbS1L is set equal to nCbSL  >>  1.

Depending on the value of PartMode, the following applies:

–	If PartMode is equal to PART_2Nx2N, the decoding process for prediction units in inter prediction mode as specified in subclause 8.5.5.2 is invoked with the width of the luma prediction block nPbW set equal to nCbSL, the height of the luma prediction block nPbH set equal to nCbSL, inter_dpcm_idc[ xCb ][ yCb ][ 0 ] and an (nCbSL)x(nCbSL) array resSamplesL as inputs, and a modified version of resSamplesL as outputs and when ChromaArrayType is not equal to 0, two (nCbSwC)x(nCbShC) arrays resSamplesCb and resSamplesCr.

–	Otherwise, if PartMode is equal to PART_2NxN, the following ordered steps apply:

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.5.5.2 is invoked with the width of the luma prediction block nPbW set equal to nCbSL, the height of the luma prediction block nPbH set equal to nCbSL  >>  1, inter_dpcm_idc[ xCb ][ yCb ][ 0 ] and an (nCbSL)x(nCbSL  >>  1) array resSamplesL as inputs, and a modified version of resSamplesL as outputs and when ChromaArrayType is not equal to 0, two (nCbSwC)x(nCbShC  >>  1) arrays resSamplesCb and resSamplesCr.

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.5.5.2 is invoked with the width of the luma prediction block nPbW set equal to nCbSL, the height of the luma prediction block nPbH set equal to nCbSL  >>  1, inter_dpcm_idc[ xCb ][ yCb+ nCbSL  >>  1 ][ 0 ] and an (nCbSL)x(nCbSL  >>  1) array resSamplesL as inputs, and a modified version of resSamplesL as outputs and when ChromaArrayType is not equal to 0, two (nCbSwC)x(nCbShC  >>  1) arrays resSamplesCb and resSamplesCr.

–	Otherwise, if PartMode is equal to PART_Nx2N, the following ordered steps apply:

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.5.5.2 is invoked with the width of the luma prediction block nPbW set equal to nCbSL  >>  1, the height of the luma prediction block nPbH set equal to nCbSL, inter_dpcm_idc[ xCb ][ yCb ][ 0 ] and an (nCbSL  >>  1)x(nCbSL) array resSamplesL as inputs, and a modified version of resSamplesL as outputs and when ChromaArrayType is not equal to 0, two (nCbSwC  >>  1)x(nCbShC) arrays resSamplesCb and resSamplesCr.

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.5.5.2 is invoked with the width of the luma prediction block nPbW set equal to nCbSL  >>  1, the height of the luma prediction block nPbH set equal to nCbSL, inter_dpcm_idc[ xCb+ nCbSL  >>  1 ][ yCb ][ 0 ] and an (nCbSL  >>  1)x(nCbSL) array resSamplesL as inputs, and a modified version of resSamplesL as outputs and when ChromaArrayType is not equal to 0, two (nCbSwC  >>  1)x(nCbShC) arrays resSamplesCb and resSamplesCr.

–	Otherwise, if PartMode is equal to PART_2NxnU, the following ordered steps apply:

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.5.5.2 is invoked with the width of the luma prediction block nPbW set equal to nCbSL, the height of the luma prediction block nPbH set equal to nCbSL  >>  2, inter_dpcm_idc[ xCb ][ yCb ][ 0 ] and an (nCbSL)x(nCbSL  >>  2) array resSamplesL as inputs, and a modified version of resSamplesL as outputs and when ChromaArrayType is not equal to 0, two (nCbSwC)x(nCbShC  >>  2) arrays resSamplesCb and resSamplesCr.

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.5.5.2 is invoked with the width of the luma prediction block nPbW set equal to nCbSL, the height of the luma prediction block nPbH set equal to nCbSL  >>  1 + nCbSL  >>  2, inter_dpcm_idc[ xCb ][ yCb + nCbSL  >>  2 ][ 0 ] and an (nCbSL)x(nCbSL  >>  1 + nCbSL  >>  2) array resSamplesL as inputs, and a modified version of resSamplesL as outputs and when ChromaArrayType is not equal to 0, two (nCbSwC)x(nCbShC  >>  1 + nCbShC  >>  2) arrays resSamplesCb and resSamplesCr.

–	Otherwise, if PartMode is equal to PART_2NxnD, the following ordered steps apply:

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.5.5.2 is invoked with the width of the luma prediction block nPbW set equal to nCbSL, the height of the luma prediction block nPbH set equal to nCbSL  >>  1 + nCbSL  >>  2, inter_dpcm_idc[ xCb ][ yCb ][ 0 ] and an (nCbSL)x(nCbSL  >>  1 + nCbSL  >>  2) array resSamplesL as inputs, and a modified version of resSamplesL as outputs and when ChromaArrayType is not equal to 0, two (nCbSwC)x(nCbShC  >>  1 + nCbShC  >>  2) arrays resSamplesCb and resSamplesCr.

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.5.5.2 is invoked with the width of the luma prediction block nPbW set equal to nCbSL, the height of the luma prediction block nPbH set equal to nCbSL  >>  2, inter_dpcm_idc[ xCb ][ yCb + nCbSL  >>  1 + nCbSL  >>  2 ][ 0 ] and an (nCbSL)x(nCbSL  >>  2) array resSamplesL as inputs, and a modified version of resSamplesL as outputs and when ChromaArrayType is not equal to 0, two (nCbSwC)x(nCbShC  >>  2) arrays resSamplesCb and resSamplesCr.

–	Otherwise, if PartMode is equal to PART_nLx2N, the following ordered steps apply:

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.5.5.2 is invoked with the width of the luma prediction block nPbW set equal to nCbSL  >>  2, the height of the luma prediction block nPbH set equal to nCbSL, inter_dpcm_idc[ xCb ][ yCb ][ 0 ] and an (nCbSL  >>  2)x(nCbSL) array resSamplesL as inputs, and a modified version of resSamplesL as outputs and when ChromaArrayType is not equal to 0, two (nCbSwC  >>  2)x(nCbShC) arrays resSamplesCb and resSamplesCr.

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.5.5.2 is invoked with the width of the luma prediction block nPbW set equal to nCbSL  >>  1 + nCbSL  >>  2, the height of the luma prediction block nPbH set equal to nCbSL, inter_dpcm_idc[ xCb + nCbSL  >>  2][ yCb ][ 0 ] and an (nCbSL  >>  1 + nCbSL  >>  2)x(nCbSL) array resSamplesL as inputs, and a modified version of resSamplesL as outputs and when ChromaArrayType is not equal to 0, two (nCbSwC  >>  1 + nCbSwC  >>  2)x(nCbShC) arrays resSamplesCb and resSamplesCr.

–	Otherwise, if PartMode is equal to PART_nRx2N, the following ordered steps apply:

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.5.5.2 is invoked with the width of the luma prediction block nPbW set equal to nCbSL  >>  1 + nCbSL  >>  2, the height of the luma prediction block nPbH set equal to nCbSL, inter_dpcm_idc[ xCb ][ yCb ][ 0 ] and an (nCbSL  >>  1 + nCbSL  >>  2)x(nCbSL) array resSamplesL as inputs, and a modified version of resSamplesL as outputs and when ChromaArrayType is not equal to 0, two (nCbSwC  >>  1 + nCbSwC  >>  2)x(nCbShC) arrays resSamplesCb and resSamplesCr.

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.5.5.2 is invoked with the width of the luma prediction block nPbW set equal to nCbSL  >>  2, the height of the luma prediction block nPbH set equal to nCbSL, inter_dpcm_idc[ xCb + nCbSL  >>  1 + nCbSL  >>  2][ yCb ][ 0 ] and an (nCbSL  >>  2)x(nCbSL) array resSamplesL as inputs, and a modified version of resSamplesL as outputs and when ChromaArrayType is not equal to 0, two (nCbSwC  >>  2)x(nCbShC) arrays resSamplesCb and resSamplesCr.

–	Otherwise (PartMode is equal to PART_NxN), the following ordered steps apply:

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.5.5.2 is invoked with the width of the luma prediction block nPbW set equal to nCbSL  >>  1, the height of the luma prediction block nPbH set equal to nCbSL  >>  1, inter_dpcm_idc[ xCb ][ yCb ][ 0 ] and an (nCbSL  >>  1)x(nCbSL  >>  1) array resSamplesL as inputs, and a modified version of resSamplesL as outputs and when ChromaArrayType is not equal to 0, two (nCbSwC  >>  1)x(nCbShC  >>  1) arrays resSamplesCb and resSamplesCr.

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.5.5.2 is invoked with the width of the luma prediction block nPbW set equal to nCbSL  >>  1, the height of the luma prediction block nPbH set equal to nCbSL  >>  1, inter_dpcm_idc[ xCb + nCbSL  >>  1][ yCb ][ 0 ] and an (nCbSL  >>  1)x(nCbSL  >>  1) array resSamplesL as inputs, and a modified version of resSamplesL as outputs and when ChromaArrayType is not equal to 0, two (nCbSwC  >>  1)x(nCbShC  >>  1) arrays resSamplesCb and resSamplesCr.

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.5.5.2 is invoked with the width of the luma prediction block nPbW set equal to nCbSL  >>  1, the height of the luma prediction block nPbH set equal to nCbSL  >>  1, inter_dpcm_idc[ xCb ][ yCb + nCbSL  >>  1][ 0 ] and an (nCbSL  >>  1)x(nCbSL  >>  1) array resSamplesL as inputs, and a modified version of resSamplesL as outputs and when ChromaArrayType is not equal to 0, two (nCbSwC  >>  1)x(nCbShC  >>  1) arrays resSamplesCb and resSamplesCr.

- The decoding process for prediction units in inter prediction mode as specified in subclause 8.5.5.2 is invoked with the width of the luma prediction block nPbW set equal to nCbSL  >>  1, the height of the luma prediction block nPbH set equal to nCbSL  >>  1, inter_dpcm_idc[ xCb + nCbSL  >>  1][ yCb + nCbSL  >>  1][ 0 ] and an (nCbSL  >>  1)x(nCbSL  >>  1) array resSamplesL as inputs, and a modified version of resSamplesL as outputs and when ChromaArrayType is not equal to 0, two (nCbSwC  >>  1)x(nCbShC  >>  1) arrays resSamplesCb and resSamplesCr.

8.5.5.2 Inter residual transform-bypass decoding process

This process is invoked when cu_transquant_bypass_flag is equal to 1 and CuPredMode[ xCb ][ yCb ] is not equal to MODE_INTR. The process for the Cb and Cr components is applied in the same way as for the luma component.

Inputs to this process are:

- a variable nPbW specifying the width of the prediction block,

- a variable nPbH specifying the height of the prediction block,

- a variable inter_dpcm_idc specifying the type of inter residual transform-bypass decoding process for either the luma component,the Cb or Cr component.

- an (nPbW)x(nPbH) array r with elements r[ x ][ y ] which is either an array relating to a residual transform-bypass block of the luma component, an array relating to a residual transform-bypass block of the Cb component or an array relating to a residual transform-bypass block of the Cr component.

Output of this process is a modified version of the (nPbW)x(nPbH) array r with elements r[ x ][ y ] containing the result of the inter residual transform-bypass decoding process.

Let f be a temporary (nPbW)x(nPbH) array with element f[ x ][ y ], which are derived by;

f[ x ][ y ] = r[ x ][ y ]   with x=0.. nPbW-1 and y=0.. nPbH-1

Depending on inter_dpcm_idc, the following applies:

- If inter_dpcm_idc is equal to 0 the array r is not modified.

Otherwise , the values of the residual samples r[x][y] are derived as follows:

    - If inter_dpcm_idc is equal to 1 (horizontal mode), the modified array r is derived by:

with x=0..nPbW-1 and y=0..nPbH-1

    - Otherwise, inter_dpcm_idc is equal to 2 (vertical mode), the modified array r is derived by:

with x=0..nPbW-1 and y=0..nPbH-1

8.6.6 Intra residual transform-bypass decoding process

This process is invoked when cu_transquant_bypass_flag is equal to 1, pred_mode_flag is equal to 1, and the applicable intra prediction mode is equal to either vertical or horizontal mode. The process for the Cb and Cr components is applied in the same way as for the luma component.

Inputs to this process are:

- a variable nT specifying the transform block size

- a variable predModeIntra specifying the intra prediction mode

- an (nT)x(nT) array r with elements r[ x ][ y ] which is either an array relating to a residual transform-bypass block of the luma component or an array relating to a residual transform-bypass block of the Cb and Cr component.

Output of this process is a modified version of the (nT)x(nT) array r with elements r[ x ][ y ] containing the result of the intra residual transform-bypass decoding process.

Let f be a temporary (nT)x(nT) array with element f[ x ][ y ], which are derived by;

f[ x ][ y ] = r[ x ][ y ]   with x=0..nT-1 and y=0..nT-1

Depending on predModeIntra, the values of the residual samples r[x][y], with x=0..nT-1 and y=0..nT-1, are derived as follows:

- If predModeIntra is equal to 26 (Intra_Vertical), residual sample r[x][y], is derived as follows:

- Otherwise (predModeIntra is equal to 10 (Intra_Horizontal)), residual sample r[x][y], is derived as follows:

# References

- S. Lee, I.-K. Kim and C. Kim, “RCE2: Test 1 - Residual DPCM for HEVC lossless coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0079, Apr. 2013.

- W. Gao, M. Zhou, P. Amon, and S. Lee, “HEVC Range Extensions Core Experiment 2 (RCE2): Intra Prediction for Lossless Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1122, Jan. 2013.

- D. Flynn, J. Sole, and T. Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 2 (for PDAM),” Document of Joint Collaborative Team on Video Coding, JCTVC-L1005, Jan. 2013.

# Patent rights declarations

The British Broadcasting Corporation (BBC) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Queen Mary, University of London may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).