| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0148 |
| --- | --- |

| Title: | AhG8: Guided Image Filtering for Screen Content Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information, Draft |  |  |
| Author(s) or Contact(s): | Thijs Vermeir Jan De Cock Glenn Van Wallendael Sebastiaan Van Leuven | Tel: Email: | +32 56 26 2379 thijs.vermeir@barco.com jan.decock@ugent.be glenn.vanwallendael@ugent.be sebastiaan.vanleuven@ugent.be |
| Source: | Barco N.V., President Kennedypark 35, 8500 Kortrijk, Belgium Ghent University – iMinds |  |  |

_____________________________

# Abstract

Downsampling the chroma components, as required in HEVC Main profile (4:2:0), generates visual artifacts for screen content. This contribution provides the results of experiments with a guided image post-processing filter. The luma component is used as a guide to enhance the chroma components. This filtered image restores the edges in the chroma component of screen-content, which have been affected due to the original 4:2:0 sub-sampling. By analyzing a local histogram per pixel, the radius of the guided image filter is reconfigured. The PSNR of the Cb and Cr components for lossless compressed streams is increased by +0.63 dB and +0.99 dB on average, respectively. The presented results are based on on-going experiments, therefore also future work is proposed.

# Introduction

The Human Visual System (HVS) is more sensitive to variations in brightness than color (chroma). Therefore it is common that the chroma components are downsampled in video compression. In HEVC this is also the case, the main supported color space is 4:2:0. This reduced resolution does not create visual artifacts for natural content. However, screen content has different characteristics like more static content and sharp edges. The viewer is also looking at the screen in another way, since typically the viewer is closer to the screen and can look only to a specific area of the screen. This combination can result in visual artifacts when screen content coding is done with HEVC Main profile (4:2:0 color space)[1]. A higher-resolution color space is required to provide visually lossless coding for screen content. Such color spaces could include 4:4:4 YUV or RGB.

A trend in the professional visualization market is that (mobile) consumer electronics devices (like smartphones and tablets) should be able to integrate with the professional visualization system. Consequently, the system should encode the video stream in a format supported by these devices. Typically these devices have hardware accelerated decoding (and/or encoding), but the support is mostly limited to the most popular profiles (HEVC Main profile). This is in conflict with the previous requirement for a 4:4:4 color space, since HEVC Main profile only supports a 4:2:0 color space. Simulcasting the stream, on a small scale, could be a valid solution for this problem but this can make the system much more expensive with the extra equipment that is needed. Also the extra complexity to manage the system should not be underestimated. In larger systems this is not a maintainable solution at all. Manufacturers of chip or SoC devices that support hardware acceleration for codecs also concentrate on the Main profile. High profile codecs are, compared to Main Profile codecs, not widely available or are not cost effective.

This contribution proposes a solution for this problem by allowing upgrading the visual quality of the HEVC Main profile stream on a professional encoder while still supporting the stream on the consumer electronics. To enhance the quality in the professional equipment the output of the decoder is additionally filtered with a guided image filter to improve the visual quality for screen content. In this contribution the filter is only working after the decoder. There are no changes needed to the encoder. This could change in future work (see 5 Future work).

# Guided Image Filtering

The guided image filter [2] uses a linear model to generate the filtered output by using a guidance image. This guidance image can be either the image itself or another image. The filter is an edge-preserving smoothing operator like the bilateral filter but with better behavior near edges. The filter transfers the structure from the guidance image to the filtered output. The filter has a fast linear time algorithm, independent of the kernel size and the intensity range. Currently this is one of the fastest edge-preserving filters.

The filter can be configured with two parameters: the radius and the regularization. The radius is defining the size of the square window around the current pixel. This window is used to generate the smoothed parameter of the filter and for edge detection. The ratio between the covariance (of guide and image) and variance (of guide) defines if the texture of the guide (like edges) should be transferred to the filtered output. With the regularization parameter this ratio can be fine-tuned.

In this contribution, the guided image filter is used with the luma component as the guide. The filtered image is the chroma component. The filter is applied for each component (Cb and Cr). Typically screen content has sharp edges where the edge slope only spans a few pixels. When downsampling edges (with less steep transitions) no extra artifacts are generated. Therefore the regularization parameter is not used dynamically and configured as a fixed property. The radius property is used to re-configure the filter for every pixel to adapt the smoothing behavior of the filter.

# Histogram-based analysis

Screen Content has typically sharp edges like it is the case around text and window borders. The characteristics however can be different for different sequences of screen content. For example dependent on the operating system and version the window borders could be sharper, have more transition colors or be transparent. This needs different configurations of the filter.

The filter is extended with a histogram-based analysis tool to define the radius. A local histogram is created in the learning phase for each pixel in the luma component. This histogram, a square window around the pixel with a fixed radius (11 pixels), is analyzed by taking three parameters into account (i.e., amount of different colors, how many times this colors are used and the difference in luminance between 2 colors). The filter is applied for all radius values. When all pixels are analyzed, the results are collected in a lookup table. This table allows to select the best radius for a set of pixel characteristics, and on average generate the best output of the filter (PSNR based). In the following tests the same content is used in the training phase as in the evaluation phase, creating the best table for this limited set of sequences.

# Results

Figure 1: Additional test sequences used in this experiment.
Left image is Office_Scada_1, right image is Video_Maps_1.

In a first phase the tests concentrate on evaluating this filter method on lossless compressed but 4:2:0 downsampled streams. In a later phase this research will be extended to lossy compressed streams. For the input of the lossless compressed images some screen content sequences from the HEVC test sequences are used (sc_cad_waveform, sc_programming, sc_video_conferencing_doc_sharing and sc_wordEditing)) and two sequences are added to this list (Office_Scada_1 and Video_Maps_1) see Figure 1. The second sequence is mixed content; it also has natural video from security cameras.

Table 1: Results of guided image filtering with fixed radius for all pixels for lossless compression, compared to the original 4:4:4 sequence

|  | Unfiltered |  | Guided Image filtering with fixed radius |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 |  | 3 |  | 5 |  | 7 |  | 9 |  | 11 |  |
| Sequence | Cb | Cr | Cb | Cr | Cb | Cr | Cb | Cr | Cb | Cr | Cb | Cr | Cb | Cr |
| Office_Scada_1 | 36.67 | 38.58 | 36.67 | 38.58 | 37.28 | 40.33 | 35.07 | 40.35 | 33.63 | 40.08 | 32.68 | 39.74 | 32.01 | 39.42 |
| sc_cad_waveform | 24.97 | 23.06 | 24.97 | 23.06 | 26.05 | 24.35 | 26.44 | 24.45 | 26.45 | 24.27 | 26.36 | 24.09 | 26.26 | 23.91 |
| Video_Maps_1 | 37.95 | 42.95 | 37.95 | 42.95 | 38.98 | 43.97 | 38.00 | 42.96 | 36.95 | 41.77 | 36.02 | 40.75 | 35.22 | 39.89 |
| sc_programming | 36.14 | 35.96 | 36.14 | 35.96 | 36.99 | 36.96 | 36.00 | 36.10 | 34.9 | 35.03 | 34.30 | 34.43 | 33.84 | 33.95 |
| sc_video_conferencing_doc_sharing | 32.70 | 31.12 | 32.70 | 31.12 | 32.91 | 31.26 | 32.64 | 31.01 | 32.4 | 30.81 | 32.24 | 30.64 | 32.09 | 30.48 |
| sc_wordEditing | 37.95 | 36.28 | 37.95 | 36.28 | 37.24 | 36.48 | 36.03 | 35.81 | 35.51 | 35.50 | 35.13 | 35.30 | 34.74 | 35.14 |

Table 1 shows the results of applying the guided image filter with the luma as the guide and chroma components as the filtered image. In the first two columns the filter is not applied. In the next columns the guided filter is used with a fixed radius on the whole frame. If a fixed radius of 3 pixels is used the best PSNR can be generated on average. However in the sc_cad_waveform sequence radius 5 is the best option.

Table 2: Results of guided image filtering when radius is defined by histogram analysis, compared to the original 4:4:4 sequence

| Sequence | Original |  | Histogram based guided image filter |  | Gain |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cb | Cr | Cb | Cr | Cb | Cr |
| Office_Scada_1 | 36.67 | 38.58 | 37.29 | 40.52 | 0.62 | 1.94 |
| sc_cad_waveform | 24.97 | 23.06 | 26.75 | 25.01 | 1.78 | 1.95 |
| Video_Maps_1 | 37.95 | 42.95 | 38.91 | 43.86 | 0.96 | 0.91 |
| sc_programming | 36.14 | 35.96 | 36.57 | 36.60 | 0.43 | 0.64 |
| sc_video_conferencing_doc_sharing | 32.70 | 31.12 | 33.11 | 31.25 | 0.41 | 0.13 |
| sc_wordEditing | 37.95 | 36.28 | 37.54 | 36.63 | -0.41 | 0.35 |
|  |  |  |  | Average | + 0.63 | + 0.99 |

Table 2 shows the results when the histogram analysis is used to define the radius. The lookup table used in this test is generated by analyzing the local histograms of all the sequences. On average there is a PSNR gain on the chroma planes of +0.63 dB and +0.99 dB. This method works best on the sc_cad_waveform sequence (see fragment in Figure 2) where a gain of +0.31dB and +0.56 dB is achieved on top of the best-fixed radius filter for this sequence (radius=5). The sc_wordEditing sequence shows a PSNR loss for the Cb component. This is because this sequence has lots of (blue) transparency, which is not detected as screen content and more smoothing than required is done. In spite of the lower PSNR there is still an improvement on subjective screen quality, as demonstrated in Figure 3.

Figure 2: Fragment from the first frame of sc_cad_waveform. Left is the original image with the artifacts. Right is the improved image with guided image filtering.

Figure 3: Fragment from sc_wordEditing: (top) unfiltered with PSNR=37.95 (bottom) filtered with PSNR = 37.54

# Future work

Table 3: Results of guided image filtering when best radius is selected for each pixel, compared to the original 4:4:4 sequence

| Sequence | Original |  | Best Radius GIF |  | Gain |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cb | Cr | Cb | Cr | Cb | Cr |
| Office_Scada_1 | 36.67 | 38.58 | 39.99 | 42.04 | 3.32 | 3.46 |
| sc_cad_waveform | 24.97 | 23.06 | 28.11 | 26.1 | 3.14 | 3.04 |
| Video_Maps_1 | 37.95 | 42.95 | 41.55 | 46.26 | 3.60 | 3.31 |
| sc_programming | 36.14 | 35.96 | 39.62 | 39.05 | 3.48 | 3.09 |
| sc_video_conferencing_doc_sharing | 32.7 | 31.12 | 34.57 | 32.74 | 1.87 | 1.62 |
| sc_wordEditing | 37.95 | 36.28 | 40.03 | 39.02 | 2.08 | 2.74 |
|  |  |  |  | Average | 2.92 | 2.88 |

This contribution is a work in progress and not all options have been explored to define the best radius (see Table 3). If the best radius is selected, the PSNR gain could go up to +2.92 dB and +2.88 dB on these sequences. It needs more investigation how the radius can be defined better from the luma component. To reduce the computational complexity the information from the encoding process (CTB size, prediction information...) can be reused to define the radius.

The lookup table is dependent on the content, so a useful path for future work could also be to do a learning phase at the encoder side and transmit the table in a SEI message to the decoder. In this way the decoder can use the best table for all sequences and should not be pre-configured with the table.

# Conclusion

This contribution demonstrates the use of guided filtering for improving screen content coding for cost efficient video compression. The proposed algorithm provides an objective improvement of visual quality (PSNR) on the chroma planes of +0.63 dB and +0.99 dB. It also demonstrates subjective improvements in the provided test sequences. However this is still a work in progress. More research has to be done to integrate into the encoding chain for improved efficiency.

# References

[1] H. Yu, X. Wang, Y. Jing, “AHG8: More investigation on screen content”, JCTVC-M0320, Incheon, Korea, Apr. 2013.

[2] He, Kaiming, Jian Sun, & Xiaoou Tang. (2013). Guided Image Filtering. IEEE Transactions on Pattern Analysis and Machina Intelligence , 35 (6), 1397-1409.

# Patent rights declaration(s)

Barco N.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ghent University - iMinds may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).