# Draft Text Specification

The proposed text changes are for the contribution JCTVC-O0277.doc for the Intra Block Copy MV difference coding fixed length scheme B. The changes are marked in yellow as compared to JCTVC-N01005_v3.doc.

#### 7.8.3.9 Motion vector difference syntax

| mvd_coding( x0, y0, refList ) { | Descriptor |
| --- | --- |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| if( !intra_mc_flag[ x0 ][ y0 ] ) { |  |
| if( abs_mvd_greater0_flag[ 0 ] ) |  |
| abs_mvd_greater1_flag[ 0 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 1 ] ) |  |
| abs_mvd_greater1_flag[ 1 ] | ae(v) |
| } |  |
| if( abs_mvd_greater0_flag[ 0 ] ) { |  |
| if( !intra_mc_flag[ x0 ][ y0 ] ) { |  |
| if( abs_mvd_greater1_flag[ 0 ] ){ |  |
| abs_mvd_minus2[ 0 ] | ae(v) |
| } |  |
| } |  |
| else { |  |
| Abs_mvd[0] | u(7) |
| } |  |
| mvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if( abs_mvd_greater0_flag[ 1 ] ) { |  |
| if( !intra_mc_flag[ x0 ][ y0 ] ) { |  |
| if( abs_mvd_greater1_flag[ 1 ] ){ |  |
| abs_mvd_minus2[ 1 ] | ae(v) |
| } |  |
| } |  |
| else { |  |
| Abs_mvd[1] |  |
| } | u(6) |
| mvd_sign_flag[ 1 ] |  |
| } | ae(v) |
| } |  |

#### 7.4.9.9 Motion vector difference semantics

…

abs_mvd_minus2[ compIdx ] plus 2 specifies the absolute value of a motion vector component difference.

When abs_mvd_minus2[ compIdx ] is not present, it is inferred to be equal to −1.

abs_mvd[compIdx] specifies the absolute value of a motion vector component difference.

When abs_mvd[ compIdx ] is not present, it is inferred to be equal to −1.

mvd_sign_flag[ compIdx ] specifies the sign of a motion vector component difference as follows

…