# Proposed text

Coding unit syntax:

| ... |  |
| --- | --- |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { |  |
| if(log2CbSize = = MinCbLog2SizeY) |  |
| part_mode | ae(v) |
| if( PartMode = = PART_2Nx2N ) |  |
| mvd_coding( x0, y0, 2) |  |
| else {/*PART_NxN*/ |  |
| mvd_coding( x0, y0, 2) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), 2) |  |
| } |  |
| } |  |
| ... |  |

In 8.4.1

…

Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0 and IntraSplitFlag is equal to 1), for the variable blkIdx proceeding over the values 0..3, the following ordered steps apply:

- The variable xPb is set equal to xCb + ( nCbS  >>  1 ) * ( blkIdx % 2 ).

- The variable yPb is set equal to yCb + ( nCbS  >>  1 ) * ( blkIdx / 2 ).

- The derivation process for the intra prediction mode as specified in subclause 8.4.2 is invoked with the luma location ( xPb, yPb ) as input.

- The general decoding process for intra blocks as specified in subclause 8.4.4.1 is invoked with the luma location ( xPb, yPb ), the variable log2TrafoSize set equal to log2CbSize − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeY[ xPb ][ yPb ], the variable predModeIntraBc set equal to intra_bc_flag[xCb][yCb], the variable bvIntra set equal to BvIntra[xPb][yPb], and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

In 8.4.4

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- a variable log2TrafoSize specifying the size of the current luma coding block.

Output of this process is the block vector bvIntra.

The variable BvpIntra[ compIdx ] specifies a block vector predictor. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1.

The block vector bvIntra is derived by the following ordered steps, for the variable compIdx proceeding over the values 0..1:

- Depending upon the number of times this process has been invoked for the current coding tree unit, the following applies:

- If this process is invoked for the first time for the current coding tree unit, bvIntra[ compIdx ] is derived as follows:

bvIntra[ compIdx ] = BvdIntra[ xPb ][ yPb ][ compIdx ]	(8-25)

- Otherwise, bvIntra[ compIdx ] is derived as follows:

bvIntra[ compIdx ] = BvdIntra[ xPb ][ yPb ][ compIdx ] + BvpIntra[ compIdx ]	(8-25)

- The value of BvpIntra[ compIdx ] is updated to be equal to bvIntra[ compIdx ].

It is a requirement of bitstream conformance that all of the following conditions are true:

–	The value of bvIntra[ 0 ] shall be greater than or equal to – ( xPb % CtbSizeY + CtbSizeY ).

–	The value of bvIntra[ 1 ] shall be greater than or equal to – ( yPb % CtbSizeY ).

–	When the derivation process for z-scan order block availability as specified in subclause 6.4.1 is invoked with ( xCurr, yCurr ) set equal to ( xPb, yPb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + bvIntra[ 0 ], yPb + bvIntra[ 1 ] ) as inputs, the output shall be equal to TRUE.

–	When the derivation process for z-scan order block availability as specified in subclause 6.4.1 is invoked with ( xCurr, yCurr ) set equal to ( xPb, yPb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + bvIntra[ 0 ] + (1<< log2TrafoSize) − 1, yPb + bvIntra[ 1 ] + (1<< log2TrafoSize) – 1 ) as inputs, the output shall be equal to TRUE.

–	One or both of the following conditions shall be true:

–	bvIntra[ 0 ] + (1<< log2TrafoSize)  <=  0

–	bvIntra[ 1 ] + (1<< log2TrafoSize)  <=  0