| Rapporteur Meeting of Questions 2, 3, 4, 12, 13, 21, 22, 24, 25, 27, & 28/16 |  |  | AVD-3813 |
| --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | STUDY GROUP 16 |  |
|  |  | Original: English |  |
| Question(s): | Q2 | Geneva, 22 – 25 June 2009 |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |
| Source: | RADVISION |  |  |
| Title: | Handling of error conditions in H.323 |  |  |
| Purpose: | Discussion |  |  |

Summary

This document outlines the problem of potentially vague specification of handling of error conditions in H.323.

# 1.	Introduction

H.323 or to be more precise H.225.0 gives very little attention to the specification of processing of the H.225.0 Call Signalling messages errors. The only place dedicated to this subject is Clause 7.1 of H.225.0.

On the other hand, ITU-T Recommendation Q.931 on which H.225.0 messages are based provides quite detailed information on the same subject. While H.225.0 states that Implementations shall follow ITU-T Rec. Q.931 as specified in H.225.0, there is a lot of confusion surrounding the cases which H.225.0 does not cover and Q.931 does.

In addition, ETSI TS 101 804 – 2 defines Conformance Test Specification for ITU-T H.225.0. This test specification apparently based on Q.931 procedures, not on the corresponding H.225.0 ones. More than this, in many cases ETSI TS 101 804 – 2 requests behavior which claims to be based on Q.931, however is not defined neither in Q.931, nor H.225.0.

The situation creates a lot of confusion regarding the document which the implementations need to follow, and must be addressed.

# 2.	Discussion

Q.931 messages consist of a message type and a list of informational elements. Q.931 has very specific rules detailing behavior of the recipient of the message if, for example, the message type is unknown, the information element is not encoded correctly, etc, etc

H.225.0 messages are based on Q.931 messages, however have added complexity of the PER encoded part inside user-user information element. In spite this added complexity, the only rule defining the behavior of the recipient of a “not understood” H.225.0 message is the following sentence in Clause 7.1 of H.225.0: “Note that the H.225.0 endpoint may, according to ITU-T Rec. Q.931, ignore all optional messages it does not support without harming interoperability, but shall respond to an unknown message with a Status message.”.

Furthermore, the aforementioned sentence leaves enough room for confusion. What is, for example, “an unknown message”? Should a syntactically invalid message be considered an unknown? What “cause value” should such Status message have?

All these facts bring us to a conclusion that to achieve interoperable implementation relevant parts of H.225.0 should be revised.

It is believed that for purposes of clarity such revision should be both more specific in defining the H.323 entity behaviour, and more straightforward in specifying which parts of Q.931 do not apply.

# 3.	Conclusion

It is proposed to discuss the problem and select the way for unambiguous specification of the error handling in H.323/H.225.0.

__________________