| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4061 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | H.248.80: Configuration Characteristic Modifiers |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This document proposes changes to draft ITU-T Recommendation H.248.80 to address configuration characteristic modifiers.

2.	Discussion

Draft Recommendation ITU-T H.248.80 contains several open editor's notes. One note addresses the syntax of the a=pcfg: attribute:

### 9.6.2	Usage of SDP Capability Negotiation Framework Attributes

{Editor’s note: The use of + and [] to designate optional and mandatory SDP parameters and how this relates to H.248 needs to be further described. the use of the "+" prefix for a parameter indicates that the entire configuration must be ignored if the parameter is not understood; otherwise, the parameter itself may be ignored. However in the context of H.248 the MGC should be responsible for checking that the SDP is valid as the MG will typically present an error if it doesn’t understand something. As this is explicit syntax (like the H.248 O-) it may be preferable to allow these prefixes by the package.}

Effectively IETF RFC5939 introduces the following syntax related to the characteristics of potential configuration:

"-" (Delete) : This allows an endpoint to indicate that certain attributes (media, session, media and session) of the actual configuration are not part of the preferred configuration.

"[ ]" (Optional): This allows an endpoint to indicate that attribute capability is optional.

"|" (Alternatives): This allows an endpoint to indicate alternate capability lists.

"+" (Mandatory): This allows an endpoint to indicate that a particular extension attribute must be supported as part of the potential configuration.

As mentioned in the editor's note H.248 already has mechanisms related to the above functionality.

## 2.1 Interworking "SDP CapNeg" and H.248.1

As described in clause 6.1/H.248.80 it is possible to map preferred configurations to the H.248 Reserve Group  and Reserve Value concepts. Clause 6.1.2/H.248.80 already provides a description of how "|" is mapped. However it does not discuss the use of "-", "[ ]' and "+".

The use of "-" is a means of indicating that the potential configuration doesn't contain attributes from the actual configuration. In mapping to H.248 concepts there is no distinction between actual and preferred configurations. All the required properties/SDP are set for a particular H.248 Group/Stream. Thus there is no need to indicate particular attributes are deleted from a H.248 Group, they are simply omitted from the applicable H.248 Descriptor.

The use of "[ ]" indicates that it is optional to support a particular capability. There is no equivalent H.248 element. A MGC may either indicate that all ReserveGroups/Values should be supported (= True) or a single instance (=False) should be chosen.

The use of "+" indicates that it is mandatory to support a particular capability for a particular potential configuration. There is no H.248 element to explicitly signal this however this behaviour can be inferred when a particular capability is added to a H.248 Group.

## 2.2 Enhanced "SDP CapNeg" H.248 Support

H.248.80 defines the "Enhanced Revised Offer/Answer Support" package that allows a MGC and MG to utilise the SDP Capability Negotiation syntax. In this case mapping of the "-", "[ ]", "|" and "+" potential configuration characteristics to H.248 concepts (Reserve Group and Reserve Value etc.) is not required. Furthermore  as the potential configuration characteristics affect only configurations other than the actual configuration there is no need to map it directly to H.248. Thus it is valid to use "-"", "[ ]", "|" and "+" within the syntax described in the "Enhanced Revised Offer/Answer Support" package.

As such it is proposed to capture the above discussion in draft H.248.80 and to remove the editor's note.

3.	Proposal

It is proposed to accept the following changes. Changes are marked up against TD 343R1/WP2 from the July, Geneva SG16 meeting.

### 6.1.1	Relationship with the H.248 Reserve Group concept

When the MGC detects the use of the Revised Offer/Answer SDP procedures and requires that the information is sent to the MG, each of the potential configurations (as well as the actual configuration) are enumerated a separate H.248 group. For more information regarding H.248 Groups see clause 7.1.8/[ITU-T H.248.1].

Note: The MGC may apply a policing function and limit the number of groups/capabilities based on internal policy. For the purposes of description this Recommendation assumes that all configurations are enumerated.

H.248 groups are represented by providing a GroupID in the binary encoding or using multiple session descriptions (i.e. SDP) separated by v= lines. Thus each group effectively represents a “potential configuration”.

The first group shall relate to the highest priority “potential configuration” I.e. the one with the lowest a=pcfg:x number. This group shall be followed by the next highest a=pcfg:x number. The final group shall be the “actual configuration”.

Each group shall contain all properties/parameters to for a valid SDP description of the configuration. In the case of the text encoding using SDP this means that all SDPCapNeg attributes (and any extension attributes) related to the a=pcfg instance should be incorporated into a valid SDP session description.

[IETF RFC5939] defines several modifiers that are related to the characteristics of a potential configuration: "-" (Delete), "[ ]" (Optional), "|" (Alternatives) and "+" (Mandatory). In incorporating SDPCapNeg attributes into a valid SDP session description these need to be taken into account.

The use of "-" is a means of indicating that the potential configuration doesn't contain attributes from the actual configuration. In mapping to H.248 concepts there is no distinction between actual and preferred configurations. All the required properties/SDP are set for a particular H.248 Group/Stream. Thus there is no need to indicate particular attributes are deleted from a H.248 Group, they are simply omitted from the applicable H.248 Descriptor.

The use of "[ ]" indicates that it is optional to support a particular capability. There is no equivalent H.248 element. A MGC may either indicate that all ReserveGroups/Values should be supported (= True) or a single instance (=False) should be chosen.

The use of "+" indicates that it is mandatory to support a particular capability for a particular potential configuration. There is no H.248 element to explicitly signal this however this behaviour can be inferred when a particular capability is added to a H.248 Group.

The use of "|" indicates alternatives. See clause 6.1.2 for more information.

However no syntax described by SDPCapNeg shall be used in the groups otherwise an error response may be generated.

For example, given the following Revised Offer/Answer SDP:

v=0

o=- 25678 753849 IN IP4 192.0.2.1

s=

c=IN IP4 192.0.2.1

t=0 0

m=audio 53456 RTP/AVP 0 18

a=tcap:1 RTP/SAVP

a=acap:1 crypto:1 AES_CM_128_HMAC_SHA1_80

inline:WVNfX19zZW1jdGwgKCkgewkyMjA7fQp9CnVubGVz|2^20|1:4

a=pcfg:1 t=1 a=1

To create a valid SDP for the first group (related to a=pcfg:1) the MGC would take RTP/AVP from the m=audio attribute and replace it with the RTP/SAVP from the a=tcap:1 attribute. It would then delete the a=tcap:1 line. It would then see that a=acap:1 introduced the crypto attribute thus it would create a new a=crypto line containing the necessary information. It would then remove the a=acap:1 line. The a=pcfg:1 would then be removed. Thus the first group would be coded:

v=0

o=- 25678 753849 IN IP4 192.0.2.1

s=

c=IN IP4 192.0.2.1

t=0 0

m=audio 53456 RTP/SAVP 0 18

a=crypto:1 AES_CM_128_HMAC_SHA1_80

inline: WVNfX19zZW1jdGwgKCkgewkyMjA7fQp9CnVubGVz|2^20|1:4

### 6.1.2	Relationship with the H.248 Reserve Value concept

SDPCapNeg also allows the ability to specify alternatives (via the use of “|”) in configuration lists. The MGC can also provide this functionality via the use of over specified property values (or SDP parameters as per ITU-T H.248.39). Over specification of properties is equivalent to providing alternatives which effectively is a “or” function in the same way as “|”.

For example, given the following Revised Offer/Answer SDP:

v=0

o=- 25678 753849 IN IP4 192.0.2.1

s=

c=IN IP4 192.0.2.1

t=0 0

m=audio 53456 RTP/AVPF 0 18

a=tcap:1 RTP/AVPF RTP/AVP RTP/SAVP RTP/SAVPF

a=pcfg:1 t=2|1 a=1

The MGC would construct the following SDP for the first H.248 Group:

v=0

o=- 25678 753849 IN IP4 192.0.2.1

s=

c=IN IP4 192.0.2.1

t=0 0

m=audio 53456 RTP/AVP RTP/AVPF 0 18

...

### 9.6.2	Usage of SDP Capability Negotiation Framework Attributes

The support of the eroas package by the MG allows the MGC and MG to use the “a=acap:”, “a=tcap:”, “a=pcfg:” and “a=acfg:” attributes in H.248 messaging between them. Therefore if the MGC uses revised Offer/Answer SDP at the session level it does not have to perform the mapping to H.248 groups as described in clause 6.1. Alternate configurations maybe specified through the use of the “a=acap:”, “a=tcap:”, “a=pcfg:” attributes. The MGC must also provide an actual configuration. Due to the H.248 connection model and the separation of multiple media into streams the MGC shall provide the required SDPCapNeg attributes in each Stream. This is effectively the same procedure as described in clause 6.1.3 except that SDPCapNeg syntax may also be used.

The MGC may also use any additional attributes related to capabilities supported by the eroas/sdpe attribute.

Note: Typically media level SDPCapNeg attributes may be supported by H.248 in the Local and Remote Descriptor. However some session level attributes may not be supported because they apply to multiple streams. As H.248 separates stream descriptions these attributes may be more appropriate at a Termination level. In this case the definition of a package (see clause 10) is desirable.

The use of Reserve Group and Reserve Value is valid for SDPCapNeg attributes. However the need for SDP Groups is diminished due to the support of the configuration attributes. As per the H.248 grouping concept if the Reserve Group and Reserve Value flag is “off” the MG shall choose one of the configurations (the preferred or actual) specified. The chosen configuration is included in the response. The response shall contain the appropriate SDPCapNeg (or valid extension) attributes. If Reserve Group is "On" and "Reserve Value" is "off" the MG shall choose a configuration in the group. If "Reserve Value" is "on" then the MG shall only remove configurations than it cannot support. Likewise for "Reserve Group" equal to "on".

The use of this package means that mapping of the "-", "[ ]", "|" and "+" modifiers related to the characteristics of a potential configuration to H.248 concepts (Reserve Group and Reserve Value etc.) is not required. Furthermore as the modifiers affect only configurations other than the actual configuration there is no need to map it directly to H.248. Thus it is valid to use "-"", "[ ]", "|" and "+" syntax when this package is used.

Rather than fully specifying the SDPCapNeg attributes the MGC may wildcard CHOOSE the “a=pcfg:” attribute. The MG should then provide any alternate preferred configurations in addition to the “actual configuration” in the reply. Even if the MGC does not wildcard CHOOSE associated attributes (e.g. “a=tcap:” and “a=acap:”) these may be provided in the response.

Note: This is in keeping with H.248.1 behaviour that allows an MG to include extra SDP lines in a response.

If the MGC sends an SDPCapNeg extension that is not supported by the MG, like any unknown SDP the MG shall send an error reply.

___________________