| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4063 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | H.248.82: Updates |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This document proposes changes to draft ITU-T Recommendation H.248.82 to address the open Editor’s notes.

2.	Discussion

Draft Recommendation ITU-T H.248.82 contains several open editor's notes, which are listed below:

## 2.1	Editor's note 1:

### 7.1.4	ECN Mode

{Editor's Note: Further specification may be needed for this property to more completely detail the mapping from the call control level "mode" concept to how it relates to the H.248 Descriptors.}

The Contributor's believe that this is an open point. The call control level "mode" and the H.248 ECN Mode property have slightly different implementations. The call control level "mode" is defined based on a bi-directional flow. The usage of the H.248 ECN mode property is based on a unidirectional H.248 Local or Remote Descriptor. Thus there are semantic issues that need to be explained. For example: what does the "mode" value "setread" mean when set on the Local Descriptor? The Local Descriptor would typically specify a "read" function rather than "set". It is proposed to explain in more detail what the values mean.

## 2.2	Editor's note 2:

7.1.6	ECN Congestion Marking

{Editor's Note: This property has been added due to the following text in clause 3.2 / [IETF ECN]:

" If the translator does not modify the media stream, it should copy the ECN bits unchanged from the incoming to the outgoing datagrams, unless it is overloaded and experiencing congestion, in which case it may mark the outgoing datagrams with an ECN-CE mark.  Such a translator passes RTCP feedback unchanged."

Further specification may be required.}

This property was accepted for inclusion into the package. The use of which is detailed in clause 7.6.1.1. Therefore the Contributors do not believe that further specification is required (at least not based on the current IETF ECN draft). Therefore it is proposed to remove the editor’s note.

## 2.3	Editor's note 3:

7.1.7	ECN SDP Usage

{Editor's note: This was previously local control. It seems this property would be a gateway wide property.}

This editor's note detailed why the property is placed in the Termination State Descriptor. Now that usage of the Termination State Descriptor is accepted, it is proposed that the editor's note shall be removed.

3.	Proposal

It is proposed to accept the following changes. Changes are marked up against TD 542R1/WP2 from the July, Geneva SG16 meeting.

### 7.1.4	ECN Mode

|  | Property Name: | ECN Mode |
| --- | --- | --- |
|  | Property ID: | mode (0x0004) |
|  | Description: | The ECN Mode parameter as defined in [IETF ECN] describes an endpoint’s ability to set and read ECN marks in UDP packets. Thus it describes a bi-directional flow. TheECN mode property in this package indicates whether the MG should set and/or read ECN marks. This property is equivalent to "mode" defined by the ABNF grammar defined by clause 6.1 [IETF ECN]. However the mode information is set in both the Local and Remote Descriptors. Thus the scope of each setting of the property is unidirectional. In order to minimise mapping at the MGC, the MG may utilise the ECN Mode value received at a call control for the ECN Mode property. Accordingly the MGC may set value: - "setonly" only in both the Local and Remote Descriptors, despite setting being only relevant to the Remote Descriptor. - "receiveonly" in both the Local and Remote Descriptors, despite reading only being relevant to the Local Descriptor. - "setread" in both the Local and Remote Descriptors, even though a particular Descriptor may only perform a set or read function. This does not imply that the MG must "set" or mark any particular ECN code point it refers to the mode that the MG shall operate in order to determine the correct ECN procedures to apply. The MG specific semantic is detailed below. |
|  | Type: | Enumeration |
|  | Possible values: | As per "mode" clause 6.1/[IETF ECN]. "setonly" (0x0001); There is agreement to use ECN at the IP connection in the LS(A)-to-RD(A) direction.. "setread" (0x0002); There is agreement to use ECN at the IP connection in both directions, i.e. the LS(A)-to-RD(A) and RS(A)-to-LD(A) direction.. "readonly" (0x0003); There is agreement to use ECN at the IP connection in RS(A)-to-LD(A) direction.. Note: For directions see naming conventions in clause 5.2.3.1/[ITU-T H.248.1]. |
|  | Default: | "setread" |
|  | Defined in: | Local / Remote |
|  | Characteristics: | Read/Write |

...

### 7.1.6	ECN Congestion Marking

|  | Property Name: | ECN Congestion Marking |
| --- | --- | --- |
|  | Property ID: | congestmark (0x0006) |
|  | Description: | This property allows the MGC to indicate whether or not RTP packets may be ECN-CE marked by the MG (or not) if it the MG itself experiences congestion. |
|  | Type: | Enumeration |
|  | Possible values: | "mgdet" (0x0001): The MG determines if ECN Congestion Marking is enabled. "cemark" (0x0002): ECN Congestion marking is enabled. "nomark" (0x0003): ECN Congestion marking is not enabled. |
|  | Default: | Provisioned. |
|  | Defined in: | Local / Remote |
|  | Characteristics: | Read Only |

### 7.1.7	ECN SDP Usage

|  | Property Name: | ECN SDP Usage |
| --- | --- | --- |
|  | Property ID: | ecnsdp (0x0007) |
|  | Description: | This property allows the MGC to determine which method for signalling the ECN "initiation method", "mode" and "ect marking" the MG supports. The MGC is may then use any method that the MG supports. |
|  | Type: | Enumeration |
|  | Possible values: | "P" (0x0001) The properties in clauses 7.1.1, 7.1.3, 7.1.4 and 7.1.5. "S" (0x0002) The SDP "a=ecn-capable-rtp" attribute. "B" (0x0003) Either of the above two methods. |
|  | Default: | Provisioned. |
|  | Defined in: | Termination State (Root Termination Only) |
|  | Characteristics: | Read Only |

__________________