| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4128a |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 25 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ETRI |  |  |  |
| Title: | A Framework for USN-based Water Quality Assessment |  |  |  |
| Purpose: | Proposal |  |  |  |

# Introduction

To make the safe and ecologically healthy water environment, we assess the change in water quality and perform water quality monitoring in the rivers, marshes, lakes and so on. Currently, the majority of water quality monitoring is done manually. For example, people go to the rivers directly, get the water samples, and test its quality at the laboratory. Another testing method is via underwater pumps at the monitoring stations that are built beside rivers.  However, the construction cost of the monitoring station is very high and the installation of the monitoring station network is a difficult task.

Furthermore, the existing water quality management system simply monitors the sensors, collects and displays the sensing data. Such system also aggregates and keeps the measured data automatically from the water quality sensors allocated at the specific area.  However these water quality management systems can’t provide the water quality dispersion or a prediction of the future trend on the water pollution.

Ubiquitous Sensor Network (USN)-based water quality assessment framework, the proposed system observes middle and small sized rivers causing the water pollution in real-time. It consists of sensor network module, the network management module and the water quality assessment module. It also interworks with the existing water quality management network for wide areas. The sensor network includes water quality sensors, sensor nodes and gateways. To establish the network using these components is very inexpensive compare with the existing water quality management system using underwater pumps at the monitoring stations.

The proposed USN-based water quality assessment framework allows the sensor nodes to be installed directly in the river, with sensors consuming less power. Using the aggregated sensing information from the sensors the system observes the water pollution level or the eutrophication of rivers, lakes, ponds, etc.

Furthermore, it estimates the water quality using the values measured from the sensors covering the specified area, and then provides the dispersion of the water pollution, tracking the water pollutant source and predicting the water quality change. Since the proposed system can monitor the amount of the polluted water in real-time, the goal of our proposal is to define the framework specifications that exactly correspond with the needs of the main environmental agent.

To address the proposed framework, following items are considered. The framework requires sensors to increase the accuracy of water pollution value, dispersion, and prediction. Compared with other USN applications the transmission mechanism must be considered because the proposed framework’s service needs the long distance data transmission among sensors and between sensor nodes and the gateway. The sensor network for the framework is constructed with almost linear topology and therefore a proper network protocol to adopt it reasonably must be considered.

IP base networking must also be considered for the interworking with the existing water quality network and for a variety of Web-based user applications. Along with the networking, the simplified and standardized network management protocol is also considered to monitor the proposed framework in real-time. It must also provide the water quality assessment model using the sensor data and the converged data for precise water pollution, dispersion, and prediction. Although the water quality sensors is able to get more stable electric power than the existing sensors, its control  like moving of the sensor location considered the loss of sensor because of natural disaster such as the flood is required.

# Framework for USN-based Water Quality Assessment

USN-based water quality assessment framework provides real-time dispersion of the water pollution, tracking the water pollutant source and prediction around the middle and small sized rivers through the analysis of the data and its converged data from the sensor network. It also fulfils the intelligent network management by detection, separation and self-diagnosis of faults of the network resources, S/W and H/W of sensors, sensor nodes, gateways, and the server.

The framework functionality can be divided into three categories, firstly, the water quality assessment sensor networking, secondly the aggregation and intelligent processing of the water quality information, and finally an appropriate water quality assessment for USN-based water quality management.

<Figure 1> shows the entire USN-based water quality assessment framework. The sensor networking function is that sensor nodes send the sensing data to the gateway by the network protocol.  Sensors measure the value of the water quality parameters (for example, pH (Potential of Hydrogen), DO (Dissolved Oxygen), EC (Electrical Conductivity), ORP (Oxidation-Reduction Potential), etc.), the flow velocity, the water level, etc.

Sensor nodes send the data or the converged data (for example, calculated flow rates) from various sensors to the gateway. The gateway also sends the aggregated sensor data to the server for the water quality assessment. Meanwhile, the interworking with the existing network can be easily performed because of its network protocol.

The aggregation and intelligent processing of the water quality information indicates that it regularly monitors the water quality parameter values measured from sensors and then filters the faults to improve the accuracy of the water quality assessment. The water parameter that can’t be measured from the sensor (for example, TN (total nitrogen), TP (total phosphorus)) directly can infer it from the aggregated sensor data.  It distinguishes sensor faults and detects, performs the separation and self-diagnosis of sensor network faults via data mining of the aggregated data. Moreover, it controls the sensing interval and the operation of the sensor wiper.

The water quality assessment function is performed on the server deal with sensor networks. It provides the modelling results of the water quality dispersion to predict exactly the quality of the water using the sensing data aggregated in real-time. It also offers the service of tracking the water pollutant source and prediction about the water quality change.

<Figure 1> USN-based Water Quality Assessment System Concept and its Framework

# Standardization issues

The water quality assessment framework as mentioned earlier is required to assess the water quality, detect the water pollutant source, and predict the water pollution in real-time. To address this, it is needed to consider many kinds of requirements compared with the existing water quality networks. Furthermore, for the interoperability between sensors and sensor nodes and between sensor nodes and the gateway, the common interfaces are also required. Thus, following items need to discuss for the common specification.

- Requirements of USN-based Water Quality Assessment

  - Sensors

  - Sensor nodes

  - Gateway

  - Water Quality Assessment Server

- Identification for Water Quality Assessment

- Reference Architecture of USN-based Water Quality Assessment

  - Interface between the sensor node and the gateway

  - Interface between the sensor and the sensor node

  - Interworking with Legacy Water Quality Assessment Networks

- Sensor Network Management for Water Quality Assessment

- Water Quality Assessment Mechanism

- Use cases of Water Quality Assessment

# Proposals

In this contribution, we propose to add “A Framework for USN-based Water Quality Assessment” into the possible future work items of Q25/16 for giving participants a hint on the work area of Q25/16 and to invite contributions.

__________________