| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22 and 25/16 |  |  |  | TD-30 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor |  |  |  |
| Title: | F/H.TPS-Arch “Telepresence System Architecture” (New): Output draft |  |  |  |
| Purpose: | Proposal |  |  |  |

1. 	Introduction

This document contains the editor’s output draft for F/H.TPS-Arch “Telepresence System Architecture”. The base text is taken from AVD-4094, 4095, 4091, 4107, 4108 and 4109. The marked up text represents changes as a result of the discussion on the above contributions as well as AVD-4084.

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| F/H.TPS-Arch Ed. 0.1 | Initial baseline text. Output from Boston meeting 07/11. Based on AVD-4084, 4094, 4095, 4091, 4107, 4108 and 4109. |

Living List Items:

- The current Telepresence Architecture builds on the H.323 architecture. Using the term H.323 telepresence endpoint may cause confusion at a future point in time if one of the Telepresence Recommendations attains anH.32x designation. This is item kept as a reminder to consider this when discussing Recommendation number allocation with the TSB.

- F.TPS-Reqs has a requirement regarding “appropriate” spatial information. This relates to that the spatial information used in a telepresence system should be applicable to the particular use case scenario. The text of F/H.TPS-Arch currently refers to spatial information. Further contribution is needed to ensure that the use of spatial information in this document is consistent with the requirements. For example: Is spatial information negotiated? Is it expressed as capabilities? Etc.

Contents

1	Scope	4

2	References	4

3	Definitions	5

3.1	Terms defined elsewhere	5

3.2	Terms defined in this Recommendation	5

4	Abbreviations and acronyms	5

5	Conventions	6

6	High Level Architecture	7

6.1	Description of sub-system components	7

6.1.1	Telepresence Endpoint	7

6.1.2	Media Control Unit	8

6.1.3	Gate Keeper	8

6.1.4	Gateway	9

6.1.5	Management System	9

7	Sub-system architectures	9

7.1 	Endpoint Architecture	9

7.1.1 Endpoint Architecture Description	10

7.2	Multipoint Control Unit	15

7.3	Gate Keeper	16

7.4	Gateway	16

8	H.323 Architecture	16

8.1 Mapping Telepresence on a H.323 Architecture	16

8.1.1 Peer-to-Peer Communication：	18

8.1.2 Multi-Point Communication	19

8.2 Peer to peer call process	19

9 	Telepresence Room Design	22

9.1	Room Layouts	22

9.2	Room characteristics	24

9.2.1 Room Dimensions and shape	24

9.2.2 Wall Floor and Ceiling Surfaces	24

9.2.3 Lighting	24

9.2.4 Acoustics	25

9.2.5 HVAC	25

9.2.6 Power Requirements	25

9.2.7 Network Connectivity	25

Draft new Recommendation F/H.TPS-Arch

Telepresence System Architecture

Summary

<Mandatory material>

Keywords

<Optional>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# 1	Scope

This Recommendation first describes a Telepresence system in a high level technology agnostic manner. This is then followed by a more in depth description of the components. The later parts of this Recommendation then describe how [ITU-T H.323] relates to a Telepresence system. It then details aspects regarding Telepresence room design.

Unless explicitly stated in the text, the interfaces depicted in the figures and functions in this Recommendation are logical and thus do not necessarily represent a particular physical implementation.

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[F.TPS-Reqs]	Recommendation ITU-T X.yyy (date), Definitions, requirements, and use cases for Telepresence Systems

[H.TPS-AV]	Recommendation ITU-T X.yyy (date), Audio/Video Parameters for Telepresence systems

[ITU-T H.245]	Recommendation ITU-T H.245 (05/2011), Control protocol for multimedia communication

[ITU-T H.320]	Recommendation ITU-T H.320 (03/2004), Narrow-band visual telephone systems and terminal equipment

[ITU-T H.323]	Recommendation ITU-T H.323 (12/2009), Packet-based multimedia communications systems

[ITU-T Q.931]	Recommendation ITU-T Q.931 (05/1998), ISDN user-network interface layer 3 specification for basic call control

# 3	Definitions

## 3.1	Terms defined elsewhere

<Normally terms defined elsewhere will simply refer to the defining document. In certain cases, it may be desirable to quote the definition to allow for a stand-alone document>

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]: <optional quoted definition>

3.1.2	<Term 2> [Reference]: <optional quoted definition>

{Editor’s note: The IETF Clue working group currently have an internet draft (draft-wenger-clue-definitions-xx)[http://datatracker.ietf.org/doc/draft-wenger-clue-definitions] defining Telepresence related terms. Rather than re-defining terms here it may be advantageous to reference the IETF terms.}

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Telepresence Media Codec:  This term (and function) is used to describe any media (i.e. audio or video) codec that would be applicable for use in a Telepresence system.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

GK	Gate Keeper

GW	Gateway

IP	Internet Protocol

ISDN	Integrated Services Digital Network

LAN	Local Area Network

LCD	Liquid Crystal Display

MCU	Multipoint Control Unit

PC	Personal Computer

PDP	Plasma Display Panel

PSTN	Public Switched Telephony Network

RAS	Registration, Admission and Status

RTP	Real Time Protocol

RTCP	RTP Control Protocol

TCU	Telepresence Control Unit

TMC	Telepresence Media Codec

TMIO	Telepresence Media Input / Output Control Unit

TMP	Telepresence Media Processor

WAN	Wide Area Network

# 5	Conventions

5.1 Icons used

Figure 1 : Icons used in this Recommendation

{Editor’s note: The above icons will be placed in alphabetical order.}

# 6	High Level Architecture

Figure 2: High level Telepresence architecture

A high level telepresence architecture includes: telepresence endpoints, legacy video conferencing endpoints, legacy audio endpoints, Multipoint Control Units (MCU), Gate Keepers (GK), Gateways (GW), the management system, etc. There may be multiple forms of telepresence endpoints, such as 3-screen endpoint, single-screen endpoint and etc. The architecture supports inter-operation of heterogeneous telepresence endpoints and inter-operation between telepresence endpoints and legacy video conferencing and audio endpoints.

There may be multiple MCUs and GKs in the architecture.

The architecture may have multiple GWs that connect to other types of networks, such as fixed and mobile telephone networks.

## 6.1	Description of sub-system components

### 6.1.1	Telepresence Endpoint

The Telepresence Endpoint is the logical point of final termination, and may consist of more than one physical device. The main functions of the telepresence endpoint include:

- Setting up call sessions through capability negotiation, capturing, encoding, packetizing and sending local media streams,

- Receiving, decoding, de-packetizing and rendering remote media streams.

In contrast to a telepresence endpoint, an MCU may also send and receive media streams, but it cannot initiate or terminate media streams through capturing and/or rendering. There may be multiple forms of telepresence endpoints, such as: 3 camera/screen endpoint, single camera/screen endpoint, audio-only endpoint and etc.

Endpoint characteristics include:

- The number of cameras, spatial relationships of cameras and capture capabilities, such as: resolution, frame rate and aspect ratio.

- Viewpoint and field of view for cameras, including overlap regions, missing regions or whether images of cameras can be composited to a seamless image in over the entire depth range.

- The number, type, size of displays, spatial relationships of displays and display capabilities, such as: resolution, frame rate, aspect ratio, and display border width.

- The number and type of presentation displays, spatial relationship of presentation displays and display capabilities.

- Spatial relationships of audio signals of microphones and capture capabilities of microphones.

- Spatial relationships of loud speakers and parameters of loud speakers.

- Display strategies for multipoint conference, such as the static or dynamic mapping of camera-to-display.

{Editor’s note: Further contribution is invited on endpoint characteristics.}

Endpoint characteristics are transmitted through signalling streams in session initialization and used for capability negotiation in inter-operation of different telepresence endpoints.

Note: [ITU-T H.245] defines a terminal as “any endpoint and may be a user's terminal or some other communication system such as an MCU or an information server”. As such the term H.323 Telepresence Terminal is equivalent to the term “Telepresence Endpoint”.

### 6.1.2	Media Control Unit

The MCU is a device that connects two or more telepresence endpoints together to one single telepresence conference. An MCU can setup and control multiple videoconferencing call sessions. After receiving media streams from multiple endpoints or cascading MCUs and GWs, an MCU processes the media streams and sends the processed media stream to an endpoint, MCU or GW.

An MCU must support video and audio mixing and media adaptation for different types of endpoints and should also support control functions, for example, auxiliary stream token control, voice switch and control based on endpoint screens, etc.

### 6.1.3	Gate Keeper

A GK is the resource management device for video conferencing and can provide resource management for endpoints, MCUs and GWs. The functions of a GK include: address resolution, access control and bandwidth management.

### 6.1.4	Gateway

A GW can connect various types of networks and provide interworking of signalling and media streams, such as interworking H.323 and SIP. It may also provide adaptation for media streams that have different characteristics e.g. codec format.

### 6.1.5	Management System

A management system mainly contains control and management functions that provide specific telepresence services, such as device management, conference scheduling and user authentication, etc.

# 7	Sub-system architectures

## 7.1 	Endpoint Architecture

Figure 3: Endpoint Architecture

A telepresence endpoint consists of the following components:

- A video capturing unit that consists of multiple main cameras and document cameras (optional) in certain spatial relationships.

- A video rendering unit that consists of multiple displays for main video streams and displays (optional) for presentation in certain spatial relationships.

- An audio capturing unit that consists of multiple microphones in certain spatial relationships.

- An audio rendering unit that consist of multiple loud speakers in certain spatial relationships.

- A Telepresence Control Unit (TPU) that connects and controls other units. It includes: a Telepresence Media Codec (TMC), Telepresence Media Processor (TMP) and Telepresence Media Input/Output (TMIO) Control units.

- A network unit, including: wired and wireless network interfaces and network switch, etc.

- An interactive unit, including: touch control panels, PCs, etc.

### 7.1.1 Endpoint Architecture Description

#### 7.1.1.1 Video Capturing Unit

A telepresence endpoint may have multiple main cameras and document cameras. Cameras connect through a telepresence media input/output control unit that can select and control multiple camera video sources.

Figure 4: Video Capturing Unit Architecture

Multiple main cameras may constitute a camera set for capturing images of participants at the telepresence endpoint. Each main camera capture images of participants in one region of the room and the multiple images of cameras are combined to a wide view angle, high resolution view. A camera set may use one of the following three models: the converging camera model, the parallel camera model and the con-optical centre camera model.

Figure 5: Camera models：converging camera model (left)，parallel camera model (middle)，con-optical centre camera model (right)

In the converging camera model, each of multiple cameras looks out directly in a different direction. The parallel camera model, in contrast to the converging camera model, all optical camera axis have the same direction. Multiple cameras capture multiple virtual images reflected by a mirror and the captured images almost have the same optical centre in the con-optical centre camera model. For the first two camera models, there may be overlap and missing regions between two images and individual images may not be able to be integrated seamlessly unless objects are extremely distant. While in the con-optical centre camera model, individual images can be stitched to a seamless image over the entire depth range. The composited image can be scaled to an image having a single camera’s resolution for inter-operation with a legacy videoconferencing endpoint that does not support this capability.

Document cameras are used to capture images of documents or objects in a conference room. There may be various spatial placements for document cameras, for example, hanging the cameras from the ceiling to capture images of objects on a desk.

#### 7.1.1.2 Video Rendering Unit

A video rendering unit may comprise of one or more main displays and presentation displays. Main displays are primarily used to display the video streams captured by local and remote main cameras and can be LCD, PDP monitors or projectors. In a fully immersive system a main display should have sufficient size to display actual-sized images. Multiple monitors are usually integrated to present a panoramic view, or a single projection screen and multiple projectors can also be used.

Figure 6: Video Rendering Unit Architecture

The telepresence media input/output control unit can dynamically arrange display patterns and contents for main displays and presentation displays in an automatic or manual way:

- Main displays may display the local and remote main video streams and presentation video streams.

- Presentation displays should only display the local and remote presentation video streams, because eye contact cannot be achieved when a main video stream is presented on a presentation display.

{Editor’s note: Further contribution is required on the above bullet point. There are scenarios where this may not be the case. E.g. where the presentation display has a camera.}

#### 7.1.1.3 Audio Capturing Unit

An audio capturing unit may comprise of multiple microphones in some spatial placement for capturing audio signals at the telepresence endpoint. This spatial placement would largely be dependent on the Telepresence endpoint configuration. For example: microphone placement may differ between single-row and multiple-row configuration. There may be no direct mapping between microphones and audio streams. Where spatial information is required the audio capture unit should support the  carriage of this information in the audio stream/s.

Figure 7: Audio Capturing Unit Architecture

Microphones may connect through a media input/output control unit that can select and control multiple audio sources. Auxiliary audio streams from a PC or other devices also connect to endpoint through media input/output control unit. Audio signals from multiple microphones may be combined into  one single audio stream for transmission.

{Editor’s note: Further contribution is required to discuss in more detail how the audio signals are combined and transmitted.}

#### 7.1.1.4 Audio Rendering Unit

An audio rendering unit may comprise of multiple loud speakers in some spatial placement for rendering audio signals of the remote telepresence endpoint. There are no direct mappings between loud speakers, microphones and displays, but the spatial information of sound from the remote endpoint can be reproduced through the audio rendering unit. Both the sound and image positions of remote participants should coincide.

Loud speakers should connect through the media input/output control unit that can select and control audio output dynamically.

Figure 8: Audio Rendering Unit Architecture

#### 7.1.1.5 Telepresence Control Unit

A telepresence control unit connects other units in a telepresence endpoint. The main functions of the telepresence control unit include:

- To provide interfaces to other functional unit in an endpoint, invoking other units to complete processing.

- To handle H.323 signalling processes, include registration, authentication, session initialization, controlling and completing sessions, capabilities negotiation for inter-operation, etc.

- To send and receive multiple media streams.

- To provide contacts and other interfaces.

- To provide device management and debug interfaces, etc.

- etc.

#### 7.1.1.6 Telepresence Media Codec

The telepresence media codec is a logical unit that be used for coding and decoding of multiple video and audio streams. Synchronization between video and audio streams is supported. The telepresence media codec can be implemented using two configurations, a centralized solution or distributed solution. A centralized solution uses a single device to code and decode multiple media streams. Synchronizing media streams for this configuration is simpler but the device requires higher processing capabilities. The distributed solution needs more than one device for coding and decoding of multiple media streams which results in more complicated synchronization strategies. However this configuration may use devices that have lower processing capabilities than that of the centralized solution.

#### 7.1.1.7 Telepresence Media Input/Output Control Unit

The telepresence media input/output control unit is a logical unit that can select and control multiple input/output media streams that include main and auxiliary video and audio streams. Video and audio capturing units, video and audio rendering units, PCs and other audio endpoints are connected to the telepresence media input/output control unit.

#### 7.1.1.8 Telepresence Media Processor

The telepresence media processor is used for processing video and audio steams to provide the required audio and visual experience. The main functions of telepresence media processor include:

- Geometric correction for multiple camera images to get better image mosaic effect.

- Colour correction for making luminance and chrominance of multiple camera images uniform.

- Geometric correction for multiple rendering video streams, that make the video outputs adapt to various types of display and get better display performance, for example: geometric correction for arc projection screen.

- Colour correction for multiple rendering video streams to get better rendering performance.

- Pre-processing for audio signals captured by audio capturing unit, such as audio de-noising and enhancement for multiple microphones, etc.

- Pre-processing for audio signals rendered by audio rendering unit, for example, improving audio quality of loud speakers.

## 7.2	Multipoint Control Unit

The main functions of an MCU are basically identical to the definitions in [ITU-T H.323], as per the following descriptions:

Access functions:

- To support accesses of various types of endpoints that include: multi-screen telepresence endpoint, single-screen telepresence endpoint and audio endpoints.

- To support multi-image composition videoconferencing.

- To support single/multiple auxiliary media streams.

- To support MCU cascading connection. Multiple media channels should be supported and can be flexibly multiplexed in cascading connection mode.

Adaptation functions:

- Adaptation for media streams according to capabilities of endpoints, such as video transcoding, audio transcoding, etc.

- Adaptation for media streams according to characteristics of endpoint, such as display aspect ratio, etc.

Control functions:

- To support voice switch and manual switch mode.

- To support switch based on endpoint or screens when an endpoint has multiple screens.

## 7.3	Gate Keeper

The main functions of GK are identical to the definitions in [ITU-T H.323].

## 7.4	Gateway

The main functions of GW are identical to the definitions in [ITU-T H.323].

# 8	H.323 Architecture

## 8.1 Mapping Telepresence on a H.323 Architecture

The Telepresence system architecture is based on the existing [ITU-T H.323] system. A Telepresence endpoint can be viewed as another kind of H.323 endpoint, as depicted in the following figure:

Figure 9: H.323 Telepresence network model

[ITU-T H.323] specifies the interfaces between the different components of a H.323 based multimedia system. This is illustrated in figure 10.

Figure 10: H.323 Telepresence system architecture

The main interfaces (reference points) in H.323 telepresence system architecture:

- A － Interface between legacy video conferencing endpoint and MCU, for transmitting conference call and control information,  completing  creation and release of media channels and media interaction.

- B － Interface between legacy video conferencing endpoint and GK, for transmitting registration, authentication and call process information using RAS messages.

- C － Interface between MCU and service system, for transmitting conference scheduling information, creating and ending conferences.

- D － Interface between MCU and GK, for registration, authentication, call process of MCU and transmitting resolving information of user addresses.

- E － Interface between GKs, transmitting resolving information of user addresses using RAS messages, etc.

- F － Interface between GK and service system, for transmitting GK’s management and configuration information, user information, billing data, etc.

- G － Interface between legacy videoconferencing endpoints, for transmitting call signalling, media streams and related control information when a terminal has a peer-to-peer communication.

- H － Interface between MCUs, for transmitting media streams between MCUs and information of MCU cascade connection and inter-operation.

- I － Interface between the MCU and PSTN/ISDN network.

- J － Interface between MCU and [ITU-T H.320] network，for inter-operation between H.323 and H.320 service network.

- K － Interface with other systems

In order to support telepresence endpoints several new interfaces based on existing ones are introduced:

- A1 － Interface between telepresence endpoint and MCU, for transmitting call and control information of telepresence conference, completing creation and release of media channels and exchanging  video and audio streams between telepresence system and MCU.

- B1 － Interface between telepresence endpoint and GK, for transmitting registration, authentication and call process information using RAS messages.

- G1 － Interface between telepresence endpoints, for transmitting call signalling, media streams and related control information when a telepresence endpoint has a peer-to-peer communication with another.

- G2 － Interface between legacy video conference endpoint and telepresence endpoint, for transmitting call signalling, media streams and related control information when a telepresence endpoint has a peer-to-peer communication with a legacy video conference endpoint.

### 8.1.1 Peer-to-Peer Communication：

Figure 11: H.323 Telepresence peer-to-peer communication

The communication between telepresence endpoints should have a single call control signalling stream, multiple video streams and a single audio stream.

The control signalling stream is used for controlling the whole calling process, for example:  enabling/disabling the sending of auxiliary streams.

### 8.1.2 Multi-Point Communication

Figure 12: H.323 Telepresence multi-point communication

Different types of telepresence endpoints may connect to an MCU.

Telepresence endpoint sends media steams according to results of capability negotiation.

The MCU completes the adaptations for media streams, for example: adaptation for single channel and multi-channel audio streams, retransmission for multiple video streams and adaptation for conference control.

## 8.2 Peer to peer call process

The peer-to-peer call processes based on the existing H.323 system is depicted as the following figure:

Figure 13: H.323 telepresence peer-to-peer call processes

The multi-point call processes based on the existing H.323 system are similar to peer-to-peer processes:

Figure 14: H.323 telepresence multi-point call processes

The whole call process consists of the following steps:

- Registration process

- Call session setup process

- Media stream transport and control process

- Call session release process

- Unregistration process

The key steps that affect inter-operability are steps 2 and 3.

The detailed processes of step 2 and 3 are depicted by the following figure. A peer-to-peer call (multi-point call is similar) example through GK is given:

Figure 15: H.323 telepresence peer-to-peer detailed call processes

The detailed signalling processes include:

- Call setup process, include RAS and [ITU-T Q.931] call signaling.

- Media setup process, mainly include mast-slave determination, capability negotiation, logical channel opening through H.245 control signaling.

- Media stream transport and control process, include media streams transmission and controls using RTP/RTCP, [ITU-T H.245] control signaling.

The key steps that affect inter-operability are steps 2 and 3.

# 9 	Telepresence Room Design

## 9.1	Room Layouts

This clause details several example room layouts. Other configurations are possible. The following figure depicts the room layout of a single row 3-screen telepresence system, with the following characteristics:

- Three main cameras and three main displays.

- Three regions for participants in the room, and each region contain two seats.

- Each main camera captures images of one region.

The missing/overlapped region of two captured images is located at the corner of the conference table and covered by borders of two main displays.

Figure 16 : 3-Screen Telepresence  Room Layout (single row)

The following figure depicts the room layout of a multi-row 3-screen telepresence system, with the following characteristics:

- Three main cameras and three main displays.

- Three regions for participants in the room, and each region contain five seats.

- Each main camera captures images of one region.

The missing/overlapped region of two captured images is located at the corner of the conference table and covered by borders of two main displays.

Figure 17 : 3-Screen Telepresence  Room Layout (multi-row)

## 9.2	Room characteristics

### 9.2.1 Room Dimensions and shape

T.B.D.

### 9.2.2 Wall Floor and Ceiling Surfaces

T.B.D.

### 9.2.3 Lighting

T.B.D.

### 9.2.4 Acoustics

T.B.D.

### 9.2.5 HVAC

T.B.D.

### 9.2.6 Power Requirements

T.B.D.

### 9.2.7 Network Connectivity

T.B.D.

Annex A

<Annex Title>

(This annex forms an integral part of this Recommendation)

<Body of annex A>

Appendix I

<Appendix Title>

(This appendix does not form an integral part of this Recommendation)

<Body of appendix I>

Bibliography

[b-ITU-T X.yyy]	ITU-T Recommendation X.yyy (date), Title

__________________