| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | AVD-4221 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21,22,25/16 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | YRP, ETRI |  |  |  |
| Title: | Technical Note: Storing OID and ID according to H.IDscheme into standard-compliant data carriers |  |  |  |
| Purpose: | Information / Report |  |  |  |

Summary

This technical note describes the current theory and practice of storing identifiers of existing legacy code systems including tag-agnostic ucode [14] as well as the particular coding scheme proposed in H.IDscheme into standard compliant tags according to the approach proposed in H.IDscheme.

The aim of the technical note is to show that unambiguous and standard-compliant encoding and embedding is possible when H.IDscheme approach is taken

# 1	Introduction

H.IDscheme adopts the approach of multi-identification schemes to support the existing identification schemes. Basically, identifier is prefixed by OID of the identification scheme so that the multimedia application can know to what code system the following identifier belongs. This use of OID is described in the following clause 2 to give the reader concrete examples.

H.IDscheme also proposes one identification scheme (its code layout) of its own, tentatively called H.XXX code in this note, to be used for multimedia application in the age of the Internet of Things and ubiquitous computing.

In order to explain how to deal with the encoding and embedding issue of H.XXX in the future, the current practice of storing the identifier of “ucode”, a code system very similar to H.XXX code, into data carrier devices is described in the following Clause 3. When H.IDscheme is adopted as Recommendation, the proposed H.XXX code can be stored in a very similar fashion. It should be stressed that the discussed storing method into data carriers follows existing ISO/IEC standards associated with the data carrier devices where applicable.

The technical note explains how to use existing identifiers a la H.IDscheme. Specifically, this report describes how existing data format standards for various data carries will be followed to store identifiers according to the approach of H.IDscheme.

Clause 2 below discusses the use of OID and OID value to describe the existing legacy code systems, and Clause 3 discusses the encoding and embedding of OID and OID value in standard compliant data carriers.

# 2	Using existing identifiers in H.IDscheme

## 2.1	General method

In the approach taken by H.IDscheme (and its companion H.IRP), we use Object Identifier (OID: defined in ISO/IEC 15961 [1]) as a header to identify the identification scheme to which each following identifier belongs. So H.IDscheme uses  a pair of OID (i.e. type of ID) and object value (i.e. value of ID) to support existing legacy coding schemes.

H.IDscheme does not describe the representation method of the pair of OID and object value for a particular API. So, for discussion purposes in this report, we describe the pair of OID and object value in URN (Uniform Resource Name) form according to RFC 2141 [5] and RFC 3061 [6] to give a concrete and unambiguous representation.

But please note that the particular encoding and embedding into a data carrier does NOT necessarily use this representation.

## 2.2	Example: ISO 15459 (ISO/IEC JTC1/SC31)

The OID for data based on ISO 15459-X is {1 0 15459 X}. Some ISO 15459-X specifications define OIDs with more arcs.

In the following example, we look at the unique identifier issued under the rules of GS1 to be presented in the approach of H.IDscheme. Let us use the example in Section A.3 of ISO/IEC 15459-4 [3] to describe the usage of OID. The Application Identifier is “8004”, and the Issuing Agency GS1 has provided the unique identifier issuer with “0098756”, that starts with the Issuing Agency Code “0”, and “000110780” has been assigned by the issuer.

In this case, the Object ID is {1 0 15459 4 2} and individual item of this item is 0 098756 000110780 (Figure 1).

Figure 1 – Unique identifier for GS1 individual asset identification 3

So, the identifier of this individual item is represented as follows in a URN form for discussion according to the approach of H.IDscheme.

urn:oid:1.0.15459.4.2:0098756000110780

## 2.3	Example: EPC

The OID of EPC is not registered. If and when OID is assigned to EPC, H.IDscheme can accommodate EPC.

Meanwhile, an OID of GS1-based data is defined in ISO/IEC 15459 series. So, GS1 identifier can be described using the approach of H.IDscheme’s OID and ID pair in a manner described in 2.2.

## 2.4	Example: Place Identifier (ISO TC211)

Place Identifier is defined in ISO/DIS 19155 [4]. When the specification number of ISO/IEC is X, it seems generally accepted that OID of data based on this specification is written as {1 0 X}. So, in this note, we follow this practice, and use {1 0 19155} as a tentative OID of Place Identifier. (This OID does not seem to have been registered today.)

Normally, Place Identifier is described based on tag URI scheme [7].

The following  Place Identifier

tag:bldgmgt.tokyo.go.jp,2008-03-18:pi:13103-2454543-A1-6893394-12B

can be described as follows in H.IDscheme’s OID and ID pair.

urn:oid:1.0.19155:tag:bldgmgt.tokyo.go.jp,2008-03-18:pi:13103-2454543-A1-6893394-12B

## 2.5	Example: IPv6

The OID of IPv6 is not registered. If and when OID is assigned to IPv6, H.IDscheme can describe IPv6 identifier.

## 2.6	Example: UUID

The OID of UUID [8] is {2 25}.

For example, a UUID “550e8400-e29b-41d4-a716-446655440000” is described as follows in H.IDscheme’s OID and ID pair.

urn:oid:2.25:550e8400-e29b-41d4-a716-446655440000

# 3	Storing an identifier with OID a la H.IDscheme in ISO/IEC Standard-compliant RFID Tags

The following sections explain how to store an identifier with OID a la H.IDscheme into standard-compliant tags.

Many such tags have associated data presentation standards that define the encoding and embedding methods to store the data into such tags. We follow the standards when such standards are available.

## 3.1	General storing method and format for ISO/IEC compliant tags

In H.IDscheme, we use Object Identifier (OID) as a header of H.IDscheme data (identifier). Memory format is defined for each ISO/IEC standard tag. We adopt the standard format to store identification information a la H.IDscheme, i.e., a pair consisting of an OID and an identifier. (In the following we may simply say “H.IDscheme” to mean identification information of a pair of an OID an identifier a la H.IDscheme)

We can store H.IDscheme into ISO/IEC standard-compliant tags that support DFSID (Data Storage Format Identifier).

## 3.2	Example: Storing H.IDscheme into ISO/IEC 18000 RFID tags (ISO/IEC JTC1/SC31)

ISO/IEC 15962 [1] is an international standard to store OID and object to RFID memory. Root OID encoded method is one of the data formats, and we select the method to store H.IDscheme. In the root OID encoded method, you will define root OID, which means that a prefix of OID for each object, and relative OID which means remaining OID.

In addition, we adopt non-directory structure as access method. The non-directory structure is the simplest memory structure. This consists of a repeating cycle of Precursor, formatted ObjectId and compacted data object.

In ISO/IEC 15962, DSFID (Data Storage Format Identifier) is needed to define the data format and the access method. In the standard, Non-directory structured Logical Memory with root-OID explicitly encoded is assigned 2. Hence, you need to set DSFID to 2.

In the following, we look at the case of  the storage method of “ucode” [14], a code system that is very similar to H.XXX code, in order  to illustrate how a code can be stored in ISO/IEC 18000 RFID tags in a manner according to H.IDscheme.

### 3.2.1	The case of storing of ucode under {0 2 440 200239 2}

Currently, {0 2 440 200239 2} is used as OID of ucode, which means that {0 2} is ITU-T, {440} is given to Japan by ITU-T, {200239} is given to T-Engine Forum (an NPO that runs uID center) by the Japanese Government (the Ministry of Internal Affairs and Communication), and the final arc {2} is given to ucode by uID center (T-Engine Forum).

Table 1: Object ID and Object value

| ObjectId |  |  | Object |  |  |
| --- | --- | --- | --- | --- | --- |
| Class Tag | Length | ObjectId | Class Tag | Length | Object |
| 0x06 | 0x07 | {0, 2, 440, 200239, 2} = {0x02, 0x83, 0x38, 0x8C, 0x9C, 0x2F, 0x02} | 0x04 | 0x10 | 128-bit ucode |

The following is a data format in root OID encoded method.

Precursor (root-OID), OID					(1)

Precursor, Relative-OID, 0x10(length), (ucode)	(2)

Terminator (0x00)						(3)

In (1), you need set a precursor and a root OID. In this example, we use { 0 2 440 200239 } as the root OID. In ISO/IEC 15962 [1], the value of OID is encoded into the following binary data {0x02, 0x83, 0x38, 0x8C, 0x9C, 0x2F}. Precursor of root OID is defined as follows. (It is described in 8.3.4 of ISO/IEC 15962)

Bit 8:	the offset and expansion bit

if bit 8 = 0	no offset is present

if bit 8 = 1	an additional octet follows as part of the Precursor

Bits 7 to 1: 	the length of the root-OID (in octets) between 1 to 126

bbbbbbb = length of the root-OID

In this example, the length of the root OID is 6 octets and then precursor is 0x06. Precursor of (2) is defined as follows. (It is described in 8.3.3 of ISO/IEC 15962.)

Bit 8: 	the offset and expansion bit

if bit 8 = 0	no offset is present

if bit 8 = 1	an additional octet follows as part of the Precursor

Bits 7 to 5: the Compaction Type Code

Bits 4 to 1: used to directly encode some RELATIVE-OID values

Values 0001 to 1110(b) shall encode a RELATIVE-OID that has only one arc and that arc has a value 1 to 14

Value 1111 indicates that the RELATIVE-OID, OBJECT IDENTIFIER or root- OID is directly encoded in subsequent bytes

In this example, compaction type code is octet strings (110), and relative OID is 0x02. The relative OID is smaller than 1110(0xb), so the precursor is 0x62. Finally, we will get the data as follows.

0x06, 0x02, 0x83, 0x38, 0x8C, 0x9C, 0x2F, 0x62, 0x10, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0x00

where the series of 0xXX denotes the value of ucode (the value of the identifier in ucode). Note that type of object 0x04 is omitted because the type of object is always 0x04.

The specification of memory format for ISO/IEC 18000-6 Type C is currently under discussion at a working group of ISO/IEC JTC1. In the draft of ISO/IEC 15962: 2011, there are two ways to store DSFID. One is using Memory Bank 01, and the other is using Memory Bank 11. Memory Bank 01 is used for UII (Unique Item Identifier). However, possible application of  ucode covers not only item but also any kinds of real world entities such as place. Hence, we use Memory Bank 11: user memory to store H.IDscheme format object for ucode. DSFID is encoded in the first octet position of this memory bank (i.e. bits position 0016 to 0716). Precursor, OID, and object are encoded from the second octet position.

### 3.2.2	The case of storing of ucode under NID OID {2 27}

Alternatively {2 27 2} can be used as short OID of ucode. {2 27} has been designated as NID OID and to be used for short OID for such application [15].

The meaning of the  components of {2 27 2} are as follows: {2} is joint ISO and ITU-T, {27} is Tag-based identifications (identification mechanisms for which identifiers are stored in a memory-limited area of a tag like an RFID, 1D or 2D barcode, etc.), and the final arc {2} is registered for ucode. This is registered in NID OID database. ObjectID {2 27 2} is encoded to {0x6B 0x02}.   In this case, the length of the root OID is 1 octet and then precursor is 0x01. Object “ucode” is encoded using the same manner explained above. Finally, we will get the data as follows.

0x01, 0x6B, 0x62, 0x10, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0xXX, 0x00

## 3.3	Example: 2D Barcode (QR code) (ISO/IEC JTC1/SC31)

There is no current standard for storing a pair of OID and related value in QRcode. When ISO/IEC defines the rule to store a pair of OID and value in QR code, we will follow the rule to store H.IDscheme in QR code.

## 3.4	Example: NFC tags (NFC forum)

NFC Forum defines NDEF (NFC Data Exchange Format) [9] to store data in NFC tags. In NDEF, there are several formats to store data. In this example, we show URI (Uniform Resource Identifier) Record. URI consists of URL (Uniform Resource Locator) and URN (Uniform Resource Name).

Let us take a look at the storing method of ucode in NFC tags.

By expressing OID using URN, the format of ucode urn can be described as follows.

urn:oid:0.2.440.200239.2:<ucode>

Note that OID {0 2 440 200239 2} is the OID given to ucode of T-Engine Forum. <ucode> is a 128-bit ID number. In NDEF format, we use the short record format to store URN of H.IDscheme. TLV (Type, Length, and Value) format is adopted in NDEF [10][11].The  first octet indicates NDEF message, and second octet indicates length of value. Third octet is NDEF header. MB (Message Begin) indicates start of a message. ME (Message End) indicates end of a message. CF (Chunk Flag) indicates that payload data is chunked. IL (ID Length presented) indicates existence of ID length field if the value is 1.  TNF (Type Name Format) indicates payload format. In this example, we select NFC Forum well-known type and URI record [12][13]. So, Type ID is “U”.

Table 2 shows URI record format. In URI record, identifier code is first, and next N-octet is URI field. We use URN as a URI identifier code in this example. The value is 0x13.

Table 2: URL Record Contents

| Name | Offset | Size | Value |
| --- | --- | --- | --- |
| Identifier code | 0 | 1 octet | URI Identifier Code |
| URI Field | 1 | N octets | UTF-8 string |

Table 3: Example of data format of ucode that follows H.IDscheme using NDEF

| Data | Content | Length | TLV | NDEF Message | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | 0x03 | 1 | T |  | NDEF Message |
| 1 | 0x2B | 1 | L |  | Length is 60 octets. |
| 2 | 0xD1 | 1 | V | NDEF header | MB=1, ME=1, CF=0, SR=1, IL=0 TNF=0x01(Well Known Type) |
| 3 | 0x01 | 1 |  | Type record length | Type record is only 1 octet. |
| 4 | 0x36 | 1 |  | Payload length | Payload is 54 octets. |
| 5 | 0x55 | 1 |  | Type record name | Type record = “U” (URI) |
| 6 | 0x13 | 1 |  | Payload | (0x13) = “urn:” |
| 7-60 |  | 53 |  |  | “oid:0.2.440.200239.2:<ucode>” |
| 61 | 0xFE | 1 | T |  | Terminator (LV is not necessary in terminator.) |

# 4	Future Work

This technical note summarizes the current theory and practice of storing identifiers of existing legacy code systems into standard-compliant data carriers according to the approach of H.IDscheme.

Once H.IDscheme is adopted as Recommendation, the newly proposed identification scheme (tentatively called H.XXX code) in H.IDscheme can be stored in standard-compliant tags in a similar fashion as ucode is stored today in the examples above (but uses different OID, of course.)

We will follow the progress of various standards to make sure unambiguous encoding and embedding in standard-compliant manner will be used widely.

# 5	Conclusion

Unambiguous standard-compliant method can be followed for embedding OID and its related value into standard-compliant data carriers. This assures that H.IDscheme once adopted should face no difficulties in having the required identification information  stored in standard-compliant data carriers such as ISO-standard RFID tags without causing any confusion on the part of  the applications

# References

- ISO/IEC 15961 Information technology — Radio frequency identification (RFID) for item management — Data protocol: application interface, 2004.

- ISO/IEC 15962 Information technology — Radio frequency identification (RFID) for item management — Data protocol: data encoding rules and logical memory functions, 2004.

- ISO/IEC 15459-4. Information technology — Unique Identifiers — Part 4 : Individual items, 2008.

- ISO/IEC 19155. Geographic information -- Place Identifier (PI) architecture, 2011-08-09.

- R. Moats. URN Syntax. RFC 2141, 1997.

- M. Mealling. A URN Namespace of Object Identifiers, RFC 3061, 2001.

- T. Kindberg. The 'tag' URI Scheme. RFC 4151, 2005.

- ISO/IEC 11578. Information technology -- Open Systems Interconnection -- Remote Procedure Call (RPC), 1996.

- NFC Forum, NFC Data Exchange Format (NDEF) Technical Specification, NDEF 1.0, NFCForum-TS-NDEF_1.0, 2006-07-24.

- NFC Forum, Type 1 Tag Operation Specification Technical Specification, NFCForum-TS-Type-1-Tag_1.0, 2007-07-09.

- NFC Forum, Type 2 Tag Operation Technical Specification, NFCForum-TS-Type-2-Tag_1.0, 2007-07-09.

- NFC Forum, NFC Record Type Definition (RTD) Technical Specification, RTD 1.0, NFCForum-TS-RTD_1.0, 2006-07-24.

- NFC Forum, URI Record Type Definition Technical Specification, RTD-URI 1.0, NFCForum-TS-RTD_URI_1.0, 2006-07-24.

- uID center “Ubiquitous Code: ucode” Version 1.A0.10 (2009)

- INTERNATIONAL STANDARD ISO/IEC 9834-9 RECOMMENDATION ITU-T X.668 Information technology – Open Systems Interconnection – Procedures for the operation of OSI Registration Authorities: Registration of object identifier arcs for applications and services using tag-based identification

__________________