| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | TD-16b |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor F.TPS-Reqs |  |  |  |
| Title: | F.TPS-Reqs “Definitions, requirements, and use cases for Telepresence Systems” (New): Output draft |  |  |  |

This document contains the editor’s output draft for F.TPS-Reqs “Definitions, requirements, and use cases for Telepresence Systems”.

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| F.TPS-Reqs Ed. 0.1 | TD-45a – Initial baseline text. Output from Andover meeting July 2011. |
| F.TPS-Reqs Ed. 0.2 | TD 649/WP2 (2011-11) – Input text to the November/December ITU-T SG16 meeting. It is a copy of the output of the Boston meeting with changes marks accepted. |
| F.TPS-Reqs Ed. 0.3 | Output text from the November/December ITU-T SG16 meeting. It incorporates changes from contributions to F.TPS-Reqs: C.654,655,656,711,744,745,750,751,756,757 |
| F.TPS-Reqs Ed. 0.4 | Input text to the November/December ITU-T SG16 meeting. It is a copy of the output of the Geneva 2012-12 meeting with changes marks accepted. |
| F.TPS-Reqs Ed. 0.5 | Output text from the ITU-T WP2/SG16 Rapporteurs meeting (Feb 2012). It incorporates changes from contributions to F.TPS-Reqs: AVD-4203,AVD-4213 |

Living List Items:

- User-xx: The telepresence system shall provide unified (standardized) user operating procedures for end-users to operate it, as for basic features (i.e. open/join and close/leave a multipoint conference).

{Editor’s note: This is not about user interface. Solicit contributions on what functions and operating procedures are needed.}

CONTENTS

1	Scope	4

2	References	4

3	Definitions	5

3.1	Terms defined elsewhere	5

3.2	Terms defined in this Recommendation	5

4	Abbreviations and acronyms	6

5	Conventions	6

6	Scenarios and Use cases	6

6.1	Use Case 1: Symmetric point-to-point meeting	6

6.2	Use Case 2: Asymmetric point-to-point meeting	7

6.3	Use Case 3: Multipoint meeting	7

6.4	Use Case 4: Presentation	7

6.5	Use Case 5: Heterogeneous systems	8

6.6	Use Case 6: Multipoint education usage	9

6.7	Use case 7: Multipoint multiview (Virtual space)	10

6.8	Use case 8: Legacy Interworking	10

7	User experience requirements	11

8	Functional requirements	12

8.1	Call control functions	12

8.2	Media control functions	12

8.3	Conference control functions	12

8.4	Collaboration functions	13

8.5	Other functions	13

9	Interoperability requirements	13

9.1	Interoperation with traditional systems	13

9.2	Interoperation between telepresence systems	13

10	Network requirements	14

11	Security requirements	14

Annex A  <Annex Title - placeholder>	14

List of Tables

Page

No table of figures entries found.

List of Figures

Page

No table of figures entries found.

Draft new Recommendation F.TPS-Reqs

Definitions, requirements, and use cases for Telepresence Systems

Summary

<Mandatory material>

Keywords

<Optional>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

1	Scope

This Recommendation provides definitions, use cases and functional requirements for telepresence systems. Specific requirements include those for:

- Call control functions

- Media control functions

- Conference control functions

- Collaboration

- Inter-operability

- Network

- Security

The requirements in this Recommendation relate to functions that a Telepresence system shall support. That is, a Telepresence system should be able to provide the functions in its design or by utilising existing mechanisms. It does not mean that the requirement must be implemented. The requirements do not mandate the deployment of a particular function. For example: Whilst a telepresence system should support mechanisms for assured QoS, it may be deployed to a network where these are not needed.

Unless the requirement mentions a specific function or device the requirements listed in this Recommendation apply to a Telepresence system as a whole. The inclusion of a requirement does not necessarily mean that something new is needed for the support of Telepresence. For example whilst there may be a requirement to support QoS for Telepresence, ultimately existing QoS mechanisms may be utilised for this.

2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.239]	ITU-T Recommendation H.239 (09/05), Role management and additional media channels for H.300-series terminals.

[ITU-T H.241]	ITU-T Recommendation H.241 (05/06), Extended video procedures and control signals for H.300-series terminals.

[ITU-T H.243]	ITU-T Recommendation H.243 (10/05), Procedures for establishing communication between three or more audiovisual terminals using digital channels up to 1920 kbit/s.

[H/F.TPS-Arch]	ITU-T Recommendation X.yyy (date), Telepresence System Architecture.

[H.TPS-AV]	ITU-T Recommendation X.yyy (date), Audio/Video Parameters for Telepresence systems.

[IETF CLUE REQ]	IETF draft-ietf-clue-telepresence-requirements-01, Requirements for Telepresence Multi-Streams.

[IETF CLUE UCS] IETF draft-ietf-clue-telepresence-use-cases-02, Use Cases for Telepresence Multi-streams.

3	Definitions

<Check in the ITU-T Terms and definitions database on the public website whether the term is already defined in another Recommendation. It may be more consistent to refer to such a definition rather than redefine it>

3.1	Terms defined elsewhere

<Normally terms defined elsewhere will simply refer to the defining document. In certain cases, it may be desirable to quote the definition to allow for a stand-alone document>

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]: <optional quoted definition>

3.1.2	<Term 2> [Reference]: <optional quoted definition>

3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Actual size: A rendered figure in a display is the same size as if the person is in the room.

3.2.2	Natural response:  Interaction between local and remote participants that is not impeded by the delay introduced by the telepresence system.

3.2.3	Telepresence: An interactive audio-visual communications experience between remote locations, where the users enjoy a strong sense of realism and presence between all participants by optimizing a variety of attributes such as audio and video quality, eye contact, body language, spatial audio, coordinated environments and natural image size.

4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| 3D | Three Dimensional |
| --- | --- |
| IPv4, IPV6 | Internet Protocol Version 4, Version 6 |
| MCU | Multipoint Control Unit |
| NAT | Network Address Traversal |
| PC | Personal Computer |
| QoS | Quality of Service |

5	Conventions

None.

6	Scenarios and Use cases

The use cases in this clause illustrate various telepresence configurations for the purposes of identification of requirements. These configurations apply to scenarios where telepresence is in use and do not include legacy systems (i.e. non-telepresence systems) unless specifically indicated in the use case. They do not imply the need for mandatory support of any particular configurations.

6.1	Use Case 1: Symmetric point-to-point meeting

In this case each of the two sites has an identical number of screens, with cameras having fixed fields of view, and one camera for each screen.  The number of screens or cameras is not relevant so long as both sites have the same number. The sound type is the same at each end. Figure 1 illustrates the use case.

Two-screen Telepresence Endpoint

Two–screen Telepresence Endpoint

IP Network

Figure 1 – Use Case 1: Symmetric point to point meeting

For further information see clause 3.1 / [IETF CLUE UCS].

6.2	Use Case 2: Asymmetric point-to-point meeting

In this case, each site has a different number of screens and cameras than the other site. The number of screens or cameras is not relevant so long as both sites have a different number. Figure 2 illustrates the use case.

Two-screen

Telepresence Endpoint

IP Network

Three-screen-- ScreenRec mean that ITU-T Q.nerein.

Telepresence Endpoint

Figure 2 – Use Case 2: Asymmetric point to point meeting

For further information see clause 3.2 / [IETF CLUE UCS].

6.3	Use Case 3: Multipoint meeting

In this use case of a multipoint telepresence conference, there are more than two sites participating. Figure 3 illustrates the use case.

For further information see clause 3.3 / [IETF CLUE UCS].

Two-screen

Telepresence Endpoint

IP Network

Three-screen

Telepresence Endpoint

Three-screen

Telepresence Endpoint

Figure 3 – Use Case 3: Multipoint meeting

6.4	Use Case 4: Presentation

This use case builds on the previous use cases that in addition to the video and audio streams showing the participants, additional streams are used for presentations. For example the additional stream may be for content sources such as: document cameras, electronic whiteboards, PC or instant messaging. Figure 4 illustrates the use case.

Two-screen

Telepresence Endpoint

IP Network

Three-screen

Telepresence Endpoint

Presentation Capture

Presentation

Display

Presentation Capture

Presentation

Display

Three-screen

Telepresence Endpoint

Two-screen

Telepresence Endpoint

IP Network

Three-screen

Telepresence Endpoint

Figure 4 – Use Case 4: Presentation

For further information see clause 3.4 / [IETF CLUE UCS].

6.5	Use Case 5: Heterogeneous systems

This use case illustrates the common meeting scenario for people to join the conference from a variety of environments, using different types of endpoint devices.  In a multi-screen immersive telepresence conference may include someone on a PC-based video conferencing system, a participant calling in by phone, and someone on a handheld device. This use case assumes that the different endpoint devices are able to participate in signalling related to Telepresence systems. The underlying technology does not determine whether a device is “legacy” or not, the deciding factor is whether it supports telepresence related protocols (or protocol extensions). Legacy systems that do not understand telepresence related protocols (or protocol extensions) are not in scope.  Figure 5 illustrates the use case.

-

Two-screen

Telepresence Endpoint

IP Network

Three-screen

Telepresence Endpoint

Presentation Capture

Presentation

Display

Presentation Capture

Presentation

Display

Three-screen

Telepresence Endpoint

Personal Computer

Mobile Phone

Figure 5 – Use Case 5: Heterogeneous systems

For further information see clause 3.5 / [IETF CLUE UCS].

6.6	Use Case 6: Multipoint education usage

This use case deviates from other use cases in that multiple video streams are not used to create an immersive conferencing experience with panoramic views at all the site.  Instead the multiple streams are dynamically used to enable full participation of remote students in a university class.  In some instances the same video stream is displayed on multiple displays in the room, in other instances an available stream is not displayed at all. Figure 6 illustrates the use case.

Figure 6 – Use Case 6: Multipoint education usage

For further information see clause 3.6 / [IETF CLUE UCS].

6.7	Use case 7: Multipoint multiview (Virtual space)

This use case describes a virtual space multipoint meeting with good eye contact and spatial layout of participants. Virtual space multipoint as defined here assumes endpoints with  multiple cameras and displays.  Usually there are the same number of cameras and displays at a given endpoint.  A camera is positioned above each display.  A key aspect of virtual space multipoint is the details of how the cameras are aimed.  The cameras are each aimed on the same area of view of the participants at the site.  Thus each camera takes a picture of the same set of people but from a different angle. Figure 7 illustrates the use case.

Figure 7 – Use Case 7: Multipoint multiview (virtual space)

For further information see clause 3.7 / [IETF CLUE UCS].

NOTE - This use case is not for 3D stereoscopic view.

6.8	Use case 8: Legacy Interworking

This use case describes a case where a legacy videoconferencing endpoint (or other heterogeneous device such as voice only endpoint) is involved in the telepresence session. The legacy endpoint does not understand additions related to telepresence functionality and interworking is required to communicate between the telepresence and legacy endpoints. Figure 8 illustrates the use case.

Figure 8 – Use Case 8: Legacy Interworking

This use case may also utilise other legacy equipment such as a legacy H.323 MCU in order to participate in a conference.

7	User experience requirements

{Editor’s Note: We need to consider if a system perspective is required, in other words use “system requirements” to capture “user experience requirements”. We also need to consider the difference between performance requirements and functional requirements, and if user experience in terms of performance metrics is out of scope.  Further contributions are needed to describe these requirements more precisely, and to limit them to not require detailed performance metrics which may not be available in the near-term. Treat the following as examples for now.

User01: The telepresence system should support actual size high-definition (includes 1080p, 1080i, etc.) imaging of videoconferencing participants at the remote end. The support of actual size is considered an advanced Telepresence feature.

{Editor’s Note: The supported HD modes need further consideration.}

User02: The telepresence system should provide a natural eye contact experience.

User03: The telepresence system should support a means of locating the speaker in a remote meeting room so that local participants can readily identify the position of different remote speakers.

User04: The telepresence system shall enable synchronization of audio and video from the remote location. Synchronization includes “lip-sync” and the sequencing of multiple media streams.

User05: The telepresence system should provide a control function to allow a user to perform an operation with a minimum of input thus enabling convenient centralized management and one-touch smart control capabilities.

User06: The telepresence system should provide harmonized environment for assuring immersive presences of remote users. Typically in order to provide immersive and realistic experiences to users, some environmental factors are aligned. These environmental factors for example include: interior design such as shape of desk, wall colour, illumination, position of light, etc.

User07: The telepresence system should support multiple video sources and multiple displays in each meeting room. The overlap of the scene from neighbouring cameras should be minimized. The output from the neighbouring cameras should provide a seamless view of the conference participants.

User-08: The telepresence system shall provide natural response on facial expression and conversational voice.

User09: The telepresence system should support the receiver end-user switching of display position.

8	Functional requirements

8.1	Call control functions

Cal01: The telepresence system shall support interoperability with other devices that send or receive a different number of media streams.

Cal02: The telepresence system shall support the description, identification and negotiation of appropriate information to enable spatially coordinated rendering of audio and video streams.

8.2	Media control functions

Med001: The telepresence system shall support communication regardless of different aspect ratio.

Med002: The telepresence system shall support a method for describing the spatial arrangement of audio and video stream in a manner which enables a satisfactory reproduction at the receiver in a spatially correct manner. Furthermore it shall support a method to enable a satisfactory spatial matching between audio and video streams coming from the same endpoints.

This requirement encompasses the requirements REQMT-1, REQMT-2 and REQMT-3 as defined in [IETF CLUE REQ].

Med003: Where endpoints send and receive different numbers of media streams the telepresence system shall provide interoperability between them. This requirement encompasses requirement REQMT-9 as defined in [IETF CLUE REQ].

Med004: The telepresence system optionally support 3D multimedia stream.

Med005: The telepresence system optionally supports a means for describing 3-Dimensional multimedia stream.

{Editor’s Note: Inclusion of 3D requirements depends on appropriate contributions to support them.}

Med006: The telepresence system should be able to add additional media streams, and remove existing media streams during the telepresence session.

Med007: The telepresence system should provide a suitable range of video frame rates, resolutions, and bit rates for presenting the shared data For example those provided by ITU-T H.239, ITU-T H.241.

8.3	Conference control functions

{Editor’s Note:  These are examples for now, that will need further discussion.}

Con01: The telepresence system shall enable switching the video from the whole of the remote meeting rooms to another meeting room, or from partial images to another image.

Con02: The telepresence system shall enable the display of all remote participants at once, including remote participants (if any) from non-telepresence systems.

Con03: The telepresence system shall enable the conference chair person to select and view video from a given meeting room or from partial seats of a meeting room.

Con04: The telepresence system shall support adding or removing an endpoint to an ongoing telepresence session. For example such as the procedures described  by ITU-T H.243.

Con05:  A telepresence system should support far-end camera control (FECC), providing means to specify the capture point of each movable camera.

8.4	Collaboration functions

Col001: It is optional for a telepresence system to provide a means of collaboration such as remote presentation, instant messaging, online documentation, whiteboard and file transferring based on existing protocols.

Col002: The telepresence system shall support a captioning function.

8.5	Other functions

This sub-clause contains requirements that are not related to the other sub-clauses in clause 8.

Other01: The telepresence system shall be able to provide various statistical information which includes but is not limited to media type, maximum number of endpoints, duration, and data volume of each media type.

{Editor’s note: we need more contributions on the precise list for reporting.}

9	Interoperability requirements

9.1	Interoperation with traditional systems

This clause presents requirements for interoperation with legacy devices/systems. When used in a conference with Telepresence system these legacy devices may not perceive telepresence.

Int01: The telepresence system shall support that both traditional videoconferencing terminals and telepresence terminals can participate in a same conference.

Int02: The telepresence system should support that a traditional videoconferencing system can connect to the telepresence system in multiple cascaded conference mode.

Int03: The telepresence system should allow audio only legacy devices to participate in a conference.

9.2	Interoperation between telepresence systems

This clause presents requirements for interoperation with devices/systems that have been enhanced for telepresence.

Int03: The telepresence system shall enable audio and video terminals based on different open standards to participate in the same conference.

Int04: It shall be possible for telepresence systems based on different open standards to participate in the same conference and experience telepresence. For example: A telepresence enhanced SIP endpoint and a telepresence enhanced H.323 endpoint via control signalling gateway.

10	Network requirements

Net001: The telepresence system should enable the reservation network resource for assuring QoS for the telepresence session.

Net002: The telepresence system should enable adaptation of the quality of media based on the network status. For example: Under best-effort network such as Internet which cannot provide strictly guaranteed network QoS, the telepresence system may need to manipulate its video quality based on network status.

Net003: The telepresence system should provide a means to traverse NATs or firewalls automatically to assure connectivity of telepresence services with minimal user operation.

{Editor’s Note:  This requirement needs more discussion.}

Net004: The telepresence system shall support both IPv4 and IPv6.

{Editor’s Note: This requirement needs more discussion. Use cases would be helpful.}

11	Security requirements

Sec001: The telepresence system is suggested to support of security mechanism for securing sessions.

Sec002: The telepresence system is suggested to support of negotiation mechanism for making a secured session.

Sec003: The telepresence system is suggested to support both signal security and media security.

Sec004: The telepresence system shall be able to authenticate each end point.

Annex A

<Annex Title - placeholder>

(This annex forms an integral part of this Recommendation)

<Body of annex A>

__________________