| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | TD-30 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editors |  |  |  |
| Title: | H.VSprot “Signalling and Protocols for visual surveillance”: Output draft |  |  |  |
| Purpose: | Information |  |  |  |

This TD contains the version of the latest draft new ITU-T Rec. H.VSprot “Signalling and Protocols for visual surveillance” as output of the ITU-T WP2/SG16 Rapporteurs meeting which was held in Geneva, 20~24 Feb, 2012.

Introduction

According to the candidate serial work items and possible roadmap discussed in the ITU-T WP2/16 Rapporteur meeting (Geneva, 26 June 2009), in the stage 1 and 2, we plan to study the requirements and architecture of visual surveillance. Then, we come into stage 3 to start the protocol work item.

Recommendation H.VSprot focuses on the detailed description of reference points, signalling and protocols based on the overall architecture defined in the ITU-T Recommendation H.VSarch.

The purpose of this new Recommendation is trying to define detailed technical characteristics for each reference points.

For the reference points between different functional entities, it needs to identify relevant protocol, such as protocols between PU and service platform, CU and service platform, PU and CU.

For the reference points in the same functional entities, identifies the internal signalling, for example defining signalling flow or information elements relevant for each reference point, such as the signalling for service control flow, media distribution flow in the service platform.

Furthermore, this recommendation will utilize and extend some existing protocols, such as SIP/RTP/RTSP/HTTP, etc. and it should give the message format as well as message text.

B.1.	Candidate serial work items

The candidate serial work items for deliverables could be:

| No. | Series | Work Item | Questions | Type | Documents | Stage |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Requirements and Service Descriptions | F.Vsreqs& H.Vsarch | Q22/16 | Standards | Rec. | Stage 1 |
| 2 | Architecture | H.Vsarch | Q21/16 | Standards | Rec. | Stage 2 |
| 3 | Protocol | H.Vsprot | Q21/16 | Standards | Rec. | Stage 2 |
| 4 | Architecture for mobile VS system | H.Vsmarch | Q21/16 | Standards | Rec. | Stage 2 |
| 5 | Customer Unit | H.Vscu | Q21/16 | Specifications | Rec. | Stage 3 |
| 6 | Premises Unit | H.Vspu | Q21/16 | Specifications | Rec. | Stage 3 |
| 7 | Service Platform | H.Vssp | Q21/16 | Specifications | Rec. | Stage 3 |
| 8 | Security | H.Vssec | TBD (SG16,17) | Standards | Rec. | Stage 4 |
| 9 | QoE | H.Vsqoe | TBD (SG12, 16) | Standards | Rec. | Stage 4 |
| 10 | QoS | H.Vsqos | TBD (SG12& 16) | Standards | Rec. | Stage 4 |
| 11 | Management | H.Vsman | TBD | Standards | Rec. | Stage 4 |
| 12 | Codecs | H.Vscodec | Q6/16 | Specifications | Rec. | Stage 5 |
| 13 | Interworking | H.Vsiwk | Q21/16 | Guideline | Supp. | Stage 5 |
| 14 | Convergence | H.Vsconv | Q21/16 | Guideline | Supp. | Stage 5 |
| … |  |  |  |  |  |  |

B. 2 – Document history

| No. | Date | Source | Action | Status | Cause |
| --- | --- | --- | --- | --- | --- |
| #01 | 2010-03-12 | Editors of H.VSprot | Draft baseline document framework from AVD-3907 of WP2/16. | Draft | Initial draft version |
| #02 | 2010-07-30 | Editors of H.VSprot | Update with some new material and do some modifications, such as: 1) Modifications for some editorial changes, references; 2) Adding the detailed structure of clause 6 which is the main part for signalling and protocols; 3) Adding materials for the signalling and protocols on CU/PU registration, media playback, Video/audio acquisition; | Update draft | Update draft version |
| #03 | 2010-12-03 | Editors of H.VSprot | Update with some new material concerning the signalling and protocols, such as: - PU list obtaining - Voice communication - Recording in PU | Update draft | Update draft version |
| #04 | 2011-03-25 | Editors of H.VSprot | Update with some new material concerning the signalling and protocols, such as: - Rcording in platform - PU-related Parameters Querying | Update draft | Update draft version |
| #05 | 2011-07-22 | Editors of H.VSprot | Update with some new material concerning the signalling and protocols for instant image snapshot service. | Update draft | Update draft version |
| #06 | 2011-11-21 | Editors of H.VSprot | Update with some new material concerning the signalling and protocols for voice/audio broadcasting service. | Update draft | Update draft version |
| #07 | 2012-02-24 | Editors of H.VSprot | Update with some new material concerning the signalling and protocols for far-end camera control. | Update draft | Update draft version |

CONTENTS

1	Scope	5

2	References	5

3	Definitions	6

3.1	Terms defined elsewhere	6

3.2	Terms defined in this Recommendation	6

4	Abbreviations	6

5	Protocol Reference Points	6

6	Signalling and Protocols	7

6.1	Service management	7

6.1.1	PU registration	7

6.1.2	CU registration	8

6.1.3	Accounting and authorization	8

6.1.4	PU list obtaining	8

6.1.5	Configuration	9

6.1.6	PU-related parameters querying	9

6.2	Alarming and linkage action (ALA)	10

6.3	Video/audio acquisition (VAA)	10

6.3.1	Media acquisition transferred by service platform	11

6.3.2	Media acquisition directly from PU	12

6.4	Instant image snapshot (IIS)	13

6.5	Recording and playback (RnP)	15

6.5.1	Recording	15

6.5.2	Playback	17

6.6	Voice (audio) communication	20

6.6.1	Voice communication	20

6.6.2	Voice/audio broadcast	22

6.7	Far-end Camera Control (FECC) 	24

Draft new ITU-T Rec. H.VSprot

Signalling and Protocols for visual surveillance

AAP Summary

[To be provided before Consent]

Summary

Recommendation H.VSprot will focus on the detailed description of reference points, signalling and protocols based on the overall architecture defined in the Recommendation H.VSarch.

For the reference points between different functional entities, it need to identifies relevant protocol, such as protocols between PU and service platform, CU and service platform, PU and CU.

For the reference points in the same functional entities, identifies the internal signaling, for example defining signaling flow or information elements relevant for each reference point, such as the signaling for service control flow, media distribution flow in the service platform.

Furthermore, this recommendation will utilize and extend some existing protocols, such as SIP/RTP/RTSP/HTTP, etc. and it should give the message format as well as message text.

Keywords

Visual Surveillance, Signalling, Control Protocols

# 1	Scope

The Visual Surveillance service is a telecommunication service focusing on video (audio included also) application technology, which is used to capture remotely multimedia (such as audio, video, image and various alarm signal) and present them to the end users with friendly manner, based on managed broadband network with ensured quality, security and reliability ensured.

The objective of this document is to describe the detailed specification of reference points, signalling and control methods, overall protocols of visual surveillance system based on the functional architecture defined in the ITU-T Recommendation H.VSarch.

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T F.743]	Recommendation ITU-T F.743 (2009), Requirements and service description for visual surveillance

[ITU-T H.VSARCH]	Recommendation ITU-T H.626 (2011), Architectural requirements for visual surveillance

[IETF RFC 2326]	IETF RFC 2326 (1998), Real Time Streaming Protocol (RTSP)

[IETF RFC 2616]	IETF RFC 2616 (1999), Hypertext Transfer Protocol -- HTTP/1.1

[IETF RFC 3261]	IETF RFC 3261 (2002), SIP: Session Initiation Protocol.

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]: <optional quoted definition>

3.1.2	<Term 2> [Reference]: <optional quoted definition>

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	<Term 3>: <definition>

# 4	Abbreviations

# 5	Protocol Reference Points

Recommendation [ITU-T H.VSARCH] describes overall architecture framework of Visual Surveillance as the following figure, and it also introduces reference points of VS architecture. This document gives more detailed description of those reference points as well as some newly defined reference points.

Figure 5-1 –Functional Architecture Framework of Visual Surveillance

# 6	Signalling and Protocols

[NOTE – This section will provide the identification of signalling or protocols.]

The following is some illustrative items:

The SIP message format should comply with the definition in RFC3261;

The RTSP message format should comply with the definition in RFC2326;

The HTTP message format should comply with HTTP1.1 defined in RFC2616;

RTP/RTCP signaling and message refer to RFC3550.

## 6.1	Service management

### 6.1.1	PU registration

The PU registration uses SIP protocol (reference to RFC3261). The request message-header uses standard REGISTER method, and CMU return a response with “401 unauthorized” or “200 OK” message. The Signalling flows are shown in Figure 6-1.

Figure 6-1 PU registration

| STEP 1: | PU uses SIP REGISTER method to implement registration to CMU, the message header (SIP header) contains “From, To, Contract” fields to show the addresses of PU and CMU; |
| --- | --- |
| STEP 2: | CMU returns a response with SIP 401 message and requests PU to send the authentication information; |
| STEP 3: | PU adds authentication information and re-sends SIP REGISTER message to the CMU; |
| STEP 4: | CMU returns SIP 200 OK message or other error code to finish the registration. |

### 6.1.2	CU registration

Figure 6-2: CU registration

CU registration has the same procedural flow and uses the same protocols as PU registration.

### 6.1.3	Accounting and authorization

### 6.1.4	PU list obtaining

After CU and PU register to service platform, CU needs to request to the SCU in service platform to obtain the PU list information and show a list to end users. The information includes the user information, static list information and dynamic list information of PUs.

Figure 6-3: Procedural flows for PU list obtaining

| STEP 1: | CU sends a HTTP GET message with the Request-URI “UserInfo” to SCU to request user information. |
| --- | --- |
| STEP 2, 3: | SCU needs to check the authentication information from CMU. CMU returns a response to SCU. |
| STEP 4: | SCU returns a HTTP 200 OK message with the user information to CU. |
| STEP 5, 6: | CU sends a HTTP GET message with the Request-URI “StaticListInfo” to SCU to request static PU list information. SCU returns a HTTP 200 OK message with the static PU list information to CU. |
| STEP 7, 8: | CU sends a HTTP GET message with the Request-URI “DynamicListInfo” to SCU to request dynamic PU list information. SCU needs to get the latest status of PUs and returns a HTTP 200 OK message as a response to CU. |
| STEP 9, 10: | SCU sends a SIP MESSAGE with the updated PU list information to CU when SCU gets the relevant status changes event. |

### 6.1.5	Configuration

### 6.1.6	PU-related parameters querying

The related parameters of PU devices include:

- Serial port parameters, include serial port numbers, baud rate, data bit rate, parity bit, flow control parameters, PTZ decoder type, PTZ decoder address, etc.

- Input parameters for switching, include channel number, channel state for input and output, etc.

- Clock parameter in PU, used by PU for clock drift correction.

- Video parameters, include video channel ID, encoding mode, encoding type, picture quality level, bit rate, frame rate, video resolution, I-frame interval, etc.;

- Video display parameters, include contrast ratio, brightness, chromaticity, saturation;

- Subtitles parameters, include, position of text (bitmap X/Y), text content, position of time stamp, etc;

- Alarm configuration parameters, specifically including the alarm type, alarm level, and parameters for instant image snapshot, etc;

- PTZ control parameters, specifically include PTZ ID, preset point index, preset point name, track number, PTZ velocity, etc;

- PU status parameters, include CPU utilization, channel status, and alarm status, etc;

- PU logs parameters, specifically include the type of log, query method, query time, etc;

The procedural flows for PU-related parameters querying are described as below:

Figure 6-4: Procedural flows for PU-related Parameters Querying

| STEP 1: | CU sends a HTTP Request message to SCU to request to query PU-related parameters. |
| --- | --- |
| STEP 2: | SCU sends a SIP INFO message to the specific PU to query the parameters. |
| STEP 3: | PU Uses SIP INFO method to return a response to SCU. The SIP INFO message body uses XML format to describe the specific parameters. |
| STEP 4: | SCU returns a HTTP response to CU with the specific parameters requested by the CU. |

## 6.2	Alarming and linkage action (ALA)

## 6.3	Video/audio acquisition (VAA)

The session control uses SIP protocol and the message body uses SDP protocol or XML text，followings are the principles:

The media transmission uses RTP protocol. The ports for media channels are not defined in this recommendation, but they usually are pairs, such as port 7000 is used for RTP and 7001 is then used for RTCP.

### 6.3.1	Media acquisition transferred by service platform

Figure 6-5: Real-time media acquisition transferred by MDU

| STEP 1: | CU sends INVITE message which includes CU media format list and CU address information to SCU, SCU transfers this message to PU. In this step can also handle authentication issue. |
| --- | --- |
| STEP 2: | PU returns a response with 200 OK message which includes media format PU selected and PU address information |
| STEP 3: | SCU modifies SDP information and then forwards 200 OK message to CU. |
| STEP 4: | CU sends ACK message including SDP information to SCU |
| STEP 5: | SCU notifies MDU to start media stream and MDU notifies this event to MSU. |
| STEP 6: | SCU sends the ACK message to PU |
| STEP 7: | SCU modifies SDP information in ACK and forwards it to PU. |
| STEP 8: | Media channels between CU and MDU, MDU and PU are established. PU sends RTP packages of media stream to MDU, and MDU transferred the media stream to CU. RTCP packages are transported between CU and MDU, MDU and PU. |
| STEP 9: | CU sends BYE message to end the session. SCU notifies MDU to stop media stream and MDU notifies this event to MSU, and MSU stops sending media stream to CU. Then SCU transferred the BYE message to PU. |
| STEP 10: | PU stops sending media stream and return a response with 200 OK message to SCU. |
| STEP 11: | SCU transfers the 200 OK message to CU. |

### 6.3.2	Media acquisition directly from PU

Figure 6-6: Real-time media acquisition directly from PU

| STEP 1: | CU sends INVITE message to SCU. In this step can also handle authentication issue. |
| --- | --- |
| STEP 2: | SCU sends the INVITE message to PU. |
| STEP 3: | PU handles the INVITE message and returns 200 OK message including SDP information. |
| STEP 4: | After receiving 200 OK message, SCU selects codec type, modifes SDP and then forwards 200 OK to CU. |
| STEP 5: | CU returns a response with ACK message including SDP answer. |
| STEP 6: | SCU modifies SDP in ACK and sends it to PU. |
| STEP 7: | PU sends RTP packages of media stream to CU. RTCP packages are transported between PU and CU. |
| STEP 8: | CU sends BYE message to end the session and SCU transfers the message to PU. |
| STEP 9: | PU stops sending media stream and returns 200 OK to SCU. |
| STEP 10: | SCU returns the 200 OK message to CU and stops sending media stream. |

## 6.4	Instant image snapshot (IIS)

The Instant Image Snapshot (IIS) function is used to get instant image information based on video from the Surveilled Object.

The following procedural flow involves implementing the snapshot action in the PU which then uploads the snapshot image to the service platform from which the CU gets access to these images.

Figure 6-7: Procedural flows for Instant Image Snapshot

Figure 6-7 shows the high level procedural flows for IIS:

| STEP 1, 2, 3, 4 | CU sends a SIP MESSAGE to SCU to request Instant image snapshot, SCU transfers this message to requested PU. PU handles this request and returns a SIP 200 OK message to SCU. SCU adds an IIS (Instant image snapshot) record in service platform and returns a SIP 200 OK message to CU. |
| --- | --- |
| STEP 5,6: | The PU sends a SIP MESSAGE to the MDU to request for the image upload address. The MDU handles this request and returns a SIP 200 OK message with the upload address to the PU. |
| STEP 7,8: | The PU uploads the image file to the MSU through HTTP POST method with the URL of the upload address. And the MSU returns a HTTP 200 OK message. |
| STEP 9,10: | The MSU sends a SIP MESSAGE to notify the SCU for the completion of image uploading action. The SCU then returns a SIP 200 OK message to the MSU. |
| STEP 11,12: | The SCU sends a SIP MESSAGE to notify the CU for the completion of image uploading action. The CU then returns a SIP 200 OK message to the SCU. |
| STEP 13,14: | The CU uses HTTP GET method to request the MSU to download the snapshot image. The MSU handles this request and returns a HTTP 200 OK message with the image file. |

## 6.5	Recording and playback (RnP)

### 6.5.1	Recording

The Recording function is used to record multimedia (such as audio and/or video) information from Surveilled Object and stored this multimedia information either in PUs or service platform.

The following figure shows the flows of start and stop recording in PU. It uses SIP MESSAGE method and uses XML format to specific parameters.

Figure 6-8: Procedural flows for recording in PU

| STEP 1,2: | CU sends a SIP MESSAGE to SCU to query the recording status of requested PU. After authentication, SCU transfers this message to requested PU. |
| --- | --- |
| STEP 3, 4: | PU returns a SIP 200 OK message to SCU with the recording status of requested PU. SCU handles this response and returns a SIP 200 OK message to CU. |
| STEP 5,6: | CU sends a SIP MESSAGE to SCU to request to start recording in PU with the parameters of recording conditions. SCU handles this request to start a new recording task or change the existed recording task according the new request conditions. Then SCU sends a SIP MESSAGE to PU to request PU to start a recording task. |
| STEP 7,8: | PU returns a SIP 200 OK message to SCU and SCU transfers SIP 200 OK message to CU. |
| STEP 9,10: | CU sends a SIP MESSAGE to SCU to request to stop recording in PU. SCU handles this request to stop the recording task. Then SCU sends a SIP MESSAGE to PU to stop recording. |
| STEP 11,12: | PU returns a SIP 200 OK message to SCU and SCU transfers SIP 200 OK message to CU. |

The recording in platform function is used to record multimedia (such as audio, image, video) information from Surveilled Object and stored in service platform.

The following figure shows the flows of start and stop recording in a service platform (media stored in the MSU). It uses HTTP protocol between CU and SCU and uses XML format to specific the parameters. It uses RTSP protocol between PU and MDU to perform video or audio recording. It uses SOAP protocol among units in the service platform, such as SCU, CMU, MDU and MSU.

Figure 6-9: Procedural flows for recording in platform

| STEP 1: | CU Sets a recording plan in platform and sends HTTP+XML message to SCU. |
| --- | --- |
| STEP 2,3,4: | SCU transfers the plan using SOAP message to CMU, CMU saves and confirms the plan, uses SOAP message to return a response to SCU, and SCU uses HTTP+XML message to return this response to CU |
| STEP 5: | SCU gets a recording event, and uses SOAP message to send a recording request to MDU. |
| STEP 6,7,8: | MDU uses SOAP message to transfer this request to MSU, and MSU uses SOAP message to return the response to MDU, and then MDU returns the response to SCU. |
| STEP 9,10: | Then, the media stream is sent from PU to MDU. After some pre-processing (such as transcoding, creating suitable file format, etc.), the MDU transfers the media data to MSU for storage and management using RTSP protocol. |
| STEP 11,12,13,14: | When the recording task is ended, MSU notifies the MDU the recording result using SOAP message, and the MDU transfers this notification to SCU. After SCU receives the notification, it returns the response to MDU, and MDU transfers the response to MSU. |
| STEP 15,16: | The SCU Uses SOAP message to notify CMU that the task is ended and CMU saves the related logs. The CMU returns a response to SCU to finish the whole recording procedure. |

### 6.5.2	Playback

The Playback function is used to play media resource which had been recorded and stored before.

According to the place of recorded media is stored, there are three different means including storing in platform, storing in PU and storing in CU.

The following shows the procedural flows for playback from PU directly.

Figure 6-10: Procedural flows for playback from PU directly

| STEP 1: | CU sends a HTTP GET request to SCU to get the URL for media resource to play. |
| --- | --- |
| STEP 2: | SCU in the platform return the URL to CU with HTTP response. |
| STEP 3: | CU sends a RTSP DESCRIBE request to PU to get the session description information of PU. |
| STEP 4: | PU returns a response with RTSP 200 OK message, includes SDP information. |
| STEP 5: | CU initiates a RTSP SETUP request to PU, and this request include the following information at least: URL of the requested media resource, Request sequence number, Transport parameters for the CU client. |
| STEP 6: | PU returns a response with RTSP 200 OK message, includes information： Status Line: RTSP/1.0 200OK, Sequence number, Session ID, Transport parameters for the CU client. |
| STEP 7: | CU sends a “RTSP PLAY" request to start getting the media packets, and this request includes the following information: - Request Line, include URL of the requested media resource, - Request sequence number, - Session ID, - Range, this parameter is optional. The RTSP server (here is PU) is recommended to support Fast Forward/Fast Rewind and Random access functions through RTSP PLAY request with standard parameters defined in RFC2326. To implement Forward/Fast Rewind function: use “Scale” and/or “Speed” parameters. A scale value of 1 indicates normal play or record at the normal forward viewing rate. If not 1, the value corresponds to the rate with respect to normal viewing rate. Example of playing in reverse at 4 times normal rate: Scale: -4 The parameter value is expressed as a decimal ratio, e.g., a value of 2.0 indicates that data is to be delivered twice as fast as normal. Example: Speed: 2.0 To implement Random access function: use “Range” parameter to indicate the starting point requested by CU. Example: Range: clock=19960213T143205Z-;time=19970123T143720Z |
| STEP 8: | PU returns a response with RTSP 200 OK message, includes information： Status Line: RTSP/1.0 200 OK, Sequence number, Session ID, Range, RTP-Info：URL of the media resource，first RTP package sequence number, RTP timestamp relative to the time in the header of range. |
| STEP 9: | Sends RTP packages of media data from PU to CU. |
| STEP 10: | RTCP packages between PU and CU, RTCP is optional. |
| STEP 11: | If need to pause the media stream, CU sends RTSP PAUSE request to PU, and this request include the following information: Request Line, include URL of the requested media resource, Request sequence number, Session ID, Range (Optional). |
| STEP 12: | PU returns a response with RTSP 200 OK message, includes information： Status Line: RTSP/1.0 200 OK, Sequence number, |
| STEP 13: | If need to stop this media stream, CU sends RTSP TEARDOWN request，and this request include the following information: Request Line, include URL of the requested media resource, Request sequence number, Session ID, |
| STEP 14: | PU returns a response with RTSP 200 OK message, includes information： Status Line: RTSP/1.0 200 OK, Sequence number. |

The following figure shows the procedural flows for playback from MSU in service platform.

Figure 6-11: Procedural flows for Playback from service platform

| STEP 1: | CU sends a HTTP GET request to SCU to get the URL for media resource to play; |
| --- | --- |
| STEP 2: | SCU transfers this request to MDU; |
| STEP 3: | MDU in the platform return the URL to SCU with HTTP response; |
| STEP 4: | SCU transfers this response to CU; |
| STEP 5 to 16: | From STEP 5 to 16, the signaling and message are the same as the steps 3 to 14 in Procedural flows for Playback from PU directly; just the RTSP media server for CU is DSU and the media server is recommended to support Fast Forward/Fast Rewind and Random access functions. |

## 6.6	Voice (audio) communication

### 6.6.1	Voice communication

The Voice Communication function is one of the advanced capabilities of Visual Surveillance, and it is used to communicate in voice between End User and Surveilled Object (Human beings).

Figure 6-12: Procedural flows for Voice communication

| STEP 1: | CU sends a SIP INVITE message with SDP information of CU to SCU to request voice communication. |
| --- | --- |
| STEP 2, 3: | SCU needs to apply resources from MDU for media stream ‘forwarding. This is an internal interface in service platform; it may be implemented based on TCP protocol. MDU returns a response to SCU with SDP information of MDU. So here don’t give recommended protocol. |
| STEP 4, 5: | SCU sends a SIP INVITE message with SDP information of MDU to PU to request voice communication. PU returns SIP 200 OK message with SDP information of PU. |
| STEP 6: | SCU sends a SIP ACK message to PU to confirm this session. |
| STEP 7: | The bidirectional audio channels are created between PU and MDU. |
| STEP 8: | PU returns SIP 200 OK message with SDP information of MDU. |
| STEP 9: | CU sends a SIP ACK message to SCU to confirm this session. |
| STEP 10: | The bidirectional audio channels are created between CU and MDU. |
| STEP 11: | RTP packages of audio media stream are transferred by MDU between CU and PU. |
| STEP 12: | To end voice communication, CU sends a SIP BYE message to SCU. |
| STEP 13, 14: | SCU notifies MDU to stop transferring media stream and release resources.MDU returns a response to SCU. These are also internal interfaces in service platform. So here don’t give recommended protocol. |
| STEP 15: | SCU returns a SIP 200 OK message to CU to end communication between CU and MDU. |
| STEP 16, 17: | SCU sends a SIP BYE message to PU, PU returns a SIP 200 OK message to SCU to end communication between PU and MDU. |

### 6.6.2	Voice/audio broadcast

The voice/audio broadcast function is one of the advanced capabilities of Visual Surveillance system, and it is used to broadcast audio stream only from CU to multiple Surveilled Objects (Human beings) through PUs.

The following procedural flow involves implementing the voice/audio broadcast action.

Figure 6-13: High level procedural flows for voice/audio broadcast

The flows for unidirectional voice communication (broadcast) shows below:

| STEP 1 | CU sends a SIP INVITE to SCU to request voice/audio broadcast action with the information of requested PUs list. |
| --- | --- |
| STEP 2,3: | After authentication, the SCU requests MDU to assign resources for broadcast action, and the MDU returns a response with the result of resources reservation to the SCU. |
| STEP 4,5: | The SCU sends SIP INVITE to each PU of the requested list for broadcast connection. Each PU creates a unidirectional media channel with the MDU and returns a response separately with SIP 200 OK message to the MDU. |
| STEP 6,7: | The SCU returns a response with SIP 200 OK message to CU. The CU creates a unidirectional media channel with the MDU and sends a SIP ACK message to the SCU. |
| STEP 8: | The SCU returns a response with SIP 200 OK message to each PU separately. |
| STEP 9: | The CU sends media stream to the MDU though the created media channel. |
| STEP 10: | The MDU broadcasts media stream to each PU through separate media channel. |
| STEP 11: | To stop broadcasting, the CU sends SIP BYE message to the SCU and releases the media channel with the MDU. |
| STEP 12,13: | The SCU requests MDU to stop broadcasting and release resources, and the MDU returns a response to the SCU. |
| STEP 14,15: | The SCU sends SIP BYE message to each PU separately. Each PU stops receiving media stream and releases the unidirectional media channel with the MDU, and then sends SIP 200 OK message to the SCU. |
| STEP 16: | The SCU sends SIP 200 OK message to the CU to complete the whole procedure. |

## 6.7	Far-end Camera Control (FECC)

The far-end control function is used to pan, tilt and zoom cameras, switch auxiliary peripherals (such as flashlight, fan and rain brush), preset position of related physical devices.

The following figure shows the procedural flows.

Figure 6-14 Procedural flows for FECC function

| STEP 1: | CU sends a SIP MESSAGE with information of far-end camera control command to SCU to request remote control of requested camera or auxiliary peripherals. |
| --- | --- |
| STEP 2: | SCU handles authentication with some information from CMU. |
| STEP 3: | After authentication, SCU transfers the SIP MESSAGE to the requested PU. |
| STEP 4: | PU parses this control commands and sends to related physical devices to implement the far-end camera control operations. |
| STEP 5: | PU returns a response to SCU with SIP 200 OK message. |
| STEP 6: | SCU transfers this SIP 200 OK message to CU to complete this operation. |

__________________