| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4279 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.1 V4 Living list: addition of a new terms |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

New terminology is proposed.

Discussion

Identifier perspective:

- Term stream is primarily a horizontal concept within a context: a stream with its associated identifier may affect a single, two or multiple terminations. The stream could be seen as termination-independent from that point of view.

- Term stream endpoint shall emphasize the binding of a stream to a specific termination; could be thus considered as a vertical concept.

The vertical view and the notion of stream endpoint (SEP) is in the meanwhile used in a couple of published and Draft H.248.x-Recommendations, but not yet explicitly defined. H.248.1 would be the appropriate place.

Further, a pair of SEPs and in general a tuple of SEPs are subsequent concept, based on the SEP. Correspondent terms are suggested.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 794 (WP 2/16), the output draft from last Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

3.1.x	stream endpoint (SEP): relates to a stream associated to a termination. A SEP is identified by the tuple of StreamID and TerminationID values.

3.1.y	stream endpoint pair (SEPP): two associated Stream Endpoints (SEP) of two Terminations within the same Context. The two SEPs share thus the same StreamID value. There is always a point-to-point topology for Stream Endpoint Pair configurations (e.g., there shall be not any further associated SEP on a third termination etc).

3.1.z	stream endpoint tuple (SEPT): the generalization of SEPP towards multiple associated Stream Endpoints (SEP) within the same Context. All SEPs share thus the same StreamID value. The stream topology is given by the topology descriptor settings.

[End Proposed Correction]

____________________