| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | TD-21 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.RTPTOPO |  |  |  |
| Title: | H.248.RTPTOPO “RTP topology dependent RTCP handling by H.248 Media Gateways with IP Terminations” (New): Output draft (Ed. 0.12) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This document contains the editor’s output for draft new H.248.RTPTOPO “RTP topology dependent RTCP handling by H.248 Media Gateways with IP Terminations”.

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.RTPTOPO Ed. 0.1 | Initial draft (TD-38) from WP 2/16 Research Triangle Park (USA) meeting (2010-11), based on AVD 4000 Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1011_Res/TD-38.zip |
| H.248.RTPTOPO Ed. 0.2 | Input (TD 458 (WP2/16)) from ITU-T SG16 Geneva meeting, March 2011 Location: http://itu.int/md/T09-SG16-110314-TD-WP2-0458 |
| H.248.RTPTOPO Ed. 0.3 | Input (TD 458R1 (WP2/16)) from ITU-T SG16 Geneva meeting, March 2011 Location: http://itu.int/md/T09-SG16-110314-TD-WP2-0458R1 |
| H.248.RTPTOPO Ed. 0.4 | Input (AVD-4051) to WP 2/16 Andover (USA) meeting (2011-07) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1107_And/AVD-4051.zip |
| H.248.RTPTOPO Ed. 0.5 | Output draft (TD-39) from WP 2/16 Andover (USA) meeting (2011-07) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1107_And/TD-39.zip |
| H.248.RTPTOPO Ed. 0.6 | Input (TD 620 (WP2/16)) from ITU-T SG16 Geneva meeting, Nov/Dec 2011 Location: http://itu.int/md/T09-SG16-111121-TD- WP2-0620/ |
| H.248.RTPTOPO Ed. 0.7 | Output (TD 620R1 (WP2/16)) from ITU-T SG16 Geneva meet., Nov/Dec 2011 Location: http://itu.int/md/T09-SG16-111121-TD- WP2-0620R1 |
| H.248.RTPTOPO Ed. 0.8 | Input (AVD-4162) to WP 2/16 Geneva (USA) meeting (2012-02) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1202_Gen/AVD-4162.zip |
| H.248.RTPTOPO Ed. 0.9 | Input (TD 748 (WP2/16)) from ITU-T SG16 Geneva meeting, Apr/May 2012 Location: http://itu.int/md/T09-SG16-120430-TD-WP2-0748 |
| H.248.RTPTOPO Ed. 0.10 | Output (TD 748R1 (WP2/16)) from ITU-T SG16 Geneva meet., Apr/May 2012 Location: http://itu.int/md/T09-SG16-120430-TD-WP2-0748R1 |
| H.248.RTPTOPO Ed. 0.11 | Input (AVD-4231) to WP 2/16 Brisbane meeting (2012-09) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/AVD-4231.zip |
| H.248.RTPTOPO Ed. 0.12 | Output (TD-21) from WP 2/16 Brisbane meeting (2012-09) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/TD-21.zip |

CONTENTS

Page

1 Scope 5 2 References 5 3 Definitions 6 3.1 Terms defined elsewhere 6 3.2 Terms defined in this Recommendation 7 4 Abbreviations 8 5 Conventions 10 6 Background to RTCP handling by H.248 MGs 10 7 RTP topologies versus RTCP handling in H.248 MGs 11 7.1 Single H.248 ”IP” Termination per Context 11 7.2 Two H.248 ”IP” Termination per Context 11 7.3 More than two H.248 ”IP” Terminations per Context 12 7.4 Others 13 7.5 Notes to the H.248 Connection Model 13 7.6 Classes of RTCP related functions 14 7.7 Relation to modes of operations of IP-to-IP media gateways 14 8 RTCP services 15 8.1 RTCP packet structure 15 8.2 Important RTCP information elements (from H.248 MG perspective) 16 8.2.1 Information elements related to the identification of an RTP entity 16 8.2.2 Other relevant information elements 16 8.3 RTCP service levels 16 8.3.1 Basic RTCP services 16 8.3.2 RTP profile dependent RTCP services 17 8.3.3 Supplementary RTCP services 17 9 Scope of RTP topology control 17 10 RTP Topology Package 18 10.1 Properties 19 10.1.1 RTP Topology 19 10.2 Events 19 10.3 Signals 19 10.4 Statistics 19 10.5 Error Codes 20 10.6 Procedures 20 10.6.1 General 20 10.6.2 MGC level information for RTP topology control 20 10.6.3 Topology dependent procedures 20 Bibliography 22 Appendix I Living List related to RTP Topology Package 23

List of Tables

Page

Table 1 – RTP Topologies vs RTCP Handling vs H.248 MG Models – Single H.248 ”IP” Termination per Context 11 Table 2 – RTP Topologies vs RTCP Handling vs H.248 MG Models – Two H.248 ”IP” Terminations per Context 12 Table 3 – RTP Topologies versus RTCP Handling versus H.248 MG Models – More than two H.248 ”IP” Terminations per Context 13 Table 4 – RTP Topologies vs RTCP Handling vs H.248 MG Models – Others 13

List of Figures

Page

Figure 1 (copy of Fig. G.3/[b-ETSI TR 183 068]): BGF modes - H.248 Local and Remote Descriptor: SDP "m="-line specification combinations 15 Figure 2 - RTCP packet structure (Compound RTCP packet) 16 Figure 3 – Level of RTCP services 18

Draft new ITU-T Recommendation H.248.RTPTOPO

Gateway control protocol: 
RTP Topology dependent RTCP Handling 
by H.248 Media Gateways with IP Terminations

AAP Summary

<Mandatory material – to be provided before Consent>

Summary

<Mandatory material>

# 1	Scope

The scope of this Recommendation is related to RTCP handling by H.248 Media Gateways (MG) [ITU-T H.248.1]. The required MG behaviour is fairly clear for the very majority of use cases. However, there are some scenarios which benefit from or even require explicit signalling indication for as an unambiguous control of MG RTCP (but also RTP) traffic handling behaviour.

This Recommendation covers

- information about history and motivation behind RTP topologies and RTCP handling and the available H.248 tool landscape (clause 6);

- overview tables concerning “RTP topology versus RTCP handling” aspects and relevant H.248 connection models (clause 7);

- RTCP service aspects (clause 8);

- scope and limitations of RTP topology control (clause 9); and

- an H.248 package for basic RTP topology control (clause 10).

This Recommendation may be useful in scenarios where an MG supports multiple RTP topologies and/or multiple RTP/RTCP services which may lead to ambiguous behaviour. In these scenarios a H.248 profile specification may use the package in this Recommendation to alleviate this behaviour.

# 2	References

{Editor’s note: this is a preliminary list of documents related to RTP/RTCP. This is just a living list.}

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (09/2005), Gateway Control Protocol: Version 3, including Amendment 2 (12/2009).

[ITU-T H.248.30]	Recommendation ITU-T H.248.30 (06/2008), Gateway control Protocol: IP NAPT traversal package.

[ITU-T H.248.48]	Recommendation ITU-T H.248.48 (02/2012), Gateway control Protocol: RTCP XR Block Reporting Package.

[ITU-T H.248.50]	Recommendation ITU-T H.248.50 (09/2010), Gateway control Protocol: NAT traversal toolkit packages.

[ITU-T H.248.57]	Recommendation ITU-T H.248.57 (06/2010), Gateway control Protocol: RTP control protocol package.

[ITU-T H.248.58]	Recommendation ITU-T H.248.58 (06/2008), Gateway control Protocol: Packages for application level H.248 statistics.

[ITU-T H.248.71]	Recommendation ITU-T H.248.71 (02/2010), Gateway control Protocol: RTCP support packages.

[IETF RFC 3550]	IETF RFC 3550 (07/2003), RTP: A Transport Protocol for Real-Time Applications.

[IETF RFC 3611]	IETF RFC 3611 (11/2003), RTP Control Protocol Extended Reports (RTCP XR).

[IETF RFC 4585]	IETF RFC 4585 (07/2006), Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF).

[IETF RFC 4961]	IETF RFC 4961 (07/2007), Symmetric RTP / RTP Control Protocol (RTCP).

[IETF RFC 5093]	IETF RFC 5093 (12/2007), BT's eXtended Network Quality RTP Control Protocol Extended Reports (RTCP XR XNQ).

[IETF RFC 5117]	IETF RFC 5117 (01/2008), RTP Topologies.

[IETF RFC 5124]	IETF RFC 5124 (02/2008), Extended Secure RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/SAVPF).

[IETF RFC 5506]	IETF RFC 5506 (04/2009), Support for Reduced-Size Real-Time Transport Control Protocol (RTCP): Opportunities and Consequences.

[IETF RFC 5725]	IETF RFC 5725 (02/2010), Post-Repair Loss RLE Report Block Type for RTP Control Protocol (RTCP) Extended Reports (XRs)).

[IETF RFC 5760]	IETF RFC 5760 (02/2010), RTP Control Protocol (RTCP) Extensions for Single-Source Multicast Sessions with Unicast Feedback.

[IETF RFC 5761]	IETF RFC 5761 (04/2010), Multiplexing RTP Data and Control Packets on a Single Port.

[IETF RFC 5968]	IETF RFC 5968 (09/2010), Guidelines for Extending the RTP Control Protocol (RTCP)).

[IETF RFC 6128]	IETF RFC 6128 (02/2011), RTP Control Protocol (RTCP) Port for Source-Specific Multicast (SSM) Sessions.

[IETF draft monarch]	IETF RFC xxxx (xxx), Monitoring Architecture for RTP.
Note: present IETF state = ‘WGLC’ (draft-ietf-avtcore-monarch)
https://datatracker.ietf.org/doc/draft-ietf-avtcore-monarch/

# 3	Definitions

## 3.1	Terms defined elsewhere

3.1.1	Bit integrity [ITU-T Q.9]: Exists when the values of the bits in each octet of a digital bit stream at the output of a device or system are unchanged from those at the input.

NOTE - Digital processing devices such as A/mu law converters, echo suppressors and digital pads must be disabled to provide bit integrity.

3.1.2	Data integrity [ITU-T T.411]: The property that data has not been altered or destroyed.

## 3.2	Terms defined in this Recommendation

3.2.1	Fully protocol unaware forwarding: Transparent forwarding, but the MG is unaware of the Lx protocol(s). The MG is thus not able to execute any kind of operation on the Lx-PDU.

NOTE 1 – Example for “fully RTP protocol unaware forwarding”: the H.248 descriptor(s) does not contain any kind of signalling information (such as H.248 events, properties, signals, statistics or SDP elements) related to the indication of RTP traffic. There would be a fully L4+ agnostic stream setting.

3.2.2	Lx-PDU integrity: In the context of packet processing, it is the basic property of data integrity. The MG must guarantee bit integrity for forwarded protocol data units (PDU) at layer Lx. Lower layer protocol control information (PCI) may be modified.

{Editor’s note #1 (2012-09):  this term would be better located in H.248.79, should we create a correspondent IG item?}

3.2.3	RTP domain: A network domain with RTP entities which share a common RTP/RTCP-related identifier space. A RTP domain is characterized therefore by a domain wide (and temporal limited, NOTE 1) uniqueness of such protocol identifiers, which typically constitute unambiguous RTP traffic flows (e.g., protocol elements SSRC, CNAME, PT).

NOTE 1 – Temporal horizon of identifier values: [b-IETF RFC 6222] differentiates the identifier lifetime in short-term persistent and long-term persistent identifier value assignments.

NOTE 2 – An end-to-end RTP traffic flow could traverse multiple RTP domains, leading to the following observations:

- The end-to-end communication association would be comprised of multiple, concatenated RTP sessions.

- The edge of RTP domains is given by RTP entities with RTP topologies of type “RTP Endsystem” or “Back-to-Back RTP Endsystem”.

{Editor’s note (2012-09): the term “RTP session”, introduced by RFC 1889/3550,  is currently evolving due to ongoing work on “RTP multiplexing” in IETF WGs AVTCORE, MMUSIC, RTCWEB and CLUE. There are already different RTP session concepts floating around. It looks like that the RTP session is always a bidirectional concept. The RTP session is also dependent on RTP topologies. The RTP session is basically constituted by a n-tuple of identifiers. An RTP session includes usually the associated RTCP control flow to the RTP media flow.  […] The term “RTP session” is frequently used in this document and should be clarified in clause 3.1 or 3.2. A “RTP session” may become a local concept (related to an RTP endpoint), rather than a global concept as e.g. related to end-to-end communication.  Contributions are solicited.}

3.2.4	RTP packet integrity: Lx-PDU integrity for PDU type “RTP packet” [IETF RFC 3550]. Thus, RTP packet integrity implies RTCP packet integrity.

{Editor’s note #2 (2012-09):  …inclusion of RTCP might be debatable …}

NOTE 1 – The embedded concept of bit integrity restricts this property to “input-output” related RTP topologies (e.g., the RTP end system topology would be excluded).

{Editor’s note #3 (2012-09):  looks like that the characteristic of “RTP packet integrity” is limited to clause 7.2 topologies only …}

NOTE 2 – RTCP packet integrity is limited to end-to-end RTCP control flows, but not related to RTCP packets originating or terminating at the MG (such as possible RTCP XR packets).

3.2.5	RTCP service: A MG service tied to a particular RTCP packet type codepoint. The RTCP PT value may be thus considered as RTCP service identifier.

NOTE 1 – This is the coarsest granularity of an RTCP service definition, but sufficient for this Recommendation. A finer grain RTCP service level could take into account further RTCP packet embedded identifiers such as the block type (BT) codepoint in case of RTCP XR or the feedback message type (FMT) codepoint in case of RTCP FB.

NOTE 2 – Another classification approach could derive generic (because usually PT-independent) RTCP service categories such as related to performance monitoring, media encryption, codec rate control, congestion control, generic feedback, keep alive, etc.

{Editor’s note (2012-09): the term “Supplementary RTCP service” should be defined. Also in order to clarify the relation to legacy supplementary services in telecommunications (see definitions in Q.9 and Q.1741).

According clause 9, a “supplementary RTCP service” is characterized by

- [Q.9 modified]: “Any (RTCP) service provided by a network in addition to its basic (RTCP) service(s).”

- [Q.1741 modified]: “A(RTCP) service which modifies or supplements a basic telecommunication (RTCP) service. Consequently, it cannot be offered to a user as a stand-alone service. It must be offered together with or in association with a basic telecommunication (RTCP) service. The same supplementary(RTCP)  service may be common to a number of basic telecommunication (RTCP) services.”

An alternative to a Q.9/Q.1741 based definition might to include also explicit definitions for

- Basic RTCP service(s)

- RTP profile dependent RTCP service(s)

and “supplementary RTCP service(s)” would be then all other services which are not (a) and (b).

Contributions are solicited.}

3.2.6	Transparent forwarding: MG packet forwarding behaviour with the characteristic of Lx-PDU integrity. This is a unidirectional characteristic of a Lx-PDU flow.

NOTE 1 – This is normally implicit, basic behaviour, when LD/RD settings are identical: “identical media descriptions” imply “transparent forwarding” by the MG by the MG under the condition that the MG could derive the intended protocol layer Lx of transparent forwarding.

NOTE 2 – There would be then the characteristic of RTP packet integrity in the context of “RTP transparent forwarding”. The MG might be RTP aware; e.g. support of RTP related statistics or event detection would not violate transparent forwarding behaviour.

NOTE 3 – Delayed transparent forwarding is a variant of transparent forward from a temporal perspective, which is characterized by transparent forwarding, but Lx-PDUs might be intentionally delayed in the outgoing direction. The traffic characteristic of the Lx-PDU flow would be then modified.
Examples:

- The application of IP traffic shaping (see clause 7.1.5/[ITU-T Y.1221]) might be one example in case of Lx equal to ‘IP’.

- The application of RTP jitter elimination, reduction or adjustment (see [ITU-T G.799.3]) might be one example in case of Lx equal to ‘RTP’

# 4	Abbreviations

This Recommendation uses the following abbreviations and acronyms:

| ASY | Asymmetric topologies |
| --- | --- |
| AVP | (RTP) Audio Visual Profile |
| AVPF | (RTP) Audio Visual Profile with Feedback |
| B2BRE | Back-to-Back RTP Endsystem |
| BGF | Border Gateway Function NOTE - synonym to Border Gateway and IP-to-IP Gateway in this Recommendation |
| BT | (RTCP) Block Type |
| CNAME | Canonical Name |
| CSRC | Contributing Source |
| FB | (RTCP) Feedback |
| FMT | (RTCP FB) Feedback Message Type |
| IP | Internet Protocol |
| LD | Local Descriptor |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| PT | (RTP) Payload Type; (RTCP) Packet Type |
| RD | Remote Descriptor |
| RR | (RTCP) Receiver Report |
| RTCP | RTP Control Protocol |
| RTP | Real-time Transport Protocol |
| RTPASM | RTP Any Source Multicast |
| RTPE | RTP Endsystem (or RTP endpoint) |
| RTPMU | RTP Multicast |
| RTPMT | RTP Media Translator |
| RTPRTM | RTP RTCP-Terminating-MCU |
| RTPSSM | RTP Source Specific Multicast |
| RTPTT | RTP Transport Translator |
| RTPVSM | RTP Video-Switch-MCU |
| SAVP | (RTP profile) Secure Audio Visual Profile |
| SDES | Source Description (RTCP Packet) |
| SR | (RTCP) Sender Report |
| SSRC | Synchronization Source |
| VoIP | Voice over Internet Protocol |
| XR | (RTCP) eXtension Report |

# 5	Conventions

None.

# 6	Background to RTCP handling by H.248 MGs

The handling of RTCP packets by H.248 media gateways is often not straightforward, sometimes not fully clear also from IETF RFC side. RTCP handling in general is tightly coupled to the RTP topology model, which drives the processing of RTP/RTCP traffic of a particular H.248 Context.

The aspect of RTP topologies is for many H.248 MG types not really relevant because the handling is obvious. For example: for all H.248 (IP, physical) connection models (as applied for VoIP MGs at the border towards circuit-switched networks). However, for H.248 (IP,IP) MGs the situation is slightly different.  In this case new RTCP services have been defined that increase the possible interactions between RTP topology and these services. This has led to possible ambiguity in the RTCP handling behaviour in the MG.

The problem also has an historical dimension: [IETF RFC 1889] and [IETF RFC 3550] were too vague on that point, which was the motivation to clarify on a more detailed level RTP topology models, published as [IETF RFC 5117]. [IETF RFC 5117] provides a more fine granular topology classification and detailed description.

NOTE 1- It may be noted that the problem is not specific to H.248, rather concerns also other gateway/policy control protocols, which are applied for IP-IP gateways or policy enforcement points. For example: [ITU-T H.248.48]  considers different RTCP processing functions (e.g., local generation of RTCP reports, forwarding/filtering of RTCP reports). [ITU-T H.248.48] is actually applicable for multiple RTP topologies and could be categorized as a supplementary RTCP service (see clause 8.3.3). It has had to consider these interaction issues.

Generally RTP topology and RTCP handling may be summarized:

- Local RTCP handling by an H.248 MG is basically linked to the applied RTP topology, and the RTP topology is given by the Terminations/Streams in a H.248 Context.

- Without this Recommendation explicit H.248 means are not defined to control RTP topology. Thus, H.248 profiles that do not utilise this Recommendation assume implicit behaviour (“which may lead to misinterpretations”).

- Existing H.248 protocol elements for RTP/RTCP control are for other, different purposes, i.e.:

  - [ITU-T H.248.1] Annex E.12 “RTP Package”: event and statistics

  - [ITU-T H.248.30] “RTCP extended performance metrics packages”:
- RTCP XR Base Package,
- RTCP XR Burst Metrics Package

  - [ITU-T H.248.48] “RTCP XR Block Reporting Package”: statistics

  - [ITU-T H.248.50] “RTP Keep Alive Request Package”: “RTP signals”

NOTE 2 – Discussion: H.248.50 signal parameter “Keep Alive Packet Type” (from signal “Send keepalive packet”) indicates some RTP/RTCP related packet types, which represent the semantic of RTP topology “RTP endsystem” (sender only). There might be then following controversial situation if the H.248 Stream endpoint were enabled with signal e.g. kar/skap/kapt/cn (indicating topology RTP endsystem) and also part of an overall RTP transport translator (given by a property setting of “rtpt/rtptopo = TR”). Such a situation may be resolved by indicating the RTP keep alive mechanism as a supplementary service (see clause 9).

  - [ITU-T H.248.57] “RTP Control Protocol Package”: L4 port allocation control

  - [ITU-T H.248.58] “RTP Application Data Package”: statistics

  - [ITU-T H.248.71] “RTCP support packages”:
- RTCP Source Description Package, 
- Received RTCP Package, 
- RTCP Feedback Message Package

The topic as such, correct RTCP handling by H.248 MGs, is important in order not to jeopardize end-to-end RTP-based services.

# 7	RTP topologies versus RTCP handling in H.248 MGs

This clause provides an attempt to structure RTP Topologies vs RTCP Handling vs H.248 MG Context configurations. The functions related to RTCP handling are labelled with “F-x” according clause 7.6. The RTP topologies are classified according H.248 connection models which are determined by the number of terminations per Context, see clauses 7.1 to 7.4.

## 7.1	Single H.248 ”IP” Termination per Context

See Table 1.

Table 1 – RTP Topologies vs RTCP Handling vs H.248 MG Models
– Single H.248 ”IP” Termination per Context

| No. | RTP Topology | RTCP Handling | H.248 Connection Model | Example |
| --- | --- | --- | --- | --- |
| I.1 | RTP Endsystem (RTPE) (aka RTP Endpoint) | F-1,2,3,5 | (RTP, non-RTP) | VoIP Residential -, Access – or Trunking MG |

## 7.2	Two H.248 ”IP” Termination per Context

See Table 2.

Table 2 – RTP Topologies vs RTCP Handling vs H.248 MG Models
– Two H.248 ”IP” Terminations per Context

| No. | RTP Topology | RTCP Handling | H.248 Connection Model | Example |
| --- | --- | --- | --- | --- |
| II.1 | Back-to-Back RTP Endsystem (B2BRE) | F-1,2,3,4,5,6,7 | (RTP, RTP) | IP-to-IP MG located at the border between two RTP domains (e.g., SRTP-to-RTP interworking) |
| II.2 | RTP Transport Translator (RTPTT) | F-1,3,4,5,6,7 F-2 if F-6 | (RTP, RTP) | IP-to-IP MG located within a single RTP domain, without any media processing |
| II.3 | RTP Media Translator (RTPMT) in general (NOTE 1) | F-1,3,4,5,6,7 F-2 if F-6 | (RTP, RTP) | IP-to-IP MG located within a single RTP domain, with processing of the RTP payload |
| II.3.1 | RTP Media Translator without media format change (RTPMTlight) | F-1,3,4,5,6,7 F-2 if F-6 | (RTP, RTP) | ditto, e.g., transpacketization (changing packetization times), jitter reduction between two IP networks |
| II.3.2 | RTP Media Translator with media format change (RTPMTfull) | F-1,3,4,5,6,7 F-2 if F-6 | (RTP, RTP) | ditto, e.g., audio transcoding |
| NOTE 1 – The RTPMT topology is sometimes further sub-classified here with II.3.1 & II.3.2 (see e.g. [b-IETF RFC 6679]). |  |  |  |  |

NOTE - Some media-level functions may be achieved by more than one RTP topology. For example, audio transcoding may be done in B2BRE or RTPMT topologies. However, there would be different RTCP handling functions.

## 7.3	More than two H.248 ”IP” Terminations per Context

See Table 3.

Table 3 – RTP Topologies versus RTCP Handling versus H.248 MG Models –
More than two H.248 ”IP” Terminations per Context

| No. | RTP Topology | RTCP Handling | H.248 Connection Model | Example |
| --- | --- | --- | --- | --- |
| III.1 | RTP Mixer (RTPM) | F-1,2,3,4,5,6 | (RTP, RTP, … RTP) | Media server |
| III.2 | RTP Video-Switch-MCU (RTPVSM) | F-1,2,3,4,5,6 | (RTP, RTP, … RTP) | Media server |
| III.3 | RTP RTCP-Terminating-MCU (RTPRTM) | F-1,2,3,4,5,6 | (RTP, RTP, … RTP) | Media server where each H.248 RTP Termination relates to the “RTP Endsystem” topology |
| III.4 | RTP Multicast (RTPMU) | TBD (see clause 7.6) | TBD |  |
| III.4.1 | RTP Source Specific Multicast (RTPSSM) | TBD (see clause 7.6) | TBD |  |
| III.4.2 | RTP Any Source Multicast (RTPASM) | TBD (see clause 7.6) | TBD |  |

## 7.4	Others

See Table 4.

Table 4 – RTP Topologies vs RTCP Handling vs H.248 MG Models –
Others

| No. | RTP Topology | RTCP Handling | H.248 Connection Model | Example |
| --- | --- | --- | --- | --- |
| IV.1 | Asymmetric topologies (RTPASY) | TBD (see clause 7.6) | (RTP, MUX, RTP … RTP) (TBD). | H.248 Contexts with internal H.248 MUX terminations … |
| IV.2 | Topology combinations | TBD (see clause 7.6) | (TBD). | Cascaded H.248 Contexts … |

## 7.5	Notes to the H.248 Connection Model

Names of terminations (TerminationID) may be arbitrarily chosen (see clause 6.2.2/[ITU-T H.248.1]), but termination naming schemes are usually given by H.248 profile specifications. Such H.248 profiles typically use the notation of “IP” for IP-based ephemeral terminations (e.g. in order not to exclude any possible IP applications, which might be concluded by a transport protocol or application protocol specific notation). This Recommendation uses“RTP” instead because there may be a difference between following connection models:

- (RTP, RTP)

  - MG is “RTP-aware”, which enforces a transport-protocol aware mode of operation

  - MG may be media aware or media agnostic

- (IP, IP)

  - MG may be “RTP-agnostic”, i.e., aware of the transport protocol (UDP), but unaware of higher protocol layers, thus RTP packets may transparently go through such an H.248 Context.

It could be noted that the termination name could carry an implicit semantic concerning configured RTP topologies and/or RTCP handling behaviour of a Context. However, it looks unfortunate to overload the termination name with such semantics (note: clause 7.1 would be an exception). This Recommendation therefore recommends that the

-H.248.TerminationID should not be used to indicate a particular RTP topology.

## 7.6	Classes of RTCP related functions

The MG could be requested to process following RTCP packets or RTCP related functions (Note: the list label is used in Tables 1 to 4):

F-1:	RTP node identifier: usage (or not) of MG-local CNAMEs (per MG?; per termination?);

F-2:	RTP source identifier: allocation (or not) of MG-local SSCRs per RTP session;

F-3:	RTCP protocol endpoint for basic services: origination/termination of RTCP packets (reports);

F-4:	RTCP packet forwarding: unmodified forwarding of RTCP packets (reports);

F-5:	RTCP packet modification: rewrite of RTCP packets;

F-6:	RTCP protocol endpoint for supplementary services: local generation of RTCP reports (such as for RTCP XR based applications);

F-7:	RTCP packet filtering: filtering of RTCP reports (e.g., either entire RTCP packet or just an individual RTCP report within a compound packet is filtered);

F-8:	others

## 7.7	Relation to modes of operations of IP-to-IP media gateways

The subject of “RTP topologies” affects all kinds of H.248 MGs, which are involved in RTP traffic processing, from PSTN residential/access/trunking MGs to media servers for large conferencing systems. Of particular interest are H.248 IP-to-IP media gateways (also known as border gateways, at the network level of access/edge, core or interconnect/peering) due to RTP traffic processing related to NAPT, NAT-T, QoS support, policy control and/or media conversion functions, based on RTP topologies according to clause 7.2.

The basic packet processing behaviour by such MGs is called “mode of operation”, leading to the aspect of possible interactions between RTP topologies and such modes of operations as defined for IP-to-IP media gateways.

There are already a number of such modes discriminated for IP-to-IP media gateways, like e.g. “media-format agnostic, transport protocol type aware”, etc. For instance, [b-ETSI TR 183 068] provides an established categorization which could be summarized according Figure 1.

Figure 1 (copy of Fig. G.3/[b-ETSI TR 183 068]): 
BGF modes - H.248 Local and Remote Descriptor: 
SDP "m="-line specification combinations

The modes of operation are related to the media description in the local and remote descriptors. The classification in Figure 1 is based on the SDP “m=”-line only.

Recommendations for H.248 profiles for IP-to-IP gateways:

A particular profile may explicitly require support of a number of RTP topologies and a number of different modes of operations. There are following two situations:

- There are not any further specification actions required in case of semantical unambiguity (e.g., no interactions concluded). The correct RTP/RTCP processing behaviour could be clearly concluded by the MG from Context specifications, without explicit RTP topology control

- In case of possible interactions, unclear semantics and/or explicit RTP topology related behaviour, then it is recommended to consider explicit RTP topology control according clause 10. The semantics of the RTP topology package takes then preceding over any other (implicit) semantics related to modes of operation.

NOTE  – the profile may also need to provide further clarification on the derived MG behaviour in the above instance.

# 8	RTCP services

Purpose of this clause is to describe the handling of RTCP packets, dependent on the RTP topology and used RTCP services.

## 8.1	RTCP packet structure

Figure 2 illustrates the basic RTCP packet structure at the example of a compound RTCP packet.

Figure 2 - RTCP packet structure (Compound RTCP packet)

The RTCP packet structures need to be considered to identify key information elements (see next subclause).

## 8.2	Important RTCP information elements (from H.248 MG perspective)

Not every information element of RTCP packets is relevant from a H.248 perspective. There is however some elements which demand careful consideration when processing RTP/RTCP traffic by H.248 MGs.

### 8.2.1	Information elements related to the identification of an RTP entity

There are primarily three information element types:

- SSRC (per originating RTP source),

- CSRC (for contributing RTP source), and

- SDES.

These elements define the spatial horizon of RTP media and RTCP control flows, i.e. the "end-to-end" significance of RTCP services. See for instance the detailed evaluation of example H.248 measurement architectures by [ITU-T H.248.48] and IETF monitoring architectures for RTP by [IETF draft- monarch].

### 8.2.2	Other relevant information elements

TBD

{Editor’s note: there might be RTCP information elements of primary interests for RTP end-systems only …FFS.}

## 8.3	RTCP service levels

### 8.3.1	Basic RTCP services

Every RTP implementation must be compliant to [IETF RFC 3550] and thus basically support all the RTCP services defined by this RFC. They may be called basic RTCP services and cover the four RTCP packet types of SR (200), RR (201), SDES (202) and BYE (203).

NOTE - The codepoint for application defined RTCP packet types APP (204) is also introduced by this RFC, but the definition of application specific services is delegated to subsequent RFCs.

Basic RTCP services are indicated to the MG by e.g. RTCP port allocation (see [ITU-T H.248.57]) for processing RTCP control flow traffic by an H.248 context.

### 8.3.2	RTP profile dependent RTCP services

RTP profiles may add additional RTCP services like e.g. the RTP profiles RTP/AVPF and RTP/SAVPF.

The use of RTP profile dependent RTCP services are basically indicated to the MG by the protocol field value in the SDP "m=" line, as part of the LD-/RD-embedded SDP media description.

### 8.3.3	Supplementary RTCP services

RTP sessions may be supplemented by further RTCP-based services. [IETF RFC 5968] defines the generic extension principles for RTCP. Such kind of supplementary services are for instance the RTCP extension report (XR) based capabilities, as defined for measurement and monitoring architectures.

Supplementary RTCP services are indicated to the MG by e.g. RTCP report/packet type specific signalling elements like e.g. H.248 packages [ITU-T H.248.30] and [ITU-T H.248.48], or correspondent SDP attributes as part of the LD-/RD-embedded SDP media description.

{Editor’s note: next step would be the completion of Tables 1 to 4 concerning RTCP handling input. The table input would have to address

- the guidelines from RFC 5117 for each topology,

- the key RTCP information elements, and

- the RTCP service levels.

One result might be that the RTP topology is basically just impacting the basic RTCP and RTP profile dependent services. And supplementary RTCP services would be an overlay on top of RTP topologies … For further studies.}

# 9	Scope of RTP topology control

There is a relation between RTP topology and RTCP services. However, the scope of RTP topology control needs to be limited due to the plethora of RTCP services and in order to handle complexity. Figure 3 summarizes again the main RTCP service categories (from clause 8).

Figure 3 – Level of RTCP services

The scope of RTP topology control, - based on the H.248 rtpt package defined by this Recommendation -, could be pragmatically limited on basic (I) and RTP profile dependent (II) RTCP services.

Example:

The H.248 RTP stream in an H.248 IP-IP MG could be configured for RTP topology “Topo-Translator”, which implies RTCP services such as

- no allocation of local SSRC values,

- no generation of RTCP RR reports, and

- forwarding of RTCP BYE packets.

The same H.248 RTP stream could be additionally enforced for H.248.48 type performance monitoring capabilities, which could lead to RTCP services such as

- allocation of local SSRC values for measurement points, or/and

- generation of RTCP XR reports.

The H.248.48 related supplementary RTCP services are not in contradiction to the RTP topology associated ones.

# 10	RTP Topology Package

|  | Package Name: | RTP Topology Package |
| --- | --- | --- |
|  | Package ID: | "rtpt" (0x????) |
|  | Description: | The package allows a MGC to indicate to a MG which particular RTP topology an RTP stream is operating under. The RTP Topologies are fundamentally defined by [IETF RFC 5117] and complemented by this Recommendation. A single H.248 Stream allows the definition of two RTP streams thus the topology is defined at a Local and Remote Descriptor level. |
|  | Version: | 1 |
|  | Extends: | None. |

## 10.1	Properties

### 10.1.1	RTP Topology

|  | Property Name: | RTP Topology |
| --- | --- | --- |
|  | Property ID: | rtptopo (0x0001) |
|  | Description: | This property indicates which RTP topology (as primarily defined by [IETF RFC 5117]) that an RTP Stream is acting under. |
|  | Type: | Enumeration |
|  | Possible values: | MG (0x0000) MG Determined PP (0x0001) Topo-Point-to-Point (Note 1) MC (0x0002) Topo-Multicast TR (0x0003) Topo-Translator MX (0x0004) Topo-Mixer VS (0x0005) Topo-Video-Switch-MCU RT (0x0006) Topo-RTCP-terminating-MCU TA (0x0007) Topo-Asymmetric TF (0x0008) Topo-Transparent-Forwarding (Note 2) NOTE 1 – A pair of two connected Stream endpoints (between two RTP terminations within a context) and PP topology is synonymous with a Back-to-Back RTP End system (B2BRE) topology from the perspective of an end-to-end RTP session. NOTE 2 – An H.248 IP-IP MG will enforce transparent forwarding (see clause 3.2.6) behaviour. |
|  | Default: | MG |
|  | Defined in: | Local / Remote |
|  | Characteristics: | Read/Write |

## 10.2	Events

None.

## 10.3	Signals

None.

## 10.4	Statistics

None.

## 10.5	Error Codes

None.

## 10.6	Procedures

### 10.6.1	General

This package allows the MGC to indicate to the MG which particular RTP topology that an RTP stream is under acting under. The type of RTP topology may have impacts on various bearer level functions. It may have an impact on RTCP handling procedures.

In most cases a MG is able to determine itself the RTP topology that is used due to the H.248 Connection Model (i.e. the combination of Terminations/Streams and associated properties in a Context). For example: it can determine the difference between a point to point topology (i.e. a two Termination Context) versus a mixer (i.e. three or more Termination Context). Thus the MGC does not need to signal the "RTP Topology" (rtpt/topo) property.

However in some cases the MG is unable to distinguish between topologies. For example: in some network scenarios it may not be able to distinguish between point to point and translator topologies. This package may be used in scenarios where a more deterministic method is needed to indicate what the RTP topology of a RTP Stream is.

It is expected that Packages extending this package or profiles utilising this package specify what the function specific impact of the RTP Topology property would be.

### 10.6.2	MGC level information for RTP topology control

RTP topology control (at MGC level) could be based on

- call/session control signalling or/and

- local policies.

### 10.6.3	Topology dependent procedures

“MG Determined” indicates the MG shall determine RTP topology and subsequent RTCP handling based on the implicit information derived from the applicable H.248 descriptors. This behaviour may also be subject to profile specification.

#### 10.6.3.1	Topology “Point-to-Point”

The MG shall provide an own, local CNAME.

The MG shall allocate local SSRC values per RTP session endpoint (which corresponds to a H.248 flow pair of RTP media and RTCP control traffic; often identical to the H.248 Stream endpoint (SEP)).

The MG shall originate/terminate RTCP packets of type:

- at least: “SDES”, “SR”, “BR” and “BYE”

[…]

#### 10.6.3.2	Topology “Multicast”

Not used.

#### 10.6.3.3	Topology “Translator”

The MG may provide an own, local CNAME.

The MG shall not allocate local SSRC values for RTP traffic under this topology.

The MG may allocate local SSRC values for supplementary RTCP services.

The MG shall not originate/terminate RTCP packets of type:

- at least: “SR”, “BR” and “BYE”

The MG may originate/terminate RTCP packets of type:

- e.g. “XR”

[…]

#### 10.6.3.4	Topology “Mixer”

Not used.

#### 10.6.3.5	Topology “Video-Switch-MCU”

Not used.

#### 10.6.3.6	Topology “RTCP-terminating-MCU”

Not used.

#### 10.6.3.7	Topology “Asymmetric”

Not used.

#### 10.6.3.8	Topology “Transparent Forwarding”

#### 10.6.3.8.1	Background to “Transparent Forwarding”

There are basically two H.248 signalling options in order to achieve a MG behaviour of “RTP transparent forwarding”:

a)	Implicit signalling

Existing H.248 signalling using LD/RD settings with “identical media descriptions”. There are two terminations (and two associated stream endpoints), which implies synchronized settings between all four local and remote descriptors (in case of a bidirectional RTP session). The MG would provide a transparent forwarding behaviour (according clause 3.2.6) for all cases of unambiguous H.248 signalling.

b)	Explicit signalling

Explicit indication via rtpt/rtptopo property (see next sub-clause).

##### 10.6.3.8.2	Usage of rtpt/rtptopo property

The H.248 Stream Descriptor could contain RTP/RTCP related information (such as LD-/RD-embedded SDP media descriptions or other H.248 signalling elements related to RTP/RTCP; or LCD level information). Other descriptors could also contain RTP/RTCP related information (e.g. related to events, signals, statistics). Such kind of H.248 information could lead to the activation of RTP/RTCP processing functions in the MG or/and ambiguity concerning the aimed RTP topology behaviour.

Property value “TF” of (rtpt/topo) property shall be then used when the MG shall transparently forward RTP packets in order to guarantee deterministic traffic processing. The MG behaviour of RTP transparent forwarding (RTPTF) includes transparent forwarding (clause 3.2.6), but not fully protocol unaware forwarding (clause 3.2.1) due to basic RTP awareness (at least by usage of rtptopo property).

Bibliography

[b-ETSI TS 183 068]	ETSI TS 183 068 v3.1.1, Telecommunications and Internet Converged Services and Protocols for Advanced Networks (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

[b-IETF RFC 6222]	IETF RFC 6222 (2011), Guidelines for Choosing RTP Control Protocol (RTCP) Canonical Names (CNAMEs).

[b-IETF RFC 6679]	IETF RFC 6679 (2012), Explicit Congestion Notification (ECN) for RTP over UDP.

Appendix I
Living List related to RTP Topology Package

(This appendix does not form an integral part of this Recommendation)

{Editor’s note: This Appendix is related to the RTP Topology Package (see clause 10):

- Level of RTP topology property: there might be following options

  - Context level (-> new Context Attribute)

          - Pro: RTP topology is a characteristic above the Stream level

          - Con: there might be different, Stream-dependent RTP topologies in case of multiple-Stream-per-Termination connection model

  - Topology descriptor  (-> syntactical or/and semantical extension)
=> not any concrete idea was presented

  - Stream level (-> new Property, see e.g. example below rtpt/rtptopo)

          - Pro: satisfies the requirement of  Stream level RTP topologies

          - Con: the MGC must ensure that all Stream endpoints, related to the RTP topology, must be configured for the same topology. Thus, any invalid signalling combination must rejected by an error code by the MG.

- Dynamic of RTP topology property value: there appears the question whether the RTP topology is a static characteristic over the lifetime of a Stream, or whether the RTP topology could even dynamically change? Some possible use cases:

          - Scenario 5.1: a Context is created with Terminations T1 (RTP) and T2 (Phy). T1 is thus in PP topology. Later on T2 is replaced (by SUBtract and MODify) by T3 (RTP), and T1 and T3 should now be connected in TR topology.
=> Transient RTP topology of T1 (PP -> TR).

          - Scenario 5.2: to a Context with two RTP Terminations (T1, T2)  in RTP Media Translator topology would be later a third RTP termination (T3) added, and the media processing should be done in “mixing mode”, thus a MX topology. 
=> again transient RTP topology of T1 and  (TR -> MX).

Contributions are solicited.}

-

____________________