| Question(s): | 2 |  | Meeting, date: |  | 14 - 25 January 2013 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 2 | Intended type of document (R-C-TD): |  | TD |
| Source: | Q2/16 Rapporteur |  |  |  |  |  |
| Title: | Update on the Advanced Multimedia System (AMS) |  |  |  |  |  |
| Contact: | Paul E. Jones Cisco Systems, Inc. USA |  |  |  | Tel: +1 919 476 2048 Fax: Email: paulej@packetizer.com |  |
| Contact: |  |  |  |  | Tel: Fax: Email: |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

# Introduction

This short document aims to provide meeting participants with an update on the current status of the work related to the Advanced Multimedia System (AMS), also known as “H.325”.

For those who might be unaware of AMS, AMS is a next-generation multimedia system that aims to separate call control signalling and application logic.  In legacy systems, the video conferencing terminal was a monolithic system that included the user interface, voice, and video capabilities.  If there were any additional capabilities, such as file transfer, application sharing, etc., those capabilities were also integrated directly into the same system.  Making changes to a deployed system generally entailed upgrading the firmware and was sometimes painful.  Further, it is difficult to introduce any new innovative features into an already-deployed terminal device.

What AMS aims to do is separate the traditional terminal into a “Container” function and a set of “Applications”.   The Container serves as the user’s primary interface, allowing the user to initiate, terminate, or otherwise control communication sessions.  The Applications register with the Container giving the user the desired features he or she would need in order to effectively communicate.  These applications might be voice, video, whiteboarding, application sharing, etc.  Further, these applications may exist on the same device with the Container or an entirely different device.

By separating the Container and Application functions, it is possible to utilize any number of different applications and introduce new applications at any time independently of the Container function.  In this way, it is possible to provide users with a much richer communication experience, even enabling modes of communication that had never been considered previously by simply introducing a new application to the Container.  It also allows the user to utilize various devices that might be near him or her, such as a video panel in a conference room or hotel room.  In effect, the approach transforms the multimedia terminal into a “platform” upon which a variety of multimedia applications may be realized, taking full advantage of the features and capabilities of packet-switched networks like the Internet.

# Documents Completed

During the last Study Period, the experts in Question 12 completed work on the requirements document for AMS titled “HSTP-AMSR - AMS Requirements”.  Prior to that, the group framed the objectives of the work in an AMS Project Description document (TD-562/WP2).

# Recent Areas of Work

During the previous study period, the experts in the Q12/16 were primarily focused on these areas:

- H.325/AMS message primitives (AVD-4339, C.824)

- AMS capability exchange, including (C.823)

  - Exchange of “application” capabilities between Containers

  - Exchange of application-specific capabilities between applications

- AMS information flows (C.822, C.676)

- Control and Indication (C&I) messages (C.675)

- AMS interfaces and reliable transport issues (C.675)

A number of interfaces have been identified for AMS as depicted in the following diagram:

Figure 1 - AMS Interfaces

Not shown in this diagram is Interface D and, per C.822, Interfaces DC and DK.  Those interfaces are to be added to the Working Assumptions document, but probably need further discussion.

# Current Work Programme

This table summarizes the work programme at the end of the previous study period:

| Acronym | Title | Editor | Consent / Approval | Reference |
| --- | --- | --- | --- | --- |
| AMS Project Description | AMS Project Description | Tyler Johnson (Tyler_Johnson@unc.edu) | N/A | TD 562/WP2 (2008-04) |
| AMS Working Assumptions | AMS Working Assumptions | TBD | N/A | AMS-0003 (2009-12) |
| AMS Catalogue | AMS Catalogue | TBD | N/A | TD 211/WP2 (2009-10) |
| H.325 (H.ams) | AMS: Advanced multimedia systems for next generation and other packet-switched networks | Paul E. Jones (paulej@packetizer.com) | 2013 | TD 671R1/WP2 (2011-11) |
| H.AMS.CIS | Common Interface Syntax | Paul E. Jones (paulej@packetizer.com) | 2013 | AMS-0001 (2009-12) |

# H.323 Enhancements to Enable H.325 Concepts

Through the discussion on H.325, there has been some interest in enhancing the existing H.323 systems with H.325-like functionality.  Essentially, an “H.323 application” could provide specific capabilities like file transfer or application sharing.  The user’s “Container” (primary H.323 terminal) would utilize RAS signalling to communicate with the H.323 “application” (e.g., file transfer application).  In this way, it would be possible to allow a device, either local or remote, to associate with the user’s terminal and for the user to then utilize those features.  See AVD-4313 and C.58 for more discussion on this topic.

# Progressing the Work

As we head into this new study period, there are several things we need to do:

- Examine the work produced to this point and make any desired changes

- Progress the work on message primitives needed to support the defined information flows and other syntactic elements of the protocol

- Close some of the open issues identified in the information flows

- Complete the core H.325 and H.AMS.cis specifications

There are additional areas where work is needed, particularly on some of the network elements discussed in the requirements document.  However, to make forward progress and to remain focused, the above listed areas should be a primary area of focus for this and the next meeting.

On the matter of signalling syntax, it was suggested to the Rapporteur by interested parties outside the ITU that we should reconsider the message encoding, too.  Presently, we are working under the assumption that messages will be encoded using XML, but it was suggested that we consider the use of JSON for message encoding.  The argument is that JSON is well-suited as a text-based communication protocol, whereas XML is more of a document-oriented syntax that is “heavier” than necessary.  This suggestion should be given consideration by the experts.

# Contributing to the Work

There are a few areas in which experts can make contributions to AMS and help drive the work forward.

First, we need volunteers to serve as editors.  Over the past few years, we have lost some valuable participants (and editors!) for various reasons, not the least of which was the unfortunate world economic climate.

Second, we need more contributions related to some of the topics above.  AMS, like other work in the ITU, is driven through contributions.  Unfortunately, there were no contributions for AMS at this meeting, but hopefully that will not be the case at the next meeting.

Third, we should consider re-starting the periodic electronic meetings.  We had good progress previously the group held electronic meetings every few weeks, but the group has not held those for quite some time.  As most people are aware, it is actually challenging to complete work during an SG16 meeting, so either having working face-to-face meetings or electronic meetings are probably necessary to progress the work.  We should discuss how best to progress the work on AMS.

_____________________