| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 23 R1 (WP 1/16) |
|  |  | English only Original: English |
| Question(s): | 3/16 | Geneva, 14 - 25 January 2013 |
| TD |  |  |
| Source: | Editor H.248.RTCPPROF |  |
| Title: | H.248.RTCPPROF "Guidelines on the use of H.248 capabilities for performance monitoring in RTP networks in H.248 Profiles" (New): Output draft Ed. 0.7 (Geneva, 14 - 25 January 2013) |  |

Summary

This document contains the editor's output draft for draft new H.248.RTCPPROF "Guidelines on the use of H.248 capabilities for performance monitoring in RTP networks in H.248 Profiles" (Ed. 0.7).

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.RTCPPROF Ed. 0.1 | Initial draft (TD-24) from WP 2/16 Geneva meeting, Feb. 2012 Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1202_Gen/TD-24.zip |
| H.248.RTCPPROF Ed. 0.2 | Input (TD 747 (WP2/16)) from ITU-T SG16 Geneva meeting, Apr/May 2012 Location: http://itu.int/md/T09-SG16-120430-TD-WP2-0747 |
| H.248.RTCPPROF Ed. 0.3 | Output (TD 747R1/WP2) from ITU-T SG16 Geneva meet., Apr/May 2012 Location: http://itu.int/md/T09-SG16-120430-TD-WP2-0747R1 |
| H.248.RTCPPROF Ed. 0.4 | Input (AVD-4234) to WP 2/16 Brisbane meeting (2012-09) Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/AVD-4234.zip |
| H.248.RTCPPROF Ed. 0.5 | Output (TD-22) from WP 2/16 Brisbane meeting (2012-09) Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/TD-22.zip |
| H.248.RTCPPROF Ed. 0.6 | Input (TD-23/WP1) from ITU-T SG16 Geneva meeting, January 2013 Location: http://www.itu.int/md/meetingdoc.asp?lang=en&parent=T13-SG16-130114-TD-WP1-0023 |
| H.248.RTCPPROF Ed. 0.7 | Output (TD 23R1/WP1) from ITU-T SG16 Geneva meet., Jan 2013 Location: |
| H.248.RTCPPROF Ed. 0.8 |  |

CONTENTS

Page

1 Scope 7 1.1 Framework and layout of this Recommendation 8 2 References 10 3 Definitions 11 3.1 Terms defined elsewhere 11 3.2 Terms defined in this Recommendation 11 4 Abbreviations and acronyms 12 5 Conventions 14 5.1 Prescriptive language for requirements specification 14 6 Relation to other ITU-T H.248.x-series Recommendations 14 6.1 ITU-T Rec. H.248.30: RTCP extended performance metrics packages 14 6.2 ITU-T Rec. H.248.48: RTCP XR Block Reporting Package 14 6.3 Expired ITU-T proposal H.248.xnq: Extended Network Quality Metrics Packages for Next Generation Networks 14 7 Classification of performance metrics 14 8 Roles provided by MG 15 8.1 H.248 MG as general measurement point (MP) 17 8.2 H.248 MG as location-dependent measurement point (MP) 17 8.2.1 Location-independent requirements 17 8.3 H.248 MG as collection point (CP) 18 8.4 H.248 MG as reporting point at IP bearer interface (RPRTCP) 18 8.5 H.248 MG as reporting point at H.248 interface (RPH.248) 18 8.6 H.248 MG as filtering point (for RTCP traffic) (FP) 18 8.7 H.248 MG as loopback point (for RTP/RTCP traffic) (LP) 19 8.8 Support of specific RTP topologies 20 9 H.248 profile specification guidelines 20 9.1 Profile Identification 20 9.2 Summary 20 9.3 Gateway Control Protocol Version 20 9.4 Connection Model 20 9.5 Context Attributes 20 9.6 Terminations 20 9.7 Descriptors 21 9.7.1 Stream Descriptor 21 9.7.2 Events Descriptor 21 9.7.3 EventBuffer Descriptor 21 9.7.4 Signals Descriptor 21 9.7.5 DigitMap Descriptor 21 9.7.6 Statistics Descriptor 21 9.7.7 ObservedEvents Descriptor 21 9.7.8 Topology Descriptor 21 9.7.9 Error Descriptor 21 9.8 Command API 22 9.8.1 Add 22 9.8.2 Modify 22 9.8.3 Subtract 22 9.8.4 Move 22 9.8.5 AuditValue 22 9.8.6 AuditCapabilities 22 9.8.7 Notify 22 9.8.8 ServiceChange 22 9.8.9 Manipulating and auditing context attributes 22 9.9 Generic command syntax and encoding 22 9.10 Transactions 22 9.11 Messages 22 9.12 Transport 22 9.13 Security 23 9.14 Packages 23 9.14.1 RTCP XR Base Package 23 9.14.2 RTCP XR Burst Metrics Package 23 9.14.3 Received RTCP XR Package 23 9.14.4 Received RTCP XR Burst Metrics Package 23 9.14.5 RTCP XR Block Reporting Package 23 9.14.6 RTCP Source Description Package 23 9.15 Mandatory support of SDP and ITU-T H.248.1 Annex C information elements 24 9.16 Optional support of SDP and ITU-T H.248.1 Annex C information elements 24 9.17 Procedures 24 9.17.1 Measurement points 24 9.17.2 Reporting point at gateway control interface (H.248 statistics) 24 Appendix I Example use cases for H.248-based Performance Monitoring in RTP networks 25 I.1 Overview 25 I.2 Use case 1: H.248 MG with connection model (RTP, non-RTP), e.g. Trunking gateway 25 I.3 Use case 2: IP-IP H.248 MG and remote SIP UE – Single control plane RP, by MG 27 I.4 Use case 3: IP-IP H.248 MG and remote SIP UE – Single control plane RP, by UE 28 I.5 Use case 4: IP-IP H.248 MG and remote SIP UE – Single location of MPs ("LP by MG") 30 I.6 Use case 5: Distributed control plane reporting with near-end measurements only (at the example of a Trunking gateway) 31 I.7 Use case 6: IP-IP H.248 MG as peering node 33 I.8 Use case 7: Multiple IP network domains – segment-based measurements 34 I.9 Use case 8: MP relocation by loopbacking 36 Appendix II Design of the expired extended network quality base (XNQ) package 38 II.1 Introduction 38 II.2 Extended Network Quality Base Package 38 Appendix III Example performance metric types 43 III.1 Background 43 III.2 Transport level metrics 43 III.2.1 RTP-agnostic transport level metrics 43 III.2.2 RTP-aware transport level metrics 43 III.3 Application level metrics 44 III.3.1 RTP-agnostic application level metrics 44 III.3.2 RTP-aware application level metrics 44 Appendix IV Relation between "IETF RTCP XR" documents and ITU-T H.248.x work products 45 IV.1 Background 45 Bibliography 48

List of Tables

Page

Table II.1 – Properties for the extended network quality base (XNQ) package 39 Table II.2 – Statistics for the extended network quality base (XNQ) package 40 Table II.3 – Error codes for the extended network quality base (XNQ) package 42 Table IV.1 – "RTCP XR"-based Performance Monitoring: Differences H.248.30 vs H.248.xnq vs H.248.48 – Part 1 47 Table IV.2 – "RTCP XR"-based Performance Monitoring: Differences H.248.30 vs H.248.xnq vs H.248.48 – Part 2 48

List of Figures

Page

Figure 1 – Scope, structuring principle and framework of this Recommendation 9 Figure A – Packet handling with loopback path options 12 Figure 2 – Protocol stack based categories of performance metrics 15 Figure 3 – RTCP-based performance measurements – H.248 IP-to-IP border gateway/router – Unidirectional, ingress only RTP/RTCP processing model 16

Draft new Recommendation ITU-T H.248.xx (ex H.248.RTCPPROF)

Gateway control protocol: 
Guidelines on the use of H.248 capabilities for performance monitoring in RTP networks in H.248 Profiles

AAP Summary

<Mandatory material – to be provided before Consent>

Summary

<Mandatory material>

Summary

This Recommendation provides guidelines on the use of #### in H.248 profiles for ###.  These guidelines may be used by other Standards Developing Organization (SDOs) when defining their H.248.1 profiles in support of ###.

# 1	Scope

The scope of this Recommendation covers performance monitoring in RTP networks with the involvement of H.248 entities. Performance monitoring as such is a network service across multiple network entities which are located in user, control and/or management planes. The scope of this Recommendation is limited to the user/control/management plane interfaces of H.248 entities. The notion of performance monitoring is used to emphasize a call/session dependent service, which is basically in contrast to the legacy performance management capability, which is a pure management plane function [ITU-T M.3010, ITU-T M.60].

Performance monitoring could be considered as a network overlay function.

This Recommendation provides (and other aspects):

- an overview of the existing "entire H.248 toolkit" for RTCP extension report (XR) support, i.e., the set of available H.248 packages in that area;

- an overview of available H.248 packages for non-XR related performance metrics, typically subject of RTCP basic report (SR, RR) capabilities;

- exemplary network use cases with different H.248 MG types (primarily physical-to-RTP gateways, media-agnostic IP-to-IP gateways, RTP-topology-aware RTP-to-RTP gateways);

- collection of functional requirements in order to motivate associated profile guidelines;

- classification of performance metrics (such as application-level versus transport-level metrics) in order to indicate applicability or not for H.248 MGs;

- principal H.248 MG tasks of

  - measurements of RTP traffic,

  - reporting of measurement data via RTCP XR reports,

  - collection of RTCP XR carried measurement data,

  - loopback of RTP (in order to relocate a measurement point),

  - filtering of RTCP XR reports,

  - modification of RTCP XR reports and/or

  - reporting of measurement data via H.248 statistics to the MGC

  - control of such tasks via H.248 that could be addressed in a H.248 profile, i.e.

    - the identification of appropriate H.248 packages

    - the detailed specification of package usage

    - the example call-dependent procedures

- the association to other related H.248.x Recommendations such as [ITU-T H.248.71], [ITU-T H.248.79], [ITU-T H.248.LOOPB] and [ITU-T H.248.RTPTOPO]

- control of measurement methods (such as interval, cumulative and alert type of measurements)

- usage of H.248.47-based conditional reporting [ITU-T H.248.47]

The Recommendation also discusses the purpose of performance monitoring (and therefore possible motivation of a use case). For example:

- capture of Grade-of-Service (GoS) related data in order to evaluate network conditions

- capture of Quality-of-Service (QoS) and Quality-of-Experience (QoE) related data in order to evaluate service conditions (e.g. as part of service level agreements)

- capture of billing/charging related data

- capture of data as input for routing algorithms

- capture of data as input for network node selection algorithms

- capture of data as input for centralized servers as support of network operations

- capture of data as part of online tests

## 1.1	Framework and layout of this Recommendation

Scope and purposes may be summarized by the diagram in Figure 1, which illustrates the framework of this Recommendation.

Figure 1 – Scope, structuring principle and framework of this Recommendation

The primary audience of this Recommendation are authors of H.248 profile specifications, which aim to support a particular network performance monitoring use case, and the consideration of a specific location and/or type of an H.248 MG.

Performance monitoring support could basically affect three areas of an H.248 profile:

- basic protocol capabilities (such as connection model);

- packages and their detailed usage; and/or

- call-dependent procedures.

This Recommendation provides guidelines for these profile areas. Typically to define a H.248 profile there are several steps that are followed, according to Figure 1, given by

- requirements stage: firstly selecting use case(s) and the identification of required MG behaviour in terms of measurements, reporting, filtering, etc. capabilities;

- protocol stage: secondly, addressing the necessary templates of the indicated profile areas and recommended input; and

- profile stage: thirdly, incorporating "guidelines" in an actual profile specification and adapting the templates to the concrete specification setting.

This Recommendation is organized as follows:

- examples use cases are collected in Appendix I;

- principal MG behaviour is defined in clause 8 by considering the set of requirements behind such MG roles (and the used prescriptive language for requirements is outlined in clause 5);

- an technology overview of existing H.248 packages is in scope of clause 6;

and finally

- guidelines for the three profile areas are subject of clause 8 (which follows the profile structuring of the profile template according to [ITU-T H.248.1]).

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2005), Gateway control protocol: Version 3, including its Amendment 2 (12/2009).

[ITU-T H.248.30]	Recommendation ITU-T H.248.30 (2007), Gateway control protocol: RTCP extended performance metrics packages.

[ITU-T H.248.47]	Recommendation ITU-T H.248.47 (2008), Gateway control protocol: Statistic conditional reporting package.

[ITU-T H.248.48]	Recommendation ITU-T H.248.48 (2012), Gateway control protocol: RTCP XR Block Reporting Package.

[ITU-T H.248.71]	Recommendation ITU-T H.248.71 (2010), Gateway control protocol: RTCP support packages.

[ITU-T H.248.76]	Recommendation ITU-T H.248.76 (2010), Gateway control protocol: Filter group package and guidelines.

[ITU-T H.248.79]	Recommendation ITU-T H.248.79 (2012), Gateway control protocol: Guidelines for Packet-Based Streams.

[ITU-T H.248.85]	Draft Recommendation ITU-T H.248.85 (201x), Gateway control protocol: Usage of Loopback in H.248.

[ITU-T H.248.RTPTOPO]	Draft Recommendation ITU-T H.248.RTPTOPO (201x), Gateway control protocol: RTP topology dependent RTCP handling by H.248 Media Gateways with IP Terminations.

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications

[IETF RFC 3611]	IETF RFC 3611 (2003), RTP Control Protocol Extended Reports (RTCP XR)

[IETF RFC 6792]	IETF RFC 6792 (11/2012), Guidelines for Use of the RTP Monitoring Framework.

{Editor’s note: to be checked, too:

[IETF RFC 6776]	IETF RFC 6776 (10/2012), Measurement Identity and Information Reporting Using a Source Description (SDES) Item and an RTCP Extended Report (XR) Block.

# 3	Definitions

This Recommendation defines the following terms:

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	measurement point (MP) (see [ITU-T H.248.71]): An MP is the physical or logical point at which measurements can be made and to which the data obtained is related. In the context of RTP and RTCP, any source of RTCP packets is a measurement point. Its measurement data is based on the RTP and RTCP traffic sent and received at that element.

3.1.2	reporting point (RP) (see [ITU-T H.248.71]): This is the point at which the measurement data is reported to another network element. In the context of RTP and RTCP, any source of RTCP packets is a reporting point, where the measurement data is reported through RTCP. In addition, any ITU-T H.248 MG is a reporting point, when reporting various measurement data through ITU-T H.248 statistics.

## 3.2	Terms defined in this Recommendation

3.2.1	application level metrics: a protocol stack based classification of a performance metric. Such a performance metric uses protocol information elements from the application layer and possibly from lower layer protocols (NOTE 1). Some application level metrics are furthermore coupled to the termination of the application protocol (NOTE 2) and/or the location of user equipment (NOTE 3).

NOTE 1 – Example: an H.248 IP-IP MG would be typically configured for a "media-aware" mode in order to have sufficient information about the carried application traffic with regards to the application level performance metric.

NOTE 2 – The termination of the application protocol correlates typically with RTP topology "RTP end system".

NOTE 3 – Example: the category of "QoE" based performance metrics as application level metrics.

3.2.2	collection point (CP) (adapted from clause 7.6.2/[ITU-T H.248.71]): An CP is the physical or logical point which records/collects remote measurement data, as received via a user plane reporting interface (here via RTCP report packets).

3.2.3	filtering point (FP) (adapted from filter definition of [ITU-T H.248.76]):

Location of policy rule enforcement for filtering information. Such specific policy rules are often simply termed as filter rules. In the context of this Recommendation,

- the filter rule conditions are primarily related to protocol control information elements related to RTP and RTCP header fields (e.g., payload/packet type, block type), but may also be based on carried measurement data; and

- the filter rule actions generally cover forwarding, blocking, counting or modification of data units at the levels of RTP/RTCP packets, reports down to individual measurement data.

NOTE 4 – Specification and syntax of such policy rules are out of scope of this Recommendation.

3.2.4	loopback point (LP) (adapted from [ITU-T H.248.85]): Location of a loopback function. In the context of this Recommendation, a loopback function comprises a copy and forward operation at the level of protocol data information, with primary scope on "IETF Media Loopback" (according clause 3.2.3/[ITU-T H.248.85]).

NOTE 5 – A loopback function is different in comparison to a redirect function, which does not copy any protocol data.

NOTE 6 – The loopback function is called "loopback mirror" in [b-IETF LOOPBACK].

{Editor's note (2012-09 meeting; the "H.248 Media Loopback" does relate to a redirect function only. There might be also use cases, in the context of performance monitoring, for such a loopback function. E.g., for offline test procedures. Hm? The primary scope is the IETF media loopback

Figure A – Packet handling with loopback path options

…}

3.2.5	Subject to profile specification: When used this term indicates that the H.248 profile template section requires further specification by a profile specification.

3.2.6	transport level metrics: a protocol stack based classification of a performance metric. Such a performance metric could be realized solely on protocol information elements from the transport layer and lower layer protocols.

NOTE 7 – Example: an H.248 IP-IP MG in "media-agnostic, transport-aware" mode could still support measurements for transport level metrics

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations:

| BGW | Border Gateway |
| --- | --- |
| BT | Block Type (RTCP XR) |
| CP | Collection Point |
| FP | Filtering Point |
| GoS | Grade of Service |
| IP | Internet Protocol |
| ISDN | Integrated Services Digital Network |
| LP | Loopback Point |
| Lx | Layer number |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| MP | Measurement Point |
| N/A | Not Applicable |
| NE | Near-end |
| NGN | Next Generation Network |
| OP | Originating Point ("source of RTP/RTCP traffic") |
| PSTN | Public Switched Telephone Network |
| QoE | Quality of Experience |
| QoS | Quality of Service |
| RP | Reporting Point |
| RPH.248 | RP H.248 (control plane interface) |
| RPRTCP | RP RTCP (user plane interface) |
| RPSIP | RP SIP (control plane interface) |
| RR | Receiver Report (RTCP) |
| RTCP | RTP Control Protocol |
| RTP | Real-time Transport Protocol |
| SDO | Standard Development Organisation |
| SDP | Session Description Protocol |
| SIP | Session Initiation Protocol |
| SR | Sender Report (RTCP) |
| TGW | Trunking Gateway |
| UDP | User Datagram Protocol |
| UE | User Equipment |
| VoIP | VoIP |
| XR | eXtension Report (RTCP) |

# 5	Conventions

## 5.1	Prescriptive language for requirements specification

This document provides a list of items (in clause 7), labelled as R-x/y, where x refers to the clause number and y a number within that clause. Such items use the following keywords with meanings as prescribed below:

- The keywords "is required to" indicate a requirement which must be strictly followed and from which no deviation is permitted if conformance to this document is to be claimed.

- The keywords "is prohibited from" indicate a requirement which must be strictly followed and from which no deviation is permitted if conformance to this document is to be claimed.

- The keywords "is recommended" indicate a requirement which is recommended but which is not absolutely required.  Thus this requirement need not be present to claim conformance.

- The keywords "can optionally" indicate an optional requirement which is permissible, without implying any sense of being recommended. This term is not intended to imply that the vendor's implementation must provide the option and the feature can be optionally enabled by the network operator/service provider.  Rather, it means the vendor may optionally provide the feature and still claim conformance with the specification.

# 6	Relation to other ITU-T H.248.x-series Recommendations

Purpose of this clause is to identify possible relations (or not) to other Recommendations and past work on H.248 profiles. An overview and more background information is provided by Appendix IV. Below are the two in force H.248 Recommendations with support of RTCP extension reports and an expired work item for additional background information.

## 6.1	ITU-T Rec. H.248.30: RTCP extended performance metrics packages

This Recommendation is tightly coupled to [IETF RFC 3611]. The RTCP performance metrics are directly mapped on H.248 statistics. Near- and far-end measurements are supported.

## 6.2	ITU-T Rec. H.248.48: RTCP XR Block Reporting Package

This Recommendation addresses the concerns of above mentioned limitations. The H.248 package and comprehensive measurement concept should obsolete above two Recommendations.

## 6.3	Expired ITU-T proposal H.248.xnq: Extended Network Quality Metrics Packages for Next Generation Networks

This expired draft (see Appendix II) was developed in order to address the limitations of the [ITU-T H.248.30] defined measurement framework, as well as the IETF work on additional RTCP performance metrics. The work on the H.248 packages was completed, but the draft was not progressed or published by ITU-T in favour of a design change for a solution that is future proof and decoupled from IETF standardization development.

# 7	Classification of performance metrics

Performance metrics for RTP traffic could be structured in several categories. The following classification characteristics are useful for this Recommendation:

- protocol stack based ("what kind of protocol information elements are part of a metric definition);

- traffic direction based ("metrics on basis of the bidirectional information of an RTP session or unidirectional only"); and

- network route based ("metrics based on network topology and RTP/RTCP traffic routes (such as metrics related to round trips, one-ways, loopbacks, etc.)").

The various categories are relevant concerning possible MG behaviour (see clause 8). Figure 2 illustrates the protocol stack based classification scheme.

Figure 2 – Protocol stack based categories of performance metrics

Appendix III provides some examples for each category. For instance, the H.248 statistics of the RTP application data package [ITU-T H.248.58] belong to the category of "RTP-agnostic application level metrics" because their calculation does not require any information from the RTP header, but their scope is application data as carried in the payload of an RTP packet.

# 8	Roles provided by MG

The overall network service of performance monitoring may be structured in a set of capabilities, which could be individually or assigned in combinations to an H.248 MG. The various roles of a MG may be illustrated by a gateway model concerning the processing of incoming RTP and RTCP packets (see Figure 3). The purpose of this clause is the characterization of such roles and the specification of associated requirements (see clauses 8.1ff).

Figure 3 – RTCP-based performance measurements – H.248 IP-to-IP border gateway/router – Unidirectional, ingress only RTP/RTCP processing model

The following separate functions are identified for an MG with an (IP, IP) connection model (Figure 3):

- Generation of local measurements (MPL);

- Reporting of local measurements (RPL,H.248) via H.248 towards MGC;

- Reporting of local measurements (RPL,RTCP) via RTCP towards remote terminals, remote H.248 MGs, remote RTP endsystems, or other RTP entities;

- Collection of remote measurements (CPR,H.248);

- Reporting of remote measurements (RPR,H.248) towards MGC;

- Loopback of incoming RTP/RTCP traffic (LPRTP) back to a remote RTP endsystem [purpose: the remote RTP endsystem may then provide a local measurement point of a remote sink, i.e., loopback allows to relocated MPs];

- Filtering of remote measurements (FPOut) in an outgoing direction, from provider A towards B (which are transported via RTCP reports); and

- Filtering of remote measurements (FPIn) in ingress direction, from provider B (which are transport via RTCP reports).

## 8.1	H.248 MG as general measurement point (MP)

MG capability related requirement:

R-8.1/1:	The MG is required to support local performance measurements.

Technology-dependent requirements related to metric types which are particularly defined for RTP traffic:

NOTE 1 - Subsequent requirements consider "grouped performance metrics" at the top-level of H.248 packages or IETF RFC documents.

R-8.1/2:	The MG is required to execute measurements for the performance metrics as defined by the RTP package [ITU-T H.248.1].

R-8.1/3:	The MG is required to execute measurements for the performance metrics as defined by the RTP application data package [ITU-T H.248.58].

R-8.1/4:	The MG is required to execute measurements for the performance metrics as defined by the Received RTCP package [ITU-T H.248.71].

R-8.1/5:	The MG is required to execute measurements for the performance metrics as defined by RTCP XR block types 1 to 7 [IETF RFC 3611].

NOTE 2 – There are two reporting options via H.248 (RPH.248): [ITU-T H.248.30] or [ITU-T H.248.48], see clause 8.5.

R-8.1/6:	The MG is required to execute measurements for the performance metrics as defined by RTCP XR block type 8 [IETF RFC 5093].

R-8.1/7:	The MG is required to execute measurements for the performance metrics as defined by other RTCP XR block types.

## 8.2	H.248 MG as location-dependent measurement point (MP)

There are two location specific aspects:

- a performance metric itself could be associated to the source, destination or interim node of an end-to-end RTP session (such as RTP packet rate at RTP sender side, as audio signal level at RTP receiver side or RTP packet delay variation observed at an interim RTP mixer); and

- the particular location of an H.248 MG within the (RTP) IP network topology.

### 8.2.1	Location-independent requirements

The following principal requirements are derived:

R-8.2.1/1:	The MG is required to support at least transport level measurements.

NOTE 1 – This is the very minimum requirement, applicable for almost all MG scenarios (related to RTP topologies, related to metric types according clause 3.2.6).

R-8.2.1/2:	The MG is required to support transport level metrics, independent of the supported H.248 connection model ("all IP" or "IP-to-non-IP") and independent of RTP topologies.

### 8.2.2	Location and metric type specific restrictions

Some performance metrics are only applicable for particular locations in a network topology. Especially application level metrics are tied to the source/sink of application level traffic, the location of the final consumer of the service (e.g., the type of metrics related to quality of experience).

R-8.2.2/1:	The MG is prohibited to support application level metrics in such a scenario due to possible violation of the metrics semantic.

NOTE 1 – This requirement is consistent with [IETF RFC 6792], see clause 3.2 "Location of RTP Monitors".

R-8.2.2/2:	The MG is required to notify the MGC of such a violation, if enforced by the MGC for such kind of measurements, by e.g. an error code.

## 8.3	H.248 MG as collection point (CP)

The collection function is based on incoming RTCP data.

R-8.3/1:	The MG is required to collect remote measurement data via incoming RTCP reports.

R-8.3/2:	The MG is required to identify remote RTP nodes (via the H.248 RTCP source description package according [ITU-T H.248.71]) in the bearer path.

## 8.4	H.248 MG as reporting point at IP bearer interface (RPRTCP)

This interface is used for the exchange of measurement data and information for the production of measurements via RTCP reports.

R-8.4/1:	The MG is required to send RTCP basic reports (SR and/or RR)..

R-8.4/2:	The MG is required to receive RTCP basic reports (SR and/or RR).

R-8.4/3:	The MG is recommended to send RTCP extension reports (XR; NOTE 1).

R-8.4/4:	The MG is recommended to receive RTCP extension reports (XR; NOTE 1).

NOTE 1 – This requirement could be further detailed to the block type (BT) level or below the BT level down to individual metrics.

## 8.5	H.248 MG as reporting point at H.248 interface (RPH.248)

This reporting capability is tightly coupled to the H.248 signalling capability according to individual H.248 packages:

R-8.5/1:	The MG is required to support H.248 statistics according to the RTP package [ITU-T H.248.1].

R-8.5/2:	The MG is required to support H.248 statistics according to the RTCP extended performance metrics package [ITU-T H.248.30].

R-8.5/3:	The MG is required to support H.248 statistics according to the RTCP XR Block Reporting package [ITU-T H.248.48].

R-8.5/4:	The MG is required to support H.248 statistics according to the RTP application data package [ITU-T H.248.58].

R-8.5/5:	The MG is required to support H.248 statistics according to the Received RTCP package [ITU-T H.248.71].

## 8.6	H.248 MG as filtering point (for RTCP traffic) (FP)

Example scenarios with filtering support are illustrated in Appendix I, see use cases 2, 3, 4, 6 and 7. The specific filter capabilities are basically given by the particular filter rule conditions and filter rule actions (see clause 3.2.3). The level of conditions and actions is useful for the derivation of filter requirements.

Identification requirements, hierarchically ordered:

R-8.6/1:	The MG is required to support filter rule conditions related to the identification of RTP/RTCP traffic as such (e.g., related to the usual 5-tuple for an RTCP control flow (under the condition of separate IP transport connections for RTP and RTCP)).

R-8.6/2:	The MG is required to support filter rule conditions related to the identification of RTCP packets based on the RTCP packet type (PT) codepoint.

R-8.6/3:	The MG is required to support filter rule conditions related to the identification of RTCP report block structures based on the RTCP block type (BT) codepoint.

R-8.6/4:	The MG is required to support filter rule conditions related to the identification of an individual metric within an RTCP report block, based on the report block structure.

Action execution requirements:

a) action type "forwarding":

R-8.6/5:	The MG is required to support unmodified forwarding of information entities according requirements R-8.6/1-4, as the default action.

b) action type "blocking":

R-8.6/6:	The MG is required to support blocking of the entire RTCP traffic.

NOTE 1 – This action may be subject of "abnormal traffic handling", but may violate end-to-end RTCP protocol applications.

R-8.6/7:	The MG is required to support blocking of specific RTCP packet types.

NOTE 2 – This action may relate to the discard of an entire RTCP packet or the removal of an individual RTCP packet unit in case of a compound RTCP packet.

R-8.6/8:	The MG is required to support blocking of specific RTCP block types.

NOTE 3 – The identified RTCP block would be entirely removed from an RTCP XR packet.

c) action type "modification":

R-8.6/9:	The MG is required to support the modification of an individual measurement value, i.e., the carried value of a specific metric type in an RTCP packet. Any concrete modification operation (such as a default value setting, value resetting, setting of a specific value) is out of scope of this Recommendation.

NOTE 4 – Such an action is typically requested by a network operator, which do not want to disclose his own measurement results to peering partners.

All above actions should be executed in a protocol correct manner, i.e., any modified RTP/L4/IP packet should be afterwards syntactically correct, which may imply the adaptation of length, padding and/or checksum information.

## 8.7	H.248 MG as loopback point (for RTP/RTCP traffic) (LP)

There are multiple loopback options:R-8.7/1:	The MG is required to support a LP in "native H.248" mode, i.e. due to a StreamMode property value LoopBack setting.

R-8.7/2:	The MG is required to support a LP according to [b-IETF LOOPBACK] in "encapsulated packet loopback" mode.

R-8.7/3:	The MG is required to support a LP according to [b-IETF LOOPBACK] in "direct loopback" mode.

R-8.7/4:	The MG is required to support a LP according to [b-IETF LOOPBACK] in "media loopback" mode.

## 8.8	Support of specific RTP topologies

The relation between RTCP services and RTP topologies in H.248 entities is described in [ITU-T H.248.RTPTOPO]. Performance monitoring based on RTCP XR belongs to supplementary RTCP services which are basically an overlay function and hence independent of RTP topologies, whereas performance monitoring using RTCP basic reports (such as SR, RR) would depend on RTP topologies.

R-8.9/1:	The MG is required to support ###.

# 9	H.248 profile specification guidelines

This clause provides guidelines for H.248 profile specifications. The structure follows the profile template according to Appendix III/[ITU-T H.248.1].

The template elements which are not applicable in this Recommendation are indicated by "Subject to profile specification".

{Editor's note (2012-02 meeting): clauses 9.1 to 9.13 may be finally shrinked if all content would be subject of profile specifications ….}

## 9.1	Profile Identification

Subject to profile specification.

## 9.2	Summary

Subject to profile specification.

## 9.3	Gateway Control Protocol Version

Subject to profile specification.

## 9.4	Connection Model

| Maximum number of contexts: | N/A |
| --- | --- |
| Maximum number of terminations per context: | N/A |
| Allowed termination type combinations in a context: | Context (one or more IP terminations) |

NOTE – The scope of this Recommendation is only applicable to H.248 gateways in IP networks.

## 9.5	Context Attributes

Subject to profile specification.

## 9.6	Terminations

Subject to profile specification.

## 9.7	Descriptors

### 9.7.1	Stream Descriptor

Subject to profile specification.

NOTE – H.248-based performance monitoring functions at RTP stream/terminations does not have implications on this Descriptor.

### 9.7.2	Events Descriptor

Subject to profile specification.

NOTE – H.248-based performance monitoring functions at RTP stream/terminations does not have implications on this Descriptor.

### 9.7.3	EventBuffer Descriptor

Subject to profile specification.

NOTE – H.248-based performance monitoring functions at RTP stream/terminations does not have implications on this Descriptor.

### 9.7.4	Signals Descriptor

Subject to profile specification.

NOTE – H.248-based performance monitoring functions at RTP stream/terminations does not have implications on this Descriptor.

### 9.7.5	DigitMap Descriptor

Subject to profile specification.

### 9.7.6	Statistics Descriptor

Subject to profile specification.

NOTE – H.248-based performance monitoring functions at RTP stream/terminations may demand for support of statistics in case of a RPH.248 requirement.

### 9.7.7	ObservedEvents Descriptor

Subject to profile specification.

NOTE – H.248-based performance monitoring functions at RTP stream/terminations does not have implications on this Descriptor.

### 9.7.8	Topology Descriptor

Subject to profile specification.

NOTE – H.248-based performance monitoring functions at RTP stream/terminations does not have implications on this Descriptor.

### 9.7.9	Error Descriptor

Which H.248.8 and package defined error codes are supported?

Error Codes sent by the MGC:

| Supported H.248.8 Error Codes: | Subject to profile specification |
| --- | --- |
| Supported Error Codes defined in packages: | Subject to profile specification |

Error Codes sent by the MG:

| Supported H.248.8 Error Codes: | All H.248.8 error codes or a list of the individual error code numbers |
| --- | --- |
| Supported Error Codes defined in packages: | Subject to profile specification |

## 9.8	Command API

NOTE – It is assumed that an Error Descriptor may be returned in any command reply.

### 9.8.1	Add

### FIXTHIS ###

### 9.8.2	Modify

### FIXTHIS ###

### 9.8.3	Subtract

### FIXTHIS ###

### 9.8.4	Move

### FIXTHIS ###

### 9.8.5	AuditValue

### FIXTHIS ###

### 9.8.6	AuditCapabilities

### FIXTHIS ###

### 9.8.7	Notify

### FIXTHIS ###

### 9.8.8	ServiceChange

### FIXTHIS ###

### 9.8.9	Manipulating and auditing context attributes

### FIXTHIS ###

## 9.9	Generic command syntax and encoding

Subject to profile specification.

NOTE – There is no impact on encoding.

## 9.10	Transactions

Subject to profile specification.

NOTE – There is no impact on Transactions.

## 9.11	Messages

Subject to profile specification.

NOTE – There is no impact on the number of transaction per message.

## 9.12	Transport

Subject to profile specification.

NOTE – No requirement on any particular H.248 transport modes. However, given the nature of these services, profile specifications could consider the use of a transport that provides large message size handling ("dependent on the amount of H.248 statistics").

## 9.13	Security

Subject to profile specification.

NOTE – There is no impact on H.248 transport security.

## 9.14	Packages

Although ITU-T H.248.RTCPPROF does not mandate any H.248 package for profile specifications that are based on this Recommendation, there may be some useful packages for supporting H.248-based performance monitoring of RTP networks. The following is a non-exhaustive list of available H.248 capabilities:

- H.248 controlled MPs:

  - [ITU-T H.248.1], Annex E.12 for …

  - [ITU-T H.248.30] for …

The use of the above Recommendations has not been taken into account in the definition of this profile specification guide.

NOTE – The above Recommendations contain many different properties, signals, events however, as they are optional to simplify the definition of this profile specification guideline, these impacts are not listed.

### 9.14.1	RTCP XR Base Package

(if [ITU-T H.248.30] approach)

### 9.14.2	RTCP XR Burst Metrics Package

(if [ITU-T H.248.30] approach)

### 9.14.3	Received RTCP XR Package

(if [ITU-T H.248.30] approach)

### 9.14.4	Received RTCP XR Burst Metrics Package

(if [ITU-T H.248.30] approach)

### 9.14.5	RTCP XR Block Reporting Package

(if [ITU-T H.248.38] approach)

### 9.14.6	RTCP Source Description Package

(possibly, if [ITU-T H.248.48] approach)

## 9.15	Mandatory support of SDP and ITU-T H.248.1 Annex C information elements

Subject to profile specification.

NOTE – ### has no impact on SDP or ITU-T H.248.1 Annex C elements.

## 9.16	Optional support of SDP and ITU-T H.248.1 Annex C information elements

Subject to profile specification.

NOTE – ### has no impact on SDP or ITU-T H.248.1 Annex C elements.

## 9.17	Procedures

### 9.17.1	Measurement points

#### 9.17.1.1	Enabling of MPs

### FIXTHIS ###

#### 9.17.1.2	Disabling of MPs

### FIXTHIS ###

### 9.17.2	Reporting point at gateway control interface (H.248 statistics)

#### 9.17.2.1	Enabling of statistics (RPH.248)

### FIXTHIS ###

#### 9.17.2.2	Disabling of statistics (RPH.248)

### FIXTHIS ###

Appendix I

Example use cases for 
H.248-based Performance Monitoring in RTP networks

(This appendix does not form an integral part of this Recommendation)

## I.1	Overview

This Appendix studies a number of example use cases, which demonstrate all type of conceivable requirements, behaviour and capabilities as possibly requested from H.248 MGs, involved in performance monitoring of RTP traffic. Each use case is illustrated by a diagram, and the set of considered capabilities is structured in multiple steps (like measurement, reporting and/or filtering points; per traffic direction) in order to point out the specific aspects.

The use cases are ordered according to increasing functionality, from perspective of H.248-based performance monitoring in RTP networks.

## I.2	Use case 1: H.248 MG with connection model (RTP, non-RTP), e.g. Trunking gateway

Figure I.1 illustrates an end-to-end media/bearer path between user equipment (UE) in a PSTN and VoIP network domain. The interworking of the two different user plane protocol stacks is provided by an H.248 physical-to-RTP media gateway (such as a trunking gateway (TGW) in this example). It may be noted that the TGW is located at the edge of the RTP domain, hence providing the "RTP endsystem" topology (according to [IETF RFC 5117]). However, the "RTP endsystem" is located at core network level, far away from the real user location ("the remote PSTN terminal UE B"). This observation is important concerning performance measurements with respect to user specific metrics (such as of type "QoE").

Figure I.1 – Use case 1: H.248 MG with connection model (RTP, non-RTP)

The measurement segment (i.e., IP network domain Xd) is bounded by UE A and H.248 MG Nk. Measurements for RTP traffic are provided by the sink location, i.e., there are MPs at the TGW in A-to-B direction (1) and the UE A in reverse direction (2).

Observation I.1.a:

The set of supported performance metrics are different in the two MPs. The H.248 MG is limited to transport level metrics only. The MG should thus provide correspondent feedback (e.g., by an error code) to a MGC, which tries to enforce the measurement of application level metrics.

The performance monitoring network is engineered in such a way, that the reporting of measurement results towards control plane is solely under responsibility of H.248 MGs ("and not of VoIP terminal equipment"). There are consequently H.248-based reporting points (RPH.248). UE A has therefore to transfer the measurements of MPT and MPA via RTCP ("the RPRTCP of UE A ") back to the H.248 MG, which collects the data (via CP) and reports them typically at a later point in time (e.g., when the call is terminated) via RPH.248 as H.248 statistics.

Observation I.1.b:

No RPRTCP support is required by the H.248 MG because there is no usage of correspondent RTCP data by UE A.

It may be further recalled again that circuit-oriented UEs (like PSTN or ISDN terminals) do not support the generation of measurements for user plane traffic and their reporting ("hence, there is no MP and RP provided by UE B in Figure I.1").

## I.3	Use case 2: IP-IP H.248 MG and remote SIP UE – Single control plane RP, by MG

The end-to-end media/bearer path is now continuously RTP based (Figure I.2). The H.248 MG provides the role of a so called border gateway (BGW), characterized by an H.248 (IP, IP) connection model.

Figure I.2 – Use case 2: IP-IP H.248 MG and remote SIP UE – Single control plane RP, by MG

Observation I.2.a:

There is a single H.248 connection model (IP, IP), but a variety of different possible RTP topologies concerning the interconnection of RTP traffic between UEs A and B (see [ITU-T H.248. RTPTOPO]). The various RTP modes of operations as assigned to the BGW may impact measurements, filtering and/or reporting capabilities.

The evaluation of RTP topology dependencies is out of scope of this example. The H.248 MG provides MPs in A-to-B (1) and B-to-A (2.1) direction. The two MPs in B-to-A direction, (2.1) and (2.2) are necessary in order to estimate the impact of incoming RTP traffic from domain Xd+1 on domain Xd.

Observation I.2.b:

The number of MPs in a unidirectional traffic path may differ between the two traffic directions, - from perspective of a particular network operator, - from perspective of the MG location in the end-to-end RTP path.

The reporting framework, - in user and control plane -, is identical as in use case #1.

Additional filtering capabilities:

- Network operator Xd wants to avoid the distribution of RTCP transferred measurement data outside his network. There is thus a FP in A-to-B direction located at the MG (5).

- There may be incoming RTCP reports from peering partner Xd+1, which could be filtered by the MG (6). Multiple filtering options might be imaginable: e.g., transparent forwarding, blocking of entire RTCP reports or early selected deletion of unwanted measurement data.

There are many conceivable variations of this use case. E.g., FP (6) could be augmented or replaced by a CP and RPH.248, e.g. in order to capture all incoming RTCP data already at the border of domain Xd.

## I.4	Use case 3: IP-IP H.248 MG and remote SIP UE – Single control plane RP, by UE

The network configuration in this example is identical to the previous use case #2, however, the control plane reporting points are removed from the H.248 MGs towards the SIP-based UEs. This is a feasible network option when there are other control plane protocols with adequate reporting capabilities (Note: SIP is such an example, which provides at least initial support, but not as complete and powerful capabilities in comparison to H.248).

Figure I.3 – Use case 3: IP-IP H.248 MG and remote SIP UE – Single control plane RP, by UE

Observation I.3.a:

The two control plane reporting concepts, - network centralized RPs located in H.248 MGs (use case #1 and #2) versus terminal-distributed RPs as provided e.g. by SIP UEs -, are not mutual exclusive design alternatives. There are pros and cons (such as cost factors, protocol support, implementation status, etc.) behind each network engineering and operation approach; their evaluation is out of scope of this Recommendation.

The H.248 MG is now requested to support RTCP-based reporting, see RPRTCP (3.1) and (3.2). The SIP UE collects the data and provides finally a SIP-based reporting (together with the UE-local measurements (2.2)).

No H.248 Statistics are requested on the H.248 MG; the RPH.248 is hence lacking in this example.

Additional filtering capabilities:

- Any FP in egress direction is not required (5), because the SIP UEs could already suppress the distribution of RTCP measurement reports.

- The ingress situation is similar as in previous use case #3.

## I.5	Use case 4: IP-IP H.248 MG and remote SIP UE – Single location of MPs ("LP by MG")

The MP is normally at the same location as the projected observation point (as in previous examples). However, the MP could be geographically relocated from the observation point in case of an available transport means for transferring the concerned traffic data from the observation point to a remote location. "Loopbacking" is such a capability: the traffic is redirected at the loopback point (LP) in the reverse direction.

Observation I.4.a:

Loopbacking permits the geographical relocation of the MP, "anywhere" in the "loopback path". It may be noted that the relocated MP doesn't have to be at the location of the originating point (OP); it may also be in between the LP and OP section.

[IETF LOOPBACK] defines a "media loopback" capability, which could be supported by H.248 entities (see [ITU-T H.248.LOOPB]). The notion of "media" is primarily related to all kind of application data carried via RTP. Figure I.4 depicts a use case with an enabled "media loopback" at the H.248 BGW.

Figure I.4 – Use case 4: IP-IP H.248 MG and remote SIP UE – Single location of MPs 
("LP by MG")

The H.248 BGW provides a loopback point (LP) for RTP traffic originating from UE A. The IETF "loopback source" and "loopback mirror" roles are assigned to UE A and BGW respectively in this example. The motivation for using an LP instead of a local MP at an H.248 MG is elaborated in more detail in clause I.8.

## I.6	Use case 5: Distributed control plane reporting with near-end measurements only (at the example of a Trunking gateway)

This is a variation of use cases #1 and #3, whereby control plane reporting is distributed between two RTP nodes (Figure I.5). The MPs are identical as in use case #1, but each RTP node is reporting their own measurements, also known as near-end measurements (see also [ITU-T H.248.71]).

UE A provides a SIP-based reporting interface (RPSIP, e.g. based on [IETF RFC 6035]). The H.248 MG (here TGW, but might be also a BGW) reports its near-end (NE) measurements (via RPH.248) in the format of H.248 statistics.

Figure I.5 – Use case 5: Distributed control plane reporting with scope on near-end measurements only (at the example of a Trunking gateway)

This use case could be attractive when:

- RTCP XR based reporting is to be avoided ("there is no RPRTCP"); or/and

- measurement and/or reporting capabilities of different RTP node implementations (here UE and MG) follow different roadmaps ("the local scope on NE-only MPs/RPs allows to decouple implementations").

It may be noted that the various control plane reporting capabilities (such as SIP [b-IETF RFC 3261], H.323 [b-ITU-T H.323], MGCP [b-IETF RFC 3435], etc. besides H.248) do not provide adequate support with regards to RTCP XR defined performance metrics and measurement configurations.

## I.7	Use case 6: IP-IP H.248 MG as peering node

Network peering is the scenario when an end-to-end media/bearer path traverses the border between two network operators, service providers, etc. Figure I.6 depicts a typical peering architecture: the last network element at the border could be an H.248 BGW, here node Nk for provider domain Xd and node Nk+1 for provider domain Xd+2. There is sometimes an interim domain Xd+1 (also known as "neutral zone" or "demilitarized zone"), e.g. realized simply as point-to-point L2 link or L3 route.

Figure I.6 – Use case 6: IP-IP H.248 MG as peering node

Provider domain Xd is of interest in use case #6 and H.248 BGW (Nk) selected control plane reporting point. Measurement, reporting and filtering is similar to the previous use cases. The H.248 BGW provides additionally the RPRTCP capability (3.1) and (3.2) for the reporting of MG-local measurements (1) and (2.1) respectively. UE A therefore also has all measurements results available: they could be used by the UE-embedded application control logic.

## I.8	Use case 7: Multiple IP network domains – segment-based measurements

Multiple IP network domains may be interconnected by H.248 BGWs (Figure I.7). Each network domain relates basically to a measurement segment from the perspective of an end-to-end media/bearer path. The primary scope of this use case is a H.248 BGW node Nk and provider domain Xd.

The fundamental challenge of this use case is the assumption that there are different network conditions in each segment, i.e., the Grade of Service (GoS) for IP traffic (here: RTP packets) diverges per network domain Xi. Each provider is interested in trying to figure out the GoS of their peering partners, besides the measurement of his own GoS values. This is in order to estimate the impact on the end-to-end QoS like the QoE conditions for users A and B.

Such an analysis is basically feasible, when the measurement segments would be bounded by H.248 BGWs, the usage of RTCP XR and the H.248.48 based "performance monitoring suite" (e.g., plus H.248.71) in order to configure MPs, CPs, RPs, LPs and/or FPs.

Figure I.7 – Use case #7: Multiple IP network domains – segment-based measurements

Performance monitoring from operator Xd (and H.248 BGW node Nk) perspective:

- capture measurements of RTP traffic entering and leaving domain Xd;, i.e.

  - MPs between Xd and Xd-1, (1.1) and (2.3), and

  - MPs between Xd and Xd+1, (1.2) and (2.1), and

- capture measurements of RTP traffic entering at their source and sink;, i.e.

  - MPs at UE A (2.2) and UE B (not shown in Fig. I.6).

- collect remote measurements via RTCP (1.1), (2.2), (2.3), …;

- distribute local measurements via RTCP (3.1), (3.2), …;

- report H.248 statistics (4.1); and

- filtering options such as (5.1), (5.2) and (6.1).

## I.9	Use case 8: MP relocation by loopbacking

The relocation of a MG-local MP to the remote RTP source endpoint might be beneficial under certain conditions (see also clause I.5). Such an MP relocation could be principally achieved by enabling a loopback point (LP), see Figure I.8.

Figure I.8 – Use case 8: MP relocation by loopbacking

This use case is even more advanced than the one of clause I.5 because the MP is relocated to a previous MG (here BGW node Nk-1), instead of the actual "loopback source" (i.e., OP of UEA here).

General discussion and evaluation of loopbacking in the context of performance monitoring [FIXTHIS: missing a verb in this sentence]:

- Prerequisites for loopbacking

- The "loopbacked media" must contain all necessary information as required by the performance metric(s). This requirement may be denoted as "location-independent measurement parity".
Here: the measurement results (of the MP's) at locations Nk-1 (with loopback) and Nk (without loopbacking) must be identical.

- This condition may be fulfilled for some metrics, but not the plethora of all envisioned RTCP XR defined metrics. The specific measurement algorithm of a performance metric determines the principal applicability (of MP relocation) or not.

- Drawbacks of loopbacking

- There is essentially a doubling of transport capacity at the loopback path.

- Motivation for MP relocation by loopbacking

- Cost factor of MG-local MP ("too expensive for node Nk"), see also clause 8/[ITU-T H.248.79];

- Congestion factor, e.g. temporary unavailability of sufficient measurement capacity;

- Implementation factor, e.g., still lacking measurement capability;

- RTCP factor, e.g., a missing "RTCP XR" interface would prevent the reporting of local measurements, whereas loopbacking just requires the usual RTP interface ("LP in favour of RPRTCP").

- MP relocation to "previous RTP node" (here node Nk-1), instead of RTP source endpoint (here: UEA)?

- Background: The relocated MP could be "somewhere" in the loopback path. There are thus two principal options in this use case (node Nk-1 or UEA).

- The option of node Nk-1 is actually out of scope of [IETF LOOPBACK], but principally feasible in network scenarios with interim H.248 IP-IP gateways (as above illustrated);

- The "node Nk-1" option provides following advantages:

  - e.g. mitigate the basic drawback of increased network transport capacity, or to the example of lacking measurement capability in UE;

  - Actually, the loopback service might be not visible at all for the UE, when node Nk-1 would act as a "virtual loopback source" in above scenario.

Thus, as an overall conclusion: MP relocation by loopbacking is not a scalable option at network level (see also [IETF LOOPBACK]), but might be beneficial and justified under specific conditions.

Appendix II

Design of the expired extended network quality base (XNQ) package

(This appendix does not form an integral part of this Recommendation)

## II.1	Introduction

The extended network quality base package is technically stable and a proven technology in the market. However, the publication was stopped in 2005, in favour of a more future-safe solution, which is now available by H.248.48. Use of the extended network quality base (H.248.xnq) is therefore deprecated.

The package usage detailed template is provided in next sub-clause, due to the fact of existing H.248 profiles which might replace that package in an up-versioned profile.

## II.2	Extended Network Quality Base Package

(if [b-ITU-T H.248.xnq] approach) The extended network quality base package is detailed in Tables II.1, II.2 and II.3.

Table II.1 – Properties for the extended network quality base (XNQ) package

| Properties | Mandatory/ Optional | Used in command | Supported values | Provisioned value | Termination/Stream Types Supported |
| --- | --- | --- | --- | --- | --- |
| Measurement Period (mp, 0x0002) | <M/O> | <ADD, MOD, MOVE, AUDITVALUE, AUDITCAP> | <Values / ALL > | <Value / Not Applicable> | <Describe> |
| Severe Degradation Threshold (splt, 0x0003) | <M/O> | <ADD, MOD, MOVE, AUDITVALUE, AUDITCAP> | <Values / ALL > | <Value / Not Applicable> | <Describe> |
| Signals | Mandatory/ Optional | Used in command |  | Duration provisioned value |  |
| None. | <M/O> | <ADD, MOD, MOVE, AUDITVALUE, AUDITCAP> |  | <Value / Not Applicable> |  |
|  | Signal parameters | Mandatory/ Optional | Supported values | Provisioned value |  |
|  | <name and Identity> | <M/O> | <Values / ALL> | <Value / Not Applicable> |  |
| Events | Mandatory/ Optional | Used in command |  |  |  |
| None. | <M/O> | <ADD, MOD, MOVE, NOTIFY, AUDITVALUE, AUDITCAP> |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values | Provisioned value |  |
|  | <name and Identity> | <M/O> | <Values / ALL> | <Value / Not Applicable> |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values | Provisioned value |  |
|  | <name and Identity> | <M/O> | <Values / ALL> | <Value / Not Applicable> |  |

Table II.2 – Statistics for the extended network quality base (XNQ) package

| Statistics | Mandatory/ Optional | Used in command | Supported values | Termination/Stream Types Supported |
| --- | --- | --- | --- | --- |
| Near-End Statistics |  |  |  |  |
| Time degraded by network problems (ntdegnet, 0x0001) | <M/O> | <ADD, MOD, MOVE, SUBTRACT, AUDITVALUE, AUDITCAP> | <Values / ALL > | <Describe> |
| Time degraded by jitter buffer adaptations (ntdegjit, 0x0002) |  |  |  |  |
| Network Degraded Seconds Count (nes, 0x0003) |  |  |  |  |
| Network Severely Degraded Seconds Count (nses, 0x0004) |  |  |  |  |
| Maximum IPDV range within RTCP cycle (nvmaxdiff, 0x0005) |  |  |  |  |
| Global Maximum IPDV range (nvrange, 0x0006) |  |  |  |  |
| IPDV Sum (nvsum, 0x0007) |  |  |  |  |
| IPDV Cycles (nvcyc, 0x0008) |  |  |  |  |
| Jitter buffer adaptation events (njbevents, 0x0009) |  |  |  |  |
| RTP cumulative packet loss (ncumpl, 0x000a) |  |  |  |  |
| Far-End Statistics |  |  |  |  |
| Time degraded by network problems (ftdegnet, 0x000b) |  |  |  |  |
| Time degraded by jitter buffer adaptations (ftdegjit, 0x000c) |  |  |  |  |
| Network Degraded Seconds Count (fes, 0x000d) |  |  |  |  |
| Network Severely Degraded Seconds Count (fses, 0x000e) |  |  |  |  |
| Maximum IPDV range within RTCP cycle (fvmaxdiff, 0x000f) |  |  |  |  |
| Global Maximum IPDV range (fvrange, 0x0010) |  |  |  |  |
| IPDV Sum (fvsum, 0x0011) |  |  |  |  |
| IPDV Cycles (fvcyc, 0x0012) |  |  |  |  |
| Jitter buffer adaptation events (fjbevents, 0x0013) |  |  |  |  |
| RTP cumulative packet loss (fcumpl, 0x0014) |  |  |  |  |
| Round-trip delay statistics |  |  |  |  |
| Minimum round trip delay (rtdmin, 0x0015) |  |  |  |  |
| Maximum round trip delay (rtdmax, 0x0016) |  |  |  |  |
| Last round trip delay (rtdnow, 0x0017) |  |  |  |  |

Table II.3 – Error codes for the extended network quality base (XNQ) package

| Error Codes | Mandatory/Optional |
| --- | --- |
| None | <M/O> |

Appendix III

Example performance metric types

(This appendix does not form an integral part of this Recommendation)

## III.1	Background

Possible classification schemas for performance metrics for RTP traffic are described in clause 7. The protocol stack based method uses the terms application and transport level metrics (see clauses 3.2.1 and 3.2.6). This appendix provides some typical examples for each category.

There are examples which may be only MG-locally generated and reported via H.248, metrics which may be reported via RTCP, and examples for both reporting options.

{Editor’s note: check [b-IETF RFC 6843] which defines a) network delay and b) end system delay metrics. Contributions solicited.}

## III.2	Transport level metrics

### III.2.1	RTP-agnostic transport level metrics

Example metrics:

- all four statistics according [ITU-T H.248.61] concerning octet and packet based IP traffic volume (for RTP sessions)

- packet delay variation [b-IETF RFC 6798]

H.248 Network package:

- statistics os and or, see clause E.11.4/[ITU-T H.248.1]

### III.2.2	RTP-aware transport level metrics

That kind of transport level metrics are RTP-aware due to consideration of RTP header information TS, SN and/or RTCP data.

Example metrics:

- Jitter buffer nominal delay [b-IETF rtcp-xr-jb]

- Jitter buffer maximum delay [b-IETF rtcp-xr-jb]

- Jitter buffer high water mark and jitter buffer low water mark [b-IETF rtcp-xr-jb]

- Sum of Burst Durations [b-IETF rtcp-xr- burst-gap-loss]

- Packets lost in bursts [b-IETF rtcp-xr- burst-gap-loss]

- … [b-IETF rtcp-xr- burst-gap-loss]

- Sum of Squares of Burst Durations [b-IETF rtcp-xr- burst-gap-loss]

- Round trip delay [IETF RFC 3611]

H.248 RTP package:

- All five statistics (ps, pr, pl, jit and delay), see clause E.12.4/[ITU-T H.248.1]

H.248 Received RTCP package:

- All five statistics (rps, ros, rpl, rcpl and rjit), see clause 7.4/[ITU-T H.248.71]

## III.3	Application level metrics

### III.3.1	RTP-agnostic application level metrics

Example metrics:

These examples are based on the pure application data stream at receiver side (to be cross-checked again):

- Unimpaired seconds [b-IETF rtcp-xr-concsec]

- Concealed seconds [b-IETF rtcp-xr-concsec]

- Severely Concealed Seconds seconds [b-IETF rtcp-xr-concsec]

- Unimpaired seconds [b-IETF rtcp-xr-concsec]

Following example relates to fax-over-IP using [ITU-T T.38] in T.38-over-RTP transport mode and an H.248 IP termination enabled with the IP FAX package [ITU-T H.248.2]:

- Pages Transferred defined by H.248 statistic ipfax/pagestrans; that's an application level metric due to data object 'page', and RTP-agnostic due to independence of RTP header information

Statistics of RTP application data package [ITU-T H.248.58] (again due to RTP payload relation only ("application data"), but independence of RTP header information):

- RTP payload octets sent and

- RTP payload octets received

### III.3.2	RTP-aware application level metrics

The measurement function must be here media format aware, based on the RTP payload type codepoint, in contrast to the examples of clause III.3.1.

Example metrics:

- MOS value (attributed by MOS type and calculation method) [b-IETF rtcp-xr-qoe]

- signal level [IETF RFC 3611]

- noise level [IETF RFC 3611]

- residual echo return loss [IETF RFC 3611]

- R factor [IETF RFC 3611]

- external R factor [IETF RFC 3611]

Appendix IV

Relation between "IETF RTCP XR" documents 
and ITU-T H.248.x work products

(This appendix does not form an integral part of this Recommendation)

## IV.1	Background

There were three major steps in ITU-T concerning the support of "RTCP XR"-based measurements and reporting. Figure IV.1 recalls again the timeline and evolution of H.248.x capabilities for support of RTCP XR.

Figure IV.1 – Timeline and evolution of H.248.x capabilities for support of RTCP XR

Figure IV.1 indicates the basic relation between ITU-T and IETF documents, however, the overall situation needs the consideration of other documents as well, see Figure IV.2.

Figure IV.2 – Link between "IETF RTCP XR" documents and ITU-T H.248.x work products

Figure IV.2 indicates three major areas:

- Performance parameters (solely defined by IETF; metric definition based RTCP XR template);

- Performance measurement & reporting variants (for decomposed gateways in the RTP/RTCP bearer plane; defined by [ITU-T H.248.30], [b-ITU-T H.248.xnq] and [ITU-T H.248.48])

- Complementary framework & capabilities (primarily for H.248.48-based performance monitoring models; such as [ITU-T H.248.71], [ITU-T H.248.79], [ITU-T H.248.LOOPB] and [ITU-T H.248.RTPTOPO])

Figure IV.2 indicates the difficulties for service providers, network operators or manufactures in identifying the right technology for a particular deployment scenario ("which is finally given by an H.248 profile specification"). For instance, it is not evident that there are self-contained H.248.48 solutions, or whether always a combination together with the H.248.71 rtcpsdes package would be required.

The next sub-clause discusses typical capabilities in scope of H.248-based performance monitoring in RTP networks.

IV.2	Capabilities versus H.248.x packages support

This section tries to compare the different approaches of [ITU-T H.248.30], [b-ITU-T H.248.xnq] and [ITU-T H.248.48], their purposes, characteristics, but also principal limitations. The different H.248.x solutions satisfy different set of requirements (or support different capability sets). The capabilities may be structured in multiple areas. Table IV.1 indicates the main categories:

Table IV.1 – "RTCP XR"-based Performance Monitoring: 
Differences H.248.30 vs H.248.xnq vs H.248.48 – Part 1

| Capability |  | (1) H.248.30 | (2) H.248.xnq | (3) H.248.48 | Comments |
| --- | --- | --- | --- | --- | --- |
| A) Supported RTCP XR metric types |  | BT = 1 - 7 (limited) | BT = 8 (limited) | BT = ... (open) | A block type (BT) combines multiple performance metrics. |
| B) Supported RTP Topologies? |  |  |  |  | see also [ITU-T H.248.RTPTOPO] |
| B.1 | RTP Endsystem (RTPE) | Yes | Yes | Yes |  |
| B.2 | Back-to-Back RTP Endsystem (B2BRE) | Yes | Yes | Yes |  |
| B.3 | RTP Translators (RTPTT, RTPMT) | No | No | Yes | „RTP translator" topology required for e.g. network peering |
| B.4 | RTP Mixer | Yes | Yes | Yes |  |
| C) H.248 MG as Reporting Point (RP) |  |  |  |  | via H.248 statistics … |
| C.1 | Only Near-End? | Yes | Yes | Yes |  |
| C.2 | Centralized RP for both directions? | No | No | Yes | ‚No' means that two MGs must be used, one for each direction |
| D) H.248 MG as Filtering Point (FP) |  | No | No | Yes | ‚No' because „not applicable" |
| E) H.248 MG types (Connection Model) |  |  |  |  |  |
| E.1 | Physical-to-RTP (e.g. H.248 RMG, AMG, TMG) | Yes | Yes | Yes |  |
| E.2 | IP-to-IP (incl. RTP-to-RTP) | No | No | Yes |  |

The content of Table IV.1 is somehow reflecting the network evolution path of the past decade, from an initially dominated network architecture with an H.248 MG at the border between an IP network and circuit-oriented networks (such as PSTN, ISDN, 2G-PLMN) towards the direction of "all-IP networks" with it's H.248 MGs as IP-IP gateway/router entities.

Table IV.2 provides some categories concerning technology status and profile availability.

Table IV.2 – "RTCP XR"-based Performance Monitoring: 
Differences H.248.30 vs H.248.xnq vs H.248.48 – Part 2

| Capability |  | (1) H.248.30 | (2) H.248.xnq | (3) H.248.48 | Comments |
| --- | --- | --- | --- | --- | --- |
| F) Part of H.248 Profile? |  |  |  |  |  |
| F.1 | Standardized Profile? | No | No | Not yet | [ITU-T H.248.48] is on the agenda for Border GWs .. |
| F.2 | Other Profiles? | - | - | - | Out of scope of this Recommendation |
| G) Non-H.248 Reporting? |  |  |  |  | of RTCP XR block types .. |
| G.1 | SIP | Yes | No | planned |  |
| G.2 | H.323 | Yes | No | open |  |
| G.3 | MGCP | Yes | No | open |  |
| G.4 | Diameter | No | No | open |  |

Bibliography

[b-ITU-T H.248.xnq]	Expired Draft Recommendation ITU-T H.248.xnq (2005), Gateway control protocol: Extended Network Quality Metrics Packages for Next Generation Networks (NGNs).
Contained in document AVD-2773:
WP2/16 Rapporteur Meeting (Geneva, 28 November - 2 December 2005), 
http://ftp3.itu.int/av-arch/avc-site/2005-2008/0511_Gen/AVD-2773.zip

[b-ITU-T H.323]	Recommendation ITU-T H.323 (2009), Packet-based multimedia communications systems.

[b-IETF RFC 3261]	IETF RFC 3261 (2002), SIP: Session Initiation Protocol.

[b-IETF RFC 3435]	IETF RFC 3435 (2003), Media Gateway Control Protocol (MGCP) Version 1.0.

[b-IETF RFC 6798]	IETF RFC 6798 (11/2012), RTP Control Protocol (RTCP) Extended Report (XR) Block for Packet Delay Variation Metric Reporting.

[b-IETF RFC 6843]	IETF RFC 6843 (01/2013), RTP Control Protocol (RTCP) Extended Report (XR) Block for Delay Metric Reporting.

[b-IETF rtcp-xr-qoe]	IETF draft-ietf-xrblock-rtcp-xr-qoe (201X), RTCP XR Blocks for QoE Metric Reporting.

[b-IETF rtcp-xr-jb]	IETF draft-ietf-xrblock-rtcp-xr-jb (201X), RTCP XR Report Block for Jitter Buffer Metric Reporting.

[b-IETF rtcp-xr- burst-gap-loss]	IETF draft-ietf-xrblock-rtcp-xr-burst-gap-loss (201X), RTCP XR Report Block for Burst/Gap Loss metric Reporting.

[b-IETF rtcp-xr- concsec]	IETF draft-ietf-xrblock-rtcp-xr-concsec (201X), RTCP XR Report Block for Concealed Seconds metric Reporting.

[b-IETF rtcp-xr- burst-gap-loss]	IETF draft-ietf-xrblock-rtcp-xr-burst-gap-loss (201X), RTCP XR Report Block for Burst/Gap Loss metric Reporting.

__________________

The "loopbacked media information" is primarily subject of the enforced loopback mode: [IETF LOOPBACK] supports the three basic modes of "encapsulated packet loopback", "direct loopback", and "media loopback". These are distinguished by the treatment of incoming RTP packets at the loopback mirror.