| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4371 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.39 (Rev): Wildcarding SDP “a=fingerprint:” |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to update H.248.39 with support of the SDP “a=fingerprint:” attribute, according RFC 4572.

Wildcarding of this SDP element is required in context of H.248.TLS and H.248.TLSPROF. Wildcarding of such a signalling element addresses requirement R-8.3.3.2.3/1 of draft H.248.TLSPROF (2013-01).

Note: there’s a parallel contribution for introducing new wildcard “~”, see AVD-4370.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 48 (WP 1/16) from the January 2013 Q.3/16 meeting.

The abbreviation list of H.248.39 should be also cross-checked.

Change proposal #1:

[Begin Proposed Correction]

## 2.2	Informative references

...

[IETF RFC 4572]	IETF RFC 4572 (2006), Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP).

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

### 6.15.10	Fingerprint attribute

The fingerprint attribute is defined by [IETF RFC 4572], used for transport security protocols such as TLS and DTLS.

Table 6-15.10/H.248.39 – Attribute “fingerprint”

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 5/RFC 4572) | – |  |
| ABNF: (Clause 5/RFC 4572) | a=fingerprint:< hash-func> <fingerprint> |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| a=fingerprint:? |  | Are invalid. Because the mandatory 2-tuple of “name of hash function” and “hash value itself“ could be in principle individually wildcarded. |  |
| a=fingerprint:? ? |  | Would give a=fingerprint:hash-func fingerprint |  |

#### 6.15.10.1	Examples

See Table 6-15.10.1:

Table 6-15.10.1/H.248.39 – Examples for attribute “fingerprint”

| H.248/SDP encoding examples with applied wildcarding: | Results: |
| --- | --- |
| Example 1: MGC: a=fingerprint:$ $ | MG would choose a hash function and hash value (‘fingerprint’); and reply to MGC, e.g.: a=fingerprint: sha-384 fingerprint |
| Example 2: MGC: a=fingerprint:md5 $ | MG would choose a hash value for md5 algorith; and reply to MGC, e.g.: a=fingerprint:md5 fingerprint |
| Example 3: MGC – 1st H.248 Command request: a=fingerprint:$ $ MGC – 2nd H.248 Command request: a=fingerprint:~ ~ | MG would choose a hash function and hash value; and reply to MGC: a=fingerprint:hash-func fingerprint MG would "maintain unchanged already specified fingerprint attribue values": a=fingerprint:~ ~ |

[End Proposed Correction]

____________________