| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4374 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.DPI: Update proposal for clause 5 and a new clause 6 (overview) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes update the convention section and the addition of a new overview chapter.

Discussion and conclusions

A new clause 6 is proposed with the purpose of

- description and relation to generic policy rule models (H.248.79);

- indication of relations to other H.248.x-series of Recommendations, which define already non-DPI and DPI policy rule capabilities; and

- description of possible MGC involvement in the DPI policy rule generation process (based on reuse of existing living list material).

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 17 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

# 5	Conventions

## 5.1	Connection endpoint naming

The reader should be familiar with the connection endpoint naming scheme according clause 5.2/[H.248.1].

## 5.2	Distinction of cases DPI and non-DPI

The concept of DPI, from perspective of layered protocol architectures, is fairly wide and includes even all protocol layers above layer 1 (see clause 3.2.5/[ITU-T Y.2770]). However, the scope of packet inspection may be basically limited, mainly related on link, network and/or transport layers. Such a limitation is/was typically motivated by service-, historical- or implementation-related aspects. Such kind of limited packet inspection is also known as shallow and medium depth packet inspection (SPI, MPI; see also clause 8.1 in [b-ITU-T Y.SupTerm].

The coarse granular distinction between DPI and non-DPI according [ITU-T Y.2770] is sufficient and also followed by this Recommendation. The notion of ‘DPI’ here means loosely policy inspections rules as supported by this Recommendation or [ITU-T H.248.69], and ‘non’DPI’ relates more to legacy packet inspection at protocol layers 2, 3 and/or 4 (i.e., SPI, MPI).

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# (new) 6	Overview

## 6.1	Policy rule model

### 6.1.1	Basic model

DPI relates to the processing of policy rules by the MG, hence the generic policy rule model (see Figure 1) according [ITU-T H.248.79] is applicable as well for this Recommendation. A DPI policy rule would be typically associated with termination “Ta” in this model, because the “inspection function” is normally executed on incoming traffic as soon as packets enterning the MG.

Figure 1 (copy of Fig. 4/[ITU-T H.248.79]) – Order of operations (within a layer) – Unidirectional packet bearer traffic models using the concept of policy rules

### 6.1.2	Extended models

Similar as other existing H.248 MG functions, they could be associated to individual contexts or at the higher level of the entire MG. An extended model is e.g. given by DPI policy rules assigned to the root termination.

## 6.2	Relation to other H.248.x-series of Recommendations

This clause provides a non-exhaustive inventory for existing H.248 services.

### 6.2.1	ITU-T H.248.79 on general guidelines for packet-based streams

Related to filtering of TCP traffic. See overview in clause 9/[ITU-T H.248.79].

### 6.2.2	ITU-T H.248.37 on policy rules for NAT-T support

The MG is inspecting transport address information in incoming IP packets (rule condition) and possibly modifying that information (rule action) ([ITU-T H.248.79]).

NOTE – Its an example of SPI due to L4/L3 related rule conditions.

### 6.2.3	ITU-T H.248.40 on policy rules for application data inactivity detection

The MG is inspecting the packet inter-arrival or/and inter-departure rate at IP transport endpoints (rule condition) and possibly notifying the MGC (rule action) ([ITU-T H.248.40]).

NOTE – Its an example of SPI due to L4/L3 related rule conditions.

### 6.2.3	ITU-T H.248.43 on policy rules for basic gate management

See e.g. the two examples in Appendix II, clauses II.2.1.3 and II.2.1.4. The illustrated policy rules are entirely based on [ITU-T H.248.43] signalling.

### 6.2.4	ITU-T H.248.48 on policy rules for RTCP report filtering

See e.g. the the example in Appendix II, clause II.2.2.5. Such kind of policy rules are basically supported by [ITU-T H.248.48].

### 6.2.5	ITU-T H.248.53 on policy rules for traffic volume policing

See e.g. the two examples of “packet size range policing” and “flow-level byterate policing” in Appendix II (clauses II.2.1.1 and II.2.1.2). The illustrated policy rules are entirely based on [ITU-T H.248.53] signalling.

### 6.2.6	ITU-T H.248.61 on policy rules for IP traffic measurements

The MG is identifying incoming IP packets according a wide variety of lookup-keys according Appendix II/([ITU-T H.248.61] (rule condition) and possibly updating statistics (rule action).

### 6.2.7	ITU-T H.248.68 on policy rules for removal of inband information

The MG is inspecting incoming media-over-packets for digit and/or tone signals (rule condition) and possibly removing that information (rule action) ([ITU-T H.248.68]).

NOTE – Its an example of MPI or even DPI, dependent on the protocol layer and encoding of tone/digit information.

### 6.2.8	ITU-T H.248.69 on policy rules for message filtering

The Messaging Filtering package (clause 13/[ITU-T H.248.69]) defines a policy rule based behaviour (based on "Sieve" as defined by [IETF RFC 5228]) for inspecting e-mail traffic (i.e., e-mail over SMTP/L4/IP packets) with the purpose of message filtering (rule actions).

### 6.2.9	ITU-T H.248.76 on policy rules for general filtering

[ITU-T H.248.76] allows to generate multiple, combined policy rules with regards to packet filtering, called “filter groups”.

### 6.2.10	ITU-T H.248.78 on policy rules for NAT-T support

The MG is inspecting address information at protocol layers L4+, L4 and L3 in incoming IP packets (rule condition) and possibly modifying the L4+ address information (rule action), in case of the MG autonomous bearer-level ALG mode (clause 6.2/[ITU-T H.248.79]).

### 6.2.11	ITU-T H.248.84 on policy rules for NAT-T support

The MG is inspecting TCP header protocol control information in incoming IP packets (rule condition) and possibly forwarding TCP packets inclusive modified TCP header protocol control information (rule action) ([ITU-T H.248.84]).

## 6.3	Control and management of DPI policy rules

The notion of “policy control” refers to the network control plane, i.e., the signalling of (DPI) policy rules by the MGC to the MG via H.248. H.248 as gateway control protocol may be then considered as policy control protocol. The notion of “policy management” refers to the network management plane, i.e., the provisioning of (DPI) policy rules by the network/element management system in the MG via configuration management (policy management) protocol(s). See also clause 7.2.1/[ITU-T Y.2770].

This Recommendation focuses on DPI policy control, however, the provisioning option of some package elements (see ###) covers as well some DPI policy management capabilities (see also Figure II.2).

The possible MGC involvement in DPI policy control is further outlined in Appendix I.

## 6.4	Packages design philosopy

{Editor’s note (2013-06 meeting): it was noted that this Recommendation is of similar type as H.248.9. Similar text as in clause 5/ H.248.9 might be useful for this Recommendation. Contributions are solicited.}

[End Proposed Correction]

____________________