| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4378 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.DPI: Update proposal for Appendix III |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to update Appendix III.

Discussion and conclusions

Appendix III illustrates four signalling methods, whereof three are proposed to be supported by H.248.DPI. This Appendix serves therefore as complementary, high-level information to the proposed packages for the main body of this Recommendation (see AVD-4375).

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 17 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

Appendix III

H.248 aspects for signalling DPI policy rules

(This appendix does not form an integral part of this Recommendation)

## III.1	Overview of principal signalling methods

Following principle signalling methods for policy rules may be identified:

1	Explicit rule signalling

1.1	Explicit rule in native H.248 syntax

1.2	Explicit rule encapsulated in container over H.248

2	Signalling of pointer to rule and rule MG-locally provisioned

3	Signalling of pointer to rule and rule MG-remotely provisioned

Figure III.1 outlines three examples for policy rules signalled between a policy decision point (here MGC) and policy enforcement point (here MG).

Figure III.1 – Principle signalling methods for policy rules

The high-level principle behind each method is as follows.

Option (1):

- the complete rule is carried by a single H.248 command.

Option (2):

- the H.248 command carries a 'pointer' (which relates to an "identifier for a rule"; also known as reference principle in [ITU-T H.248.69]),

- the rule would be already available in a MG local data base,

- the identifier correlates rule and correspondent H.248 context/termination/stream.

Option (3):

- as in (2), the H.248 command carries a 'pointer' (which relates to an "identifier for a rule"),

- the rule would be already available in a MG remote data base,

- the MG issues a query to that data base for requesting the rule,

- the interface between MG and remote data base would be typically “non-H.248” based (and out of scope of ITU-T H.248.x-series of Recommendations),

- the downloaded rule is assigned to H.248 context/termination/stream.

Method (1) may be further categorized, see Figure III.2:

Figure III.2 – Two sub-options for the first method

Sub-option (1.1):

- the rule is encoded in native H.248 syntax, i.e., based on (existing or new) H.248 properties, events, signals, and/or statistics, incl. error codes.

Sub-option (1.2):

- the rule itself is specified in a non-H.248 language (also known as policy specification language (PSL), see Appendix IV),

- H.248 provides support of a "container" for carrying the rule,

- Note: a  "container" relates to a data object in general. The container principle is already used by some H.248 packages. See e.g., the data block property in [ITU-T H.248.45].

The several options are not mutually exclusive. Multiple options could be principally applied in parallel, which implies a carefully evaluation of possible interaction issues.

## III.2	H.248 support

Table III.2 summarizes present H.248 capabilities for support of the outlined methods.

Table III.2 – Signalling methods versus H.248 support

| Method | Present H.248 support | Comment/reference |
| --- | --- | --- |
| 1.1 |  |  | Not supported. [Primarily due to the existence of sufficient PSLs with DPI policy rule specification support, see Appendix IV.] | Main advantage would be a tight coupling and alignment with existing H.248 protocol elements (see e.g. clause 6.x). |  |  |
| 1.2 | Supported by inspection rule base package, container for rule property. |  |  |  |  | see clauses 7.1.2 and 7.6.1.2 |  |
| 2 | Supported by inspection rule base package, pointer to rule conditions property. | see clauses 7.1.1 and 7.6.1.3 |
| 3 | Support, as method 2. | see clauses 7.1.1 and 7.6.1.4 |

[End Proposed Correction]

____________________