| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4389 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.RTCPPROF: Update proposal for Table IV.1 |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

The column related to H.248.48 in Table IV.1 isn’t yet fully completed concerning the indication of valid block types.

This contribution proposes a resolution.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 23 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

…

Table IV.1 – "RTCP XR"-based Performance Monitoring: 
Differences H.248.30 vs H.248.xnq vs H.248.48 – Part 1

| Capability |  | (1) H.248.30 | (2) H.248.xnq | (3) H.248.48 | Comments |
| --- | --- | --- | --- | --- | --- |
| A) Supported RTCP XR metric types |  | BT = 1 - 7 (limited) | BT = 8 (limited) | BT = ”All” (NOTE 1) | A block type (BT) combines multiple performance metrics. |
| B) Supported RTP Topologies? |  |  |  |  | see also [b-ITU-T H.248.RTPTOPO] |
| B.1 | RTP Endsystem (RTPE) | Yes | Yes | Yes |  |
| B.2 | Back-to-Back RTP Endsystem (B2BRE) | Yes | Yes | Yes |  |
| B.3 | RTP Translators (RTPTT, RTPMT) | No | No | Yes | „RTP translator" topology required for e.g. network peering |
| B.4 | RTP Mixer | Yes | Yes | Yes |  |
| C) H.248 MG as Reporting Point (RP) |  |  |  |  | via H.248 statistics … |
| C.1 | Only Near-End? | Yes | Yes | Yes |  |
| C.2 | Centralized RP for both directions? | No | No | Yes | ‚No' means that two MGs must be used, one for each direction |
| D) H.248 MG as Filtering Point (FP) |  | No | No | Yes | ‚No' because „not applicable" |
| E) H.248 MG types (Connection Model) |  |  |  |  |  |
| E.1 | Physical-to-RTP (e.g. H.248 RMG, AMG, TMG) | Yes | Yes | Yes |  |
| E.2 | IP-to-IP (incl. RTP-to-RTP) | No | No | Yes |  |
| NOTE 1 – All RTCP XR block types as registered with IANA, see [b-IANA RTCP XR]. |  |  |  |  |  |

The content of Table IV.1 is somehow reflecting the network evolution path of the past decade, from an initially dominated network architecture with an H.248 MG at the border between an IP network and circuit-oriented networks (such as PSTN, ISDN, 2G-PLMN) towards the direction of "all-IP networks" with it's H.248 MGs as IP-IP gateway/router entities.

…

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

Bibliography

…

[b-IANA RTCP XR]	IANA Registry of RTCP XR Block Types at http://iana.org/assignments/rtcp-xr-block-types/rtcp-xr-block-types.xhtml

[End Proposed Correction]

____________________