| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4401 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: Clause 9 – Update of TLS capability negotiation package |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes package updates according the agreements of the 2013-01 meeting.

1	Introduction

Contributions C-4 and C-14 from last meeting (2013-01) resulted in an agreement in drafting three individual H.248 packages for TLS bearer control.

Copy of COM 16 – C 14/Figure 1 – H.248.TLS – Package redesign proposal – Overview

This contribution continues the update process as initiated by C-20 of the TLS session connection control package.

2	Discussion

There are basically following changes proposed:

- the TLS profile concept needs to be detached from individual package specifications (“due to the overall character”), thus moved in a separate chapter;

- TLS capability negotiation: there are two fundamental options, - either the MGC uses a TLS profile(s) indication or the MGC want to influence individual TLS protocol parameters

Furthermore, a lot of editorial enhancements is suggested, however, all descriptions of the procedures are not yet fully consistent to the still pending question whether we stick to support the above mentioned options only (or also a third one), and how strictly each method would be. Therefore a number of addional editor’s notes were added, in order to clean up the text at the future meeting.

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 20 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

# 9	TLS capability negotiation package

|  | Package Name: | TLS capability negotiation package |
| --- | --- | --- |
|  | Package ID: | tlscn (0x####) |
|  | Description: | ## When this package is not used by an H.248 profile, then the MG will autonomously negotiate TLS protocol configurations with the peer TLS node. This package provides control tools for the MGC for influencing bearer-level TLS negotiations between the MG and the remote TLS endpoint. The MGC could impact TLS negotiations in terms of - TLS protocol parameter values (such as TLS version); - Quality/goals of negotiation results; - Time limits for negotiations (e.g. due to realtime aspects of the overall application/service); or/and - Handling of unsuccessful negotiations. There are two options for negotiation of TLS protocol configurations: - MGC may indicate a TLS profile (or multiple) as such; - MGC may control individual TLS protocol parameters. |
|  | Version: | 1 |
|  | Extends: | None. |

## 9.1	Properties

### 9.1.1	TLS Versions

|  | Property Name: | TLS Versions |
| --- | --- | --- |
|  | Property ID: | tlsv (0x0001) |
|  | Description: | This property indicates which TLS versions should be negotiated with the remote TLS endpoint. |
|  | Type: | Sub-list of Strings |
|  | Possible values: | Each string consists of 4 hexadecimal digits and represents the TLS version as defined in the TLS protocol version related IETF RFCs (e.g. [IETF RFC 5246, Appendix A.1]). The hexadecimal representation of the major version is coded in the first two characters, while the hexadecimal representation of the minor version is coded in the last two characters of the string. The hexadecimal representation of the versions omits the “0x”, and if needed, a leading “0” is added for padding. Over-decadic digits are to be represented by lower case characters (“a”..”f”). |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) {Editor’s note (2012-02): the scope needs to be clarified, i.e., either Root or LocalControl level per H.248 profile, or both. Contributions are solicited. => (2013-06): might be addressed in clause 9.6.2} |
|  | Characteristics: | Read/Write |

### 9.1.2	Cipher Suites

|  | Property Name: | Chipher Suites |
| --- | --- | --- |
|  | Property ID: | cs (0x0002) |
|  | Description: | This property indicates the cipher suites and their precedence for the negotiation with the remote TLS-endpoint. |
|  | Type: | Sublist of Strings |
|  | Possible values: | Each string consists of 4 hexadecimal digits and represents the cipher suite in accordance to the TLS Cipher Suite Registry of IANA (http://www.iana.org/assignments/tls-parameters/tls-parameters.xml#tls-parameters-3 ). The first value of the value pair as defined by the TLS Cipher Suite Registry of IANA shall be coded into the first two characters, and the second value shall be coded into the last two characters of the string. Thus each value in its hexadecimal representation shall be converted into the double-hexdigit string. The “0x” shall be omitted and if needed, a “0” is used for padding. Over-decadic digits are to be represented by lower case characters (“a”..”f”). |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write, |

### 9.1.3	Compression Methods

|  | Property Name: | Compression Methods |
| --- | --- | --- |
|  | Property ID: | cm (0x0003) |
|  | Description: | This property declares the list of compression methods and their preference for the negotiation with the remote TLS-endpoint. |
|  | Type: | Sub-list of unsigned Integer |
|  | Possible values: | The values correspond to the values allocated by IANA for TLS Compression Method Identifiers (see http://www.iana.org/assignments/comp-meth-ids/comp-meth-ids.xml ). |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

### 9.1.4	Support for Session Resumption

|  | Property Name: | Support for Session Resumption |
| --- | --- | --- |
|  | Property ID: | ssr (0x0004) |
|  | Description: | This property declares the support for the TLS session resumption {Editor’s note (2013-06 meeting): the capability of TLS session resumption, - from perspective of H.248 entities -, is still not fully clear. Description should provide more info … )} |
|  | Type: | Boolean |
|  | Possible values: | False Session Resumption shall not be supported True Session Resumption shall be supported |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

### 9.1.5	Support for Renegotiation of the Security Context

|  | Property Name: | Support of Renegotiation of the Security Context |
| --- | --- | --- |
|  | Property ID: | srsc (0x0005) |
|  | Description: | This property declares whether the renegotiation of the security context shall be supported. |
|  | Type: | Boolean |
|  | Possible values: | False Renegotiation shall not be supported True Renegotiation shall be supported |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

### 9.1.6	Renegotiation Period

|  | Property Name: | Renegotiation Period |
| --- | --- | --- |
|  | Property ID: | rp (0x0006) |
|  | Description: | This property declares the period in seconds after which a renegotiation of the security context shall be initiated by the MG. The value 0 indicates that the MG shall not initiate a renegotiation. {Editor’s note (2013-01 meeting): Do we need other criteria for a renegotiation trigger, like nbr fragments sent/rec., nbr octets sent/rec., etc?} |
|  | Type: | Unsigned Integer |
|  | Possible values: | Any non-negative value |
|  | Default: | 0, unless provisioned otherwise |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

### 9.1.7	TLS Domain Profile Identifier

### {Contributor’s note (2013-06 meeting): this property should be moved to 9.1.1 …}

|  | Property Name: | TLS Domain Profile Identifier |
| --- | --- | --- |
|  | Property ID: | dpid (0x0001) |
|  | Description: | This property identifies the TLS domain profile that shall be ideally used by the MG to setup the TLS security session. The actually applied, so called TLS endpoint profile is the match result of locally supported TLS capabilities by the MG (termed as TLS MG profile) with requested TLS domain profiles. ## It should be noted that the normal case would be aligned TLS profile capabilities, i.e.: ## applied “TLS endpoint profile” = requested “TLS domain profile”. |
|  | Type: | String ## {Contributor’s note (2013-06 meeting): or “sublist of String” due to the fact that there might be multiple profiles! We are tending to plural because we are proposing “concrete TLS profile specs” as out of scope of this Recommendation ….} |
|  | Possible values: | Any string as agreed between MGC and MG. Note: should be subject of H.248 profile definitions. |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

### 9.1.8	Client Authentication Required

|  | Property Name: | Client Authentication Required |
| --- | --- | --- |
|  | Property ID: | car (0x0008) |
|  | Description: | This property declares if the MG – if acting as a TLS-server – shall authenticate the remote TLS client. |
|  | Type: | Boolean |
|  | Possible values: | False In case the MG acts as a TLS-server, the client shall not be authenticated. True In case the MG acts as a TLS-server, the client shall be authenticated. |
|  | Default: | True, unless provisioned otherwise |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

## 9.2	Events

None.

## 9.3	Signals

None.

## 9.4	Statistics

None.

## 9.5	Error codes

None.

## 9.6	Procedures

### 9.6.1	General default behaviour (MGC does not influence TLS capability negotiations)

Package tlscn is optional for H.248 profile specifications with TLS support. When the package is not used, then the MG shall independently negotiate TLS configuration(s) with remote TLS endpoints.

Such MG-autonomous TLS capability negotiations shall be compliant to [IETF RFC 5246]. The negotiations would be based on the TLS MG profile and possibly preprovisioned negotiation policies.

### 9.6.2	MGC-controlled TLS capability negotiations: Options

There are two options for MGC-controlled TLS capability negotiations, with the aim to determine the TLS domain profile instance, which is to be used by the MG to form the TLS endpoint profile (by also taking into account the TLS MG profile), see Table X:

Table X – Variants of MGC-controlled TLS capability negotiations

| Method | H.248 elements |  |  | Protocol procedures, see: |  |
| --- | --- | --- | --- | --- | --- |
| a) TLS profile approach | Property: - tlscn/dpid |  |  | clause 9.6.4 |  |
| b) Individual TLS protocol parameter control | Properties: - tlscn/tlsv - tlscn/cs - tlscn/cm - tlscn/ssr - tlscn/srsc - tlscn/rp - tlscn/car | clause 9.6.5 |

{Editor’s note (2013-06 meeting): theoretically, a third Method c) might be added: 
Explicit signalling of tlscn/dpid and additionally explicit signalling of some other tlscn/x properties with x != dpid. Or should this be explicitly forbidden and thus be rejected by the MG? Contributions are solicited….}

### 9.6.3 (ex 8.6.1)	Determining TLS Related Capabilities

9.6.3.1	Auditing supported TLS protocol profiles

The MGC may access supported TLS protocol profile(s) by auditing property tlscn/dpid. The identifier of a particular TLS protocol profile relates to a reference on a TLS protocol profile description, which might be for instance explicitly part of an H.248 profile definition or documented anywhere else.

9.6.3.2	Auditing individual supported TLS protocol capabilities

The MGC may determine the properties of the TLS MG profile by auditing the value of Supported TLS Versions (tlsv), Supported Cipher Suites (scs), Supported Compression Methods (scm) and Support for Session Resumption (ssr). Normally the audit will be performed on the Root termination.

The Supported TLS Versions property provides the TLS-versions supported by the MG without any specific sequence.

The Supported Cipher Suite property shall include the cipher suite TLS_NULL_WITH_NULL_NULL (“0000”), which is used as the initial cipher suite during the first handshake for the TLS-session, but which must not be used for a cipher suite negotiation.

The list of supported cipher suites and the list of supported compression methods are provided without any specific sequence.

TLS session resumption may be used to avoid the need for a negotiation of the new security context. The Support for Session Resumption property provides the MGC with an indication whether the MG supports this feature.

{Editor’s note (2013-01 meeting): TLS-resumption use cases to be identified.}

### 9.6.4	Indication of TLS profile(s) to MG by MGC

The MGC may request TLS domain profile(s) support from the MG via property dpid, leading to following MG behaviour:

- Built intersection between locally supported TLS capabilities (the “TLS MG profile”) with indicated TLS domain profile(s), resulting in a TLS endpoint profile (see also clause 13);

- The TLS endpoint profile shall be the baseline TLS negotiations with the remote TLS entity (during TLS security session establishment/resumption procedures).

(To be checked again)

### 9.6.5	Indication of individual TLS protocol capabilities to MG by MGC

{Editor’s note (2013-01 meeting): In this case the “TLD Domain profile” instance to be used by the MG is (partially) defined by the MGC by the actually signalled tlscn/x properties with x != dpid, isn’t it? Those tlscn/x properties, which are not explicitly signalled by the MGC assume their values as provisioned in MG’s “MG TLD profile”, - to be cross-checked again.}

9.6.5.1 (ex 8.6.4.1)	TLS protocol parameter “TLS version”

The list of TLS versions used for the TLS procedures of the session is determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the TLS MG profile. This list is supposed to be provisioned (ie., the property value for dpid) on the MG as typically default configuration.

NOTE – The TLS endpoint profile again (see clause 9.x.y) relates to the intersection/match of TLS MG profile (as provisioned on MG) and individual explicitly signalled tlscn/x properties with “x != dpid”.

In case the MGC signals the property TLS Versions (tlsv), the MG shall use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

In case the MG is acting as a TLS client, the highest TLS versions as defined by the specific TLS-endpoint profile are included into the initial TLS-handshake message (ClientHello). In case the MG acts as a TLS server, the MG shall select the highest TLS version that is present in the list of TLS-versions offered from the TLS client and in the TLS versions as defined by the TLS endpoint profile. If there is no match between both lists the TLS-handshake is regarded as failed.

{Editor’s note (2012-09 meeting): Error cases need to be worked out, e.g. in case not all TLS-versions received from the MGC are supported by the MG}

9.6.5.2 (ex 8.6.4.2)	TLS protocol parameter “cipher suite”

The prioritized list of cipher suites used for the TLS handshakes of the session are determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the TLS MG profile. The TLS domain profile may specify different cipher suites for TLS clients and TLS server, thus the property TLS endpoint role (er) must be taken into account.

In case the MGC signals property Chipher Suites (cs), the MG shall use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

The MG will negotiate the cipher suite according to the TLS endpoint profile. In case the MG is acting as a TLS client, the cipher suites are included into the first TLS-handshake message (ClientHello) in the same sequence as defined by the TLS endpoint profile. In case the MG is acting as a TLS server, the Cipher Suites property is used to negotiate the cipher suite with the client according to the procedure that is applicable for the negotiated TLS version.

9.6.5.3 (ex 8.6.4.3)	TLS protocol parameter “compression method”

The list of compression methods used for the TLS handshakes of the session are determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the TLS MG profile. The TLS domain profile may specify different compression methods for TLS clients and TLS server, thus the property TLS endpoint role (er) must be taken into account.

In case the MGC signals property Compression Methods (cm), the MG shall use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

The negotiation of the compression methods follows the same principle as the negotiation of the cipher suite. The compression methods as defined by the TLS endpoint profile provides the MG with the list of methods to be used for the negotiation as defined by the procedure that is applicable for the negotiated TLS version.

9.6.5.4 (ex 8.6.4.)4	TLS protocol procedure “client authentication”

The MG’s TLS server implementation shall support the client authentication.

In case the TLS endpoint in the MG is configured as a TLS server, the client authentication indication is taken from the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid).

In case the MGC signals property Client Authentication Required (car), the MG shall use this value instead. In this case the MGC must ensure the consistency of the value with the requirements of the TLS domain.

9.6.5.5 (ex 8.6.4.5)	TLS protocol procedure “self-signed client certificates”

In case the TLS endpoint in the MG is configured as a TLS server and client authentication is required, the type of client authentication is implicitly defined by the SDP attribute “a=fingerprint”. If present in the Remote Descriptor, the client certificate shall be self-signed and shall be verified against the fingerprint.

9.6.5.6 (ex 8.6.4.6)	TLS protocol procedure “signed client certificates”

In case the TLS endpoint in the MG is configured as a TLS server, client authentication is required and no SDP fingerprint attribute is present, the client certificate will be verified against a list of root certificates as defined for the TLS domain profile. This list of root certificates is supposed to be configured per dpid on the MG.

9.6.5.7 (ex 8.6.4.7)	TLS protocol procedure “server authentication”

In case the TLS endpoint in the MG is configured as a TLS server, the MG shall provide the server certificate to the client as defined by the TLS domain profile.

9.6.5.8 (ex 8.6.4.8)	TLS protocol procedure “session resumption”

FIX THIS

9.6.5.9 (ex 8.6.4.9)	TLS protocol procedure “renegotiation support”

The TLS implementation in the MG shall provide support for renegotiation.

Whether renegotiation for a TLS session shall be enabled is determined by the TLS domain profile.

In case the MGC signals property Support of Renegotiation of the Security Context (srsc), the MG shall use this value instead.

### 9.6.5.10 (ex 8.6.6)	TLS protocol procedure “security context renegotiation”

The behaviour of the renegotiation of the security context is defined by the TLS endpoint profile, which indicates if renegotiation shall be supported or not, taking into account property Renegotiation Period (rp).

In case the TLS endpoint profile disables renegotiation, the MG shall never initiate a renegotiation. If receiving a renegotiation request from the remote TLS-endpoint, the MG shall respond with a TLS no_renegotiation alert, as defined by the procedure that is applicable by the negotiated TLS-version.

If renegotiation shall be supported, the MG shall accept and process a renegotiation request from the remote TLS endpoint. The initiation of the renegotiation procedure by the MG depends on the value of the Renegotiation Period property. If this property set to value “0”, no renegotiation shall be initiated. If set to any other non-zero value, a timer with the duration of the Renegotiation Period is started each time a TLS-handshake completed successfully. On expiration of the timer the MG shall trigger the renegotiation as defined by the procedure that is applicable by the negotiated TLS-version. In case the MG receives a trigger for the renegotiation from the remote TLS-endpoint while this timer is running, the MG shall cancel and restart this timer.

{Editor’s note (2012-09 meeting): Current assumption is that no explicit MGC control of the renegotiation is required, e.g. trigger renegotiation through a signal.
However, to be considered: “If the MGC provides the property tlscn/rp, then the MG will use this value instead of the provisioned default TLD Domain profile’s “rp” value }

### 8.6.7	TLS-Session Resumption

FIXTHIS …

# 13	The TLS profile concept

## 13.1 (ex 8.6.4)	TLS protocol profiles

A MG may be requested to support more than one TLS domain. For each such TLS domain different “TLS services” may be required from the MG. For each TLS domain those required TLS behaviours are defined by a domain-sopecific TLS domain profile.

Generally the TLS domain profile may be provisioned on the MG. E.g., when using the tlscn package, then property TLS Domain Profile Identifier (dpid) shall be used to refer to a TLS domain profile. This property shall be then aligned between the MG and the MGC, as the MGC may use it to indicate to the MG which TLS domain profile should be applicable for the creation of a TLS endpoint in the MG.

Important to note: any concrete, detailed, normative specifications of TLS protocol profiles are out of scope of this Recommendation!

The TLS related parameters and procedures supported by the TLS implementation in the MG are referred to as TLS MG profile”(see also clause 3.2.x).

{Editor’s note (2012-09 meeting): Thus, if the “dpid” property is not signalled, does the applied assumed “default” TLS Domain profile correspond to the TLS client/server role specific subset of this “MG TLS profile”?}

On creation of a TLS endpoint in the MG, the intersection of the selected TLS domain profile(s) and the TLS MG profile will be build. This intersection may be termed as the TLS endpoint profile. It defines the TLS related parameters and procedures the MG will use to establish the TLS session on the TLS-enabled endpoint (i.e., the MG H.248 SEP/termination).

Figure 6 illustrates the principle relation between the profiles.

Figure 6 – Example: Intersection of the TLS domain profile 1 and the TLS MG profile

Normally, the result of intersection would be a TLS endpoint profile that exactly matches with the requested TLS domain profile(s). However, there might be partial matches in principle as well, as depicted by the example in Figure 6.

## 13.2	Illustration of TLS profile concept

The indication and negotiation of media configurations across the H.248 interface is subject of [ITU-T H.248.80]. A TLS configuration relates effectively to a subset of a media configuration (see clause 3.2.1.2/[ITU-T H.248.80]). Table Y opposes correspondent terms and profile types:

Table Y – Analogon between H.248.80 media configurations and TLS profile types

| H.248.80 generic media configuration | TLS-specific media configuration |  | H.248.TLS TLS profile term: |  |  |
| --- | --- | --- | --- | --- | --- |
| Overall term: |  |  |  |  |  |  |  |
| Configuration (clause 3.2.1 / [ITU-T H.248.80]) | TLS configuration |  | TLS profile |  |  |
| Qualified term: |  |  |  |  |  |  |  |
| Prefered configuration (e.g., prefered codec list) |  | Prefered TLS configuration (NOTE 1) |  | TLS domain profile |  |  |
| Supported configuration (e.g., supported codec list) |  | Supported TLS configuration |  | TLS MG profile |  |  |
| Actual configuration (e.g., actual codec list) | Actual TLS configuration | TLS endpoint profile |
| NOTE 1 – There is a subtle difference between H.248.80 and this Recommendation: there are not any SIP-level SDP Offer/Answer negotiations concerning TLS configurations due to the fact that TLS provides itself an embedded TLS handshake protocol for TLS-level negotiations. Thus, there is neither a “Offered TLS configuration” nor “Answered TLS configuration” at application control protocol level. |  |  |

Figure x1 provides an illustration of the three TLS profile types, as in use during TLS security session establishment:

Figure x1 – Illustration of TLS profile concept

In general, the TLS MG profile reflect the status of implemented TLS capabilities, it may be therefore principally a subset, superset or equal to a requested TLS domain profile.

H.248 profiles could also simplify network solutions by mandating supported TLS capabilities, which could lead to the equation of “TLS MG profile equal to TLS domain profile”, resulting in “TLS endpoint profile also equal to TLS domain profile”.

{Editor’s note (2012-09 meeting): two clarifications are requested:

- notion of “overall TLS profle”( what exactly? required?)

- relation of “MG TLS profile” to “TLS domain profile(s)”: subset, equal, superset, - all options possible?

Contributions are solicited.}

## 13.3 (ex 8.7)	Example for the TLS MG profile concept

Appendix ??? provides examples for TLS domain profiles .Table 2 depicts an example for a TLS MG profile. It also shows which properties can be audited on the Root termination by the MGC.

Table 2 – Example of a TLS MG profile

| Scope | Example | Corresponding H.248 signalling element for property audit by MGC |
| --- | --- | --- |
| TLS versions | TLS1.1 TLS1.2 | tlscn/tlsv |
| Cipher suites | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA | tlscn/cs |
| Compression methods | NULL, DEFLATE, LZS | tlscn/cm |
| Client authentication required | yes | tlscn/car |
| Resumption support | no | tlscn/ssr |
| Extensions |  |  |

The following Table 3 shows an example how the intersection of a TLS domain profile and the TLS MG profile is built to determine the values for the TLS endpoint profile. The properties of the TLS endpoint profile will be used by the MG for the TLS handshake. Table 3 does not indicate which properties can be provided by the MGC.

Table 3 – Example of an intersection between a TLS domain profile and a TLS MG profile

| Property | TLS Domain profile | TLS MG profile | TLS endpoint property value |
| --- | --- | --- | --- |
| TLS version | Mandatory: TLS1.1 Optional: TLS1.2 | TLS1.1, TLS1.2 | TLS1.1, TLS1.2 |
| Cipher suite | Mandatory: TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_NULL_SHA Optional: TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_DH_RSA_WITH_AES_256_CBC_SHA256 | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA, TLS_RSA_WITH_RC4_128_MD5, TLS_RSA_WITH_DES_CBC_SHA | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_NULL_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA |
| Compression method | NULL, LZS | NULL | NULL |
| Client authentication required | Yes | Yes | Yes |
| Root certificates | List: A, B, C | List: X, A, B | List: A, B Provisioned per TLS domain profile |
| MG certificate |  | List of certificates, one certificate per TLS domain profile | Provisioned per TLS domain profile |
| Support for renegotiation | No | Yes | No |
| Support for resumption | Yes | No | No |

{Editor’s note (2012-09 meeting): A clear view on the MG certificate is still missing. It is used during the TLS handshake, thus it is a part of the “set of TLS related parameter and procedures” (current definition of a TLS profile), but it is neither defined for the TLS domain profile nor (currently) for the TLS MG profile. The MG certificate is network element specific and TLS-domain specific.}

[End Proposed Correction]