| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4414 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.RTPTOPO: Appendix I - Update proposals |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes a resolution to the editor’s note in Appendix I, which primarily requests more information about the examples and conclusions concerning usability of the implicit signalling method (for the purpose of “transparent forwarding”).

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 18 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

Appendix I
Examples of transparent forwarding

(This appendix does not form an integral part of this Recommendation)

## I.1	Introduction

The MG behaviour of “RTP transparent forwarding” may be enforced by either implicit or explicit H.248 signalling (see clause 10.6.3.8.1). The explicit method is semantically straightforward, whereas implicit signalling is often conditional. Some examples shall illustrate the uncontroversial and controversial usage of implicit signalling.

There is an (IP, IP) connection model in all examples. The signalling of a single stream endpoint is shown.

## I.2	Convention

The examples use an abstracted command notation, see Table I.1. There are three key areas of interest in such H.248 commands:

Table I.1 –Implicit signalling – abstracted H.248 command format 
with principal areas related to RTP

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| Add = ip/$ ; Termination Ta, SEP S1 … Stream = 1 { LocalControl {<”RTP impacting elements Y/N?”>} Local {… m=… a=… }, Remote { m=… a=… } other Descriptors{<”RTP impacting elements Y/N?”>} …}, Add = ip/$ ; Termination Tb, SEP S1 … Stream = 1 { LocalControl {<” RTP impacting elements Y/N?”>} Local {… m=… a=… }, Remote { m=… a=… } other Descriptors{<”RTP impacting elements Y/N?”>} …} | There are basically three areas in H.248 commands which could contain information indicating the existence of RTP bearer traffic: - SDP media descriptions embedded in LD/RD as part of the Stream Descriptor; relevant information would be primarily subject of the “m=” and “a=” lines; - H.248 properties in the LCD (again embedded in Stream Descriptor); - H.248 properties in other descriptors (e.g., Events Descriptor, Statistics Descriptor). |

## I.3	Examples for implicit signalling

The method of implicit signalling is described in clause 10.6.3.8.1 as option (a).

### I.3.1	Example 1 “UDP payload transparent forwarding”

Table I.2 illustrates “UDP payload transparent forwarding”:

Table I.2 – Example 1 “UDP payload transparent forwarding”

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/… Transaction = … { Context = … { Add = ip/$ ; Termination Ta, SEP S1 {Media { Stream = 1 { LocalControl { ... ; NOTE 1 } ... Local { v=0 c=IN IP4 <IP_addr> m=- <port> udp – }, Remote { v=0 c=IN IP4 <IP_addr> m=- <port> udp – }…}, Add = ip/$ ; Termination Tb, SEP S1 {Media { Stream = 1 { LocalControl { ... } ... Local { v=0 c=IN IP4 <IP_addr> m=- <port> udp – }, Remote { v=0 c=IN IP4 <IP_addr> m=- <port> udp – }…}, … | The RTP session shall be configured for transparent forwarding for both traffic directions. Prerequisites: - All four “m=” lines SHALL be identical. Example here: - Transport ‘UDP’ is indicated, which implies “RTP agnostic” to the MG. - The MG could be requested for local NAPT, which is a L4-dependent function (thus “UDP” indication). Conclusions: - This setting does not provide “UDP transparent forwarding” (which was also not requested) due to UDP checksum updates. - It leads rather to “UDP payload transparent forwarding” … - … which could be “RTP transparent forwarding” in case of RTP-over-UDP bearer traffic |

Discussion/conclusions of example 1:

- requested and actual forwarding mode (“UDP payload transparent forwarding”) are identical;

- implicit signalling is thus unambiguous.

### I.3.2	Example 2 “fully RTP protocol unaware forwarding”

Signalling according Table I.3 should lead to a MG behaviour of “fully RTP protocol unaware forwarding”:

Table I.3 – Example 2

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| Add = ip/$ ; Termination Ta, SEP S1 … Stream = 1 { LocalControl {<”RTP impacting elements Y/N?”>} Local {… m=- - - - }, Remote { m=- - - - } other Descriptors{<”RTP impacting elements Y/N?”>} …}, Add = ip/$ ; Termination Tb, SEP S1 … Stream = 1 { LocalControl {<” RTP impacting elements Y/N?”>} Local {… m=- - - - }, Remote { m=- - - - } other Descriptors{<”RTP impacting elements Y/N?”>} …} | The so called BGW mode of operation (in TISPAN BGF, 3GPP IMS-AGW, 3GPP TrGW, MSF SBG) with the characteristic of: - media type agnostic - media format agnostic - transport protocol type agnostic and - L4 port agnostic. The “-“ notation was introduced by TISPAN in border gateway profiles. |

Discussion/conclusions of example 2:

- The MG would implicitly provide “fully RTP protocol unaware forwarding” if there would be not any other signalling elements related to RTP.

- The MGC knows whether the implicit signalling would be unambiguous or not (due to the indicated condition). The MGC may therefore provide an additional explicit indication in case of guaranteed transparent forwarding service requirements.

### I.3.3	Example 3 “additional RTCP awareness”

Example 3: same as example 2, but with additional property “rtcph/rsb = ON” (see [ITU-T H.248.57]) in the LocalControl Descriptor (LCD)

Discussion/conclusions of example 3:

- That’s an explicit indication of RTP bearer traffic, the MG becomes RTP aware.

- However, there are basically three meaningful RTP translator options (PP, TR and TF) with different impact on RTP packets, thus pure implicit signalling would be controversial.

- Unambiguous behaviour might be achieved by explicit assignment of PP or TR topology (with different RTCP handling, but common SSRC/CSRC modifications) in case of non-transparent forwarding.

- Transparent forwarding however could not be achieved via implicit signalling, requires rather the explicit assignment of the TF topology.

### I.3.4	Example 4 “fully specified SDP media description”

Example 4: same semantic aimed as with example 2, but fully specified SDP media description, rather than the “-“ option, see Table I.4:

Table I.4 – Example 4

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| Add = ip/$ ; Termination Ta, SEP S1 … Stream = 1 { LocalControl {<”RTP impacting elements Y/N?”>} Local {… m=audio <port> rtp/avp x1 x2 a=rtpmap:x1 … a=ptime: … }, Remote { m=audio <port> rtp/avp x1 x2 a=rtpmap:x1 … a=ptime: … } other Descriptors{<”RTP impacting elements Y/N?”>} …}, Add = ip/$ ; Termination Tb, SEP S1 … Stream = 1 { LocalControl {<” RTP impacting elements Y/N?”>} Local {… m=audio <port> rtp/avp x1 x2 a=rtpmap:x1 … a=ptime: … }, Remote { m=audio <port> rtp/avp x1 x2 a=rtpmap:x1 … a=ptime: … } other Descriptors{<”RTP impacting elements Y/N?”>} …} | This option was added by 3GPP profiles (in comparison to TISPAN BGF). The MG has to compare all four SDP media descriptions, and if fully identical then to conclude a correspondent RTP topology behaviour. But which one exactly? |

Discussion/conclusions of example 4:

- This example should be functionally identical as example 2, i.e. leading to the same MG behaviour, but there are subtle difference between examples 2 and 4 when considering the dynamics of H.248 signalling.

- Problem: the final establishment of the H.248 stream might be subject of multiple signalling cycles (ADD, MOD, …). The MG becomes normally already RTP aware (even media aware) with the very first H.248 command request. But which topology? B2BRE, media translator, transport translator, transparent forwarding?

- The MG could of course wait till the stream is finally established, but that would be an unsatisfying situation for performance optimized implementations, which may want to achieve e.g. the minimization of call and bearer establishment delays.

- The explicit indication of the aimed RTP topology (MG, PP, TR or TF) in the first command request could solve the ambiguity.

### I.3.5	Example 5 “transport protocol translation for RTP bearer traffic”

Table I.5 illustrates the example RTP-over-UDP to RTP-over-TCP interworking (but could be also RTP-over-UDP to RTP-over-DCCP, etc.).

Table I.5 – Example 5

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| Add = ip/$ ; Termination Ta, SEP S1 … Stream = 1 { LocalControl {<”RTP impacting elements Y/N?”>} Local {… m=- <port> UDP - }, Remote { m=- <port> UDP - } other Descriptors{<”RTP impacting elements Y/N?”>} …}, Add = ip/$ ; Termination Tb, SEP S1 … Stream = 1 { LocalControl {<” RTP impacting elements Y/N?”>} Local {… m=- <port> TCP/RTP/AVP - }, Remote { m=- <port> TCP/RTP/AVP - } other Descriptors{<”RTP impacting elements Y/N?”>} …} | Stream endpoint SEP Ta/S1 is L4+ agnostic, the MG would be RTP unaware. However, SEP Tb/S1 indicates RTP as L4+ protocol. The complete settings lead to the conclusion of transport protocol translation for RTP bearer traffic. |

Discussion/conclusions of example 5:

- That is a different signalling variant, but similar concerning conclusions as example 3.

- The MG would exclude the PP topology (because PP would be based on value “RTP/AVP” instead of “UDP” at Ta/S1 side.

- However, there are still two meaningful RTP translator options (TR and TF), thus, not unambiguous.

- Thus, guaranteed transparent forwarding would require rather the explicit assignment of the TF topology-

### I.3.6	Example 6 “transparent forwarding of encrypted RTP traffic”

This use case relates e.g. to so-called end-to-end media security scenarios where an IP-IP MG is located in the middle of the end-to-end SRTP-enabled RTP session. There are H.248 profiles which use the “m=” line transport codepoint “RTP/SAVP” for the indication of SRTP traffic to the MG, see Table I.6:

Table I.6 – Example 6

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| Add = ip/$ ; Termination Ta, SEP S1 … Stream = 1 { LocalControl {<”RTP impacting elements Y/N?”>} Local {… m=- <port> RTP/SAVP - }, Remote { m=- <port> RTP/SAVP - } other Descriptors{<”RTP impacting elements Y/N?”>} …}, Add = ip/$ ; Termination Tb, SEP S1 … Stream = 1 { LocalControl {<” RTP impacting elements Y/N?”>} Local {… m=- <port> RTP/SAVP - }, Remote { m=- <port> RTP/SAVP - } other Descriptors{<”RTP impacting elements Y/N?”>} …} | SRTP traffic is indicated by the “m=” line, in a media agnostic manner. The complete settings lead to the conclusion of transparent forwarding of SRTP bearer traffic. |

Discussion/conclusions of example 6:

- There could be an ambiguous situation without explicit RTP topology indication (e.g. MG could conclude B2BRE (= PP)), which would be controversial.

- Required would be a transparent forwarding behaviour, which would be satisfied by RTPTT and TF topologies.

- Solution: unambiguous semantic by explicit indication of rtpt/rtptopo = TF.

[End Proposed Correction]

____________________

{Editor's note (2013-01): it was requested to provide a more detailed section on  “Discussion/conclusions” per use case. Primarily whether an example could be really unambiguously interpreted …Contributions are solicited.}