| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4422 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLSPROF: clause 8.2 - Client/server role assignment |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to update clause 8.2.

Discussion

Review of application scenario BFCP-over-TLS/TCP lead to the conclusion of decoupled client/server roles between TLS and underlying transport protocol.

This aspect needs to be clarified in clause 8.2.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 20 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

# 2	References

…

[IETF RFC 4583]	IETF RFC 4583 (2006), Session Description Protocol (SDP) Format for Binary Floor Control Protocol (BFCP) Streams.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| BFCP | Binary Floor Control Protocol |
| --- | --- |
| CA | Certification Authority |
| DB | Data Base |
| IP | Internet Protocol |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| PSK | Pre-Shared Key |
| SEP | Stream Endpoint |
| TCP | Transport Control Protocol |
| TLS | Transport Layer Security |

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

## 8.2	Client/Server Mode

### 8.2.1	TLS client/server role assignments

R-8.2.1/1: The MG is required to support both the TLS server and TLS client mode.

The TLS standards do not make any correlation between the initiation of the underlying transport connection and the TLS server/client mode. Thus this mode is decoupled from the (TCP) SDP “a=setup:” attribute, as used for TLS-based SDP indications (see clause 4/[IETF RFC 4572]).

R-8.2.1/2: The MGC is required to indicate the TLS server/client role to the MG.

NOTE – There is not any existing TLS related SDP defined which represents TLS client/server semantic. [IETF RFC 4572] intentionally delegates such control information up to the application (layer).

R-8.2.1/3: The MG is required to select the TLS client or TLS server role based on the MGC request.

### 8.2.2	Client/server role assignments to TLS transport protocols

R-8.2.2/1: Any IP transport protocol level client/server role assignment shall be basically independent of TLS server and TLS client mode.

Justification:

- TLS is principally designed to be independent from IP transport protocols [IETF RFC 5246], in order to support multiple transport options (e.g., TCP, SCTP).

- SCTP (as TLS transport candidate) does not provide a client/server concept.

- Application protocol BFCP: TCP client/server roles are decoupled from TLS client/server roles (in case of BFCP-over-TLS/TCP), see [IETF RFC 4583].

[End Proposed Correction]

____________________