| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4426 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June, 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.40 IG: Semantical clarification of traffic components as part of inactivity detection |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution tries to clarify the frequently asked question concerning the notion of “application data” traffic in H.248.40 (FAQ: "Which packet types are subject of the detection logic?").

1	Background

The context of H.248.40 usage was apparently fairly straightforward when H.248.40 was developped in 2006/07. Some weaknesses (from specification perspective) where in the meanwhile observed, primarily related to following three items:

- The definition of the adid/ipstop event itself (in clause 6.2.1) is lacking the precise semantics related to the detection condition. 
This information is contained in the scope section (clause 1).

- Notion of “application data”, leading to the question that there might be potentially application (protocol) specific detection conditions.

- Notion of “IP data packet”, hence there are obviously also IP control packets? Is there any relation to H.248 media flow and H.248 control flow components?

2	Discussion

All three items could be clarified by strictly using the detection condition of clause 1 as semantics in clause 6:

H.248.40, clause 1:

This Recommendation defines an event which is related to one or more IP 2-tuples. An individual 2-tuple is given by <IP address, IP port> of an IP flow of a H.248 stream or termination. The set of conditions for inactivity detection is related to IP packet arrival and/or departure events for all 2-tuples of a stream/termination. The condition of packet arrivals or departures respectively is controlled via a dedicated parameter (called "direction").

We’d like to suggest to update the package specification in clause 6, and to add a new Appendix for some concrete examples.

The proposed clarification is not changing any existing syntax/semantic of this H.248 package in our understanding.

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against Recommendation H.248.40 (03/2013).

Change proposal #1:

[Begin Proposed Correction]

# 1	Scope

This Recommendation allows a media gateway controller to request the media gateway to detect that after a certain period of time no Internet protocol application data has flowed on a particular termination/stream. The ability to detect if Internet protocol application data flow has stopped or has not started is useful to avoid dead lock in latching scenarios and also may be of use to detect hanging bearers.

This Recommendation defines an event which is related to one or more IP 2-tuples. An individual 2-tuple is given by <IP address, IP port> of an IP flow of a H.248 stream or termination. The set of conditions for inactivity detection is related to IP packet arrival and/or departure events for all 2-tuples of a stream/termination. The condition of packet arrivals or departures respectively is controlled via a dedicated parameter (called "direction").

The flexibility of inactivity detection logic configurations allows the usage of H.248.40 for various applications (see also appendices I to III).

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 6	Application data inactivity detection package

|  | Package Name: | Application data inactivity detection package |
| --- | --- | --- |
|  | Package ID: | adid (0x009c) |
|  | Description: | This package enables the MGC to be notified when the MG has detected that no IP application data flow has been detected on a termination/stream. The event ("ipstop") is related to one or more IP 2-tuples. An individual 2-tuple is given by <IP address, IP port> of an IP flow of a H.248 stream or termination. The set of conditions for inactivity detection is related to IP packet arrival and/or departure events for all 2 tuples of a stream/termination. NOTE 1 – The notion of "IP application data" was historically selected in order to differentiate from "IP application control" traffic (related to the MGC level IP signalling path for application control protocol traffic and the MG level IP bearer path for associated application data traffic.) The condition of packet arrivals or departures respectively is controlled via a dedicated parameter (called "direction"). |
|  | Version: | 1 |
|  | Extends: | None |

## 6.1	Properties

None.

## 6.2	Events

### 6.2.1	IP flow stop detection

|  | Event Name: | IP flow stop detection |
| --- | --- | --- |
|  | Event ID: | ipstop (0x0001) |
|  | Description: | This event detects if there has been no direction-dependent application data for a set (detection time) interval of time. In cases where it has been indicated that multiple IP ports are associated with a flow (e.g., RTP and RTCP), the detection of no application data flow must be done on all ports before the event is triggered. When applied at a termination level the adid/ipstop event shall be notified when it has been determined that application data has stopped on all of the streams associated with the termination. If, after the initial reporting of the event, and if the event remains active, the detection time elapses without detection of application data, the event is reported again. This may occur multiple times. This event shall be detected irrespective of the StreamMode. For example, even if the stream is set to SendOnly and packets are received, this will be taken into account. |

#### 6.2.1.1	EventDescriptor parameters

##### 6.2.1.1.1	Detection time

|  | Parameter Name: | Detection time |
| --- | --- | --- |
|  | Parameter ID: | dt (0x0001) |
|  | Description: | This is the interval of time after which if no application data flow is detected, the IP Flow Stop Detection event is triggered. The MG checks over intervals of detection time dt if any application data traffic has occurred. If no application data has arrived, then the adid/ipstop event is triggered. NOTE – This may result that more than the detection time has passed between the IP data flow stop and the detection of the event. |
|  | Type: | Integer |
|  | Optional: | Yes (if default is provisioned). |
|  | Possible values: | Any positive number of seconds. |
|  | Default: | Provisioned NOTE: a) Application-independent versus application-specific values: Either a single, global default value or multiple, application specific default values may be provisioned. There is typically a single value configured in case of a provisioning approach, valid for all applications. Multiple, application-specific default values could be also provisioned. The value selection could be tied to application information like the LD/RD-embedded media description. The specific selection method is out of scope of this package definition and may be e.g. defined in an H.248 profile specification. b) Stream- versus Termination-level event arming: When applied at a Termination level, where might be aggregated traffic based on multiple applications (in case of multiple streams). The value selection may be again application-specific or a single value. However in this case value selection must be based on the fact that the event will be notified when only all streams on the Termination have timed out. |

6.2.1.1.2	Direction

|  | Parameter Name: | Direction |
| --- | --- | --- |
|  | Parameter ID: | dir (0x0002) |
|  | Description: | With this parameter, the MGC indicates to the MG which direction of the data flow should be monitored to detect inactivity. Incoming direction means from the outside of the context. Outgoing direction means towards the outside of the context. If direction is set to "BOTH", the MG will generate the event if no data is sent nor received on the termination to/from the outside of the context for an interval of detection time dt. |
|  | Type: | Enumeration |
|  | Optional: | Yes |
|  | Possible values: | "IN" (0x0001) Incoming direction "OUT" (0x0002) Outgoing direction "BOTH" (0x0003) Both directions |
|  | Default: | Both |

#### 6.2.1.2	ObservedEventsDescriptor parameters

None.

## 6.3	Signals

None.

## 6.4	Statistics

None.

## 6.5	Error codes

None.

## 6.6	Procedures

To detect application data inactivity, the MGC should set the adid/ipstop event with an appropriate "detection time" and the appropriate "direction" on the applicable H.248 Stream/Termination. The adid/ipstop event is notified to the MGC:

•	If the MGC has set dir to "IN" and no IP data packets have been received by the MG from the network in that stream/termination by the expiry of the detection time (dt).

•	If the MGC has set dir to "OUT" and no IP data packets have been sent by the MG to the network in that stream/termination by the expiry of the detection time (dt).

•	If the MGC has set direction to "BOTH" and no IP data packets have been sent to nor received from the network in that stream/termination by the expiry of the detection time (dt).

On reception of a NOTIFY.req with the adid/ipstop event, the MGC should take appropriate action.

The triggered action on MGC level may have to take into account:

•	Service features (e.g., unidirectional applications, muted microphone in conference services, etc.).

•	Bearer service configuration settings (e.g., enabled silence suppression mode in case of speech telephony, RTP sessions without RTCP, etc.).

•	H.248 Termination configurations (e.g., H.248 StreamMode Property equals "Inactive").

It is recommended to set timer detection time (dt) with a relevant value, e.g., a multiple of half the round-trip delay or a multiple of the typical packet interarrival time. The mean interarrival time may be IP application-specific (e.g., codec type), may depend on application level framing protocol usage (e.g., RTP packetization time, RTCP transmission interval, etc.) or service-specific (e.g., muted microphone in case of voice over RTP, or suspended streaming in case of RTSP-controlled multimedia streaming).

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

Appendix IV

Examples for IP application data traffic

(This appendix does not form an integral part of this Recommendation)

The ipstop event related detection logic for application data inactivity is based on the observation of 2-tuple(s) given by <IP address, IP port> pairs, i.e., IP transport endpoint identifiers (at the level of a stream or termination).

Such a condition comprises following IP traffic components, such as

- ALL IP packets related to a H.248 media flow and H.248 control flow components

- Bearer and service specific examples:

  - RTP session: RTCP and RTP, and not only RTP

  - T.38 session: e.g. all UDPTL/UDP packets in case of T.38-over-UDP

  - XoTCP: all TCP packets

  - WWW services: all HTTP/TCP packets

  - Streaming service: e.g., RTSP, RTP, RTCP

  - WebRTC service: all UDP packets (for audio, video, text, data, security key exchange, RTCP XR, etc)

But out scope of adid logic:

- ICMP/IP packets

- IP packets related to IP routing protocols (e.g., OSPF/IP packets)

[End Proposed Correction]

____________________