| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4466 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | Q5 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ETRI |  |  |  |
| Title: | Additional requirements for telepresence systems |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

The contribution proposes a media control function and two other functions to be included within section 8 of F.TPS-Reqs.

# References

[IETF CLUE UCS]	IETF draft-ietf-clue-telepresence-use-cases-05, Use Cases for Telepresence Multi-streams.

# Discussion

Section 8 of F.TPS-Reqs describes functional requirements of the telepresence system. We suggest some additional issues that need to be discussed for the telepresence systems.

- Considering of telepresence system’s suggestion of optimal media characteristics to be used for the conferencing consists of heterogeneous devices.

[IETF CLUE UCS] says that it is common in meeting scenarios for people to join the conference from a variety of environments, using different types of endpoint devices; the devices will be PC, handheld and so on; where the H/W and network states of each participating device will greatly differ.

Therefore, it is recommended for the telepresence system to suggest the optimal media characteristics according to the profile of heterogeneous devices. For instance, the optimal media characteristics suggested by the telepresence system are depicted in the following figure.

Two-screen

Telepresence endpoint

IP Network

Three-screen

Telepresence endpoint

Presentation capture

Presentation

screen

Presentation capture

Presentation

screen

Three-screen

Telepresence endpoint

Less than 480P

More than 1080P

More than 1080P

More than 1080P

Less than 720P

Personal Computer

Mobile Phone

- Considering of telepresence system’s reporting of conferencing states.

In the middle of conferencing, a participant may want to review the purpose of meeting, or to know the name of specific participant.  To satisfy this user’s desire, a telepresence system can have means of reporting meeting information to users, such as the name of conference, the list of current participants, the conference chairman, and so on.

Therefore, it is recommended for the telepresence system to provide information related to conference states which include the name of conference, the list of participants, the conference chairman, and so on.

- Considering of telepresence system’s reporting of network conditions.

When the video or audio quality of meeting goes bad, a user may be curious about the current network states to decide whether the meeting should be restarted.

For instances, when a network condition goes bad and a telepresence system tells its user about the current network conditions, then the user can decide his or her fate; if the user believes that the network problem will be recovered in a soon can endure if not leaves the conference.

Therefore, it is recommended for the telepresence system to be able to provide information related to network conditions which related to QoS/QoE such as the network bandwidth, network delay and so on.

# Proposal

According to the discussion above, we propose some additional requirements to be included in section 8 of F.TPS-Reqs.

Med xxx: The telepresence system is recommended to be able to propose optimal media characteristics for the conferencing from a variety of environments using different types of endpoint devices.

Other xxx: The telepresence system is recommended to be able to provide information related to conference states which include the name of conference, the list of participants, and the conference chairman, but are not limited to.

Other xxx: The telepresence system is recommended to be able to provide information related to network conditions which related to QoS/QoE such as the network bandwidth, network delay, loss rate and jitter, but are not limited to.

__________________