| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD XX (WP 1/16) |
|  |  | English only Original: English |
| Question(s): | 21/16 | Geneva, 28 October-8 November 2013 |
| TD |  |  |
| Source: | Editors H.IVSReqs |  |
| Title: | H.IVSReqs "Requirements for intelligent visual surveillance" (New) : Output draft |  |

Introduction

This document contains the editors’ output draft for H.IVSReqs "Requirements for intelligent visual surveillance" (New) from the SG16, Geneva, 28 October – 8 November 2013 meeting.

CONTENTS

1	Scope	4

2	References	4

3	Definitions	4

3.1 Terms defined elsewhere	4

3.2 Terms Defined within this document	5

4	Abbreviations	5

5	Scenarios for IVS	5

5.1	Target Recognition	5

5.2	Event Recognition	7

5.3	Data Analysis	8

5.4	Intelligent alarm and linkage	10

5.5	Intelligent video diagnosis	11

5.6	Intelligent data acquisition	12

5.7	Intelligent video retrieval	13

5.8	Intelligent video synopsis	14

6	Reference Architecture	14

7	Requirements for IVS	15

Appendix I  <Appendix Title>	15

Bibliography	16

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure 5-1 – Facial recognition 5 Figure 5-2 – Target recognition (vehicle) 6 Figure 5-3 – Target recognition (miscellaneous object) 6 Figure 5-4 – Target recognition (miscellaneous object) 6 Figure 5-5 – Region detection 7 Figure 5-6 – Region stranded 7 Figure.5-7 – Tripwire 8 Figure.5-8 – Directed tripwire and region 8 Figure 5-9 Pedestrian flow analysis 9 Fig 5-4-1 Examples of regional security alarm and linkage 10 Fig 5-4-2 traffic violation 11 Fig 5-5-1 video diagnosis 12 Fig 5-6-1 Face recognition 12 Fig 5-6-2 Vehicle license plate recognition 13 Fig 5-8-1 Intelligent video synopsis 14 Figure 6-1: Function components of IVS 14

Draft new Recommendation H.IVSReqs

Requirements for intelligent visual surveillance

AAP Summary

[To be provided before Consent]

Summary

[To be provided before Consent]

Keywords

Intelligent visual surveillance, requirement

1	Scope

The Visual Surveillance service is a telecommunication service focusing on visual (audio included also) application technology, which is used to capture remotely multimedia (such as audio, visual, image and various alarm signal) and present them to the end users with friendly manner, based on managed broadband network with ensured quality, security and reliability ensured.

Intelligent visual surveillance could automatically identify specific objects, behaviours or attitudes in video. It transforms the video into data to be transmitted or archived so that the visual surveillance system can act accordingly.

This Recommendation identifies the scenarios and requirements for intelligent visual surveillance.

2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below.

[ITU-T F.743]	Recommendation ITU-T F.743 (2009), Requirements and service description for visual surveillance.

[ITU-T H.626]	Recommendation ITU-T H.626 (2012), Architectural requirements for visual surveillance.

3	Definitions

3.1 Terms defined elsewhere

This document uses the following terms defined elsewhere:

TBD

3.2 Terms Defined within this document

This document defines the following terms

TBD

4	Abbreviations

| TBD |  |
| --- | --- |

5	Scenarios for IVS

5.1	Target Recognition

5.1.1	Human

It identifies human behaviour according to it’s motion characteristic, such as  running, crossing the border, retention, loitering, falling, theft, tracking, fighting, etc. Specifically, as shown in Figure 5-1, Facial recognition is that it detects face in video and extracts its characteristic. Then this application compares one person with another image or a database of images of individuals, and gets the comparison result. This application might be used in airport, port, railway station, coach station and other public places.

Human image

Facial recognition

Facial

location

Facial feature extraction

comparison

Facial database

Final result

Result of Facial

recognition

Result of Facial

location

Facial feature

Figure 5-1 – Facial recognition

5.1.2	Vehicle (car, ship, aircraft)

As shown in Figure 5-2, it identifies vehicle behaviour according to its motion characteristic, such as speeding, crossing the border, retention, and so on. Vehicle recognition can tell its type and colour. The type can be achieved using some rules about the size of the vehicle. A vehicle that is small than a certain size might be a car and something larger might be a truck. More specific recognition can be achieved using colour, but the colour of an object is dependent on the ambient light. This application might be used to expressway and crossroads.

Figure 5-2 – Target recognition (vehicle)

5.1.3	Miscellaneous objects

As shown in Figures 5-3 and 5-4, miscellaneous objects include moving objects except human and vehicles, for example animals, smoke, fire, pollutants. Specifically, smoke and fire recognition can be used in forests, tunnels, oil and gas station. As shown in Figure 5-4, this system can distinguish between smoke and other objects that are moving in the video.

Figure 5-3 – Target recognition (miscellaneous object)

Figure 5-4 – Target recognition (miscellaneous object)

  - Event Recognition

Editor’s note: Further contributions may be needed to reword and refine some description of the following scenarioes.

    - Perimeter Prevention

Perimeter prevention / boundary detection of some key areas which develops the virtual cordon or virtual warning region can determine the direction of the target, identify the target object, such as person, transportation and so on.

Typical applications of perimeter prevention include the following 4 categories:

- Region detection: Alarm output must be based on the polygon region which is defined by users. When an object enters or leaves, alarm output could be triggered.

Figure 5-5 – Region detection

- Region stranded: Alarm output must be based on the polygon region which is defined by users. When the retention time of an object exceeds a prescribed threshold, alarm output could be triggered.

Figure 5-6 – Region stranded

- Tripwire: A line that is drawn within the surveillance area. When an object crosses that line an alarm is set off.

Figure.5-7 – Tripwire

- Directed tripwire: A line that is drawn within the surveillance area. When an object along the specified direction crosses that line, an alarm is set off.

A virtual cordon is drawn within the video image. When an object which is depending on different targets beyond that line, an alarm is set off. As shown in Figure 5-4, the direction of the virtual cordon can be unidirectional or bidirectional.

(a) Illegal intruder                                           (b) Retrograde detection

Figure.5-8 – Directed tripwire and region

  - Data Analysis

5.3.1	Traffic / pedestrian flow analysis

According to intelligent tracking and recognition technology, Traffic / pedestrian flow analysis system based on the moving target presents video surveillance, motion analysis and feature classification within the specified single or multiple video monitoring area. This can accurately detect the data of pedestrian flow, as shown in Fig 5-1.

Figure 5-9 Pedestrian flow analysis

Through real-time monitoring of pedestrian flow to provide requirements of high-quality service, to provide real-time pedestrian flow detection and analysis of multi-regional and multi-period, intelligent pedestrian flow analysis solution can help managers to understand of pedestrian flow faster and more directly, to make related decision for the best planning, scheduling and layout and to help customers get more business opportunities. Intelligent pedestrian flow analysis solution solves a variety of scenarios, which include the statistics and analysis of pedestrian flow in shopping mall, bus station, parking area, entertainment, stadium, theater and other public places.

- Statistics and analysis of pedestrian flow in shopping mall

Through detection and statistics of pedestrian flow, it is possible to explore the internal law for the company and to provide objective, scientific basis by developing and adjusting business tactics, public tactics, promotion tactics, pricing tactics, stock option, store layout, service mode and content.

- Statistics and analysis of pedestrian flow in bus station

Through automatic, intelligent, accurate acquisition of on and off pedestrian flow, it is possible to detect and manage analysis of vehicle operation information for each site. Transportation owners and managers should use traffic statistics as the basis of vehicle planning, vehicle scheduling and operation management.

- Statistics and analysis of pedestrian flow in parking area

Pedestrian flow statistics help security departments to determine the peak time, the area of pedestrian flow to arrange for additional person and to help managers to study on pedestrian flow distribution. This can effectively raise property values.

- Statistics and analysis of pedestrian flow in entertainment, stadium and theater

By accurate feature data of pedestrian flow, managers can reasonably deploy staffs, accurately plan of future activities through determining the time, locating and arranging for appropriate staffs. Based on variation of pedestrian flow, it is possible to make a reasonable arrangement according to the busiest day or the busiest week of every month, or the busy entrance.

Pedestrian flow statistics is generally based on cameras of a passageway. Cameras are mostly installed indoors, so access position of staffs and outside light can be controlled. Traffic statistics are generally in the outdoor, such as relatively wide multi-lane roads. So it is difficult to control the orientation of the vehicle, to control the light and weather conditions. Intelligent traffic analysis product is more mature in pedestrian flow statistics, but traffic statistics should be improved.

  - Intelligent alarm and linkage

5.4.1 Regional security alarm and linkage

- Step 1: Bob is a security. His job is to find suspicious personnel and abnormal behaviour by watching real-time video from the important residential areas using surveillance system in the room.

- Step 2: Bob opens the intelligent video surveillance system, and draws a virtual line or a polygon with alarm direction as electronic fence on the video images. When some object crossing the electronic fence and direction consistent with the setting, the system will issue sound and light alarms which alert related personnel to deal with.

- Step 3: Leonard lives in this residential area. He entered the underground -garage from the exit. Since Bob had set the electronic fence, the intelligent surveillance system identified Leonard’s abnormal behaviour and raised the alarm. Bob receives the alarm and warns Leonard to correct his behaviour through radio device. In addition, the public security on patrol in the area receives SMS sent by the alarm linkage system to remind him something happened in the garage at the same time.

Similarly, this intelligent surveillance system can detect other abnormal behaviours, such as climbing over the fence, entering the danger zone. Also user can adjust alarming sensitivity in order to exclude interferences, such as bird, illumination changing.

Fig 5-4-1 Examples of regional security alarm and linkage

5.4.2 Intelligent traffic violations monitoring

- Step 1: Sheldon is a traffic police, and his job is to find violation vehicle by surveillance video. Traffic monitoring system can capture violation vehicle images as evidence.

- Step 2: Raj drives through a monitoring camera which is deployed by the intelligent violation detection system. Because of retrograde, his illegal behaviour is captured by the HD camera.

- Step3: The system distinguishes Raj’s vehicle license plate, and sends picture to Raj by Email or SMS automatically to remind him accepting the punishment.

This system can also detect illegal parking, speeding and other violations automatically.

Fig 5-4-2 traffic violation

  - Intelligent video diagnosis

- Step 1: Howard is a video surveillance maintenance staff. He needs to ensure that the 100 surveillance cameras in this office building are in normal running.

- Step2: The intelligent video system can diagnose whether cameras within the building are in normal working or not. Through routine inspection, this system helps Howard to find the abnormal camera, and prompt the location and type.

- Step3: Howard sends an equipment maintenance worker to maintain the abnormal camera.

Similarly, Howard can get alarms for picture quality abnormalities, such as snow, blur and colour cast.

Fig 5-5-1 video diagnosis

  - Intelligent data acquisition

5.6.1 Face recognition

Step 1: Bill works in human resources department and work attendance management is part of his job.

Step2: Two HD cameras were installed at the entrance to capturing in and out video of the entrance every day. Face recognition system can recognizes the face image automatically.

Step3: The system compares those face images with company face image database, and correlate them with the time. Thus Bill can acquire daily staff access attendance reports, and execute staff attendance management easily.

Fig 5-6-1 Face recognition

5.6.2 Vehicle license plate recognition

Step1: Peter is a traffic police, and works on vehicle information management department. Through the license plate recognition system, it can snaps license plate across the intersection.

Step2: Firstly, the system locates the position of license plate. Then it identifies the plate colour, characters and vehicle types using intelligent image analysis technology and stored in information database.

Peter can use this database for fake plate vehicles management, vehicle tracking, etc.

Fig 5-6-2 Vehicle license plate recognition

  - Intelligent video retrieval

Step 1: Steven is a police working in criminal investigation department. When the criminal case occurred, Steven and his colleagues first collect all the surveillance videos locating in that area.

Step 2: Steven use intelligent video retrieval system to processing the videos. The system extracts moving objects information such as size, colour, and motion direction and establishes target index database. This database transforms the non-structured video into the structured index description.

Step3: The scattered target feature clues can quickly locate suspicious objects in the video using the index. For example, Steven searches a red car in several surveillance videos. With the help of intelligent video retrieval system, Steven can quickly locate the red car appeared in all the videos. It assists Steven to searching important information in the massive video data.

Fig 5-7-1 Intelligent video retrieval

  - Intelligent video synopsis

Step 1: Amy is a company staff. In order to look after the elderly and child, she installed (installed) a camera in the house. She wants to browse key information happened in her house quickly.

Step 2: First of all, Amy imports the daytime videos to the video synopsis system. After target recognition is finished, target and related information are indexed.

Step3: As Amy’s requirement, different targets are stitched to the same background scene and combined together. After that, the video length is compressed and Amy can get a five-minute compressed video instead of one-hour.

Fig 5-8-1 Intelligent video synopsis

6	Reference Architecture

Figure 6-1: Function components of IVS

Figure6-1 shows a proposed overall function architecture for IVS, based on the architecture defined in the Recommendation ITU-T H.VSarch showed on the right part，functional architecture for Recommendation H.IVSReqs focuses on a visual surveillance system with intelligent units, including IVM( Intelligent Video Management) server and IVU(Intelligent Video Unit) and APP, showed on the left part in figure 6-1.

The architecture also contains ICU(Intelligent Customer Unit)  and IPU(Intelligent Premises Unit) showed on the right part in  Figure 6-1.

IVU: IVU automatically identify specific objects, and output recognition results to IVM. The recognize information includes event trigger information and data acquisition information. IVU can include more than one intelligent analysis algorithms which can be added or deleted according to the requirements.

IVM: IVM supports users in configuring intelligent application strategies and dynamically scheduling video sources according to the intelligent application strategies. IVM dynamically schedule IVU and accept registration, deletion, capability report of IVU. IVM can be invoked by an intelligent application from the CMU, and then sends interface information to CMU.

APP : APP is the third-party data application analysis system, which is not the standard unit in general. APP can accept data from the platform, and then configure data analysis algorithms and strategies. can achieve the realization of the data analysis. APP can send alarm information to the platform if needed .In addi This tion, APP can be built to meet individual requirements of users. With the consideration that alarm processing is the main function and has heavy load, APP should be stripped from CMU, which make the function of platform units clearer and make the platform strong extendibility.

ICU: In order to achieve comprehensive intelligent video recognition function, we can add CIV(Client Intelligent Video) module to CU. CIV recognizes required information from input videos, and output recognition results. In addition, CIV can search designated records from video files. The Identifiable information includes event trigger information and data acquisition information.

IPU:  IPU just adds PIV(Premises Intelligent Video)  module to PU, PIV is intelligent recognition module of PU, which can be achieved in PU devices and be PU integration. PIV can recognize required information from input videos, and output recognition results. The recognize information includes event trigger information and data acquisition information, etc.

7	Requirements for IVS

[TBD]

Appendix I

<Appendix Title>

(This appendix does not form an integral part of this Recommendation)

[TBD]

Bibliography

[TBD]	TBD

_______________