| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 80R1 (WP 1/16) |
|  |  | English only Original: English |
| Question(s): | 3/16 | Geneva, 28 October - 8 November 2013 |
| TD |  |  |
| Source: | Editor H.248.DTLS |  |
| Title: | H.248.DTLS "Gateway control protocol: H.248 support for control of transport security using DTLS" (New): Output draft Ed. 0.4 |  |

Summary

This document contains the editor’s output draft ed. 0.4 for draft new ITU-T H.248.DTLS "Gateway control protocol: H.248 support for control of transport security using DTLS".

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.DTLS Ed. 0.1 | Initial draft (AVD-4419) to WP 1/16 Oslo meeting (2013-06) Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1202_Gen/TD-23.zip |
| H.248.DTLS Ed. 0.2 | Output (TD-34) from WP 1/16 Oslo meeting (2013-06) Location: http://ftp3.itu.int/av-arch/avc-site/2013-2016/1306_Osl/TD-34.zip |
| H.248.DTLS Ed. 0.3 | Input (TD 80/WP1) to SG16 Geneva meeting, Oct/Nov 2013 Location: http://www.itu.int/md/T13-SG16-131028-TD-WP1-0080/en |
| H.248.DTLS Ed. 0.4 | Output (TD 80R1/WP1) from SG16 Geneva meeting, Oct/Nov 2013 Location: http://www.itu.int/md/T13-SG16-131028-TD-WP1-0080/en |
| H.248.DTLS Ed. 0.5 |  |
| H.248.DTLS Ed. 0.6 |  |
| H.248.DTLS Ed. 0.7 |  |
| H.248.DTLS Ed. 0.8 |  |
| H.248.DTLS Ed. 0.9 |  |
| H.248.DTLS Ed. 0.10 |  |

CONTENTS

Page

1 Scope 5 2 References 5 3 Definitions 6 3.1 Terms defined elsewhere 6 3.2 Terms defined in this Recommendation 6 4 Abbreviations and acronyms 6 5 Conventions 7 6 Use case descriptions 7 7 Models 7 8 Basic session control package (for DTLS) 7 9 Extended session control package (for DTLS) 7 10 Capability negotiation package (for DTLS) 7 11 Session maintenance package (for DTLS) 7 12 Traffic volume metrics package (for DTLS) 7 Annex A <Annex Title> 8 Appendix I <Appendix Title> 9 Appendix I Signalling flows for basic DTLS session establishment and release 9 I.1 Overview 9 I.2 Conventions 9 I.3 Establishment of DTLS security sessions 9 I.3.1 Successful establishment – Terminating side 9 I.3.2 Successful establishment – Originating side 10 I.3.3 Unsuccessful establishment 11 I.4 Release of DTLS security sessions 11 I.4.1 Successful release – Terminating side 11 I.4.2 Successful release – Originating side 12 I.4.3 Unsuccessful release 13 Appendix IV Living List related to DTLS and DTLS-SRTP 13 IV.1 DTLS protocol 13 IV.2 DTLS related services 13 IV.3 DTLS in scope of H.248.TLS? 14 IV.4 DTLS use cases 14 IV.4.1 Use cases from TLS-to-DTLS interworking perspective 14 IV.4.2 Use cases from SRTP "key exchange" perspective 15 IV.4.3 Two interpretations for "DTLS-SRTP"? 16 IV.4.4 Use cases from WEBRTC perspective 17 IV.4.5 Use cases from Telepresence perspective 18 Bibliography 18

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure I.1 – Successful establishment – Terminating side 10 Figure I.2 – Successful establishment – Originating side 11 Figure I.3 – Successful release – Terminating side 12 Figure I.4 – Successful release – Originating side 12 Figure IV.1 – TLS/DTLS protocol historic 13 Figure IV.2 – Example TLS-to-DTLS interworking 14 Figure IV.3 – Media-path decoupled key exchange using SDP (via SIP & H.248) 15 Figure IV.4 – Media-path coupled key exchange using DTLS 15 Figure IV.5 – Interpretation "DTLS only used for SRTP key exchange" 16 Figure IV.6 – Interpretation "Secure RTP traffic using DTLS" 16 Figure IV.7 – DTLS use cases: "WebRTC data" & "WebRTC SRTP key exchange" 17

Draft new ITU-T Recommendation H.248.DTLS

Gateway control protocol: 
H.248 support for control of transport security using DTLS

AAP Summary

<Mandatory material – to be provided before Consent>

Summary

<Mandatory material>

Keywords

H.248, TLS, DTLS

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# 1	Scope

The DTLS protocol [b-IETF RFC 4347] and [IETF RFC 6347] is derived and thus aligned with the TLS protocol [IETF RFC 5246]. There are consequently many commonalities between the control of DTLS bearers and TLS bearers in H.248 gateways.

H.248-controlled TLS bearers are subject of [ITU-T H.248.TLS].

The purpose of this Recommendation is to define usage of [ITU-T H.248.TLS] for DTLS bearers.

In scope of this Recommendation are in particular

- description of DTLS specific use cases;

- description of MG bearer plane differences between DTLS and TLS;

- usage of TLS-defined H.248 packages for DTLS bearer types; and

- procedures for the specific application of DTLS-SRTP [IETF RFC 5764].

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway control protocol: Version 3.

[ITU-T H.248.TLS]	Recommendation ITU-T H.248.TLS (2014), Gateway control protocol: H.248 packages for control of transport security using TLS.

[ITU-T H.248.TLSPROF]	Recommendation ITU-T H.248.TLSPROF (2014), Gateway control protocol: Guidelines on the use of H.248 capabilities for transport security in TLS networks in H.248 Profiles.

[IETF RFC 5246]	IETF RFC 5246 (2006), The Transport Layer Security (TLS) Protocol Version 1.2.

[IETF RFC 5764]	IETF RFC 5764 (2010), Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP).

[IETF RFC 6347]	IETF RFC 6347 (2012), Datagram Transport Layer Security Version 1.2.

# 3	Definitions

<Check in the ITU-T Terms and definitions database on the public website whether the term is already defined in another Recommendation. It may be more consistent to refer to such a definition rather than redefine it>

## 3.1	Terms defined elsewhere

<Normally terms defined elsewhere will simply refer to the defining document. In certain cases, it may be desirable to quote the definition to allow for a stand-alone document>

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]: <optional quoted definition>

3.1.2	<Term 2> [Reference]: <optional quoted definition>

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	<Term 3>: <definition>

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| DTLS | Datagram Transport Layer Security |
| --- | --- |
| IP | Internet Protocol |
| IPv4 | Internet Protocol Version 4 |
| IPv6 | Internet Protocol Version 6 |
| L3 | Layer three |
| L4 | Layer four |
| L4+ | Above layer four |
| MAC | Message Authentication Code |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| SDP | Session Description Protocol |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SIP | Session Initiation Protocol |
| SSL | Secure Sockets Layer |
| TCP | Transmission Control Protocol |
| TLS | Transport Layer Security |
| UDP | User Datagram Protocol |

# 5	Conventions

<Describe any particular notation, style, presentation, etc. used within the Recommendation, if any>

# 6	Use case descriptions

<Clause 6 onwards contains the body of the Recommendation>

# 7	Models

FIXTHIS

# 8	Basic session control package (for DTLS)

FIXTHIS

# 9	Extended session control package (for DTLS)

FIXTHIS

# 10	Capability negotiation package (for DTLS)

FIXTHIS

# 11	Session maintenance package (for DTLS)

FIXTHIS

# 12	Traffic volume metrics package (for DTLS)

FIXTHIS

Annex A

<Annex Title>

(This annex forms an integral part of this Recommendation.)

<Body of annex A>

Appendix I

<Appendix Title>

(This appendix does not form an integral part of this Recommendation.)

<Body of appendix I>

Appendix I

Signalling flows for 
basic DTLS session establishment and release

(This appendix does not form an integral part of this Recommendation.)

## I.1	Overview

The tlsbsc package (clause 8/[ITU-T H.248.TCP]) defines the basic TLS security session control with scope on support for establishment and release. Basic DTLS session control reuses the solution defined by this package.

## I.2	Conventions

An H.248 context with a single SEPP; only one SEP (labelled as T1(S1)) is considered. The MG bearer interface (DTLS) is highlighted besides the H.248 interface.

Furthermore all Figures indicate possible Event notifications to the MGC by the MG. The particular event(s) would be related to state changes of the local DTLS session endpoint.

It has to be noted that some DTLS specifics are not visible at the considered level of abstraction, such as

- Bearer type indication (via SDP "m=" line) when creating a DTLS-enabled SEP / Termination; and

- DTLS timeouts and retransmissions (during e.g. establishment handshake procedures) (i.e., related to [IETF RFC 6347]/Figure 3 "DTLS Timeout and Retransmission State Machine").

## I.3	Establishment of DTLS security sessions

### I.3.1	Successful establishment – Terminating side

See Figure I.1, termed as use case (E.1):

Figure I.1 – Successful establishment – Terminating side

More details for this example:

- The MGC blocks the start of DTLS session establishment (1).

- Early incoming DTLS messages are discarded by the MG (2).

- The MGC unblocks the DTLS SEP (3).

- DTLS session establishment (5 to 7); MG acting as DTLS server.

- Optional notification of MGC concerning available DTLS session for application data transfer (8), if subscription to event by MGC (4).

### I.3.2	Successful establishment – Originating side

See Figure I.2, use case (E.2).

Figure I.2 – Successful establishment – Originating side

More details for this example:

- Steps 1 to 3 as in clause I.3.1

- DTLS session establishment (5 to 7), triggered by MGC (4); MG acting as DTLS client.

- Optional notification of MGC concerning available DTLS session for application data transfer (9), if subscription to event by MGC (8).

### I.3.3	Unsuccessful establishment

The DTLS security session negotiation handshake is basically not completed.

## I.4	Release of DTLS security sessions

### I.4.1	Successful release – Terminating side

See Figure I.3, termed as use case (R.1).

Figure I.3 – Successful release – Terminating side

More details for this example:

- MG receives an incoming DTLS close_notify alert (2), the indication for DTLS session release.

- The MG acknowledges the release request (3).

- Optional notification of MGC concerning successfully released DTLS session (4), if subscription to event by MGC (1).

### I.4.2	Successful release – Originating side

See Figure I.4, use cases (R.2) illustrates an outgoing DTLS session release:

Figure I.4 – Successful release – Originating side

More details for this example:

- MGC initiates the DTLS session release (1).

- MG sends an outgoing DTLS close_notify alert (2), the indication for DTLS session release to wards remote DTLS endpoint X.

- Successful release (3).

- Optional notification of MGC concerning successfully released DTLS session (5), if subscription to event by MGC (4).

### I.4.3	Unsuccessful release

An unsuccessful DTLS session release procedure would imply still that the MG remains in H.248 state Established and that the MGC could possibly suspect a yet established DTLS session. There are multiple options how such protocol deadlocks could be resolved. The situation could be e.g. cleared by the subtraction of the termination by the MGC.

Appendix L
Living List related to DTLS and DTLS-SRTP

(This appendix does not form an integral part of this Recommendation)

## L.1	DTLS protocol

DTLS is derived from TLS, could be thus considered as a single security protocol family. DTLS versioning numbers are (somehow) linked to TLS protocol versions.

Figure L.1 – TLS/DTLS protocol historic

## L.2	DTLS related services

There are two slightly different applications:

- DTLS as "transport security” means for UDP/IP bearer traffic

- DTLS as "key exchange” means for RTP/UDP/IP bearer traffic using media security according SRTP

## L.3	DTLS in scope of H.248.TLS?

There are basically following options:

- DTLS covered by H.248.TLS?

- DTLS in a self-contained, new H.248.DTLS?

- DTLS added to H.248.77 due to link with DTLS-SRTP?

- DTLS covered by H.248.WEBRTC due usage by WebRTC service.

Option #2 (or #1) seems to be preferable.

## L.4	DTLS use cases

Initial list of possible use cases:

### L.4.1	Use cases from TLS-to-DTLS interworking perspective

Figure L.2 provides an interworking example (here: DTLS based secure WebRTC data service).

Figure L.2 – Example TLS-to-DTLS interworking

### L.4.2	Use cases from SRTP "key exchange" perspective

SRTP-based media encryption, - there are many key exchange options for SRTP. For example the two outlined in the sub-clauses below.

#### L.4.2.1	Media-path decoupled key exchange using SDP (via SIP & H.248)

Figure L.3 recalls again the protocol stack for SDES-based key exchange for SRTP, as in scope of [b-ITU-T H.248.77] or as part of the 3GPP H.248 Iq profile ([b-ETSI TS 129 334]):

Figure L.3 – Media-path decoupled key exchange using SDP (via SIP & H.248)

#### L.4.2.2	Media-path coupled key exchange using DTLS

Figure L.4 illustrates the alternative of a DTLS-based key exchange for SRTP:

Figure L.4 – Media-path coupled key exchange using DTLS

### L.4.3	Two interpretations for "DTLS-SRTP"?

[b-IETF RFC 5764]:

This document describes DTLS-SRTP, a SRTP extension for DTLS that

combines the performance and encryption flexibility benefits of SRTP

with the flexibility and convenience of DTLS-integrated key and

association management.  DTLS-SRTP can be viewed in two equivalent

ways: as a new key management method for SRTP, and a new RTP-specific

data format for DTLS.

I.e., there are two possible interpretations from protocol stack point of view, see Figures L.5 and L.6.

Figure L.5 – Interpretation "DTLS only used for SRTP key exchange"

Figure L.6 – Interpretation "Secure RTP traffic using DTLS"

It has to be noted that these interpretations might be incorrect. To be confirmed.

### L.4.4	Use cases from WEBRTC perspective

DTLS use cases from WebRTC perspective.

Figure L.7 – DTLS use cases: "WebRTC data" & "WebRTC SRTP key exchange"

In Figure L.7:

- WebRTC data traffic

- WebRTC DTLS-based key exchange for SRTP

### L.4.5	Use cases from Telepresence perspective

Are there any? TBD

Bibliography

[b-IETF RFC 4347]	IETF RFC 4347 (2006), Datagram Transport Layer Security.

__________________