| Question(s): | 2 |  | Meeting, date: |  | 28 October - 8 November 2013 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 1 | Intended type of document (R-C-TD): |  | C |
| Source: | Cisco Systems, Inc. |  |  |  |  |  |
| Title: | URN Identifiers in H.325 |  |  |  |  |  |
| Contact: | Paul E. Jones Cisco Systems, Inc. USA |  |  |  | Tel: +1 919 476 2048 Fax: Email: paulej@packetizer.com |  |
| Contact: |  |  |  |  | Tel: Fax: Email: |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

# Background

During the recent meeting of Q2/16 in Oslo, Norway, experts discussed possible ways to identify protocol parameters, capabilities, etc. within H.325.  For example, if an application would like to advertise itself as the “video” application, how does it do that?  If the video application would like to advertise a set of capabilities, how are those capabilities identified?

Within H.323 systems, the answer to those questions vary.  Sometimes we utilize an OID, sometimes we utilize a single integer, and sometimes capabilities are defined within the ASN.1 syntax.

For H.325, we could use a mixture of identifiers, as well.  Since H.325 will be specified using JSON, we could consider string identifiers and integers. Integers work well within the context of a single application in some cases, but are not well-suited for capability advertisement since meaning would be ambiguous.  For this, we need to utilize a text string with defined structure.

As a possible solution, we discussed identifying capabilities and features using URIs.  URIs would enable the ITU-T or software developers to identify capabilities and features within a namespace each controls.  URIs are structured text strings that are well-suited for use in JSON.

# Discussion

The question before us now is which URI scheme(s) to utilize.  Since these URIs are only used as identifiers, it makes little difference to the processing entities.  The receiver of these URIs will just perform a simple string comparison when looking for particular features or capabilities.

Considering the H.325 “video” application as an example, we could utilize:

- http://itu.int/ns/h.325/video

- urn:oid:0.0.8.325.1.0

- urn:itu:t:rec:h.325:p:video

While all of these are fictitious, they are nonetheless reasonable sample representatives of the types of URIs we could utilize.

The “http” scheme is interesting since it could be utilized outside of H.325 to enable a human to learn more about the particular identifier.  As an example, see http://packetizer.com/ns/activated.

The URN “oid” namespace (IETF RFC 3061) has certain appeal since we use OIDs within H.323 systems today.  A concern with using OIDs, particularly in a protocol like H.325, is that the text strings are somewhat cryptic.  If you compare the “oid” URN with the other two alternatives, that is clear.  OIDs are well suited for compact encoding in H.323 systems, but neither significant space nor readability benefits exit with the “oid” URNs.

The last example shows a URN that utilizes the “itu” namespace.  This one does not offer any particular advantage over the “http” variant in terms of readability or utility outside H.325.  However, in consulting experts in this area, it was generally viewed that URNs are the best choice for the task at hand since URNs are identifiers that are independent from a particular protocol and persist through time.  Plus if structured properly, URNs can help software developers and those trying to troubleshoot the protocol since the text strings are readable by humans and should be generally understood.

# Proposal

This contribution proposes the use of URNs in the “itu” namespace for use within H.325 core specifications and application protocols defined by the ITU.  (This proposal does not intend to restrict use of any particular URI scheme from being used by any third-party application.)  In order to utilize the “itu” namespace, however, it will first be necessary to register the namespace with IANA as per the procedures described in IETF RFC 3406.  An initial proposal is attached to this contribution, which must be submitted to the IETF for review and publication.

_____________________