| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4481 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.SEPLINK: clause 1 to 6 - update proposal |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides update proposals for clauses 1 to 6.

1	Introduction

Purpose of this contribution:

- provide initial text for blank sections and

- resolve editor's notes

2	Discussion

The editor's note(s) are addressed by parallel contributions AVD-4483 and AVD-4482. The terminology used is based on the syntax changes (ABNF based) of the linktopo property (AVD-4482). AVD-4483 provides some illustrations and categorizes three basic interlinkage configurations.

The proposed changes by this contribution should be consistent with the parallel SEPLINK contributions.

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4477).

Change proposals:

[Begin Proposed Correction]

# 1	Scope

The protocol stack of an H.248 media gateway bearer interface may contain principally connectionless and connection-oriented protocols. The concerned protocol layer(s) from H.248 perspective, i.e., which are in scope of the H.248 Stream Descriptors, are typically a network layer (L3) and higher. Connection-oriented protocols are known from L2 onwards in general, and in case of IP from L4 and higher concerning with respect to basic reference models for layered protocol architectures.

Connection-oriented protocols are characterized by explicit bearer connection control procedures, related to the establishment, modification and/or release of a bearer connection. Normally, the MG as slave is triggered by the master MGC for a particular bearer connection control procedure, and then autonomously executing such a procedure at the bearer interface related to a particual H.248 termination (T) / stream endpoint (SEP).

The end-to-end bearer plane connectivity implies typically scenarios there multiple SEPs are interconnected in an H.248 context. SEP-individual bearer control procedures are then often interlinked; e.g. an incoming bearer connection establishment procedure (at one SEP) could lead to an outgoing bearer connection establishment procedure (at a partner SEP), - at the same or at a different protocol layer.

An MGC may wish to individually control each of these procedures (so called MGC-strictly controlled mode). But there are many use cases where the MGC could benefit from a delegation of "interlinkage work" down to the MG itself.

Such kind of behaviour is the purpose of this Recommendation.

An H.248 package, called stream endpoint interlinkage package (text codepoint 'seplink'), is defined in a rather generic manner, which represents a fairly flexible tool for a wide range of interlinkage options.

The supported interlinkage

- is limited to a single context,

- is direction dependent,

and allows the interlinkage

- of protocol endpoints, at the same or different layers, in different SEPs ("inter-SEP interlinkage"), and

- of protocol endpoints at different layers within an individual SEP ("intra-SEP interlinkage"),

and

- of an individual SEP again may be armed with one or multiple interlinkage configurations.

## 1.1	Applicability statements

The capability of "interlinkage" relates to the MG internal forwarding of protocol control information (PCI) between different SEPs and/or different protocol layers. A particular layered protocol architecture is thus associated with a SEP.

The interlinkage capability is basically limited on protocol stack segments, which may be unambiguously described via the mechanism according clause 7.1.1.

The defined interlinkage capability is basically generic, i.e., applicable for all kind of connection-oriented bearer protocols. However, the focus of the initial package version is tightly coupled with protocols related to IP.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.TCP]	Draft Recommendation ITU-T H.248.TCP (201X), Gateway control protocol: TCP support packages.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	stream endpoint (SEP) [clause 3.2.10/ITU-T H.248.1]: relates to a stream associated to a Termination. A SEP is identified by the tuple of StreamID and TerminationID values.

3.1.2	stream endpoint pair (SEPP) [clause 3.2.11/ITU-T H.248.1]: two associated Stream Endpoints (SEP) of two Terminations within the same Context. The two SEPs share thus the same StreamID value. There is always a point-to-point topology for Stream Endpoint Pair configurations (e.g., there is no further associated SEP on a third Termination etc).

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	stream endpoint tuple (SEPT): the generalization of SEPP towards multiple associated Stream Endpoints (SEP) within the same Context. All SEPs share thus the same StreamID value. The stream topology is given by the topology descriptor settings.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| EP | End Point |
| --- | --- |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| SCTP | Stream Control Transmission Protocol |
| SDP | Session Description Protocol |
| SEP | Stream End Point |
| SEPP | Stream End Point Pair |
| SEPT | Stream End Point Tuple |
| T | Termination |
| TCP | Transmission Control Protocol |
| TLS | Transport Layer Security |

# 5	Conventions

None.

# 6	Motivation, use cases and models

H.248 controlled Media Gateways may terminate multiple connection-oriented transport layer protocols, like TCP, TLS, SCTP.

This is outlined in the following Figure 1 where in a given H.248 context each Stream Endpoint (SEP) is supposed to host at least one or more connection-oriented transport layer endpoints.

Figure 1 – Two-termination context with multiple connection-oriented protocols 
in the SEP-specific protocol stack (segments)

The interlinkage concept is constituted by following characteristics:

- the interlinkage scope is limited to a single context;

- an interlinkage configuration is defined via an H.248 Property, which itself is linked to a particular stream endpoint/termination;

- a specific external, incoming bearer control procedure (e.g., establishment event) is immediately triggering the execution of an external, outgoing bearer control procedure, at the same or another termination in the context;

- a single interlinkage configuration acts in an unidirectional manner; and

- multiple interlinkage configurations in parallel may be defined for a single SEP.

Thus, three principal interlinkage types may be differentiated (see also illustration in Appendix I.2):

- inter-SEP interlinkage (when source and interlinked transport endpoints are located at different terminations);

- intra-SEP interlinkage (when source and interlinked transport endpoints are located at the same termination); and

- multi-SEP interlinkage (when a single stimuli is used to trigger multiple outgoing bearer procedures).

The following Figure 2 outlines the various interlinkage options enabling an MG autonomous establishment/release of a connection/session-oriented partner transport protocol endpoint (again at the example of a stream endpoint pair (SEPP)):

Figure 2 – Principle interlinkage options (here: two-termination context) 
from perspective of a single SEP (here: SEP-A)

Example interlinkage configurations (in Figure 2):

- Intra-SEP interlinkage from lower to higher protocol layer: within one SEP a lower layer connection oriented transport protocol endpoint (EP) triggers an upper layer connection oriented transport protocol EP. 
Note that this doesn’t mean that lower layer service access point is used by the upper layer transport protocol directly, i.e. there might reside other connectionless protocol layers in between.

- Intra-SEP interlinkage from higher to lower protocol layer: within one SEP an upper layer connection oriented transport protocol EP triggers a lower layer connection oriented transport protocol EP. 
Note that this doesn’t mean that lower layer service access point is used by the upper layer transport protocol directly, i.e. there might reside other connectionless protocol layers in between.

- Inter-SEP interlinkage: Given a SEP pair SEP-A triggers the connection oriented transport protocol of the peer SEP-B. Either the same or a different transport protocol can be interlinked, thus for example SEP-A TCP endpoint could trigger a SEP-B TCP endpoint, or a SEP-A TCP endpoint could trigger e.g. a SEP-B SCTP endpoint.

The first three interlinkage configurations are part of SEP-A. When configuration (3) should act in a bidirectional manner, then following interlinkage configuration needs to be associated with SEP-B:

- Inter-SEP interlinkage: Given a SEP pair SEP-A receives a trigger regarding a connection oriented transport protocol from its peer SEP-B. Either the same or different transport protocol can be interlinked, thus for example SEP-B TCP endpoint could trigger a SEP-A TCP endpoint, or a SEP-B TCP endpoint could trigger e.g. a SEP-A SCTP endpoint.

There are following principal limitations:

- Horizontal protocol layer interlinkage (i.e., related to the stream endpoint pair (SEPP)) is only permitted between SEPs belong to the same H.248 stream.

- Vertical protocol layer interlinkage is only permitted within a SEP, i.e. between its supported transport protocol layers.

A H.248 context may contain more than two terminations; where more than two terminations host SEPs belonging to the same H.248 stream. In this case the SEP interlinkage capabilitiy needs to be able to address a dedicated stream endpoint tuple (SEPT).

For example, following interlinkage scheme should be supported in case of an H.248 context containing three SEPs (same H.248 streamID).

Figure 3 – Interlinkage example for a three-termination context

Whereas SEPP (SEP-A, SEP-B) can trigger each other regarding the identified transport protocol endpoints, the SEP-A connection oriented transport protocol EP may trigger SEP-C connection oriented transport protocol EP in addition. However SEPP (SEP-B, SEP-C) doesn’t have any interlinkage, whereas also SEP-C connection oriented transport protocol EP cannot trigger SEP-A connection oriented transport protocol EP in any way.

[End Proposed Correction]

____________________