| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4500 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLSPROF: update of profile guidelines |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

Purpose of this contribution is to complete missing content of clause 10 and associated Appendix I.

Discussion

Clause 10 and associated Appendix I was started at the last meeting due to the consolidated and stable base of requirements according to clauses 6 to 9. The remaining empty sections in clause 10 and associated Appendix I could be completed.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4475).

Change proposal:

[Begin Proposed Correction]

# 10	H.248 profile specification guidelines

This clause provides guidelines for H.248 profile specifications. The structure follows the profile template according to Appendix III/[ITU-T H.248.1].

The template elements which are not applicable in this Recommendation are indicated by "Subject to profile specification".

Any profile guidelines are primarily dependent on the concerned network configuration and use case. The guidelines in this clause are therefore basically conditional. Two exemplary use cases are considered (as described in Appendix I), termed as capability set CSA and CSB.

## 10.1	Profile Identification

Subject to profile specification.

## 10.2	Summary

Subject to profile specification.

## 10.3	Gateway Control Protocol Version

Subject to profile specification.

## 10.4	Connection Model

| Maximum number of contexts: | Subject to profile specification. |
| --- | --- |
| Maximum number of terminations per context: | Subject to profile specification. Examples: IF CSA THEN "2". IF CSB THEN "2". |
| Allowed termination type combinations in a context: | Subject to profile specification. |

## 10.5	Context Attributes

Subject to profile specification.

## 10.6	Terminations

Subject to profile specification.

## 10.7	Descriptors

### 10.7.1	TerminationState Descriptor

Subject to profile specification.

### 10.7.2	Stream Descriptor

Subject to profile specification.

### 10.7.3	Events Descriptor

IF CSA OR CSB THEN following table:

| Events settable on termination types and stream types: | Yes |  |  |  |
| --- | --- | --- | --- | --- |
| If yes | EventID | Termination type | Stream type |
|  | See clause 10.14.3.1 - tlsbsc/BNCChange | ALL except ROOT | TLS |
|  | See clause 10.14.3.6 - tcpbcc/BNCChange | ALL except ROOT | ANY (if TCP) |

All other aspects related to Events Descriptor (e.g., EventBuffer Control, KeepActive, Notification Behaviour) are "Subject to profile specification".

### 10.7.4	EventBuffer Descriptor

Subject to profile specification.

### 10.7.5	Signals Descriptor

IF CSA OR CSB THEN following table:

| The setting of signals is dependent on termination or streams types: | Yes |  |  |
| --- | --- | --- | --- |
| If yes | SignalID | Termination Type | Stream Type/ID |
|  | See clause 10.14.3.1 - tlsbsc/EstBNC - tlsbsc/RelBNC | ALL except ROOT | TLS |
|  | See clause 10.14.3.6 - tcpbcc/EstBNC - tcpbcc/RelBNC | ALL except ROOT | ANY (if TCP) |

All other aspects related to Signals Descriptor (e.g., Signal Direction, Signal List) are "Subject to profile specification".

### 10.7.6	DigitMap Descriptor

Subject to profile specification.

### 10.7.7	Statistics Descriptor

IF CSA THEN none.

IF CSB THEN following table entries:

| Statistics supported on: | Both (i.e., stream and termination level) |
| --- | --- |

| Statistics reported on Subtract: | Yes |  |
| --- | --- | --- |
| If yes | StatisticIDs reported: | tlstv/recadpduo tlstv/sentadpduo tlstv/recado tlstv/sentado |

### 10.7.8	ObservedEvents Descriptor

Subject to profile specification.

### 10.7.9	Topology Descriptor

Subject to profile specification.

### 10.7.10	Error Descriptor

Subject to profile specification.

NOTE – The TLS session maintenance package may be used for enhanced abnormal TLS session handling (but this is beyond the example capability sets CSA and CSB).

## 10.8	Command API

NOTE – It is assumed that an Error Descriptor may be returned in any command reply.

### 10.8.1	Add

Subject to profile specification.

### 10.8.2	Modify

Subject to profile specification.

### 10.8.3	Subtract

Subject to profile specification.

### 10.8.4	Move

Subject to profile specification.

### 10.8.5	AuditValue

Subject to profile specification.

### 10.8.6	AuditCapabilities

Subject to profile specification.

### 10.8.7	Notify

Subject to profile specification.

### 10.8.8	ServiceChange

Subject to profile specification.

### 10.8.9	Manipulating and auditing context attributes

Subject to profile specification.

## 10.9	Generic command syntax and encoding

Subject to profile specification.

## 10.10	Transactions

Subject to profile specification.

NOTE – There is no impact on Transactions.

## 10.11	Messages

Subject to profile specification.

## 10.12	Transport

Subject to profile specification.

NOTE – Usage of bearer security may demand for an secured H.248 transport mode, too.

## 10.13	Security

Subject to profile specification.

NOTE – It need to be identified whether the H.248 MG might be a potential point of security attacks (due to possible security threats at IP, TCP and/or TLS layer).

## 10.14	Packages

### 10.14.1	Mandatory Package

Mandatory: specifies the packages that shall be supported in this profile.

Examples:

IF CSA OR CSB THEN "following table":

| Mandatory packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | PackageID | Version | Termination Types Supported |
| "TLS basic session control package" [ITU-T H.248.TLS] | tlsbsc, (0x####) | v1 | TLS |
| "TCP basic connection control package" [ITU-T H.248.TCP] | tcpbcc, (0x####) | v1 | ANY (if TCP) |

### 10.14.2	Optional Package

Examples:

IF CSA OR CSB THEN "following table":

| Optional packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | PackageID | Version | Termination Types Supported |
| "TLS extended session control package" [ITU-T H.248.TLS] | tlsesc, (0x####) | v1 | TLS |
| "TCP extended connection control package" [ITU-T H.248.TCP] | tcpecc, (0x####) | v1 | ANY (if TCP) |
| "Stream endpoint interlinkage package" [ITU-T H.248.SEPLINK] | seplink, (0x####) | v1 | TLS, TCP |

Examples:

IF CSB THEN "following table":

| Optional packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | PackageID | Version | Termination Types Supported |
| "TLS capability negotiation package" [ITU-T H.248.TLS] | tlscn, (0x####) | v1 | TLS |
| "TLS session maintenance package" [ITU-T H.248.TLS] | tlsm, (0x####) | v1 | TLS |
| "TLS traffic volume metrics package" [ITU-T H.248.TLS] | tlstv, (0x####) | v1 | TLS |
| "NAT-traversal peer-to-peer package" [ITU-T H.248.84] | nattp2p, (0x010d) | v1 | ANY (if TCP) |
| "TCP hole punching package" [ITU-T H.248.84] | tcphp, (0x010e) | v1 | ANY (if TCP) |
| "TCP traffic volume metrics package" [ITU-T H.248.84] | tcptv, (0x010f) | v1 | ANY (if TCP) |
| "TCP connection control metrics package" [ITU-T H.248.84] | tcpccm, (0x0110) | v1 | ANY (if TCP) |
| "TCP connection quality metrics package" [ITU-T H.248.84] | tcpcqm, (0x0111) | v1 | ANY (if TCP) |
| "TCP retransmission metrics package" [ITU-T H.248.TCP] | tcprm, (0x####) | v1 | ANY (if TCP) |

### 10.14.3	Package usage information

The following is a non-exhaustive list of package usage indications.

#### 10.14.3.1	TLS basic session control package

Example:

IF CSA OR CSB THEN "usage detail see table …":

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| bceb, (0x0001) | O (NOTE 1) | ADD, MOD | ALL | "Un-blocked " | TLS |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| EstBNC, (0x0003) | M | ADD, MOD |  | - |  |
| RelBNC, (0x0004) | M | ADD, MOD |  | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| BNCChange, (0x0001) | M | ADD, MOD |  |  |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |
| NOTE 1 – Required for e.g. too early incoming TLS messages (due to security threat), or delayed TLS session establishment (due to multiple SDP O/A cycles, H.248 two-stage resource reservation, to await firstly successful L4 connectivity, etc). |  |  |  |  |  |

#### 10.14.3.2	TLS extended session control package

Example:

IF CSA OR CSB THEN "usage detail see clause 10.14.3.10 Stream endpoint interlinkage package.

Background:

- Package signalling elements are reused from Stream endpoint interlinkage package for TLS extended session control.

- Package procedures are defined in the TLS extended session control package ("the generic seplink package procedures are adapted to TLS specific procedures by tlsesc package").

#### 10.14.3.3	TLS capability negotiation package

Example:

IF CSB THEN "usage detail see table …":

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| dpid, (0x0001) | O | ADD, MOD | ALL | - | TLS |
| tlsv, (0x0002) | O | ADD, MOD | ALL | "0303" (NOTE 1) | TLS |
| cs, (0x0003) | O | ADD, MOD | ALL | - | TLS |
| cm, (0x0004) | O | ADD, MOD | ALL | - | TLS |
| ssr, (0x0005) | O | ADD, MOD | ALL | FALSE | TLS |
| srsc, (0x0006) | O | ADD, MOD | ALL | FALSE | TLS |
| rp, (0x0007) | O | ADD, MOD | ALL | 0 | TLS |
| car, (0x0008) | O | ADD, MOD | ALL | - | TLS |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None | - | - |  | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |
| NOTE 1 – This codepoint relates to "TLS v1.2". |  |  |  |  |  |

#### 10.14.3.4	TLS session maintenance package

Example:

IF CSB THEN "usage detail see table …":

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| mgcea, (0x0004) | O | ADD, MOD |  | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| mgea, (0x0001) | O | ADD, MOD |  |  |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 10.14.3.5	TLS traffic volume metrics package

Example:

IF CSB THEN "usage detail see table …":

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| recadfrag, (0x0001) | Not supported. | - | - |  | - |
| sentadfrag, (0x0002) | Not supported. | - | - |  | - |
| rectmfrag, (0x0003) | Not supported. | - | - |  | - |
| senttmfrag, (0x0004) | Not supported. | - | - |  | - |
| recado, (0x0005) | M | SUB | ALL |  | TLS |
| sentado, (0x0006) | M | SUB | ALL |  | TLS |
| rectmo, (0x0007) | Not supported. | - | - |  | - |
| senttmo, (0x0008) | Not supported. | - | - |  | - |
| recadpduo, (0x0009) | M | SUB | ALL |  | TLS |
| sentadpduo, (0x000a) | M | SUB | ALL |  | TLS |
| rectmpduo, (0x000b) | Not supported. | - | - |  | - |
| senttmpduo, (0x000c) | Not supported. | - | - |  | - |
| recadpco, (0x000d) | Not supported. | - | - |  | - |
| sentadpco, (0x000e) | Not supported. | - | - |  | - |
| rectmpco, (0x000f) | Not supported. | - | - |  | - |
| senttmpco, (0x0010) | Not supported. | - | - |  | - |
| recadcfo, (0x0011) | Not supported. | - | - |  | - |
| sentadcfo, (0x0012) | Not supported. | - | - |  | - |
| rectmcfo, (0x0013) | Not supported. | - | - |  | - |
| senttmcfo, (0x0014) | Not supported. | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 10.14.3.6	TCP basic connection control package

Example:

IF CSA OR CSB THEN "usage detail see table …":

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| bceb, (0x0001) | O (NOTE 1) | ADD, MOD | ALL | "Un-blocked " | ANY (if TCP) |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| EstBNC, (0x0001) | M | ADD, MOD |  | - |  |
| RelBNC, (0x0002) | M | ADD, MOD |  | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| BNCChange, (0x0001) | M | ADD, MOD |  |  |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |
| NOTE 1 – Required for e.g. too early incoming TCP packets (due to security threats), or delayed TLS session establishment (due to multiple SDP O/A cycles, H.248 two-stage resource reservation, to decouple the establishment of the two TCP connection segments (in case of TCP proxy), etc). |  |  |  |  |  |

#### 10.14.3.7	TCP extended connection control package

Example:

IF CSA OR CSB THEN "usage detail see clause 10.14.3.10 Stream endpoint interlinkage package.

Background:

- Package signalling elements are reused from Stream endpoint interlinkage package for TCP extended connection control.

- Package procedures are defined in the TCP extended connection control package ("the generic seplink package procedures are adapted to TCP specific procedures by tcpecc package").

#### 10.14.3.8	NAT-traversal peer-to-peer package

Example:

IF CSB THEN "usage detail see table …":

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| rnatip, (0x0001) | O | ADD, MOD |  |  |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 10.14.3.9	TCP hole punching package

Example:

IF CSB THEN "usage detail see table …":

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| rnat, (0x0001) | O | ADD, MOD |  |  |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| rstrx, (0x0001) | O | AUDITVALUE, SUB | ALL |  | TCP |
| synrx, (0x0002) | O | AUDITVALUE, SUB | ALL |  | TCP |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 10.14.3.10	TCP traffic volume metrics package

Example:

IF CSB THEN "usage detail see table …":

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| tcpos, (0x0001) | O | SUB | ALL |  | TCP |
| tcpor, (0x0002) | O | SUB | ALL |  | TCP |
| tcpps, (0x0003) | Not supported. | - | - |  | - |
| tcppr, (0x0004) | Not supported. | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 10.14.3.11	TCP connection control metrics package

Example:

IF CSB THEN "usage detail see table …":

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| tcpest, (0x0001) | M | SUB | ALL |  | TCP |
| tcpsyntx, (0x0002) | O | SUB | ALL |  | TCP |
| tcpsynrx, (0x0003) | O | SUB | ALL |  | TCP |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 10.14.3.12	TCP connection quality metrics package

Example:

IF CSB THEN "usage detail see table …":

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| tcprttest, (0x0001) | O | SUB | ALL |  | TCP |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 10.14.3.13	TCP retransmission metrics package

Example:

IF CSB THEN "usage detail see table …":

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| tcpros, (0x0001) | Not supported. | - | - |  | - |
| tcprps, (0x0002) | O | SUB | ALL |  | TCP |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 10.14.3.14	Stream endpoint interlinkage package

Example:

IF CSA OR CSB THEN "usage detail see table …":

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| linktopo, (0x0001) | O | ADD, MOD | only TLS and TCP endpoints | empty list | TLS, TCP |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None | - | - |  | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

## 10.15	Mandatory support of SDP and ITU-T H.248.1 Annex C information elements

At least the following:

| Supported Annex C and SDP information elements: |  |  |
| --- | --- | --- |
| Information Element | Annex C Support | SDP Support |
| "m="-line <proto> | not supported (in this example) | Value(s): ALL IANA registered codepoints with "TLS" protocol Purpose: bearer type indication ("TLS security session" Dependent on required MG behaviour for "TLS/L4 enabled terminations" (here: L4 = 'TCP'): - application-aware: "<L4>/TLS/<application>" - application-agnostic: "<L4>/TLS/–" |
|  |  | Value(s): ALL IANA registered codepoints with "TCP" protocol Purpose: bearer type indication ("TCP bearer connection" Dependent on required MG behaviour for "TCP enabled terminations": - application-aware: "TCP/<application>" application-agnostic: "TCP/–" |
| "a=fingerprint:" | not supported (in this example) | Value(s): all Purpose: TLS authentication procedures |

Rest: subject to profile specification.

## 10.16	Optional support of SDP and ITU-T H.248.1 Annex C information elements

At least the following:

| Supported Annex C and SDP information elements: |  |  |
| --- | --- | --- |
| Information Element | Annex C Support | SDP Support |
| "a=setup:" | not supported (in this example) | IF CSB THEN "TCP merge/relay mode" indication (for L3/L4 NAT-T) (NOTE 1) |
| NOTE 1 – Usage of this SDP attribute is defined in clause 13/[ITU-T H.248.84]. |  |  |

Rest: subject to profile specification.

## 10.17	Procedures

The initial release of this Recommendation is focusing on the previous profile elements. Specific guidelines for the procedural section are for further studies.

Appendix I

Use case specific capability sets – Two examples

(This appendix does not form an integral part of this Recommendation)

## I.1	Overview

Profile content is mainly use case dependent as outlined in clause 1. In order to demonstrate profile specification guidelines in clause 10, this Recommendation considers two exemplary use cases:

- Capability set 'A' (CSA):

  - a TCP-to-TCP media gateway,

  - connected to a single TLS domain,

  - supporting TLS-to-non-TLS interworking and TLS-to-TLS transparent forwarding,

  - optional support of stream endpoint interlinkage capability with regards to the establishment and release of bearer entities TCP, TLS and combined TLS/TCP (in addition to MGC strict control of individual stream endpoints);

  - only support of a single TLS authentication principle,

  - only support of certificates issued by a dertification authority;

  - only support of a minimum, fixed TLS profile (without any options),

  - any orthogonal L3/L4 NAT traversal support is disregarded, and

  - the MGC should be offloaded as best as possible from TLS security session and TCP bearer connection control.

- Capability set 'B' (CSB):

  - a TCP-to-TCP media gateway,

  - support of multiple TLS domains,

  - supporting TLS-to-non-TLS interworking, TLS-to-TLS transparent forwarding and additional TLS-to-TLS* interworking,

  - optional support of stream endpoint interlinkage capability with regards to the establishment and release of bearer entities TCP, TLS and combined TLS/TCP (in addition to MGC strict control of individual stream endpoints);

  - only support of a single TLS authentication principle,

  - support of self-signed certificates besides certificates issued by a dertification authority;

  - only support of a minimum, fixed TLS profile (without any options),

  - support of L3/L4 NAT traversal,

  - optional MGC-influenced TLS session negotiations, and

  - optional support of basic TLS performance monitoring (limited on bitrate related traffic volume measurements at the two measurement points "L4 – TLS record layer" and "TLS – application layer").

Capability set 'B' (CSB) is therefore a superset of capability set 'A' (CSA)-

Tables I.I and I.2 illustrate exemplary set of requirements (based on clauses 6 to 9) for profiling the two gateway types.

Table I.1 – MG example for capability set 'A' (CSA)

| Functional area | Capabilities |
| --- | --- |
| Requirements for control the MG mode of operation: - TLS transport mode - MG mode of operation for case of TLS-over-TCP transport : | - R-6.1/1, - R-6.2/1, R-6.2/2, R-6.2/3, R-6.2/4, R-6.2/5 |
| Requirements given by a TLS Profile concept: - Requirements to a TLS profile as such - Requirements to elements of TLS profiles | - R-7.1/1, - R-7.2/1, R-7.2/2, R-7.2/3, R-7.2/4, R-7.2/6, R-7.2/7 (NOTE 1) |
| Requirements on TLS procedures: - Selection of the TLS domain - Client/Server Mode - Behavioural Requirements for Authentication - Renegotiation of security parameters - Termination of the TLS/TCP session - Reporting unsuccessful TLS connection setup - TLS Statistics - Auditing of TLS related capabilities by the MGC | - R-8.1/2 (single TLS domain case), - R-8.2.1/all, R-8.2.2/1, - R-8.3.1/all, R-8.3.2/1, R-8.3.3/all, R-8.3.3.1/all - R-8.4/1, R-8.4/3, R-8.4/4, - R-8.5.all/all, - R-8.6/1, - - - - |
| Performance and resource aspects | - |
| NOTE 1 – No support of TLS session resumption. |  |

Table I.2 – MG example for capability set 'B' (CSB)

| Functional area | Capabilities |
| --- | --- |
| Requirements for control the MG mode of operation: - TLS transport mode - MG mode of operation for case of TLS-over-TCP transport : | - R-6.1/1, - R-6.2/1, R-6.2/2, R-6.2/3, R-6.2/4, R-6.2/5 |
| Requirements given by a TLS Profile concept: - Requirements to a TLS profile as such - Requirements to elements of TLS profiles | - R-7.1/1, - R-7.2/1, R-7.2/2, R-7.2/3, R-7.2/4, R-7.2/6, R-7.2/7 (NOTE 2) |
| Requirements on TLS procedures: - Selection of the TLS domain - Client/Server Mode - Behavioural Requirements for Authentication - Renegotiation of security parameters - Termination of the TLS/TCP session - Reporting unsuccessful TLS connection setup - TLS Statistics - Auditing of TLS related capabilities by the MGC | - R-8.1/1, R-8.1/3, R-8.1/4, - R-8.2.1/all, R-8.2.2/1, - R-8.3.1/all, R-8.3.2/1, R-8.3.3/all, R-8.3.3.1/all, R-8.3.3.2/all, - R-8.4/1, R-8.4/2, R-8.4/3, R-8.4/4, - R-8.5.all/all, - R-8.6/1, - - R-8.7/1, R-8.7/2, - - R-8.8/1, |
| Performance and resource aspects | - R-9/1 |
| NOTE 2 – No support of TLS session resumption. |  |

[End Proposed Correction]

____________________