| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4561 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Ericsson |  |  |  |
| Title: | H.248.78: Source of replaced address information |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution from Ericsson proposes some redesign of the raip and saip properties to address an editor’s note.

# Introduction

The output draft of the new revision of H.248.78 from October meeting specifies the MG located Bearer Level ALG package which includes a property to indicate what source should be used as a replacement of address information in L4+ layers. However, as it was pointed out during the October meeting, if both source address information and destination address information are to be replaced, there may be scenarios where two different sources are needed, one as a replacement of the source address information and another one as a replacement of the destination address information.

# Discussion

In order to address the issue outlined in the introduction, the mgbalg/saip property can be unfolded into two, one as replacement of the source address information and another one as a replacement of the destination address information.

At the same time, the mgbalg/raip property can be easily made redundant by having an additional value of the saip property which indicates “no replacement”.

# Conclusion

The following changes are proposed in the Recommendation:

[Begin Correction]

### 8.1.3	Indication of "source of replaced source address information"

|  | Property name: | Source of replaced source address information part |
| --- | --- | --- |
|  | Property ID: | sosaip (0x0003) |
|  | Description: | Communication addresses in Lx-PDUs (protocol data unit at layer Lx; here primarily layers concerned above L3) contain address information of the traffic source and/or traffic sink. The ALG function may be required for the replacement of source information, for example as a result of a particular translation behaviour of remote NAT devices in the IP bearer path. The translation of source address information only is a very common ALG scenario. This property allows the MGC to indicate the used source(s) of replaced source address information. |
|  | Type: | Sublist of Enumeration |
|  | Possible values: | A single value or a sublist, according to the value of ulpf NR [0x00] No replacement SD [0x01] Contained in H.248 Stream Descriptor PR [0x02] Provisioned in MG Contributor’s note: value IP seems not applicable for the replacement of source information. If the value is a sublist, the order of elements in the list shall correspond to the list of affected protocols as indicated by ulpf. |
|  | Default: | Provisioned {Editor's note (2013-10): a default value should be provided.} |
|  | Defined in: | LocalControl on an individual stream or TerminationState on the root termination. The variants of non-root and root termination level shall be mutually exclusive. An H.248 profile should indicate the supported variant. |
|  | Characteristics: | Read/write |

### 8.1.4	Indication of "source of replaced destination address information"

|  | Property name: | Source of replaced destination address information part |
| --- | --- | --- |
|  | Property ID: | sodaip (0x0004) |
|  | Description: | Communication addresses in Lx-PDUs (protocol data unit at layer Lx; here primarily layers concerned above L3) contain address information of the traffic source and/or traffic sink. The ALG function may be required for the replacement of destination information, for example as a result of a particular translation behaviour of remote NAT devices in the IP bearer path. This property allows the MGC to indicate the used source of replaced destination address information. |
|  | Type: | Sublist of Enumeration |
|  | Possible values: | A single value or a sublist, according to the value of ulpf NR [0x00] No replacement SD [0x01] Contained in H.248 Stream Descriptor PR [0x02] Provisioned in MG IP [0x03] Source IP (transport) address information of incoming packets If the value is a sublist, the order of elements in the list shall correspond to the list of affected protocols as indicated by ulpf. Contributor’s note: is the PR value relevant for the destination address? Seems unlikely. |
|  | Default: | Provisioned {Editor's note (2013-10): a default value should be provided.} |
|  | Defined in: | LocalControl on an individual stream or TerminationState on the root termination. The variants of non-root and root termination level shall be mutually exclusive. An H.248 profile should indicate the supported variant. |
|  | Characteristics: | Read/write |

[End Correction]

__________________