| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4567 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Ericsson |  |  |  |
| Title: | H.248.TLS: TLS role assumption, blocking semantics and interlinkage |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution from Ericsson proposes changes in the H.248.TLS draft in relation to the TLS role assumption and the scope of the ‘Bearer connection establishment blocking’ property.

# Introduction

The use of the term “TLS role assignment” is used throughout the draft in spite of the fact that there is no explicit role assignment by the MGC. There is only an implicit assignment for the client role together with the initiation of the establishment procedure if this is ordered by the MGC. In no case there is a role assignment prior to the initiation of the establishment procedure that could serve as preparation.

The semantics of the bceb property have been discussed during the last two meetings, as to whether or not they affect outgoing establishment procedures, also in the light of the new H.248.SEPLINK draft.

# Discussion

In the starting phases of the H.248.TLS draft Q3 considered the option of MGC explicitly assigning a role to the TLS endpoint (client or server). This role assignment could set the TLS endpoint in preparedness to receive a TLS ClientHello (server role) or to initiate an outgoing establishment procedure, i.e. send a TCP ClientHello (server role). This option has been later on abandoned in favour of considering that the TLS endpoint role is assumed as a consequence of either an incoming establishment procedure (server role) or an outgoing establishment procedure (client role). Although since then, the text has been further elaborated to try to be consistent with this approach, there are still descriptions that seem to be based on the existence of a role assignment or may mislead the reader into this belief. This contribution proposes changes to make the text consistent throughout the draft with the following facts:

a) there is no explicit role assignment

b) the role is assumed (adopted) as a consequence of incoming or outgoing establishment procedures

c) the MGC has the ability to trigger outgoing procedures and to allow or prevent initiation of outgoing procedures triggered by procedures in the associated SEP [H.248.SEPLINK] or by procedures in another layer of the same SEP [H.248.SEPLINK]

d) the MGC has the ability to prevent incoming procedures

Furthermore, since the MGC has always control of outgoing procedures, via explicit use of the EstBNC signal or by allowing interlinkage procedures [H.248.SEPLINK] it is unnecessary to let these procedures be controlled additionally by the bceb property. The use of the bceb property should only affect the incoming procedures.

# Conclusion

It is proposed to adopt following changes in order to address the issues subject of the discussion above.

[Begin Correction]

### 8.1.1	Incoming Security session establishment blocking

|  | Property Name: | Incoming security session establishment blocking |
| --- | --- | --- |
|  | Property ID: | bceb (0x0001) NOTE 1 - Identifier aligned with H.248 tcpbcc package. |
|  | Description: | This property defines the MG availability to accept or reject incoming TLS session establishment procedures. NOTE 2 - See Annex A concerning a state model based illustration. |
|  | Type: | Enumeration |
|  | Possible values: | See Table 2 Table 2 – Security session establishment preparedness (of TLS endpoints) Value; MG behaviour. "blocked" (0x0001); - Bearer resources are prepared (i.e., resources for TLS endpoint are reserved and allocated to H.248 termination/SEP); - TLS role: not determined; prevents assumption of server role. - TLS session still closed; - H.248 counterpart state = "Blocked" (Figure A.1); - Any incoming TLS packet shall be silently discarded (e.g., TLS ClientHello requests shall not be acknowledged). - Any outgoing TLS session control procedures are not affected: - Signal tlsbsc/EstBNC: start of TLS session establishment by sending of TLS ClientHello; TLS client role is effectively assigned - Start TLS session establishment by sending of TLS ClientHello triggered by interlinkage procedures [H.248.SEPLINK]; SEP adopts TLS client role. - Signal EstBNC (clause 8.3.1) implicitly overwrites the value to "unblocked".. "unblocked" (0x0002); - MG shall be ready for incoming TLS security session control procedures; - TLS role assignment: not determined but prepares for a TLS server role effectively: - Incoming TLS ClientHello: TLS server role is confirmed; - Any outgoing TLS session control procedures are not affected - Signal tlsbsc/EstBNC: start of TLS session establishment by sending of TLS ClientHello; TLS client role is effectively assigned - Start TLS session establishment by sending of TLS ClientHello triggered by interlinkage procedures [H.248.SEPLINK]; SEP adopts TLS client role. - TLS session state: transitioning from closed to …; - H.248 counterpart state = "Idle" (Figure A.1);. |
|  | Default: | "Unblocked" NOTE - H.248 profile specifications could redefine other default values. |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

[End Correction]

[Begin Correction]

### 8.6.2	TLS endpoint role assumption

The TLS role, client or server, is primarily relevant during the establishment phase of communication (of perspective of H.248 gateways). The MGC does not explicitly assign the SEP a TLS role. It is an incoming or outgoing TLS session establishment procedure which leads to the assumption of a TLS role. The SEP is basically prepared for an incoming or an outgoing TLS session establishment procedure since the moment it is created with an appropriate media descriptor. The MGC may prevent the assumption of a server role by setting the bceb property to “blocked”. The MGC may effectively determine the SEP to assume the client role by sending the EstBNC signal. The SEP may also assume the client role as a consequence of interlinkage procedures [H.248.SEPLINK]. If these procedures are allowed the SEP may assume a client role as a consequence of e.g. an incoming procedure in the associated SEP.  Table 4 summarizes MGC actions and their results in TLS role assignments.

Table 4 – TLS endpoint role assumption (at establishment phase)

|  | H.248 indication | MG behaviour: | TLS role assumed by SEP |
| --- | --- | --- | --- |
| 1 | 1) H.248 SEP state "blocked", due to explicit signalling of bceb property | Any incoming TLS PDU shall be silently discarded. | Not yet determined |
| 2 | 2) H.248 SEP state "unblocked" either due to explicit signalling of bceb property or correspondent default value | Incoming (TLS) ClientHello leads to subsequent completion of the establishment related handshake procedure. | Server role |
| 3 | H.248 signal EstBNC. Value of bceb property not relevant. | Start of TLS session establishment related handshake procedure | Client role |
| 4 | Interlinkage procedures allowed according to [H.248.SEPLINK] Value of bcep property not relevant (although this or similar properties may be relevant at the incoming side of the interlinkage). | An interlinkage establishment procedure[H.248.SEPLINK], e.g. an outgoing TCP establishment procedure in the same SEP leads to the start of 3-way handshake for TCP connection establishment. | Client role |

[End Correction]

[Begin Correction]

### 8.6.3	TLS security session establishment

The incoming and outgoing side of TLS security session protocol procedures is given by the assigned TLS role (see above clause 8.6.2) by the MGC.

If the SEP assumes the TLS client role as a result of an EstBNC signal or of an interlinkage procedure [H.248.SEPLINK], the MG shall initiate and complete the TLS security session setup handshake without any further involvement of the MGC.

If the SEP assumes the server role, as a result of an incoming TLS ClientHello message, and being allowed to do so by the MGC by having the bceb property set to “unblocked”, the MG shall complete the TLS security session setup handshake without any further involvement of the MGC.

The end of the TLS security session establishment phase may be summarized according Table 5:

Table 5 – TLS security session establishment - Results

| TLS role | Result of TLS security session establishment | MGC notification? |
| --- | --- | --- |
| Client (i.e., outgoing establishment side) | Successful | MGC dependent on subscription of event and parameter value tlsbsc/BNCChange{Type=[Est]} . |
|  | Unsuccessful | Unsuccessful establishment may be indicated to the MGC via a correspondent error code. {Editor's note (2013-11): error code is not possible … because bearer control procedures are still ongoing when H.248 command.reply is already sent. Either a Notify or ServiceChange based method …} |
| Server (i.e., incoming establishment side) | Successful | As client side. |
|  | Unsuccessful | As client side. |

[End Correction]

__________________