| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4571 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TCP: New clause "SDP for gateway control - IANA considerations" |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This is a parallel activity related to IANA registry consideration concerning H.248 specific SDP codepoints (see AVD-4569). This contribution provides the evaluation results and proposal for H.248.TCP.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4473).

Change proposal:

[Begin Proposed Correction]

# x	SDP for gateway control - IANA considerations

## x.1	Purpose

The H.248 gateway control protocol reuses normally the SDP codepoints as defined by the IETF or other organizations. However, gateway control may demand for additional SDP codepoints. The ones idendified by this Recommendation are collected in this clause and are subject of registration with IANA.

## x.2	SDP codepoints related to SDP "m=" line <proto> element

This Recommendation identifies following bearer type indication requirements (Table x1):

Table x1 – SDP codepoints related to SDP "m=" line <proto> element

| Type | SDP name (value) | Use case | IANA status |
| --- | --- | --- | --- |
| proto | "TCP" | application agnostic bearer type indication | already supported |
| proto | "TCP/<application>" | application aware bearer type indication | some application specific codepoints are already supported |

Thus, not any additional SDP codepoints needs to be registered with IANA.

## x.3	SDP codepoints related to SDP attributes

None.

[End Proposed Correction]

____________________