| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 239 (PLEN/16) |
|  |  | English only Original: English |
| Question(s): | 3/16 | Sapporo, Japan, 30 June – 11 July 2014 |
| TD |  |  |
| Source: | Editor H.Sup.14 |  |
| Title: | Approval: H.Sup.14 (ex H.Sup.IANA) "SDP codepoints for gateway control – Release 1" (New) |  |

This TD contains draft draft new H.Sup.14: "SDP codepoints for gateway control – Release 1" for Approval at this SG16 meeting.

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.SUPP.IANA Ed. 0.1 | Initial draft (AVD-4570) to WP 1/16 Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/AVD-4570.zip |
| H.SUPP.IANA Ed. 0.2 | Output (TD-32) from WP 1/16 Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/TD-32.zip |
| H.SUPP.IANA Ed. 0.3 | Input text to the Sapporo, 30 June-11 July 2014 meeting Location: http://www.itu.int/md/T13-SG16-140630-TD-WP1-0166 |
| H.SUPP.IANA Ed. 0.4 | Output draft from the Sapporo, 30 June-11 July 2014 meeting includes changes due to: C.468. Location: http://www.itu.int/md/T13-SG16-140630-TD-PLEN-0239 |

History

| Edition | Recommendation | Approval | Study Group |  |
| --- | --- | --- | --- | --- |
| 1.0 | ITU-T H Suppl. 14 | 2014-07-11 | 16 |  |

CONTENTS

Page

1 Scope 4 2 References 4 3 Definitions 5 3.1 Terms defined elsewhere 5 3.2 Terms defined in this Supplement 5 4 Abbreviations and acronyms 5 5 Conventions 5 6 Registration process 5 7 SDP codepoints for IANA registration 7 7.1 SDP codepoints related to wildcarding 7 7.2 SDP codepoints related to SDP "m=" line <proto> element 7 7.3 SDP codepoints related to SDP "a=" lines 7 7.3.1 SDP attribute "ITU-T H.248 package" 7

List of Tables

Page

Table 1 – SDP codepoints related to wildcarding 7 Table 2 – SDP codepoints related to SDP "m=" line <proto> element 7

List of Figures

Page

Figure 1 – IANA SDP registry – Process for H.248-specific SDP registrations 6

New Supplement 14 to the ITU-T H-Series (ex H.Supp.IANA)

Gateway Control Protocol:
ITU-T H.248.x-series - SDP codepoints for gateway control – Release 1

Summary

Gateway control protocols may require additional SDP codepoints, - beyond e.g. SDP information used by call control protocols -, for specific interworking functions supported at bearer plane level.

Supplement 14 to ITU-T H-series Recommendations summarizes SDP codepoints that have been identified in the time-frame from June 2000 to July 2014. It identifies SDP codepoints that meet ITU-T H.248.x sub-series requirements for gateway control protocols and are for general use by the wider standards community.

This Supplement – Release 1 covers:

•	SDP codepoints for the H.248 based control of TLS and DTLS traffic in H.248 media gateways.

This Supplement is used as base input for the IETF driven registration process of SDP codepoints with IANA.

# 1	Scope

Gateway control protocols (such as ITU-T H.248 [ITU-T H.248.1]) may require additional SDP codepoints, - beyond e.g. SDP information used by call control protocols -, for specific interworking functions supported at bearer plane level.

Supplement 14 to ITU-T H-series Recommendations [ITU-T H.248.x] summarizes SDP codepoints that have been identified in the time-frame from June 2000 to July 2014. It identifies SDP codepoints that meet ITU-T H.248.x sub-series requirements for gateway control protocols and are for general use by the wider standards community.

This Supplement – Release 1 covers:

- SDP codepoints for the H.248 based control of TLS traffic [ITU-T H.248.90] and DTLS traffic [ITU-T H.248.93] in H.248 media gateways; and

- Preliminary SDP codepoint information for SCTP traffic [ITU-T H.248.SCTP].

This Supplement is used as base input for the IETF driven registration process of SDP codepoints with IANA.

# 2	References

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway Control Protocol: Version 3.

[ITU-T H.248.x]	The ITU-T H.248.x-series of Recommendations (Gateway Control Protocol).

[ITU-T H.248.15]	Recommendation ITU-T H.248.15 (03/2013), Gateway Control Protocol: SDP ITU-T H.248 package attribute.

[ITU-T H.248.39]	Recommendation ITU-T H.248.39 (05/2006), Gateway Control Protocol: H.248 SDP parameter identification and wildcarding.

[ITU-T H.248.90]	Recommendation ITU-T H.248.90 (07/2014), Gateway control protocol: H.248 packages for control of transport security using TLS.

[ITU-T H.248.93]	Recommendation ITU-T H.248.93 (07/2014), Gateway control protocol: H.248 packages for control of transport security using DTLS.

[ITU-T H.248.SCTP]	Draft Recommendation ITU-T H.248.SCTP (07/2014), Gateway control protocol: H.248 support for control of SCTP bearer connections.

[IETF RFC 4566]	IETF RFC 4566 (2006), SDP: Session Description Protocol.

[IETF RFC 4572]	IETF RFC 4572 (2006), Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP).

[IETF IANA SDP]	IETF draft-schwarz-mmusic-for-gw (06/2014), SDP codepoints for gateway control. URL: http://tools.ietf.org/html/draft-schwarz-mmusic-sdp-for-gw-00

[IETF SCTP SDP]	draft-ietf-mmusic-sctp-sdp (02/2014), Stream Control Transmission Protocol (SCTP)-Based Media Transport in the Session Description Protocol (SDP).
URL: http://tools.ietf.org/html/draft-ietf-mmusic-sctp-sdp-06

# 3	Definitions

## 3.1	Terms defined elsewhere

None.

## 3.2	Terms defined in this Supplement

3.2.1	Codepoint: The combination of a "signalling parameter" plus assigned "value" in protocol engineering. The "value" represents a codepoint (or code position) in the code space.

# 4	Abbreviations and acronyms

This Supplement uses the following abbreviations and acronyms:

| RFC | (IETF) Request for Comments |
| --- | --- |
| SDP | Session Description Protocol |

# 5	Conventions

None

# 6	Registration process

IANA registrations should fundamentally be based on IETF Standards-Track RFCs, primarily in case of technologies owned by the IETF, such as SDP. The ITU-T could, but should not bypass the IETF in case of SDP related registrations with IANA. The ultimate goal of unambiguous protocol semantics implies a single instance, which reviews and coordinates registration requests with IANA. Figure 1 summarizes the principle process followed by ITU-T Study Group 16 with respect to IANA registration requests originated by ITU-T H.248 protocol:

Figure 1 – IANA SDP registry – Process for H.248-specific SDP registrations

The process in more detail:

- ITU-T: H.248 protocol specifications reuse existing, IANA-registered SDP codepoints, but can identify new SDP codepoints in addition, as e.g. subject of dedicated use cases.
Such extensions to the H.248 gateway control protocol are normally related to new H.248 package definitions, which are published as part of the H.248.x-series of ITU-T Recommendations. However, new SDP codepoints could be also derived as the result of (package-less) H.248 procedures or specific SDP usage according to an H.248 profile.

- ITU-T: This Supplement is used as a central place for 
a) the documentation of already registered H.248-specific SDP codepoints with IANA, and
b) the collection of newly identified codepoints.

- IETF: working group MMUSIC is presently responsible for the SDP. The content of the ITU-T Supplement is mapped on an IETF draft, for example [IETF IANA SDP] (which later becomes an RFC), which becomes the baseline for the IANA registration request.

- IANA: registration on request by IETF.

The process is not a singular activity, rather open for recurrent updates because

- ITU-T Supplements support "Release" versions, and

- IETF RFCs could be replaced (obsoleted) by new RFCs.

# 7	SDP codepoints for IANA registration

## 7.1	SDP codepoints related to wildcarding

The H.248-specific SDP wildcards “-”, “~”, “r”, “l” and “[0-F]” (see [ITU-T H.248.39]) could be used in various SDP lines and line fields.

Table 1 – SDP codepoints related to wildcarding

| Type | SDP name (value) | Reference |
| --- | --- | --- |
| Various SDP field elements | “-”, “~”, “r”, “l” and “[0-F]” | [ITU-T H.248.39] |

Such wildcards represent a codepoint space, but are out of scope of any IANA registry.

NOTE – The wildcard characters are compliant with the SDP grammar according to [IETF RFC 4566].

## 7.2	SDP codepoints related to SDP "m=" line <proto> element

See Table 2:

Table 2 – SDP codepoints related to SDP "m=" line <proto> element

| Type | SDP name (value) | Reference |
| --- | --- | --- |
| proto | "TLS" | [ITU-T H.248.90] |
| proto | "TCP/TLS" "SCTP/TLS" | [ITU-T H.248.90] NOTE 1 |
| proto | "DTLS" | [ITU-T H.248.93] |
| proto | "UDP/DTLS" "DCCP/DTLS" | [ITU-T H.248.93] |
| proto | "SCTP" | [ITU-T H.248.SCTP] NOTE 2 |
| proto | "SCTP/DTLS" | [ITU-T H.248.SCTP] NOTE 2 |
| proto | "DTLS/SCTP" | [ITU-T H.248.SCTP] NOTE 2 |
| NOTE 1 – Codepoint "TCP/TLS" is already registered, based on [IETF RFC 4572]. NOTE 2 – The table entry may be deleted when the IETF draft [IETF SCTP SDP] “SCTP- based media transport in the SDP” becomes an RFC. |  |  |

## 7.3	SDP codepoints related to SDP "a=" lines

### 7.3.1	SDP attribute "ITU-T H.248 package"

Recommendation [ITU-T H.248.15] defines an ITU-T specific extension for SDP. The SDP attribute "a=h248item:" allows for the carriage of general ITU-T H.248 properties in the local and remote descriptor in the textual ITU-T H.248 encoding.

____________________