| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4670 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.Sup.ALTC |  |  |  |
| Title: | H.Sup.ALTC "Gateway Control Protocol: Alternate IP connectivity support by H.248 gateway control protocol" (New): Input draft Ed. 0.5 |  |  |  |
| Purpose: | Meeting input (Draft Supplement) |  |  |  |

Summary

This document contains the editor's input draft for draft new H.Sup.ALTC "Gateway Control Protocol: Alternate IP connectivity support by H.248 gateway control protocol" (Ed. 0.5).

# Changes versus Ed. 0.4 (TD 221 R1 (WP 1/16) (2015-02)):

- no changes from Editor's side

__________________

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.Sup.ALTC Ed. 0.1 | Initial draft (AVD-4655) to WP 1/16 Seoul (KR) meeting (2014-11) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1411_Seo/AVD-4655.zip |
| H.Sup.ALTC Ed. 0.2 | Output (TD-23) from WP 1/16 Seoul (KR) meeting (2014-11) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1411_Seo/TD-23.zip |
| H.Sup.ALTC Ed. 0.3 | Input (TD 221/WP1) to ITU-T SG16 Geneva meeting, Feb 2015 Location: http://www.itu.int/md/T13-SG16-150209-TD-WP1-0221/en |
| H.Sup.ALTC Ed. 0.4 | Output (TD 221R1/WP1) from ITU-T SG16 Geneva meeting, Feb 2015 Location: http://www.itu.int/md/T13-SG16-150209-TD-WP1-0221/en |
| H.Sup.ALTC Ed. 0.5 | Input (AVD-4670) to WP 1/16 Guilin meeting (2015-06) Location: |
| H.Sup.ALTC Ed. 0.6 | Output (TD-##) from WP 1/16 Guilin meeting (2015-06) Location: |
| H.Sup.ALTC Ed. 0.7 |  |
| H.Sup.ALTC Ed. 0.8 |  |
| H.Sup.ALTC Ed. 0.9 |  |
| H.Sup.ALTC Ed. 0.10 |  |

CONTENTS

Page

1 Scope 7 2 References 7 3 Definitions 8 3.1 Terms defined elsewhere 8 3.2 Terms defined in this Supplement 8 4 Abbreviations and acronyms 8 5 Conventions 9 6 Network model 9 7 Support of alternate IP connectivity: principle options from H.248 side 10 7.1 Solution options: basic H.248 context models and H.248 media descriptor elements 10 7.1.1 H.248 ALTC context model "M1": reservation of two H.248 contexts 10 7.1.2 H.248 ALTC context model "M2": reservation of two H.248 terminations 11 7.1.3 H.248 ALTC context model "M3": reservation of two H.248 stream endpoints 12 7.1.4 H.248 ALTC context model "M4": reservation of two H.248 media groups 13 7.1.5 H.248 ALTC context model "M5": reservation of two H.248 stream groups 14 7.1.6 H.248 ALTC context model "M6": reservation of a "dual-stack IP interface" 15 7.2 Solution options: complementary H.248 resource usage indication information 16 7.2.1 H.248 signalling element "S1": SDP-based ALTC call type indication 16 7.2.2 H.248 signalling element "S2": Termination connection model package (H.248.51) 16 7.2.3 H.248 signalling element "S3": usage of media mixer resources (H.248.9) 16 7.2.4 H.248 signalling element "S4": Resource management package(s) (H.248.63) 16 7.2.5 H.248 signalling element "S5": SDP Offer/Answer model (H.248.80) 16 8 Requirements (from H.248 profile perspective) 16 8.1 Communication service: number of media components 16 8.2 Communication service: early end-to-end IP connectivity 16 8.3 Call control signaling: number of negotiation cycles 17 8.4 Gateway control signaling: resource management process in H.248 MG 17 8.5 Gateway control signaling: explicit ALTC "call" tagging 18 8.6 Gateway control signaling: specific resource components in H.248 MG 18 9 Discussion 18 9.1 Call control signalling: examples of SIP SDP offers 18 9.1.1 SIP SDP offer example 1: monomedia audio telephony 18 9.1.2 SIP SDP offer example 2: multimedia WebRTC call 19 9.2 Gateway control signalling: examples 19 9.2.1 Example H.248 signalling in case of "application aware IWF" 20 9.2.1.1 Usage of H.248 ALTC context model "M1" (two contexts) 20 9.2.1.2 Usage of H.248 ALTC context model "M2" (single context with two terminations for ALTC) 26 9.2.1.3 Usage of H.248 ALTC context model "M3" (single termination with two SEPs for ALTC) 28 9.2.1.4 Usage of H.248 ALTC context model "M4" (two media groups for ALTC) 29 9.2.1.5 Usage of H.248 ALTC context model "M5" (two stream groups) 35 9.2.1.6 Usage of H.248 ALTC context model "M6" (dual stack IP interface) 35 9.2.2 Example H.248 signalling in case of "application agnostic IWF" 35 9.2.2.1 Usage of H.248 ALTC context model "M1" (two contexts) 35 9.2.2.2 Usage of H.248 ALTC context model "M2" (single context with two terminations for ALTC) 37 9.2.2.3 Usage of H.248 ALTC context model "M3" (single termination with two SEPs for ALTC) 38 9.2.2.4 Usage of H.248 ALTC context model "M3" (two media groups for ALTC) 38 9.2.2.5 Usage of H.248 ALTC context model "M5" (two stream groups) 39 10 Profile guidelines 39 Appendix I Discussion of the various H.248 solution options – Signalling examples 40 I.1 #### 40 Appendix II Discussion of the various H.248 solution options – Summary 41 II.1 Summary table 41 II.2 Recommended solution(s) 42 Appendix III Example H.248 profile: 3GPP Iq profile 43 III.1 Summary of stage 2 requirements 43 Bibliography 44

List of Tables

Page

Table 9.1.1 – Example command encoding – (SIP) SDP Offer example 1 18 Table 9.1.2 – Example command encoding – (SIP) SDP Offer example 2 19 Table 9.2.1.1/1 – Example command encoding – Step 1 – MGC request (for local connection point only in outgoing direction) 20 Table 9.2.1.1/2 – Example command encoding – Step 2 – MG reply 22 Table 9.2.1.1/3a – Example command encoding – Step 3a – MGC request 23 Table 9.2.1.1/4a – Example command encoding – Step 4a – MG reply 24 Table 9.2.1.1/3b – Example command encoding – Step 3b – MGC request 24 Table 9.2.1.1/4b – Example command encoding – Step 4b – MG reply 25 Table 9.2.1.2/1 – Example command encoding – Step 1 – MGC request (for local connection point only in outgoing direction) 26 Table 9.2.1.2/2 – Example command encoding – Step 2 – MG reply 27 Table 9.2.1.2/3a – Example command encoding – Step 3a – MGC request 27 Table 9.2.1.2/4a – Example command encoding – Step 4a – MG reply 28 Table 9.2.1.3/1 – Example command encoding – Step 1 – MGC request (for local connection point only in outgoing direction) 28 Table 9.2.1.3/2 – Example command encoding – Step 2 – MG reply 29 Table 9.2.1.3/3a – Example command encoding – Step 3a – MGC request 29 Table 9.2.1.3/4a – Example command encoding – Step 4a – MG reply 29 Table 9.2.1.4/1 – Example command encoding – Step 1 – MGC request (for local connection point only in outgoing direction) 30 Table 9.2.1.4/2 – Example command encoding – Step 2 – MG reply 31 Table 9.2.1.4/3a – Example command encoding – Step 3a – MGC request 32 Table 9.2.1.4/4a – Example command encoding – Step 4a – MG reply 33 Table 9.2.1.4/3b – Example command encoding – Step 3b – MGC request 33 Table 9.2.1.4/4b – Example command encoding – Step 4b – MG reply 34 Table 9.2.2.1/1 – Example command encoding – Step 1 – MGC request 35 Table 9.2.2.1/2 – Example command encoding – Step 2 – MG reply 37 Table 9.2.2.1/3 – Example command encoding – Step 3 – MGC request 37 Table 9.2.2.1/4 – Example command encoding – Step 4 – MG reply 37 Table 9.2.2.2/1 – Example command encoding – Step 1 – MGC request 37 Table 9.2.2.2/2 – Example command encoding – Step 2 – MG reply 37 Table 9.2.2.2/3 – Example command encoding – Step 3 – MGC request 38 Table 9.2.2.2/4 – Example command encoding – Step 4 – MG reply 38 Table 9.2.2.3/1 – Example command encoding – Step 1 – MGC request 38 Table 9.2.2.3/2 – Example command encoding – Step 2 – MG reply 38 Table 9.2.2.3/3 – Example command encoding – Step 3 – MGC request 38 Table 9.2.2.3/4 – Example command encoding – Step 4 – MG reply 38 Table 9.2.2.4/1 – Example command encoding – Step 1 – MGC request 39 Table 9.2.2.4/2 – Example command encoding – Step 2 – MG reply 39 Table 9.2.2.4/3 – Example command encoding – Step 3 – MGC request 39 Table 9.2.2.4/4 – Example command encoding – Step 4 – MG reply 39 Table II.1 – Evaluation summary 41

List of Figures

Page

Figure 1 – Network model for alternate IP connectivity from H.248 perspective 9 Figure 2 – H.248 ALTC context model "M1": reservation of two H.248 contexts 10 Figure 3 – H.248 ALTC context model "M2a": reservation of two H.248 terminations 11 Figure 4 – H.248 ALTC context model "M2b": reservation of two H.248 terminations 12 Figure 5 – H.248 ALTC context model "M3": reservation of two H.248 stream endpoints 13 Figure 6 – H.248 ALTC context model "M4": reservation of two H.248 media groups 14 Figure 7 – Usage of "stream grouping" capabilities according H.248.STGROUP 14 Figure 8 – H.248 ALTC context model "M5": reservation of two H.248 stream groups 15 Figure 9 – H.248 ALTC context model "M6": reservation of dual-stack capable SEPs 15

Draft New Supplement # to the ITU-T H-Series (ex H.Sup.ALTC)

Gateway control protocol: 
Alternate IP connectivity support by H.248 entities

Summary

<Mandatory material>

Keywords

ALTC

Introduction

The alternate IP connectivity capability results from transitioning scenarios in evolving IP version 4 networks towards IP version 6. The reality of IPv4-IPv6 coexistence introduces heterogeneous scenarios with combinations of IPv4 and IPv6 nodes, some of which are capable of supporting both IPv4 and IPv6 dual-stack (DS) and some of which are capable of supporting only IPv4 or only IPv6. In this context, H.248 entities could be involved in alternate IP connectivity scenarios, at least when the H.248 media gateway support dual-stack (DS) capable IP interfaces.

{Editor's note (2014-11): the main body (clause 7) of the Supplement should finally document the "recommended solution" …}

# 1	Scope

The capability of alternate IP connectivity (ALTC) is basically supported by H.248 entities, without any need to extend the existing H.248 protocol capabilities. However, there are in principle multiple solutions possible how a particular H.248 profile could define ALTC support.

The purpose of this Supplement is to provide guidance on this topic. The Supplement is structured as follows:

- network model for ALTC with focus on H.248 entities (clause 6);

- discussion of principle ALTC solution options from H.248 side (clause 7);

- typical H.248 signalling requirements related to ALTC (clause 8), i.e., the so-called stage 2 information for H.248 profiles;

- evaluation and discussion of signalling examples in order to challenge H.248 ALTC solutions from clause 7 (clause 9); and

- H.248 profile guidelines, i.e., stage 3 recommendations (clause 10).

{Editor's note (2014-11): the document is positioned as Supplement because we don't expect any normative H.248 extensions, at least at this stage of the discussion …}

# 2	References

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.39]	Recommendation ITU-T H.248.39 (10/2014), Gateway control protocol: H.248 SDP parameter identification and wildcarding.

[ITU-T H.248.80]	Recommendation ITU-T H.248.80 (01/2014), Gateway control protocol: Usage of the revised SDP offer/answer model with ITU-T H.248.

[IETF RFC 6947]	IETF RFC 6947 (05/2013), The Session Description Protocol (SDP) Alternate Connectivity (ALTC) Attribute.

[ETSI TS 183 018]	ETSI TS 183 018 V3.5.2 (01/2010), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Resource and Admission Control: H.248 Profile Version 3 for controlling Border Gateway Functions (BGF) in the Resource and Admission Control Subsystem (RACS); Protocol specification.

[ETSI TR 183 068]	ETSI TR 183 068 V3.1.1 (08/2009), Telecommunications and Internet converged Services and Protocols for Advanced Networks (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

# 3	Definitions

## 3.1	Terms defined elsewhere

None.

## 3.2	Terms defined in this Supplement

3.2.1	Alternate (IP) Connectivity: mechanism to request initially resources (e.g., from a terminal or gateway) for multiple IP addresses of different address families (e.g., IPv4 and IPv6), and release again immediately after successful connectivity establishment the unused resources.

3.2.2	Early media: unidirectional, end-to-end user plane connectivity from called connection endpoint back to calling connection endpoint, which allows e.g., the calling party to receive application data (i.e., "backward early media") already before the final call establishment phase is completed inclusive a successfully established bidirectional end-to-end user plane connection.

NOTE 1 – From H.248 media gateway perspective the capability of "early media" support implies the context-internal cut-through of an H.248 media stream. Unidirectional support is achieved by StreamMode settings of "SendOnly" and "RecvOnly" at the concerned H.248 SEPP (or SEPT in general).

NOTE 2 – The notion of "early media" originates from SIP-based call control signalling, see [b-IETF RFC 5009]. The RFC describes the use cases of "backward early media" and "forward early media".

# 4	Abbreviations and acronyms

This Supplement uses the following abbreviations and acronyms:

| ALTC | Alternate (IP) Connectivity |
| --- | --- |
| B2BIH | Back-to-Back IP Host (mode according to [ITU-T H.248.64])] |
| DS | Dual-Stack (parallel support of V4 and V6) |
| GCP | Gateway Control Protocol |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| NA(P)T-PT | Network Address (and Port) Translation plus (Internet) Protocol (Version) Translation |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SEPT | Stream Endpoint Tuple |
| UA | (SIP) User Agent |
| UE | User Equipment |
| V4 | Internet Protocol version 4 |
| V6 | Internet Protocol version 6 |

# 5	Conventions

None.

# 6	Network model

There will be two interconnected IP network domains (labelled as X and Y in Figure 1) in case of ALTC support by H.248 gateways:

Figure 1 – Network model for alternate IP connectivity from H.248 perspective

It is sufficient to consider an end-to-end network configuration where at least one domain supports IPv4 and IPv6 connectivity and dualstack capable IP host entities (here domain Y). The SIP B2BUA entity receives a SDP offer with SDP "a=altc" qualified media descriptions from X or Y, dependent on the call establishment direction. The SIP level SDP offer/answer procedures are subject of [IETF RFC 6947] and out of scope of this Recommendation.

The H.248 IP-to-IP media gateway is most likely configured for a B2BIH mode, especially due to potential IP version translation in case of a final V4-to-V6 configuration. This is also known as NAT46 (of NAT64) mode. IP version translation is only possible in B2BIH mode (and not in IPR mode).

Therefore the assumption of B2BIH mode, despite the fact of scenarios with final V4-only or V6-only end-to-end IP connectivity.

# 7	Support of alternate IP connectivity: principle options from H.248 side

The SDP based ALTC qualification of media descriptions is firstly only required at the SIP-based call control level. H.248 gateway control signalling for ALTC support does not necessarily require such kind of SDP tagging due to alternative solutions concerning media plane IP resource reservations.

## 7.1	Solution options: basic H.248 context models and H.248 media descriptor elements

The illustrations take the example of a dualmedia call with audio and video over RTP, inclusive RTCP control flows.

### 7.1.1	H.248 ALTC context model "M1": reservation of two H.248 contexts

Figure 2 illustrates how such a model could look like. Termination TX serves the bearer traffic towards UE_X and is out of consideration in this Supplement. The bearer traffic for the preferred IP connection is assigned to Termination TY,P. Both Terminations (TX and TY,P) are created in Context Cz1. The alternate IP connection is assigned to Termination TY,A, which is created in a separate Context Cz2.

Figure 2 – H.248 ALTC context model "M1": reservation of two H.248 contexts

When the final IP connection option is negotiated, when either

- Termination TY,P is deleted again (via H.248 SUBtract command) and replaced by Termination TY,A (via H.248 MOVe command); or

- Termination TY,A is deleted (via H.248 SUBtract command).

The 2nd Context Cz2 disappears again in both cases.

The motivation for the "two context" model (M1) might be the isolation of resources associated with the alternate IP connection, given the assumption of an estimated low probability that these resources will be finally used.

{Comment (2014-11-11): the final decision between for one of the two options is usually not 50:50 from MGC perspective (due to certain a-prio knowledge about the likely selected IP connection, e.g., due to operator policies, registration information from UE side, etc. An H.248 profile might benefit from such information by trying to opimize resource usage: e.g., both terminations T_Y,P and T_Y,A are prepared not adequately; T_Y,P is configured as usual (with much as needed information) whereas T_Y,A is created with minimum information only.}

Model (M1) looks also fairly attractive from H.248 control perspective because Context Cz1 represents the usual context configuration, without any ALTC specific aspects.

### 7.1.2	H.248 ALTC context model "M2": reservation of two H.248 terminations

The alternate IP connection is assigned to a self-contained Termination TY,A in model M2, similar as in previous model M1. However, there is only a single context from the very beginning, see Figure 3:

Figure 3 – H.248 ALTC context model "M2a": reservation of two H.248 terminations

It is obvious that there are some context-internal resource management aspects during the temporary phase when both Terminations TY,P and TY,A exists:

- the IWF resources for media processing (here: audio transcoding IWF and video transparent forwarding IWF) might be already prepared and internally connected to the external IP interfaces …

- … raising the question of the internal H.248 stream topology (e.g., topologies [TX(S1), TY,P(S1), oneway] plus [TX(S1), TY,A(S1), oneway]).

Hence, there might be the option of shared resources (e.g., all three SEPs S1 are connected to the same audio transcoder entity), or separate resources (i.e., for "preferred" and "alternate" SEPs S1) are prepared. This is a remarkable aspect of model M2a, but should be implementation specific (to be confirmed), hence out of scope of this Supplement.

Purpose of model M2a is to serve the possible requirement of early media (see R-8.2/2 in clause 8). If such a requirements does not exist in an H.248 profile, then the consideration of a simpler model might be sufficient, see model M2b in Figure 4:

Figure 4 – H.248 ALTC context model "M2b": reservation of two H.248 terminations

When the final IP connection option is negotiated, when either

- Termination TY,P is deleted again (via H.248 SUBtract command) or

- Termination TY,A is deleted (via H.248 SUBtract command).

The context internal cut-through occurs either "as soon as possible" (M2a) or with the final context configuration (M2b).

{Comment (2014-11-11): context model M2a implies usage of the H.248 Topology descriptor! Should be described in the text. There are side effects: the termination names must be known in order to specify the topologies, which isn't the case in the first signalling cycles when the local connection endpoints are wildcard. There are consequently two signalling cycles required in case of model M2a.
And the same is true actually for model M2b under condition of early media support (which implies a topology specification)}

### 7.1.3	H.248 ALTC context model "M3": reservation of two H.248 stream endpoints

NOTE – Model M3 is conditional, limited on communication services related to single-stream-per-termination structures!

The example of a monomedia call (here: audio) is considered, resulting in a single H.248 stream (S1) for the preferred IP configuration (Figure 5). The alternate IP connection resources are defined via a separate H.248 stream descriptor (see SEP S2). Both SEPs S1 and S2 belong to the same Termination TY:

Figure 5 – H.248 ALTC context model "M3": reservation of two H.248 stream endpoints

When the final IP connection option is negotiated, when either

- SEP TY(S1) is deleted again (via H.248 SUBtract command) or

- SEP TY(S2) is deleted (via H.248 SUBtract command).

The interconnection of SEP TY(S1) with SEP TX(S1) is straightforward, and the interconnection of SEP TY(S2) with SEP TX(S1) may require capabilities as defined by [ITU-T H.248.STGROUP] (to be confirmed; see also parallel discussion on C-680 "H.248.STGROUP: model discussion (4) for use case "Alternate IP connectivity"" (note: currently parked in a "living list Appendix" of H.248.STGROUP).).

Model M3 looks attractive due to the simplicity of single terminations only. However, the context model does not provide a general purpose solution as mentioned in the beginning.

### 7.1.4	H.248 ALTC context model "M4": reservation of two H.248 media groups

Another approach may be through the use of the H.248 LocalControl properties ReserveValue and ReserveGroup, which would allow two IP addresses to be indicated. The semantic of these LocalControl properties is that only one resource is used at a time. The unused one can be removed at a later date. There would be two H.248 media groups reserved (to be confirmed), see Figure 5:

Figure 6 – H.248 ALTC context model "M4": reservation of two H.248 media groups

Model M4 may result in quite extensive H.248 stream descriptors …

### 7.1.5	H.248 ALTC context model "M5": reservation of two H.248 stream groups

{FIXTHIS: insert model based on H.248.STGROUP 
Note (2015-02): sub-clause might be updated in a next meeting dependent on the discussion of C-680 "H.248.STGROUP: model discussion (4) for use case "Alternate IP connectivity""  (note: currently parked in a "living list Appendix" of H.248.STGROUP)…}

Figure 7 illustrates the context-internal grouping principle of H.248 streams. There would be one H.248 stream group per media type:

Figure 7 – Usage of "stream grouping" capabilities according H.248.STGROUP

Figure 8 shows the synonym H.248 ALTC context model "M5" according to the style as used for the previous models M1 to M4.

Figure 8 – H.248 ALTC context model "M5": reservation of two H.248 stream groups

### 7.1.6	H.248 ALTC context model "M6": reservation of a "dual-stack IP interface"

… model of a single resource component "dual-stack IP interface" (as kind of alias addresses) … 
e.g., indication via
a) SDP "c="-line: a new registered address type, 
    e.g.  c=IN IP46 <connection-address(es)>   or
b) a new H.248 media stream property 
for further studies …

Figure 9 – H.248 ALTC context model "M6": reservation of dual-stack capable SEPs

## 7.2	Solution options: complementary H.248 resource usage indication information

The qualification of an H.248 context (or stream) for "ALTC awareness" might improve the MG level resource management.

### 7.2.1	H.248 signalling element "S1": SDP-based ALTC call type indication

It could be imagined that the occurrence of SDP attribute "a=altc:" in the H.248 Local and/or Remote descriptor will be assigned the semantic, that the MG could qualify associated resources of type "ALTC". E.g., resources which are only temporarily reserved, but finally not used and released immediately after call establishment completion.

FIXTHIS – check applicability

### 7.2.2	H.248 signalling element "S2": Termination connection model package (H.248.51)

FIXTHIS – check applicability

### 7.2.3	H.248 signalling element "S3": usage of media mixer resources (H.248.9)

FIXTHIS – check applicability

### 7.2.4	H.248 signalling element "S4": Resource management package(s) (H.248.63)

FIXTHIS – check applicability

### 7.2.5	H.248 signalling element "S5": SDP Offer/Answer model (H.248.80)

The SDP-based ALTC signalling method at SIP level is independent of the SDP offer/answer model (see clause 4.2.4/[IETF RFC 6947]), thus, the SDP attribute "a=altc:" may be used in the legacy SDP offer/answer (see clause 3.2.3.1/[ITU-T H.248.80]) and revised SDP offer/answer (see clause 3.2.3.2/[ITU-T H.248.80]) syntax.

# 8	Requirements (from H.248 profile perspective)

There are multiple solutions for ALTC support (see previous clause), but some of them are conditional dependent on typically H.248 profile dependent requirements. The purpose of this clause is to highlight such kind of capabilities.

## 8.1	Communication service: number of media components

This aspect affects normally the number of required H.248 streams (and therefore also the internal context topology).

R-8.1/1: Support of single media descriptions only per H.248 context ("monomedia call").

R-8.1/2: Support of multiple media descriptions per H.248 context ("multimedia call").

## 8.2	Communication service: early end-to-end IP connectivity

The ALTC feature is subject of the call establishment phase. There are options how fast the end-to-end bearer establishment is finalized.

R-8.2/1: End-to-end IP connectivity should be available with completion of the end-to-end call control signalling.

R-8.2/2: End-to-end IP connectivity should be available as soon as possible, i.e., the MG should internally cut-through the bearer path as soon as fully specified media and transport descriptions are available for individual traffic directions.

NOTE – Requirement R-8.1/2 is also known as "early media", e.g., the called communication endpoint could already sent media in backward direction to the calling communication endpoint when acknowledging an incoming communication request.

It has to be noted that the temporary phase between a partial H.248 context configuration ("which allows early media") and the final complete H.248 context configuration is typically lasting only a few tens or hundreds of milliseconds. Thus, support of requirement R-8.2/1 should be normally more than sufficient for call controlled communication services, inclusive the category of realtime services.

There are two follow-up requirements for the early media capability (R-8.2/2):

R-8.2/3: The MG should support unidirectional early media, from the terminating (altc) UE back to the originating UE.

R-8.2/4: The MG should support bidirectional early media between both UEs.

Above requirements impact the context internal model and topology, e.g. oneway or bothway topology, and may reduce the set context model options (as outlined in clause 7.1).

## 8.3	Call control signaling: number of negotiation cycles

The indication and negotiation of media configurations at call control signalling level could theoretically subject of multiple (SDP offer/answer) negotiation cycles. The number of cycles is usually only bounded by performance objectives concerning call establishment delays.

R-8.3/1: Network solutions aims for single cycle negotiations, i.e., there is only one SDP offer and one SDP answer at SIP level.

R-8.3/2: Network solutions allow multi cycle negotiations.

Normally, the number of H.248 resource reservation cycles is decoupled and independent from call control procedures, however, incoming SIP messages usually acting as direct stimuli for H.248 signalling actions.

## 8.4	Gateway control signaling: resource management process in H.248 MG

Following resource management models might be worth of consideration:

- model of resource "protocol stack":

- monolithic resource model, i.e., an entire SDP media description is modelled as a single MG resource;

- decomposed resource model by separating the protocol stack in a lower segment of network/transport resources and a upper segment of the actual media-related resources.

NOTE – The decomposed model (1b) is already anchored in the ETSI TISPAN Ia profile (see [ETSI TS 183 018], clause 5.17.1.11 "One-Stage and Two-Stage BGF Resource Reservation") by splitting the protocol stack in SDP "c="-line and SDP "m="-line resource partitions.

It is obvious that there will be trade-off decisions between "early end-to-end connectivity and media path usage" versus "MG resource management complexity". It is not the purpose of this Supplement to evaluate this topic in more detail, but there might be at least following requirements postulated due to H.248 profiles according to [ETSI TS 183 018]:

R-8.4/1: The MGC-to-MG resource reservation process follows the process that available SDP "m="-line is already signalled together with "c="-line information.

R-8.4/2: The MGC-to-MG resource reservation process supports the process that the MGC would provide in a first phase IP connection resources only ("which would be here SDP "c="-line plus "a=altc:"-lines"), without yet media type / media format information. The "m="-line part would be then subjedt of the second phase in case of media aware type of IWF.

## 8.5	Gateway control signaling: explicit ALTC "call" tagging

The ALTC network service might be basically fully transparent to H.248 entities (e.g., in case of a lacking ALTC signalling element at the H.248 interface), however, the MG might support optimized resource management when knowing that a particular call contains "ALTC resource components". Resulting in following requirements:

R-8.5/1: ALTC could be transparent to the H.248 MG. The MG processes "ALTC context configuration" requests as regular communication services.

NOTE – This requirement could result in inefficient MG resource usage and decreased MG capacity.

R-8.5/2: Support of explicit ALTC context tagging (via a ContextAttribute, or Termination-level property, or SDP element, or …).

## 8.6	Gateway control signaling: specific resource components in H.248 MG

For further studies ("inserted due to email comments"):

R-8.6/1: ALTC should be supported by "conferencing resources".

R-8.6/2: …

# 9	Discussion

The various solution options from clause 7 are evaluated by considering example signalling scenarios.

## 9.1	Call control signalling: examples of SIP SDP offers

### 9.1.1	SIP SDP offer example 1: monomedia audio telephony

A typical SIP-level SDP offer is illustrated in Table 9.1.1:

Table 9.1.1 – Example command encoding – (SIP) SDP Offer example 1

| (SIP) SDP encoding | Comments |
| --- | --- |
| v=0 o=... s=... t=... c=IN IP4 <UE_X_IPv4_addr_data> m=audio <UE_X_IPv4_port_audio> RTP/AVP <fmt> … ; SDP attributes for audio format description(s) a=altc:1 IP6 <UE_X_IPv6_addr_data> <UE_X_IPv6_port_audio> a=altc:2 IP4 <UE_X_IPv4_addr_data> <UE_X_IPv4_port_audio> … | The "altc" attribute is a media-level-only attribute and MUST NOT appear at the SDP session-level. The "c="-line is at session-level in example 1. |

### 9.1.2	SIP SDP offer example 2: multimedia WebRTC call

Consideration of a dualmedia WebRTC call with media components audio and video. A typical SIP-level SDP offer is illustrated in Table 9.1.2:

Table 9.1.2 – Example command encoding – (SIP) SDP Offer example 2

| (SIP) SDP encoding | Comments |
| --- | --- |
| v=0 o=... s=... t=... a=group:BUNDLE audio video ; m=audio <UE_X_IPv4_port_audio_rtp> RTP/SAVPF 103 104 0 8 106 105 13 126 c=IN IP4 <UE_X_IPv4_addr_audio_rtp> a=rtcp:<UE_X_IPv4_port_audio_rtcp> IN IP4 <UE_X_IPv4_addr&port_audio_rtcp> a=mid:audio a=rtcp-mux [attributes for media formats, RTP media multiplexing, …] a=sendrecv a=altc:1 IP6 <UE_X_IPv6_addr_audio data> <UE_X_IPv6_port_audio> a=altc:2 IP4 <UE_X_IPv4_addr_audio data> <UE_X_IPv4_port_audio> ; m=video <UE_X_IPv4_port_video_rtp> RTP/SAVPF 100 101 102 c=IN IP4 <UE_X_IP_addr_video_rtp> a=rtcp:<UE_X_IPv4_port_video_rtcp> IN IP4 <UE_X_IPv4_addr&port_video_rtcp> a=mid:video a=rtcp-mux [attributes for media formats, RTP media multiplexing, …] a=sendrcv a=altc:1 IP6 <UE_X_IPv6_addr_video data> <UE_X_IPv6_port_video> a=altc:2 IP4 <UE_X_IPv4_addr_video data> <UE_X_IPv4_port_video> … | There are two media descriptions for audio and video. Both are grouped via bundling. The SIP UA provides a multihomed IP host capability (inclusive dual-stack IP interfaces) and wants to use different IP interfaces for each media component. Hence, there are SDP media-level "c="-lines (in contrast to example 1). There are consequently two pairs of "a=altc:"-lines, one per media description. The SDP offer considers that IPv4 is most likely to be used for the session, but IPv6 is preferred. Furthermore, RTCP control flows could be either transport multiplexed (via "a=rtcp-mux") or transport unmultiplexed by allocation of an explicit transport port (via "a=rtcp:"). |

'The combination of "bundling of audio and video" and separate IP connection endpoints for audio and video isn't a contradiction (in the SDP offer). When the SDP answerer doesn't support bundling then just all bundle related SDP ("a=group:BUNDLE" and "a=mid:") would be ignored and media type individual IP connections would be used. When the SDP answerer supports bundling then the first IP connection would be shared by both media components.

Proceeding of evaluation:

The 2nd SDP offer example should be more suitable for the demonstration of ALTC support by H.248 entities due to variety of IP interfaces, alternatives, transport multiplexing and RTCP port allocation options.

It has to be noted that the typical ICE functionality (as part of WebRTC call control signalling) is not considered here. There would not be any added value, see see clause 4.2.3/[IETF RFC 6947] "Interaction with ICE".

## 9.2	Gateway control signalling: examples

(SIP) SDP Offer example 2 is used for below H.248 signalling examples. H.248 signalling illustrates only the "ALTC side" of an H.248 context.

{Editor's note (2015-02): it was commented that the signalling examples might be simplified by indicating a single media type only instead of two (her audio and video).
Background for two media types in the signalling examples:
Idea: "ALTC" relates actually to a vertical split of the "protocol stack" in a lower layer "network" segment and an upper layer "media" segment, hence the rough two classes of SDP "c="- and "m="-line related information. Both classes relate to two categories of MG resources. Any H.248 transaction cycle will lead to reservation, allocation, de-allocation or modification of such resources. It seems to be beneficial to separate also correspondent signalling. E.g., if not separated, then any sub-sequent H.248 transaction cycle must repeat all signalling information related to the "media" segment. There seems to be benefit from avoiding such redundancy.
This is a quantitative criteria. However, there isn't yet any proof of this assumption because the signalling examples are not yet complete.)

### 9.2.1	Example H.248 signalling in case of "application aware IWF"

Thus, there would be fully specified media descriptions.

#### 9.2.1.1	Usage of H.248 ALTC context model "M1" (two contexts)

##### 9.2.1.1.1	First transaction cycle: preparation of local IP connection endpoints

The SIP-level SDP offer stimulates an H.248 transaction request from MGC to MG (Table 9.2.1.1/1). The MGC selects the two context model "M1" (as illustrated in Figure 2).

Notes to SIP (SDP) to H.248 (SDP) mapping:

- Decision (by MGC) concerning the order of preferred IP connectivity:

Resources of local destination (inclusive local source) IP connection endpoints (i.e., LD(A,P) and LS(A,P) are supposed to be managed at MG level, which implies SDP wildcarding of such resources in the H.248 local destination descriptor. The MGC shall therefore request the preferred IP connection endpoint resources in context Cz,1 (example here: IPv6 due the network operator policy for IPv6 preference in IP domain Y).

- Decision (by MGC) concerning "RTP /RTCP transport multiplexing versus explicit RTCP transport address allocation":

MGC considers that is most likely to be used for the IP connection segment towards UE_B. The SDP "a=rtcp:" information is therefore removed from the SDP media description in H.248 stream descriptors.

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.1.1/1 and Table 9.2.1.1/2:

Table 9.2.1.1/1 – Example command encoding – Step 1 – MGC request 
(for local connection point only in outgoing direction)

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { ; Context CZ1 Add = ip/$/$/$ { ; Termination TY,P Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ; NOTE 1 ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP6 $ m=audio $ RTP/SAVPF 103 104 0 8 106 105 13 126 a=rtcp-mux [attributes for media formats, RTP media multipl., …] … }, Remote { ; still empty (because SDP answer from UE Y not yet received } Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ; NOTE 1 ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP6 $ m=video $ RTP/SAVPF 100 101 102 a=rtcp-mux [attributes for media formats, RTP media multipl., …] , Remote { ; still empty (because SDP answer from UE Y not yet received } … } Transaction = 2 { Context = $ { ; Context CZ2 Add = ip/$/$/$ { ; Termination TY,A Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = Inactive, ; NOTE 2 ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP4 $ m=audio $ RTP/SAVPF 103 104 0 8 106 105 13 126 a=rtcp-mux [attributes for media formats, RTP media multipl., …] … }, Remote { ; still empty (because SDP answer from UE Y not yet received } Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = Inactive, ; NOTE 2 ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP4 $ m=video $ RTP/SAVPF 100 101 102 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, Remote { ; still empty (because SDP answer from UE Y not yet received } … } | The ADD.request command for Termination TX is omitted. Some observations: - SDP attributes "a=altc:" are not used in H.248 descriptors. - The IP connection endpoints for IPv4 and IPv6 belong essentially to separate IP address realms. They can't be inherently part of the same IP address space. That characteristic is reflected by two separate codepoints of property ipdc/realm. NOTE 1 – The StreamMode property setting of value "Inactive" at SEPs at termination TY,P prevents support of "early media". NOTE 2 – The StreamMode property setting of value "Inactive" at SEPs at termination TY,A is required because these SEPs are still isolated in context Cz.2. |

A possible example MG reply is indicated in Table 9.2.1.1/2:

Table 9.2.1.1/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 1 { Context = Cz1 { ; Context CZ1 Add = ip/y/p/1 { ; Termination TY,P Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=audio <IP_Y,P_IPv6_port_audio> RTP/SAVPF 103 104 0 8 106 105 13 126 a=rtcp-mux … }, Remote { ; still empty (because SDP answer from UE Y not yet received } Stream = 2 { ; SEP S2 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_video> m=video <IP_Y,P_IPv6_port_video> RTP/SAVPF 100 101 102 a=rtcp-mux … }, Remote { ; still empty (because SDP answer from UE Y not yet received } … } Reply = 2 { Context = Cz2 { ; Context CZ2 Add = ip/y/a/1 { ; Termination TY,A Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=audio <IP_Y,P_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] … }, Remote { ; still empty (because SDP answer from UE Y not yet received } Stream = 2 { ; SEP S2 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_video> m=video <IP_Y,P_IPv4_port_video> RTP/SAVPF 100 a=rtcp-mux ## [attributes for media formats, RTP media multipl., …] }, Remote { ; still empty (because SDP answer from UE Y not yet received } … } | The media format list is reduced to a single media format (here '103' in case of audio) due to ReserveValue setting of FALSE. The decision made by the MG is dependent on the Stream Descriptor settings at termination TX. |

##### 9.2.1.1.2	Second transaction cycle: decision to use preferred IP connection

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.1.1/3a and Table 9.2.1.1/.4a:

Table 9.2.1.1/3a – Example command encoding – Step 3a – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 3 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY,P Media { Stream = 1 { ; SEP S1 LocalControl { ## ipdc/realm = <realm_Y_IPv6>, Mode = SendRecv, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, ; LocalDescriptor don't need to be modified anymore (NOTE 1) Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_audio> m=audio <IP_UE_Y_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] … }, } Stream = 2 { ; SEP S2 LocalControl { ## ipdc/realm = <realm_Y_IPv6>, Mode = SendRecv, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, ; LocalDescriptor don't need to be modified anymore Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_video> m=video <IP_UE_Y_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux ## [attributes for media formats, RTP media multipl., …] } … } Transaction = 4 { Context = Cz2 { ; Context CZ2 Subtract = ip/y/a/1 { ; Termination TY,A } } | ## IPv6 is the selected IP version, hence termination TY,P is kept and completety specified by addition of the still missing remote destination information (via H.248 MODify command). ## NOTE 1 – The LD with the reduced media format list don't need to be repeated again because still stored (as it should be) at MG side. ## And termination TY,A is removed via H.248 SUBtract. |

A possible example MG reply is indicated in Table 9.2.1.1/4a:

Table 9.2.1.1/4a – Example command encoding – Step 4a – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 3 { Context = Cz1 { ; Context CZ1 Add = ip/y/p/1 { ; Termination TY,P Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=audio <IP_Y,P_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux … }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_audio> m=audio <IP_UE_Y_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux … }, Stream = 2 { ; SEP S2 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_video> m=video <IP_Y,P_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux … }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_video> m=video <IP_UE_Y_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux } …} |  |

##### 9.2.1.1.3	Second transaction cycle: decision to use alternate IP connection

IPv4 is the selected IP version, hence termination TY,P is subtracted and replaced by termination TY,A (using an H.248 MOVE command). See Table 9.2.1.1/3b and Table 9.2.1.1/.4b:

Table 9.2.1.1/3b – Example command encoding – Step 3b – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 3 { Context = Cz1 { ; Context CZ1 ; step 1 – removal of TY,P via SUBtract Subtract = ip/y/p/1 { ; Termination TY,P }, ; step 2 – replacement by TY,A via MOVe Move = ip/y/a/1 { ; Termination TY,A Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = SendRecv, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, ; LocalDescriptor don't need to be modified anymore Remote { v=0 c=IN IP4 <IP_UE_Y_IPv4_addr_audio> m=audio <IP_UE_Y_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] … }, } Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = SendRecv, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, ; LocalDescriptor don't need to be modified anymore Remote { v=0 c=IN IP4 <IP_UE_Y_IPv4_addr_video> m=video <IP_UE_Y_IPv4_port_video> RTP/SAVPF 100 101 102 a=rtcp-mux ## [attributes for media formats, RTP media multipl., …] } … ## } | IPv4 is the selected IP version, hence termination TY,P is subtracted and replaced by termination TY,A (using an H.248 MOVE command). Both H.248 command requests are subject of context Cz1. It has to be noted that the Stream identifier values in the moved termination has to be consistent with the removed termination. |

A possible example MG reply is indicated in Table 9.2.1.1/4b:

Table 9.2.1.1/4b – Example command encoding – Step 4b – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 3 { Context = Cz1 { ; Context CZ1 Move = ip/y/a/1 { ; Termination TY,A Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=audio <IP_Y,P_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux … }, Remote { v=0 c=IN IP4 <IP_UE_Y_IPv4_addr_audio> m=audio <IP_UE_Y_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux … }, Stream = 2 { ; SEP S2 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_video> m=video <IP_Y,P_IPv4_port_video> RTP/SAVPF 100 a=rtcp-mux … }, Remote { v=0 c=IN IP4 <IP_UE_Y_IPv4_addr_video> m=video <IP_UE_Y_IPv4_port_video> RTP/SAVPF 100 a=rtcp-mux } …} |  |

#### 9.2.1.2	Usage of H.248 ALTC context model "M2" (single context with two terminations for ALTC)

##### 9.2.1.2.1	First transaction cycle: preparation of local IP connection endpoints

The option without early media support is selected, i.e., there will be not an early context internal cut-through of the H.248 streams. This is achieved be keeping the StreamMode inactive. (tbc)

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.1.2/1 and Table 9.2.1.2/2:

Table 9.2.1.2/1 – Example command encoding – Step 1 – MGC request 
(for local connection point only in outgoing direction)

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { ; Context CZ1 Topology{}, Add = ip/$/$/$ { ; Termination TY,P Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP6 $ m=audio $ RTP/SAVPF 103 104 0 8 106 105 13 126 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, Remote { ; still empty (because SDP answer from UE Y not yet received } Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP6 $ m=video $ RTP/SAVPF 100 101 102 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, Remote { ; still empty (because SDP answer from UE Y not yet received } ; 2nd Termination for the same context Add = ip/$/$/$ { ; Termination TY,A Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP4 $ m=audio $ RTP/SAVPF 103 104 0 8 106 105 13 126 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, Remote { ; still empty (because SDP answer from UE Y not yet received } Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP4 $ m=video $ RTP/SAVPF 100 101 102 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, Remote { ; still empty (because SDP answer from UE Y not yet received } } | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.1.2/2:

Table 9.2.1.2/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

##### 9.2.1.2.2	Second transaction cycle: decision to use preferred IP connection

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.1.2/3a and Table 9.2.1.2/.4a:

Table 9.2.1.2/3a – Example command encoding – Step 3a – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.1.2/4a:

Table 9.2.1.2/4a – Example command encoding – Step 4a – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

##### 9.2.1.2.3	Second transaction cycle: decision to use alternate IP connection

FIXTHIS

#### 9.2.1.3	Usage of H.248 ALTC context model "M3" (single termination with two SEPs for ALTC)

{Editor's note (2015-02): it was questioned whether M3 could be realized at all from protocol syntax perspective. )

This method is conditional, applicable for single-stream-per-termination structures only, such as an audio call only in this example. The first SEP is used for the resources tight to the preferred IP connection, the second contains resources related to the alternate IP connection.

##### 9.2.1.3.1	First transaction cycle: preparation of local IP connection endpoints

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.1.3/1 and Table 9.2.1.3/2:

Table 9.2.1.3/1 – Example command encoding – Step 1 – MGC request 
(for local connection point only in outgoing direction)

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { ; Context CZ1 Add = ip/$/$/$ { ; Termination TY,P Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP6 $ m=audio $ RTP/SAVPF 103 104 0 8 106 105 13 126 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, Remote { ; still empty (because SDP answer from UE Y not yet received } Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP4 $ m=audio $ RTP/SAVPF 103 104 0 8 106 105 13 126 a=rtcp-mux [attributes for media formats, RTP media multipl., …] Remote { ; still empty (because SDP answer from UE Y not yet received } …} |  |

A possible example MG reply is indicated in Table 9.2.1.3/2:

Table 9.2.1.3/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

##### 9.2.1.3.2	Second transaction cycle: decision to use preferred IP connection

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.1.3/3a and Table 9.2.1.3/.4a:

Table 9.2.1.3/3a – Example command encoding – Step 3a – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.1.3/4a:

Table 9.2.1.3/4a – Example command encoding – Step 4a – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

##### 9.2.1.3.3	Second transaction cycle: decision to use alternate IP connection

FIXTHIS

#### 9.2.1.4	Usage of H.248 ALTC context model "M4" (two media groups for ALTC)

##### 9.2.1.4.1	First transaction cycle: preparation of local IP connection endpoints

There are two media groups (preferred media group and alternate media group) required per media type (audio and video). Such a resource reservation is achieved by a setting of the LocalControl properties combination of {ReserveValue = "False" AND ReserveGroup = "True"}.

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.1.4/1 and Table 9.2.1.4/2:

Table 9.2.1.4/1 – Example command encoding – Step 1 – MGC request 
(for local connection point only in outgoing direction)

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { ; Context CZ1 Add = ip/$/$/$ { ; Termination TY Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ReservedGroup = ON, ; two media groups! ReservedValue = OFF, ; the default value ... }, Local { v=0 ; 1st media group for audio c=IN IP6 $ m=audio $ RTP/SAVPF 103 104 0 8 106 105 13 126 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of 1st media group v=0 ; 2nd media group for audio c=IN IP4 $ m=audio $ RTP/SAVPF 103 104 0 8 106 105 13 126 a=rtcp-mux [attributes for media formats, RTP media multipl., …] … ; end of 2nd media group }, Remote { ; still empty (because SDP answer from UE Y not yet received } Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 ; 1st media group for video c=IN IP6 $ m=video $ RTP/SAVPF 100 101 102 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of 1st media group v=0 ; 2nd media group for video c=IN IP4 $ m=video $ RTP/SAVPF 100 101 102 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, ; end of 2nd media group }, Remote { ; still empty (because SDP answer from UE Y not yet received } … } |  |

Following issue was noticed in the H.248 stream descriptor in above command requests:

Problem:

- the IP domain connection indication (via property ipdc/realm) in Version 1 of the IP Domain Connection package is on LocalControl descriptor level

- the IP connection endpoint information in the SDP media description must be consistent with the IP domain indication

- … hence one H.248 media group would be in violation with the IP domain indication!

Background:

- IPv4 and IPv6 belong inherently to two different IP address realms (or briefly IP realms, aka H.248.41 “IP domains”).

- Thus, the preferred IP connection and alternate IP connection are routed through different IP address realms.

- There are two IP domain connection code points …

- and the IP realm identifier is mandatory signalling information due to the assumption of multiple IP address realms (e.g., IMS AN vs CN side …).

The above is no longer an issue as Version 2 of the IP Domain Connection package has the ability to signal multiple IP realms.

A possible example MG reply is indicated in Table 9.2.1.4/2.

Table 9.2.1.4/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 1 { Context = Cz1 { ; Context CZ1 Add = ip/y/p/1 { ; Termination TY Media { Stream = 1 { ; SEP S1 Local { v=0 ; 1st media group for audio c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=audio <IP_Y,P_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of 1st media group v=0 ; 2nd media group for audio c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=audio <IP_Y,P_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of 2nd media group }, Remote { ; still empty (because SDP answer from UE Y not yet received } Stream = 2 { ; SEP S2 Local { v=0 ; 1st media group for video c=IN IP6 <IP_Y,P_IPv6_addr_video> m=video <IP_Y,P_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of 1st media group v=0 ; 2nd media group for video c=IN IP4 <IP_Y,P_IPv4_addr_video> m=video <IP_Y,P_IPv4_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of 2nd media group }, Remote { ; still empty (because SDP answer from UE Y not yet received } …} | The media format list is reduced to a single media format (here '103' in case of audio) due to ReserveValue setting of FALSE. The decision made by the MG is dependent on the Stream Descriptor settings at termination TX. |

##### 9.2.1.4.2	Second transaction cycle: decision to use preferred IP connection

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.1.4/3a and Table 9.2.1.4/.4a.

Table 9.2.1.4/3a – Example command encoding – Step 3a – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 2 { Context = Cz1 { ; Context CZ1 Add = ip/y/p/1 { ; Termination TY Media { Stream = 1 { ; SEP S1 LocalControl { ; NOTE 1 ipdc/realm = <realm_Y_IPv6>, ; NOTE 2 Mode = SendRecv, ; NOTE 3 ReservedGroup = OFF, ; NOTE 4 ReservedValue = OFF, ; the default value ... }, Local { ; NOTE 5 v=0 ; selected media group for audio c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=audio <IP_Y,P_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of "media group" ; no second media group for audio }, Remote { v=0 ; only a single media group for audio c=IN IP6 <IP_UE_Y_IPv6_addr_audio> m=audio <IP_UE_Y_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { ; NOTE 5 v=0 ; selected media group for video c=IN IP6 <IP_Y,P_IPv6_addr_video> m=video <IP_Y,P_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of "media group" ; no second media group for video }, Remote { v=0 ; only a single media group for video c=IN IP6 <IP_UE_Y_IPv6_addr_video> m=video <IP_UE_Y_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } …} | Preferred option is IPv6, i.e., the first H.248 media group remains and the second one is just not included in the transaction request. NOTE 1 – It may be noted that the LCD has to be included again (in contrast to e.g. model M1) due to changed property values NOTE 2 – The IP domain indication is now correct because unambiguous. NOTE 3 – Now, "early" media enabled (TBC). NOTE 4 – ReserveGroup disabled again due to selection of a single media group. NOTE 5 – It may be noted that the LD has to be completely included (in contrast to e.g. model M1) with the selected H.248 media group. |

A possible example MG reply is indicated in Table 9.2.1.4/4a.

Table 9.2.1.4/4a – Example command encoding – Step 4a – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 1 { Context = Cz1 { ; Context CZ1 Add = ip/y/p/1 { ; Termination TY Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=audio <IP_Y,P_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_audio> m=audio <IP_UE_Y_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux } Stream = 2 { ; SEP S2 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_video> m=video <IP_Y,P_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_video> m=video <IP_UE_Y_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux } …} | FIXTHIS |

##### 9.2.1.4.3	Second transaction cycle: decision to use alternate IP connection

IPv4 is selected. The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.1.4/3b and Table 9.2.1.4/.4b.

Table 9.2.1.4/3b – Example command encoding – Step 3b – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 2 { Context = Cz1 { ; Context CZ1 Add = ip/y/p/1 { ; Termination TY Media { Stream = 1 { ; SEP S1 LocalControl { ; NOTE 1 ipdc/realm = <realm_Y_IPv4>, ; NOTE 2 Mode = SendRecv, ; NOTE 3 ReservedGroup = OFF, ; NOTE 4 ReservedValue = OFF, ; the default value ... }, Local { ; NOTE 5 v=0 ; selected media group for audio c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=audio <IP_Y,P_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of "media group" ; no second media group for audio }, Remote { v=0 ; only a single media group for audio c=IN IP4 <IP_UE_Y_IPv4_addr_audio> m=audio <IP_UE_Y_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { ; NOTE 5 v=0 ; selected media group for video c=IN IP4 <IP_Y,P_IPv4_addr_video> m=video <IP_Y,P_IPv4_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of "media group" ; no second media group for video }, Remote { v=0 ; only a single media group for video c=IN IP4 <IP_UE_Y_IPv4_addr_video> m=video <IP_UE_Y_IPv4_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } …} | Alternate option is IPv4, i.e., the first H.248 media group remains and the second one is just not included in the transaction request. NOTE 1 – It may be noted that the LCD has to be included again (in contrast to e.g. model M1) due to changed property values NOTE 2 – The IP domain indication is now correct because unambiguous. NOTE 3 – Now, "early" media enabled (TBC). NOTE 4 – ReserveGroup disabled again due to selection of a single media group. NOTE 5 – It may be noted that the LD has to be completely included (in contrast to e.g. model M1) with the selected H.248 media group. |

A possible example MG reply is indicated in Table 9.2.1.4/4b.

Table 9.2.1.4/4b – Example command encoding – Step 4b – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 1 { Context = Cz1 { ; Context CZ1 Add = ip/y/p/1 { ; Termination TY Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=audio <IP_Y,P_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux }, Remote { v=0 c=IN IP4 <IP_UE_Y_IPv4_addr_audio> m=audio <IP_UE_Y_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux } Stream = 2 { ; SEP S2 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_video> m=video <IP_Y,P_IPv4_port_video> RTP/SAVPF 100 a=rtcp-mux }, Remote { v=0 c=IN IP4 <IP_UE_Y_IPv4_addr_video> m=video <IP_UE_Y_IPv4_port_video> RTP/SAVPF 100 a=rtcp-mux } …} | FIXTHIS |

#### 9.2.1.5	Usage of H.248 ALTC context model "M5" (two stream groups)

FIXTHIS

#### 9.2.1.6	Usage of H.248 ALTC context model "M6" (dual stack IP interface)

For further studies.

### 9.2.2	Example H.248 signalling in case of "application agnostic IWF"

#### 9.2.2.1	Usage of H.248 ALTC context model "M1" (two contexts)

The usage of an (initial) application agnostic signalling at the ALTC context side may result in the huge benefit of fairly short media descriptions within the H.248 stream descriptors. The gain depends on the list size of media format types, ReserveValue settings and number of media attributes per media format type.

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.2.1/1 and Table 9.2.2.1/2:

Table 9.2.2.1/1 – Example command encoding – Step 1 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { ; Context CZ1 Add = ip/$/$/$ { ; Termination TY,P Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ; NOTE 1 ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP6 $ m=- $ RTP/SAVPF - ; NOTE 3 a=rtcp-mux }, Remote { ; still empty (because SDP answer from UE Y not yet received } Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ; NOTE 1 ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP6 $ m=- $ RTP/SAVPF - NOTE 3 a=rtcp-mux , Remote { ; still empty (because SDP answer from UE Y not yet received } … } Transaction = 2 { Context = $ { ; Context CZ2 Add = ip/$/$/$ { ; Termination TY,A Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = Inactive, ; NOTE 2 ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP4 $ m=- $ RTP/SAVPF - NOTE 3 a=rtcp-mux }, Remote { ; still empty (because SDP answer from UE Y not yet received } Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = Inactive, ; NOTE 2 ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP4 $ m=- $ RTP/SAVPF - NOTE 3 a=rtcp-mux }, Remote { ; still empty (because SDP answer from UE Y not yet received } … } | Additional comments in comparison the the application-aware case (see Table 9.2.1.1/1): NOTE 3 – "Application agnostic" results in a - media type agnostic, - media format agnostic and - transport protocol aware configuration. Transport protocol awareness (RTP/SAVPF) is used as indication for RTP media flow and RTCP control flows due to request of RTP / RTCP transport multiplexing. The ReserveValue property would be essentially not effective in case of a TRUE setting. |

A possible example MG reply is indicated in Table 9.2.2.1/2:

Table 9.2.2.1/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.2.1/3 and Table 9.2.2.1/.4:

Table 9.2.2.1/3 – Example command encoding – Step 3 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.2.1/4:

Table 9.2.2.1/4 – Example command encoding – Step 4 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

#### 9.2.2.2	Usage of H.248 ALTC context model "M2" (single context with two terminations for ALTC)

FIXTHIS

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.2.2/1 and Table 9.2.2.2/2:

Table 9.2.2.2/1 – Example command encoding – Step 1 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.2.2/2:

Table 9.2.2.2/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.2.2/3 and Table 9.2.2.2/.4:

Table 9.2.2.2/3 – Example command encoding – Step 3 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.2.2/4:

Table 9.2.2.2/4 – Example command encoding – Step 4 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

#### 9.2.2.3	Usage of H.248 ALTC context model "M3" (single termination with two SEPs for ALTC)

FIXTHIS

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.2.3/1 and Table 9.2.2.3/2:

Table 9.2.2.3/1 – Example command encoding – Step 1 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.2.3/2:

Table 9.2.2.3/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.2.3/3 and Table 9.2.2.3/.4:

Table 9.2.2.3/3 – Example command encoding – Step 3 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.2.3/4:

Table 9.2.2.3/4 – Example command encoding – Step 4 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

#### 9.2.2.4	Usage of H.248 ALTC context model "M3" (two media groups for ALTC)

FIXTHIS

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.2.4/1 and Table 9.2.2.4/2:

Table 9.2.2.4/1 – Example command encoding – Step 1 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.2.4/2:

Table 9.2.2.4/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.2.4/3 and Table 9.2.2.4/.4:

Table 9.2.2.4/3 – Example command encoding – Step 3 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.2.4/4:

Table 9.2.2.4/4 – Example command encoding – Step 4 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

#### 9.2.2.5	Usage of H.248 ALTC context model "M5" (two stream groups)

FIXTHIS

# 10	Profile guidelines

FIXTHIS

Appendix I

Discussion of the various H.248 solution options 
 – Signalling examples

## I.1	####

{Editor's note (2014-11): this Appendix is a placeholder for the text from current clause 9. All the signalling examples should be moved from clause 9 to this Appendix. Clause 9 would finally just indicated the preferred / recommended method(s).}

Appendix II

Discussion of the various H.248 solution options 
 – Summary

## II.1	Summary table

Table II.1 summarize aspects which might be positive or not in particular situations. There majority of statements is "soft" (because conditional) besides a few "hard" facts (because in violation with protocol syntax/semantics or limited context model):

{Contributor's note (2014-11-12): table updates are still ongoing, dependent on signalling example discussions …}

Table II.1 – Evaluation summary

| H.248 solution option | Pro | Con | Comments |
| --- | --- | --- | --- |
| M1: two context model | - isolation of "alternate MG resources" in separate context - context of "preferred IP connectivity" is fully identical to the usual non-ALTC context configuration | - usage of MOVE command - support of early media not possible (because limited on preferred IP connection only) |  |
| M2: two termination model | - isolation of "alternate MG resources" in separate termination - support of early media possible, but dependent on topology signalling | - Topology descriptor required - thus, two H.248 transaction cycles | The Stream topology subject could be considered as blocking item. |
| M3: two stream model | - single termination | - conditional (only monomedia call types) - static H.248 StreamID values imply removal of S2 AND modification of S1, possibly inclusive IP version change | The StreamID subject could be considered as blocking item. |
| M4: two media groups model | - the semantic of ReserveGroup matches actually the semantic required for ALTC | - protocol issue: obvious contradiction with LCD level IP domain indication versus LD / RD level usage of "two IP address realms" in Version 1 of the IPDC package. - No issue with version 2 of the IPDC package. | Protocol contradiction of H.248.41 usage together with ReserveGroup and IP address realm specific media groups only in IPDC package version 1. |
| M5: two stream groups model | - straightforward specification of Context-internal stream topology (without Topology descriptor); - Context-internal resource configuration ("IWFs") remains unchanged after ALTC decision - Stream grouping allows to decouple the MG internal and MG external resources related to H.248 media streams | - FIXTHIS | dependent on progress of draft H.248.STGROUP, i.e., solution not yet available |
| M6: dualstack IP interface resource | - ATLC decision would result in the simple modification of a single signalling parameter value | - protocol extensions (SDP or/and H.248 required) | Protocol extension subject could be considered as blocking item. |

## II.2	Recommended solution(s)

Three out of six candidates could be most likely rejected: M2, M3 and M6.

Thus, M1, M4 and M5 would be the remaining candidates.

Option M4 is not possible due to interaction with existing H.248.41.

Option M5 would be an interesting alternative dependent on progress of H.248.STGROUP.

[…]

Appendix III

Example H.248 profile: 3GPP Iq profile

## III.1	Summary of stage 2 requirements

{Contributor's note (2014-11): there might be following stage 2 summary after a quick check:

…for further studies}

Bibliography

[b-ETSI TR 183 046]	ETSI TR 183 046 V3.3.1 (08/2009), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); SDP Interworking between Call/Session Control Protocols (SIP/SDP, RTSP/SDP; etc.) and the Gateway Control Protocol (H.248/SDP).

_________________________

See e.g. Figure G.2 in [ETSI TR 183 068].