| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4685 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.RTPMUX: update proposals to clauses 6 to 9 |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides some editorial update proposals for the main body of draft H.248.RTPMUX.

# Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD 4679 as input draft for this Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

# 6	Overview of RTP multiplexing

## 6.1	Introduction

Multiple, RTP-based traffic flow components may be multiplexed at multiple, different levels (from the perspective of a layered protocol architecture).

A RTP-based traffic flow is given by a packet flow with a common identifier (also known as flow identifier or traffic descriptor). A particular multiplexing method uses specific information elements of such a traffic descriptor, which are mandatory for de-multiplexing again. Figure 1 summarizes the three major multiplexing techniques for RTP traffic, as well as the used header elements.

Figure 2 – Different multiplexing methods for RTP traffic 
and their used RTP header fields

In more detail:

- RTP transport multiplexing (see clause 6.3) uses the 8-bit field of Marker bit (M) and 7-bit Payload Type (PT), which together comprise the 8-bit Packet Type (PT);

- RTP media multiplexing (see clause 6.4) uses the 32-bit SSRC (hence, also known as SSRC multiplexing);

- UDP payload multiplexing (see clause 6.5) is based on the first octet of the UDP payload, which relates to the RTP header field indicated in Figure 2.

## 6.2	Protocol history: multiplexing framework defined for original RTP

Clause 5.2/[IETF RFC 3550] "Multiplexing RTP Sessions" describes the general multiplexing principles for RTP.

## 6.3	RTP transport multiplexing

RTP transport multiplexing represents the case when a RTP media flow and its associated RTCP control flow share a common IP transport address endpoint. In a normal, non-multiplexed scenario, RTP and RTCP use different endpoint identifiers on layer 4 (e.g., UDP ports in case of RTP-over-UDP transport).

Enabling of RTP transport multiplexing is achieved by the allocation of the same L4 port for RTCP as allocated for RTP.

Port allocation rules for RTCP at H.248 interfaces are subject of [ITU-T H.248.57], which covers both, the transport unmultiplexed and multiplexed cases.

RTP transport multiplexing is an extension to [IETF RFC 3550], motivated and defined by [IETF RFC 5761].

### 6.3.1	Limitations of RTP transport multiplexing

Clauses 4 and 5 in [IETF RFC 5761] describe a number of constraints, interactions and conditions to be considered when applying RTP transport multiplexing.

### 6.3.2	Control of RTP transport multiplexing

Control of RTP transport multiplexing:

- Using SDP: via SDP attribute "a=rtcp-mux" (see clause 8/[IETF RFC 5761]). The usage of this SDP element at H.248 interfaces is defined by [ITU-T H.248.57].

### 6.3.3	Relation to (RTP, non-RTP media) multiplexing in general

RTP transport multiplexing is orthogonal to other kinds of multiplexing mechanisms (such as application-specific media level multiplexing options).

RTP transport multiplexing is basically (RTP) application agnostic, only driven by NAT traversal aspects.

## 6.4	RTP media multiplexing

RTP media multiplexing represents the case when multiple RTP streams of different RTP sources share a common IP transport address endpoint. RTP media multiplexing is also known as SSRC multiplexing because the different RTP sources could be separated on SSRC basis. In a normal, non-multiplexed scenario, individual RTP sources use different endpoint identifiers on layer 4 (e.g., UDP ports in case of RTP-over-UDP transport).

Enabling of RTP media multiplexing is achieved at SIP-based call control level via the SDP method "bundling" (see [###]). Bundling again transformed in H.248 stream grouping for H.248 signalling.

## 6.5	UDP payload multiplexing

UDP payload multiplexing represents the case of the specific combination of

- UDP as transport protocol for RTP and RTCP plus

- secure RTP (SRTP and SRTCP) plus

- DTLS-based SRTP key exchange between the SRTP endpoints plus

- ICE/STUN –based NAT traversal support for the UDP connection.

This case refers as "DTLS-SRTP" and is defined by [IETF RFC 5764]. A single UDP connection is then used as multiplexing structure for the three traffic flow components related to L4+ protocols SRTP, DTLS and STUN. The demultiplexing method is defined in clause 5.1.2 of [IETF RFC 5764], using the leading byte of the UDP packet payload.

Enabling of UDP payload multiplexing is achieved at SIP-based call control level via the indication of "DTLS-SRTP" in RTP related SDP media descriptions (see [###]). H.248 gateway control signaling indicates the application of UDP payload multiplexing at a particular H.248 termination/stream implicitly, via the signalling of the SRTP key management scheme [ITU-T H.248.77], usage of ICE/STUN [ITU-T H.248.50] and DTLS related bearer control [ITU-T H.248.93].

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 7	Use cases with RTP multiplexing

## 7.1	General note

There are theoretically at least following reasons for the application of RTP multiplexing:

- maximize the IP transport capacity efficiency (colloquially also known as "bandwith efficiency");

- maximize the likelihood of successful  NAT traversal by minimizing the number of "pinholes" (i.e., number of IP transport connections);

- minimize the call establishment delay in case of ICE-based NAT traversal procedures by minimizing the number of IP transport connections, because every candidate IP transport connection must be firstly tested for end-to-end connectivity before; or/and

- "piggy back" the DTLS and STUN packet flows on the same RTP-over-UDP connection in case of DTLS-SRTP.

Scenario (4) is already applicable for a single RTP stream ("monomedia call"), but all other multiplexing schemes take only effect for communication services with multiple RTP streams ("multimedia call").

The dominant use cases are driven by NAT traversal (2, 3) for multimedia communication services. This clause focuses on such scenarios.

## 7.2	Use case "multimedia call in NGN/IMS"

FIXTHIS

## 7.3	Use case "Web Real-Time Communication"

[b-ITU-T H.248.WEBRTC] defines an H.248 WebRTC gateway, which is required to support RTP multiplexing an inherent defined capability of this service. See [b-ITU-T H.248.WEBRTC], e.g. clause 6.3 ("WebRTC protocol stacks inclusive multiplexing usage") and clause 8.4.3 ("RTP multiplexing requirements for WebRTC").

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

# 8	H.248 control of RTP multiplexing

## 8.1	General principle

Multiplexing of multiple H.248 Streams onto one multiplexed Stream is primarily indicated by setting the same address/port information on the applicable H.248 Streams.

## 8.2	RTP media multiplexing

Additionally the MGC may set the “association” (mgroup/groupse) property indicating which H.248 Streams on a Termination are associated together for the purposes of multiplexing, with the grouping type of “BUNDLE”.

[ITU-T H.248.STGROUP] describes these methods.

## 8.3	RTP transport multiplexing

Solely controlled via [ITU-T H.248.57], any kind of H.248 stream grouping [ITU-T H.248.STGROUP] is not required, even not applicable (due to the lack of a correspondent grouping semantic).

[End Proposed Correction]

Change proposal #4:

[Begin Proposed Correction]

# 9	RTP multiplexing examples

{Editor’s note: The scope of the examples need further consideration. The examples below show the use of grouping i.e. the use of the bundle semantic and H.248.STGROUP. It needs to be considered whether the description of RTP multiplexing should focus on the mechanism defined in H.248.STGROUP.}

This clause provides example use cases of RTP multiplexing. In each of the examples, Terminations (T1) and (T2) have five Streams. StreamIDs (1) and (2) are audio streams. StreamIDs (3), (4) and (5) are video streams. These RTP and H.248 streams are bi-directional. The MGC indicates Stream multiplexing by setting the same address/port information on the streams to be multiplexed and by setting the “BUNDLE” grouping semantic tying the Streams together.

There are two basic RTP multiplexing methods, -

- RTP/RTCP transport multiplexing (see clause 6.3) and

- RTP media multiplexing ("SSRC multiplexing") (see clause 6.4).

The examples in clauses 9.1 and 9.2 show RTP media multiplexing.

## 9.1 Video stream multiplexing

In this example the video streams are multiplexed together on Termination (T1) using the “BUNDLE” grouping semantic. The scenario is illustrated in Figure 3.

Figure 3 – Example: Video Stream Multiplexing connection model

Tables 2 and 3 provide an example H.248 command request and response establishing the connection on Termination T1 illustrated in Figure 3.

The MG reply to the command request for multiplexed streams in Table 2 is shown in Table 3.

Table 2 – Example H.248 command request for multiplexed streams

| H.248 encoding | Comments |
| --- | --- |
| MEGACO/3 [123.123.123.4]:55555 Transaction = 10004 { Context = 1 { Add = T1 { Media { TermationState { mgroup/groupse = [“BUNDLE 3 4 5”]}, Stream = 1 { Local { v=0 c=IN IP4 $ m=audio $ RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 32 } } Stream = 2 { Local { v=0 c=IN IP4 $ m=audio $ RTP/AVP 8 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30002 RTP/AVP 8 } } Stream = 3 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30004 RTP/AVP 32 } } Stream = 4 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 33 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30004 RTP/AVP 33 } } Stream = 5 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 34 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30004 RTP/AVP 34 } } } } } } | Termination (T1) is added with the two audio streams and three video streams. The “BUNDLE” grouping semantic is used to indicate that Streams 3, 4 and 5 are multiplexed together. |

Table 3 – Example reply to a H.248 command request for multiplexed streams

| H.248 encoding | Comments |
| --- | --- |
| MEGACO/3 [124.124.124.222]:55555 Reply = 10004 { Context = 1 { Add=T1 { Media { Stream = 1 { Local { v=0 c=IN IP4 200.200.200.200 m=audio 50000 RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 32 } } Stream = 2 { Local { v=0 c=IN IP4 200.200.200.200 m=audio 50002 RTP/AVP 8 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30002 RTP/AVP 8 } } Stream = 3 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50004 RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30004 RTP/AVP 32 } } Stream = 4 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50004 RTP/AVP 33 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30004 RTP/AVP 33 } } Stream = 5 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50004 RTP/AVP 34 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30004 RTP/AVP 34 } } } } } } |  |

## 9.2 Audio and Video Stream in separate multiplexes

In this example video streams are multiplexed over a transport connection and the audio streams are multiplexed over a separate transport connection on Termination (T1) using the “BUNDLE” grouping semantic. The scenario is illustrated in figure 4.

Figure 4 – Example: Video Stream Multiplexing connection model

Tables 4 and 5 provide an example H.248 command request and response establishing the connection illustrated in Figure 4. The MG reply is described in Table 5.

Table 4 – Example H.248 command request for multiple multiplexed streams

| H.248 encoding | Comments |
| --- | --- |
| MEGACO/3 [123.123.123.4]:55555 Transaction = 10004 { Context = 1 { Add = T1 { Media { TermationState { mgroup/groupse = [“BUNDLE 1 2”, “BUNDLE 3 4 5”]}, Stream = 1 { Local { v=0 c=IN IP4 $ m=audio $ RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 32 } } Stream = 2 { Local { v=0 c=IN IP4 $ m=audio $ RTP/AVP 8 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 8 } } Stream = 3 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 32 } } Stream = 4 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 33 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 33 } } Stream = 5 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 34 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 34 } } } } } } | Termination (T1) is added with the two audio streams and three video streams. The “BUNDLE” grouping semantic is used to indicate that Streams 3, 4 and 5 are multiplexed together and that Streams 2 and 3 are multiplexed together. |

Table 5 – Example reply to a H.248 command request for multiple multiplexed streams

| H.248 encoding | Comments |
| --- | --- |
| MEGACO/3 [124.124.124.222]:55555 Reply = 10004 { Context = 1 { Add=T1{ Media { Stream = 1 { Local { v=0 c=IN IP4 200.200.200.200 m=audio 50000 RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 32 } } Stream = 2 { Local { v=0 c=IN IP4 200.200.200.200 m=audio 50000 RTP/AVP 8 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 8 } } Stream = 3 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50002 RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 32 } } Stream = 4 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50002 RTP/AVP 33 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 33 } } Stream = 5 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50002 RTP/AVP 34 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 34 } } } } } } |  |

[End Proposed Correction]

____________________