| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4686 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.STGROUP: consistency check of terminology (clauses 3 and 6) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides some update proposals with regards to ongoing terminology discussions.

NOTE to Q.3 agenda: AVD-4684 (H.248.RTPMUX) needs to be discussed first before this contribution.

1	Items of discussion and conclusions

Item #1 – "consistency about notion of stream semantics":

We are using so far "stream semantics", "grouping semantics", "stream grouping semantics" and "grouping stream semantics", caused by the fact that term "(RFC 5888) stream semantics" was extended to "(H.248) stream grouping" (via "media grouping").

We should be consistent and decide which term should be used. We are tending to "stream grouping semantics" concerning H.248 aspects, which would affect clause 3.2. Any text about SDP aspects would still be not affected and refer to "(RFC 5888) stream semantics" (clause 3.1.2)

Item #2 – "consistency about multiplexing":

The definition for term multiplexing is contained in H.Sup13.

Item #3  – "RTP stream":

AVD-4684 (H.248.RTPMUX) provides a definition proposal, derived from the latest draft [IETF GRPTAX]. This is a RTP-specific stream definition. We need furthermore a definition of generic RFC 5888 stream due to the fact that RFC 5888 is not explicitly qualifiying the semantic of "media stream". However, a RFC 5888 media stream could be implicitly tight to the "a=mid"-based identifier because such an identifier relates to a specific subset of the overall SDP media description.

Correspondent definition updates/proposals are provided, as well as a new convention on the technology specific (media) stream interrelationships.

Item #4  – Clause 6.2 versus [IETF GRPTAX] processing model:

We are not sure whether clause 6.2 is entirely consistent with [IETF GRPTAX], see also Figure A. It might be beneficial to differentiate between RTP source, RTP stream and RTP media transport.

Figure A – Functional "media" processing model of [IETF GRPTAX] – Sender side

2	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4680, the input draft for this meeting.

NOTE – Some changes are complemented by electronic comments in order to provide more background information or editorial hints. It's at the discretion of the meeting and/or editor to handle these comments (deletion or conversion to editor note).

Change proposal #1:

[Begin Proposed Correction]

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.x1	media (clause 2.1/[IETF GRPTAX]|[b-ITU-T H.248.RTPMUX]): a sequence of synthetic or physical Stimuli (sound waves, photons, key-strokes), represented in digital form. Synthesized media is typically generated directly in the digital domain.

3.1.1		(H.248 media) stream (clause 3.2.9/[ITU-T H.248.1]): Bidirectional media or control flow received/sent by a media gateway as part of a call or conference.

3.1.2		Stream Semantics [IETF RFC 5888]: the purpose for which the media streams are grouped.

NOTE – The notion of media stream in the definition is RFC-context specific, see clause 3.2.x1.

3.1.x2	RTP stream (clause 2.1.10/[IETF GRPTAX]|[b-ITU-T H.248.RTPMUX]): A stream of RTP packets containing media data, source or redundant. The RTP stream is identified by an SSRC belonging to a particular RTP session.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Aggregation: The MG internal process of combining two or more component streams on entering the H.248 context in the MG into one single stream, called the aggregation stream. The way the component streams are combined is dependent on the grouping stream semantics defined for the component streams. It may consist of selecting one of the component streams, applying a reparation procedure to one of the component streams based on the information from another component stream, etc. In the reverse direction it may consist on delivering media to only one of them at a time, providing a replica, generating repairing or redundancy information, etc.

3.2.2	Aggregation stream: a stream that sources media to two or more component streams and/ or sinks media from two or more component streams which are related to each other with a certain stream grouping semantics. The aggregation stream exists only in the interior of the context. Within one context a stream may be defined as an aggregation stream in one termination and as a normal stream in another termination.

3.2.3	Component stream: A stream that acts as a source or sink to an aggregation or de-aggregation stream. Also, each of the streams defined as belonging to a semantic group.

3.2.4	De-aggregation: The MG internal process of splitting a stream (called the de-aggregation stream) on entering the H.248 context in the MG into multiple component streams. The way component streams are generated from a aggregation stream is dependent of the stream grouping semantics. It may consist on delivering media to only one of them at a time, etc. In the reverse direction it may consist of sending multiple component streams over the same de-aggregation stream.

3.2.5	De-aggregation stream: a stream acting as a media sink to two or more component streams and/ or sources media from two or more component streams that are related to each other with a certain stream grouping semantics. The component streams exist on the interior of the context whereas a de-aggregation stream represents an external stream. Within one context, a stream may be defined as a component stream in one termination and as a normal stream in another termination.

3.2.6	Group (or Stream Group): Streams that share the same stream grouping semantic are part of the same group. A stream may be part of several groups. Whether the streams in a group are aggregated or not depends on the stream grouping semantic. Primarily [IETF RFC 5888] is utilised to group the streams and to provide the semantics.

3.2.x1	(RFC 5888 media) stream: the traffic description given by a particular SDP media description, which is unambiguously identifiable withing the overall SDP session description. The SDP media stream identification attribute ("a=mid:") provides media stream identifier role at protocol level.

3.2.7	Multiplexed stream: Multiple media streams are combined into a lower layer protocol.

[End Proposed Correction]

Change proposal #5:

[Begin Proposed Correction]

# 5	Conventions

## 5.1	Terminology relationships related to grouping

This Recommendation uses the terms “Grouping”, “Aggregation”, “De-aggregation” and “Multiplexing”. As per the definitions:

- Grouping describes the process using H.248 to group streams together that share a common stream semantic.

- Aggregation describes the process of using H.248 to indicate that the media of several streams is aggregated in a single stream.

- De-aggregation describes the process of using H.248 to indicate that the media of a single stream is deaggregated in multiple streams.

- Multiplexing is the process used to carry multiple streams over the same lower layer protocol (see also clause 3.1/[b-ITU-T H.Sup13]).

## 5.2	Terminology relationships related to the semantic of "stream"

The notions of "media stream" and "stream" are synonym in context of this Recommendation because the original and RTP specific meaning of media (see clause 3.1.x1) is used here in the general sense (e.g., IP application data information).

There are usually 1:1 mappings between

- RTP traffic: a RTP stream and a H.248 stream, and

- general: a RFC 5888 stream and a H.248 stream,

especially in case of H.248 stream grouping models with its purpose in limiting the scope of an H.248 component stream on a single RTP stream or RFC 5888 stream.

However, the H.248 protocol architectures supports in principle also N:1 mapping models for above stream types.

[End Proposed Correction]

Change proposal #4:

[Begin Proposed Correction]

## 6.2	Stream multiplexing using transport address information

Typically, a single media stream is assigned to a single H.248 Stream endpoint by specifying a unique address/port combination in the Local Descriptor per H.248 Stream on a Termination along with the associated media properties. In general, to indicate the use of a multiplexed media stream where multiple streams are sent using the same transport address, the MGC requests the same address/port in the Local Descriptor for all H.248 Streams that are to make up the multiplexed media stream.

As such when the same transport address/port is used for several streams there is an implicit grouping semantic which effectively multiplexes several streams over the same transport.

For text-based H.248, SDP is used for each Local Descriptor. In order to request to the same address/port the following conditions must be must:

- The value of the c-line must be the same for each Local Descriptor

- The value of m-line [IETF RFC 4566] <port> field is the same for each Local Descriptor.

A MGC may fully specify the address/port information for multiple H.248 Streams in a single Termination in an Add.req command. However in many cases the MGC will wildcard “choose” this information, in order for the MG to provide the address/port. In the case where wildcarding is used the MGC can:

- request the address/port information for one of the H.248 Streams involved in the multiplex and then use the same address/port information in subsequent Mod.req/s for additional H.248 Streams, or

- request address/port information for all the H.248 Streams involved in the multiplex and then select the address/port from one of the H.248 Streams and then send a subsequent Modify.request to modify the address/port information. This has the disadvantage over option a) that some resources may have been allocated to the previous address/port and these allocations would have to be changed.

The scope of above multiplexing scheme affects the local connection endpoint (see clause 5.1/[ITU-T H.248.1]) only (due to H.248 Local Descriptor scope).

{Contributor's note: following changes are based on the understanding that the used term "RTP stream" should be rather replaced "(RTP) media transport" according the taxonomy draft, because each RTP stream got his own SSCR and we got here two RTP sources due to different media types audio and video, each with an own SSRC. The changes are dependent on H.248.RTPMUX agreements (AVD-4684).}

The use case of multiplexing multiple RTP streams on a single IP transport address shall be demonstrated at a dual-media call based on audio and video components. There are then two media type specific RTP sources (with separate SSRC value assignments) at sender side.  The two RTP sources generate two RTP streams which share a single (RTP) media transport. Figure 1 shows example syntax for a Termination (T1) utilising the two RTP media transports and Termination (T2) utilising one RTP media transport  for the two RTP streams and their two H.248 Streams (due to the applied 1:1 mapping between RTP streams and H.248 streams here).

| Two H.248 Streams, two RTP Streams, single RTP media transport | Two H.248 Streams, two RTP Streams, two RTP media transports |
| --- | --- |
| MEGACO/3 [123.123.123.4]:55555 Transaction = 10003 { Context = 1 { Add = T1 { Media { Stream = 1 { Local { v=0 c=IN IP4 124.124.124.222 m=audio 2222 RTP/AVP 4 } } Stream = 2 { Local { v=0 c=IN IP4 124.124.124.222 m=video 2223 RTP/AVP 34 } } } } } } | MEGACO/3 [123.123.123.4]:55555 Transaction = 10004 { Context = 1 { Add = T2 { Media { Stream = 1 { Local { v=0 c=IN IP4 124.124.124.222 m=audio 2222 RTP/AVP 4 } } Stream = 2 { Local { v=0 c=IN IP4 124.124.124.222 m=video 2222 RTP/AVP 34 } } } } } } |

Figure 1 – Example single and multiplexed RTP media transport

It has to be noted that above example only illustrates RTP media multiplexing (see [IETF RTPMUX]) because there are not any explicit syntactical elements indicated related to RTCP port allocation. Leading to the assumption that additional RTP transport multiplexing is not used here due to default behaviour according to [b-ITU-T H.248.57].

On reception of an Add, Modify or Move request command with several Local Descriptors from the same Termination containing the same address/port information the MG shall treat the associated H.248 Streams as a multiplexed media stream. Multiplexing is applied according to the type or RTP profile of the stream (in case of RTP based services). For example, in the case that a RTP profile RTP/AVP is given RTP multiplexing is applied. Each media stream component of the overall multiplexed RTP traffic is coded according to the information Local Descriptor / H.248 Stream for that particular media.

RTP multiplexing as such (with its three variants) is described by [IETF RTPMUX].

This multiplexed media stream may contain more than one media type.

Multiple multiplexed media streams are also possible on a Termination.

Individual components of the media multiplex can be modified by changing the Local and/or Remote Descriptors of the related H.248 Stream. If the address/port information of a H.248 Stream is changed to be different from the other components of the multiplexed media stream then the media defined by the H.248 Stream will longer be part of the multiplex and it will be its own media stream.

Clause 8.6.3 describes several grouping semantics where the use of transport address for multiplexing may be used.

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

Bibliography

[b-ITU-T H.248.RTPMUX]	Recommendation ITU-T H.248.RTPMUX (##/2016), Gateway control protocol: H.248 support for RTP multiplexing.

[b-ITU-T H.Sup.13]	Supplement ITU-T H.Sup.13 (10/2015), Gateway Control Protocol: ITU-T H.248.x-series - Supplement on common H.248 terminology – Release 2.

{Editor’s note: this reference is not yet confirmed, dependent whether one or multiple common RTP related  terms would be moved from H.248.RTPMUX to H.Sup13).

[b-IANA group semantics]	IANA registry for "Semantics for the "group" SDP Attribute".
URL: http://www.iana.org/assignments/sdp-parameters/sdp-parameters.xhtml#sdp-parameters-13

[b-IETF RFC 4091]	IETF RFC 4091 (07/2005), The Alternative Network Address Types (ANAT) Semantics for the Session Description Protocol (SDP) Grouping Framework.

[b-IETF CLUESIG]	IETF draft-ietf-clue-signaling (xx/xxxx), CLUE Signaling.

[b-IETF FLUTE]		IETF draft-ietf-rmt-flute-sdp-03 (09/2012), SDP Descriptors for FLUTE.

[End Proposed Correction]

____________________

In more detail: the two RTP sources are physically assigned to the IP local source connection endpoint, but the multiplexing scheme affects essentially the IP local destination connection endpoint where the correspondent RTP sinks would be located. However, an SSRC identifies only the RTP source side.
So what's the actual multiplexing effect from H.248 MG perspective? Answer: the MG will receive two RTP streams, which share a single IP local destination transport address. The two received RTP streams are separated and assigned to its RTP sink functions based on individual SSRC values, which are used by the correspondent remote RTP source entities.

Assumption of address symmetry between local source and local destination endpoint.