| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4688 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.STGROUP: clause L.2 – discussion and update proposal for "use case #2: WebRTC" |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides an update proposal for clause L.2 related to "use case #2 "WebRTC".

1	Discussion

The only new updates are related to the previously defined stream grouping semantic at SCTP association level (clause 11.1/H.248.SCTP).

More important: there wasn't yet any comprehensive technical discussion about this use case at the last 2015-02 meeting. Thus, there are still some pending questions for clarification arround this use case.

Technical feedback would be appreciated.

2	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4680, the input draft for this meeting.

NOTE – Some changes are complemented by electronic comments in order to provide more background information or editorial hints. It's at the discretion of the meeting and/or editor to handle these comments (deletion or conversion to editor note).

Change proposal #1:

[Begin Proposed Correction]

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

## L.2	Use case #2: WebRTC

### L.2.1	Use case #2.1: WebRTC with RTP media multiplexing and separated H.248 streams for WebRTC DTLS session and underyling IP transport connection

The user plane protocol stack for WebRTC (see also clause 6.3/[b-ITU-T H.248.WEBRTC]) represents a hierarchical layered protocol architecture with multiplexing options at various level. The general WebRTC protocol stack mandates the usage of stream grouping capabilities in order to model such an H.248 termination in H.248 WebRTC gateways. The first example (#2.1) tries to model the most complex scenario, constituted by a maximum degree of multiplexing as well as multiple, dynamic WebRTC data channels. The considered WebRTC communication service is characterized by

- usage of RTP / RTCP transport multiplexing;

- usage of RTP media multiplexing (aka RTP SSRC multiplexing);

- usage of a single IP transport connection (UDP or TCP) for all WebRTC application flow components;

- usage of a single DTLS connection for a) DTLS-SRTP based key management for SRTP and b) DTLS transport of SCTP protocol data units;
{Editor's note (2015-06): the single DTLS connection option seems to be confirmed from the latest IETF email discussions.}
 and

- multiple WebRTC data application instances.

The WebRTC protocol stack and its traffic flows could be mapped on a number of H.248 streams (see Figure L.4) using following stream grouping principles:

- shared use of an (RTP)-connection for bundling audio and video;

- shared use of an (SCTP)-association for WebRTC data channels; and

- protocol stack segment realizing the tunnel endpoint (DTLS/L4).

{Editor's note (2015-02): there is an ongoing discussion about the set of entities which constitute actually the "tunnel endpoint" (TEP). There might be three meanigful solutions:

S1: TEPS1 = "DTLS protocol" only,
justified by the fact that the tunnel layer start above the native stack (L.e., "L4/IP/…") and ends before the served user instance of the tunnel, which would be the SCTP (because SCTP provides the multiplexing option for DCs based on the SCTP stream concept; or

S2: TEPS2 = "DTLS/L4 protocol stack segment",
due to the native coupling of the transport level security protocol DTLS with the transport layer; or

S3: TEPS3 = "(SCTP)-association/(DTLS)-session/(L4)-connection" (using a X.200 type of convention in denoting layer-dependent "protocol objects"),  justified by the fact that the X.200 convention allows to model the SCTP (layer) in the sub-entities of (SCTP)-association-endpoint and (SCTP)-stream-endpoint objects, which could be mapped on separated H.248 SEPs. Hence we would get a more fine granular object model for the "WEBRTC TEP" as located at H.248 WEBRTC gateway side.

Discussion is ongoing. The present text denotes the tunnel according to solution S2.}

The first two grouping principles are rather straighforward because a single protocol (RTP, SCTP) is used for multiple, separated application traffic flows. And the actual application flows are mapped on individual H.248 SEPs and the protocol layer used for aggregation gets a separate (but optional) SEP assignment.

The 3rd list item isn't straightforward because conditional, primarily dependent on NAT traversal aspects (see clause L.5 on "SCTP model").

Figure L.4 – Use case #2.1: WebRTC with RTP media multiplexing and separated H.248 streams for WebRTC DTLS session and underyling IP transport connection 
– Context stream grouping model

Figure L.4 higlights already the usage of the three mgroup properties. There are three SEPs which serve stream deaggregation (S8, S9 and S10) and one SEP for stream aggregation (S3).

Table L.2.1 illustrate example H.248 signalling with regards to the initial creation of the context and all four stream endpoints:

Table L.2.1 – Example H.248 media descriptor

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| ; Termination T1 Context = C1 { ; Context C1 Add = ip/$/$/$ { ; Termination T1 Media{ TerminationState { mgroup/groupse = ["BUNDLE 1 2", "SCTP 4 5 6 7", "SCTP-TRANSPORT 8", "DTLS-TRANSPORT 9"] ; Semantics "SCTP-TRANSPORT", ; "DTLS-TRANSPORT are not yet specified; }, Stream = 1 { ; Audio/SRTP LocalControl { rtcph/rsb = On, ; transport multiplexed RCTP Mode = SendReceive, ReservedValue = On, ... }, Local { v=0 c=IN IP6 $ m=audio $ UDP/TLS/RTP/SAVP 101 0 13 126 a=rtcp-mux a=rtpmap:101 opus/48000/2 a=rtpmap:0 PCMU/8000 a=rtpmap:13 CN/8000 a=rtpmap:126 telephone-event/8000 a=ssrc:$ cname:$ a=ice-ufrag:$ a=ice-pwd:$ a=fingerprint:$ $ }, Remote { v=0 c=IN IP6 <UE_IP_BUNDLE> m=audio <UE_port_BUNDLE> UDP/TLS/RTP/SAVP 101 0 13 126 a=rtcp-mux a=rtpmap:101 opus/48000/2 a=rtpmap:0 PCMU/8000 a=rtpmap:13 CN/8000 a=rtpmap:126 telephone-event/8000 a=ssrc:<UE_SSRC_audio> cname:<UE_CNAME> a=ice-ufrag:<UE_ICE_ufrag_BUNDLE> a=ice-pwd:<UE_ICE_pwd_BUNDLE> a=fingerprint: <UE_hash_func_BUNDLE> <UE_fingerprint_BUNDLE> } }, Stream = 2 { ; Video/SRTP LocalControl { rtcph/rsb = On, ; transport multiplexed RCTP Mode = SendReceive, ReservedValue = On, ... }, Local { v=0 c=IN IP6 $ m=video $ UDP/TLS/RTP/SAVPF 96 97 a=rtcp-mux a=rtpmap:96 VP8/90000 a=rtpmap:97 H264/90000 a=ssrc:$ cname:$ a=ice-ufrag:$ a=ice-pwd:$ a=fingerprint:$ $ }, Remote { v=0 c=IN IP6 <UE_IP_BUNDLE> m=video <UE_port_BUNDLE> UDP/TLS/RTP/SAVPF 96 97 a=rtcp-mux a=rtpmap:96 VP8/90000 a=rtpmap:97 H264/90000 a=ssrc:<UE_SSRC_video> cname:<UE_CNAME> a=ice-ufrag:<UE_ICE_ufrag_BUNDLE> a=ice-pwd:<UE_ICE_pwd_BUNDLE> a=fingerprint: <UE_hash_func_BUNDLE> <UE_fingerprint_BUNDLE> } }, Stream = 3 { ; Ctxt internal aggregation stream ; for statistics LocalControl { Mode = SendReceive, ## ; Stream mode intentionally set to SendReceive ; in order to activate statistics counting. ; This was clarified in the H.248.1 IG. mgroup/stragg = [1,2] ... }, ; Assumption: Local and Remote descriptors ; are not needed, and hence are left ; unspecified. Statistics { ; traffic volume in octets rtpad/payloados, ; for audio & video at app- rtpad/payloador ; lication-level } }, Stream = 4 { ; Text/T.140/DC LocalControl { ; Mode = SendReceive, ... }, Local { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel ; How do we know that this SEP does not ; model L4 and DTLS connection endpoints? ; Should we just use "SCTP" as <proto> value? ; But this is already defined in ; draft-ietf-mmusic-sctp-sdp for the ; plain SCTP-over-IP case. ; Above m-line does not contain a UDP port, ; and the c-line also does not contain any ; L3 information. ; Also, there is no further DTLS related ; information, like fingerprint value. ## ; Potentially this could be used as indication ## ; that no L4 and DTLS connection endpoints are ; modeled by this SEP. ; How do we know that this SEP does also ; not model the SCTP association endpoint? ; Because it is part of the SEP list ; associated with the "WRTC-DC" semantic in ; T1's groupse value. ; The fmtp attribute is not present, as its ; only parameter is the "max-message-size" ; parameter. However, this is an SCTP ; association level parameter, which occurs ; just once in S8, which models the SCTP ; association endpoint. a=dcmap:0 subprotocol="text-t.140" ; Do we need a subprotocol value at all? ; Assumingly yes, if we want to use the ; "data channel" m-line above. ; In this case the subprotocol value ; would need to unambiguously define ; the exact sub-protocol and application ; protocol formats. Proposing "text-t.140". ; "text-t.140" is not defined so far. ; The dcmap attribute does not contain ; further data channel parameters. ; This especially implies ordered and ; reliable transmission, such that no ; other redundancy schemes are required. ; Similar to ; draft-ejzak-mmusic-msrp-usage-data-channel ; a dedicated "T.140-usage-data-channel" ; document would be needed. }, Remote { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel a=dcmap:0 subprotocol="text-t.140" } }, Stream = 5 { ; IM(text)/MSRP/DC LocalControl { Mode = SendReceive, ... }, Local { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel a=dcmap:1 subprotocol="msrp" a=dcsa:1 path:msrps://<MG_authority_webrtc>/\ <Remote_session_chat>;dc ; Assumption: MG operates in MSRP BL ALG mode, ; thus it does not have an own MSRP session id ; such that the Local's path value references ; the MSRP session id used by the remote far ; end MSRP session endpoint. ; For the same reason no a=dcsa attribute ; is present containing the ; accept-types attribute. }, Remote { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel a=dcmap:1 subprotocol="msrp" a=dcsa:1 path:msrps://<UE_authority>/\ <UE_session_chat>;dc } }, Stream = 6 { ; IM(file)/MSRP/DC LocalControl { Mode = SendReceive, ... }, Local { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel a=dcmap:2 subprotocol="msrp"; a=dcsa:2 path:msrps://<MG_authority_webrtc>/\ <Remote_session_file_transfer>;dc ; Similar comments as to T1.S5 }, Remote { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel a=dcmap:2 subprotocol="msrp"; a=dcsa:2 path:msrps://<UE_authority>/\ <UE_session_file_transfer>;dc } }, Stream = 7 { ; FloorCtrl/BFCP/DC LocalControl { Mode = SendReceive, ... }, Local { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel a=dcmap:3 subprotocol="bfcp"; ; TBC - Further BFCP related attributes? ; Sub-protocol "bfcp" is not defined yet. ; The dcmap attribute does not contain ; further data channel parameters. ; This especially implies ordered and ; reliable transmission. ; Similar to ; draft-mmusic-msrp-usage-data-channel ; a dedicated "bfcp-usage-data-channel" ; document would be needed. }, Remote { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel a=dcmap:3 subprotocol="bfcp"; ; TBC - Further BFCP related attributes? } }, Stream = 8 { ; SCTP association LocalControl { Mode = SendReceive, mgroup/strdeagg = [4,5,6,7] ; deaggregation ... }, Local { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel ; Similar comment as to T1.S4: ; How do we know that this SEP does not ; model L4 and DTLS connection endpoints? ; Should we just use "SCTP" as <proto> value? ; But this is already defined in ; draft-ietf-mmusic-sctp-sdp-08 for the ; plain SCTP-over-IP case. ; Above m-line does not contain a UDP port, ; and the c-line also does not contain any ; L3 information. ; Also, there is no further DTLS related ; information, like fingerprint value. ; Potentially this could be used as indication ; that no L4 and DTLS connection endpoints are ; modeled by this SEP. ; How do we know that this SEP does ; model the SCTP association endpoint? ; Because of its strdeagg value, which ; associates it to the SEP list in T1's ; groupse value, whose semantic is "WRTC-DC". a=fmtp:webrtc-datachannel \ max-message-size=$ a=sctp-port=$ ; Assumption: MGC lets MG choose local SCTP ; port and maximal message size. }, Remote { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel a=fmtp:webrtc-datachannel \ max-message-size=<UE_SCTP_max_message_size> a=sctp-port=<UE_SCTP_port> } }, Stream = 9 { ; DTLS connection LocalControl { Mode = SendReceive, tlsbsc/bceb="unblocked", mgroup/strdeagg = [8] ; Seems we cannot use the seplink v1 (!) linktopo ; property in order to request T1.S8 to ; trigger SCTP association establishment as ; soon as the DTLS session / connection is ; establshed. Reason: The linktopo property ; allows for vertical interlinkage of ; different protocol layers, but only within ; the same SEP. But here SCTP is handled by ; S8, whereas DTLS is handled by S9. ; Potential seplink extension: Allow the ; linktopo property to optionally trigger ; establishment or release procedures on ; another H.248 stream (only within same ; termination or also in other ; termination(s)?). => see proposal C-685 ... }, Local { v=0 c=- - - m=- – DTLS/SCTP – ; Media format list intentionally agnostic ; as this SEP does not need to know ; anything regarding SCTP usage. ; Similar comment as to T1.S4: ; How do we know that this SEP does not ; model the L4 connection endpoints? ; Should we just use "DTLS" as <proto> value? ; Would be an H.248 specific <proto> value. ; Above m-line does not contain an L4 port, ; and the c-line also does not contain any ; L3 information. ; Potentially this could be used as indication ; that no L4 connection endpoint is ; modeled by this SEP. ; How do we know that this SEP does ; model the DTLS session or connection ; endpoint? ; Because of its strdeagg value, which ; associates it to the SEP list in T1's ; groupse value, whose semantic is ; "SCTP-TRANSPORT", and because its <proto> ; value indicates "DTLS/..." and the ; fingerprint attribute is present and valid. ; How do we know that this SEP does not model ; an SCTP association endpoint? ; This could also be derived from the above ; mentioned strdeagg value. (But just using ; "DTLS" as m-line <proto> value might make ; this clearer.) ; Media type is intentionally agnostic as this ; SEP does not need to know anything related ; to upper layer application. a=fingerprint:$ $ ; TBC – Do we need "setup"? If yes, is ; "actpass" the correct value (in order to ; indicate DTLS endpoint mode, see H.248.84)? ; H.248.93 re-uses .90's tlsbsc package, which ; contains the bcep property and the ; EstBNC signal for (D)TLS role specific ; signaling and processing ("package-full ; approach"). }, Remote { v=0 c=- - - m=- – DTLS/SCTP – a=fingerprint:<UE_hash_func_data> <UE_fingerprint_data> } }, Stream = 10 { ; L4 connection: UDP or TCP LocalControl { ipdc/realm = <webrtc_realm>, Mode = SendReceive, mgroup/strdeagg = [9] ... }, Local { v=0 c=IN IP6 $ m=- $ udp – ; Media type and media format agnostic, as ; this SEP does not need to know anything ; related to UDP payload formats. ; <proto> value "udp" as per RFC 4566. ; Assumption: ICE might also conclude with ; a TCP candicate pair, which would result ; in an SCTP/DTLS/RFC 4571/TCP/IP ; transport protocol stack. a=ice-ufrag:$ a=ice-pwd:$ }, Remote { v=0 c=IN IP6 <UE_IP_data> m=- <UE_port_data> udp – a=ice-ufrag:<UE_ICE_ufrag_data> a=ice-pwd:<UE_ICE_pwd_data> } } } }} | General comments: SDP "a=setup:" attribute: This SDP attribute could be used for multiple self-contained protocol layers (here TCP, DTLS and SCTP) from IETF side, e.g., in call control signalling. This is not possible here in case of an hierarchical layered protocol architecture due to the above mentioned semantical overloading of this SDP attribute. Therefore, the control of the bearer control procedure for establishment of a a) (TCP)-connection, b) (DTLS)-session/-connection, and c) (SCTP)-association is subject of other signalling elements (H.248 event/signal based control, or ICE-related SDP in case of TCP). Stream-specific comments: Stream 1: - audio with codec options OPUS and G.711 µ-law; - RTCP transport multiplexed (according to the rules of clause [b-ITU-T H.248.57]; Stream 2: - video with codec options VP8 and H.264; - RTCP transport multiplexed (according to the rules of clause [b-ITU-T H.248.57]; Stream 3: - represents an context-internal aggregation stream (for all RTP traffic); - allows a simple configuration related to the performance monitoring of the entire RTP traffic for audio and video; Stream 4: - WebRTC data channel with sub-protocol 'T.140' for WebRTC application flow "text conversation"; Stream 5: - WebRTC data channel with sub-protocol 'MSRP' for WebRTC application flow "instant messaging (text-based data)"; Stream 6: - WebRTC data channel with sub-protocol 'MSRP' for WebRTC application flow "instant messaging (file-based data)"; Stream 7: - WebRTC data channel with sub-protocol 'BFCP' for WebRTC application flow "conference/floor control)"; Stream 8: - represents an context-external aggregation stream (for all SCTP traffic); - the SEP represents the actual H.248 (SCTP)-bearer-connection-endpoint, i.e., handles all real SCTP packets with the MG-external network interface; and "forwards" the SCPT traffic to the SCTP-Stream-dependent MG-internal IWFs (as configured by H.248 streams S4 to S7); - thus, the usual protocol configuration of the (SCTP)-connection-endpoint is subject of this SEP; Stream 9: - SEP realizes the H.248 (DTLS)-bearer-connection-endpoint; - this SEP is used for a) DTLS-SRTP key exchange and b) DTLS application traffic 'SCTP'; - thus, represents an context-external aggregation stream (for all SCTP traffic); Stream 10: - SEP realizes the H.248 (L4)-bearer-connection-endpoint; |
| ; Termination T2 Context = C1 { ; Context C1 Add = ip/$/$/$ { ; Termination T2 Media{ TerminationState { ... }, Stream = 1 { ; Audio/RTP LocalControl { ipdc/realm = <core_realm>, Mode = SendReceive, ... }, Local { v=0 c=IN IP6 $ m=audio $ RTP/AVP 101 0 13 126 a=rtpmap:101 opus/48000/2 a=rtpmap:0 PCMU/8000 a=rtpmap:13 CN/8000 a=rtpmap:126 telephone-event/8000 }, Remote { v=0 c=IN IP6 <Remote_IP_audio> m=audio <Remote_port_audio> RTP/AVP 101 0 13 126 a=rtpmap:101 opus/48000/2 a=rtpmap:0 PCMU/8000 a=rtpmap:13 CN/8000 a=rtpmap:126 telephone-event/8000 } }, Stream = 2 { ; Video/RTP LocalControl { ipdc/realm = <core_realm>, Mode = SendReceive, ... }, Local { v=0 c=IN IP6 $ m=audio $ RTP/AVPF 96 97 a=rtpmap:96 VP8/90000 a=rtpmap:97 H264/90000 }, Remote { v=0 c=IN IP6 <Remote_IP_video> m=audio <Remote_port_video> RTP/AVPF 96 97 a=rtpmap:96 VP8/90000 a=rtpmap:97 H264/90000 } }, Stream = 4 { ; Text/T.140/RTP LocalControl { ipdc/realm = <core_realm>, Mode = SendReceive, ... }, Local { v=0 c=IN IP6 $ m=text $ RTP/AVP 94 95 a=rtpmap:95 t140/1000/1 a=rtpmap:94 red/1000/1 a=fmtp:94 95/95/95 }, Remote { v=0 c=IN IP6 <Remote_IP_text> m=text <Remote_port_text> RTP/AVP 94 95 a=rtpmap:95 t140/1000/1 a=rtpmap:94 red/1000/1 a=fmtp:94 95/95/95 } }, Stream = 5 { ; IM(text)/MSRP/TCP LocalControl { ipdc/realm = <core_realm>, Mode = SendReceive, ... }, Local { v=0 c=IN IP6 $ m=text $ TCP/MSRP – a=path:msrp://<MG_authority_core>/\ <UE_session_chat>;tcp a=setup:actpass ; Similar comments as to T1.S5 }, Remote { v=0 c=IN IP6 <Remote_IP_chat> m=text <Remote_port_chat> TCP/MSRP – a=path:msrp://<Remote_authority>/\ <Remote_session_chat>;tcp } }, Stream = 6 { ; IM(file)/MSRP/TCP LocalControl { ipdc/realm = <core_realm>, Mode = SendReceive, ... }, Local { v=0 c=IN IP6 $ m=text $ TCP/MSRP – a=path:msrp://<MG_authority_core>/\ <UE_session_file_transfer>;tcp a=setup:actpass ; Similar comments as to T1.S5 }, Remote { v=0 c=IN IP6 <Remote_IP_chat> m=text <Remote_port_chat> TCP/MSRP – a=path:msrp://<Remote_authority>/\ <Remote_session_file_transfer>;tcp } }, Stream = 7 { ; FloorCtrl/BFCP/TCP LocalControl { ipdc/realm = <core_realm>, Mode = SendReceive, ... }, Local { v=0 c=IN IP6 $ m=application $ TCP/BFCP – ; TBC – further BFCP related attributes? ; a=floorctrl might not be needed as we want ; to establish an end-to-end BFCP session. ; "confid" or "userid" attributes required? }, Remote { v=0 c=IN IP6 <Remote_IP_floor_ctrl> m=application <Remote_port_floor_ctrl> \ TCP/BFCP – ; TBC – further BFCP related attributes? } } } }} |  |
| NOTE 1 – |  |

{Editor's note (2015-02): above signalling example is just a first draft and needs further work because dependent on parallel discussions related to
1) stream deaggregation (C-698),
2) codepoint for parameter Semantics (C-682),
3) seplink enhancements (C-685) and
4) "IETF SDP drafts".
Furthermore, the example is expected to be extended in order to illustrate as well the bearer control procedures related to
5) DTLS establishment, 
6) SCTP establishment and
7) SCTP StreamID handling in more detail.

Furthermore, the creation of the ten SEPs in T1 might be mapped on multiple H.248 command request/reply cycles in order to illustrate signalling dynamics (see H.248.SCTP). For instance:
Step 1: creation of S1 to S3, S9 and S10 ("the WebRTC communication is initially starting with audio and video, DTLS session is immediately established in order to execute DTLS-SRTP");
Step 2: creation of S8 ("additional WebRTC data components are expected for future use, thus the SCTP association is established despite the fact of still lacking concrete WebRTC data channel open requests");
Step 3: creation and removal of S4 to S7 ("dependent on dynamic usage of WebRTC DCs").

Furthermore, the example could be extended to illustrate a L4 protocol switchover from UDP to TCP (or vice versa) during the active communication phase ("which results in an H.248 MODify.request for S10").

Discussion are ongoing.}

Some observations:

- IP address realm indication is mandatory for H.248 WebRTC gateways. The isolation of the (L4)-connection-endpoint (UDP or TCP here) inclusive the underlying IP (IPv4 or IPv6) in a self-contained SEP (here S10) allows an unambiguous IP realm indication (via ipdc/realm property), i.e., avoids redundancy by repeating that information in all SEPs "at upper layer protocols".

- Protocol replacement within a particular SEP (here: L4 protocol switchover process from UDP to TCP or vice versa during active communication phase, driven by NAT traversal issues: the isolation of the (L4)-connection-endpoint in a self-contained SEP seems to be beneficial from H.248 signalling perspective ("by limiting the required signalling information on the parts only, which are actually affected");

- FIXTHIS

### L.2.2	Use case #2.2: WebRTC without RTP media multiplexing and a single SEP for the "SCTP/DTLS/L4/IP" stack segment

{Editor's note (2015-02): use case #2.1 illustrates the most granular model concerning the mapping on individual H.248 SEPs. Such a mapping is feasible (thanks to H.248.STGROUP) and beneficial for specific H.248 WebRTC gateway scenarios. However, there are "WebRTC domains" without the full-fledged usage of the WebRTC service suite, a simplified NAT traversal context, a data-less WebRTC service profile, etc etc. Such scenarios could lead to simplified stream grouping models. Such a use case is subject of this clause.}

FIXTHIS.

[End Proposed Correction]

____________________