| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4693 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.WEBRTC: clause 8 – update of requirements |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides update proposals in order to progress the requirements related section of this Draft.

1	Discussion

The main changes are related to following topics:

1.1	Usage of DTLS

We considered multiple models for DTLS usage with WebRTC in the meanwhile:

Model IV seems to be finally the applied protocol usage, i.e., a single DTLS connection shared by SCTP and SRTP key exchange. Whether the underlying DTLS session is of type resumable or not will be most likely not specified.

1.2	Protocol stack models in clause 6.3

Both transport models (UDP- and TCP-based) should be able to be combined in a stack like the following:

1.3	Requirements related to TCP-based media transport in clause 8.5

First input is proposed.

1.4	Cross-check of Appendix L.4

We reviewed again Table L.4.1. We are proposing new text on following two items:

- H.248 termination type indication:

| 10. draft-ietf-rtcweb-rtp-usage-17 “Web Real-Time Communication (WebRTC): Media Transport and Use of RTP” | This draft outlines requirements for the use of RTP in WEBRTC. | Much of the required information can be signalled between a MGC and MG via SDP however other requirements relate to specific handling of RTP that is typically not signalled by the MGC. A MG must know whether the H.248 Termination relates to a WebRTC client to ensure that all the requirements are implemented. This area requires further investigation. |
| --- | --- | --- |

=> new sub-clause 8.12

- NAT traversal and TURN support

| 14.3.2 RFC5766 “Traversal Using Relays around NAT (TURN): Relay Extensions to Session Traversal Utilities for NAT (STUN)” |  | TURN is supported by H.248.50. |
| --- | --- | --- |
| 14.3.3 RFC6156 “TURN Extension for IPv4/IPv6 Transition” |  | H.248.50 supports TURN however it currently does not specify the usage of RFC6156 which allows the use of the REQUESTED-ADDRESS-FAMILY attribute (e.g. clause 4.1/RFC6156). RTCWeb indicates this must be supported. H.248.50 should be updated to indicate that this may be used with a possible new codepoint in clause 7.3 for the MG TURN client package. |
| 14.3.4 RFC6062 “Traversal Using Relays around NAT (TURN) Extensions for TCP Allocations” | The RFC defines additional STUN methods and attributes to be used with TURN for TCP address allocations. | H.248.50 currently doesn’t make reference to this RFC. The support of RFC6062 should be considered by revised H.248.50. |

=> new text in sub-clause 8.1.2

2	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4663, the input draft for this meeting.

Change proposal #1:

[Begin Proposed Correction]

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.37]	Recommendation ITU-T H.248.37 (06/2008), Gateway control protocol: IP NAPT traversal package.

[ITU-T H.248.39]	Recommendation ITU-T H.248.39 (07/2014), Gateway control protocol: H.248 SDP parameter identification and wildcarding.

[ITU-T H.248.48]	Recommendation ITU-T H.248.48 (02/2012), Gateway control protocol: RTCP XR block reporting package.

[ITU-T H.248.50]	Recommendation ITU-T H.248.50 (09/2010), Gateway control protocol: NAT traversal toolkit packages.

{Editor’s note (2015-06): WebRTC requires the Revision of this Recommendation. The references should be therefore updated as soon as Revision consented/approved.}

[ITU-T H.248.52]	Recommendation ITU-T H.248.52 (06/2008), Gateway control protocol: QoS support packages.

[ITU-T H.248.53]	Recommendation ITU-T H.248.53 (03/2009), Gateway control protocol: Traffic management packages.

[ITU-T H.248.57]	Recommendation ITU-T H.248.57 (07/2014), Gateway control protocol: RTP control protocol package.

[ITU-T H.248.71]	Recommendation ITU-T H.248.71 (02/2010), Gateway control protocol: RTCP support packages.

[ITU-T H.248.77]	Recommendation ITU-T H.248.77 (09/2010), Gateway control protocol: Secure real-time transport protocol (SRTP) package and procedures.

{Editor’s note (2015-06): WebRTC requires the Revision of this Recommendation. The references should be therefore updated as soon as Revision consented/approved.}

[ITU-T H.248.78]	Recommendation ITU-T H.248.78 (04/2015), Gateway control protocol: Bearer-level backhauling and application level gateway.

[ITU-T H.248.87]	Recommendation ITU-T H.248.87 (01/2014), Gateway control protocol: Guidelines on the use of ITU-T H.248 capabilities for performance monitoring in RTP networks in ITU-T H.248 profiles.

[ITU-T H.248.88]	Recommendation ITU-T H.248.88 (01/2014), Gateway control protocol: RTP topology dependent RTCP handling by ITU-T H.248 media gateways with IP terminations.

[ITU-T H.248.92]	Recommendation ITU-T H.248.92 (07/2014), Gateway control protocol: Stream endpoint interlinkage package.

{Editor’s note (2015-06): WebRTC could benefit from the Revision of this Recommendation. The references should be therefore updated as soon as Revision consented/approved.}

[ITU-T H.248.93]	Recommendation ITU-T H.248.93 (07/2014), Gateway control protocol: H.248 support for control of transport security using DTLS.

[ITU-T H.248.RTPMUX]	Draft Recommendation ITU-T H.248.RTPMUX (XX/2014), Gateway control protocol: H.248 support for RTP multiplexing.

[ITU-T H.248.SCTP]	Draft Recommendation ITU-T H.248.SCTP (XX/2014), Gateway control protocol: H.248 support for control of SCTP bearer connections.

[ITU-T H.248.STGROUP]	Draft Recommendation ITU-T H.248.STGROUP (XX/2014), Gateway control protocol: H.248 Stream grouping and aggregation.

[ITU-T H.320]	Recommendation ITU-T H.320 (03/2004), Narrow-band visual telephone systems and terminal equipment.

[ITU-T H.324]	Recommendation ITU-T H.324 (04/2009), Terminal for low bit-rate multimedia communication.

[ITU-T T.140]	Recommendation ITU-T T.140 (02/1998), Protocol for multimedia application text conversation.

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.

[IETF RFC 4585]	IETF RFC 4585 (2006), Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF).

[IETF RFC 4961]	IETF RFC 4961 (2007), Symmetric RTP / RTP Control Protocol (RTCP).

[IETF RFC 5109]	IETF RFC 5109 (2007), RTP Payload Format for Generic Forward Error Correction.

[IETF RFC 5124]	IETF RFC 5124 (2008), Extended Secure RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/SAVPF).

[IETF RFC 5506]	IETF RFC 5506 (2009), Support for Reduced-Size Real-Time Transport Control Protocol (RTCP): Opportunities and Consequences.

[IETF RFC 5576]	IETF RFC 5576 (2009), Source-Specific Media Attributes in the Session Description Protocol (SDP).

[IETF RFC 5764]	IETF RFC 5764 (2010), Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP).

[IETF RFC 7007]	IETF RFC 7007 (2013), Update to Remove DVI4 from the Recommended Codecs for the RTP Profile for Audio and Video Conferences with Minimal Control (RTP/AVP).

[IETF rtcweb-overview]	IETF draft-ietf-rtcweb-overview (2015), Overview: Real Time Protocols for Brower-based Applications.

[IETF rtp-usage]	IETF draft-ietf-rtcweb-rtp-usage (2015), Web Real-Time Communication (WebRTC): Media Transport and Use of RTP.

[IETF rtcweb-secur]	IETF draft-ietf-rtcweb-security (2015), Security Considerations for WebRTC.

[IETF rtcweb- security-arch]	IETF draft-ietf-rtcweb-security-arch (2015), WebRTC Security Architecture.

[IETF rtcweb-transports]	IETF draft-ietf-rtcweb-transports (2015), Transports for RTCWEB.

[IETF xrblock-webrtc]	IETF draft-singh-xrblock-webrtc-additional-stats (201#), Additional RTP Control Protocol (RTCP) Extended Report (XR) Metrics for WebRTC Statistics API.

[IETF DCEP]	IETF draft-ietf-rtcweb-data-protocol-09 (xxxx), WebRTC Data Channel Establishment Protocol.

[IETF WebRTCDC] 	IETF draft-ietf-rtcweb-data-channel-12 (xxxx), WebRTC Data Channels.

[IETF DCSDP]	IETF draft-ietf-mmusic-data-channel-sdpneg (2015), SDP-based "SCTP over DTLS" data channel negotiation.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation refers to following document related to all used terms in context of the DTLS protocol: [b-IETF tls-terms].

This Recommendation uses the following terms defined elsewhere:

3.1.1	media path [IETF rtcweb-overview]: The path that media data follows from one WebRTC endpoint to another.

NOTE – Relates to the bearer plane interface (at MG) in case of WEBRTC H.248 gateway.

3.1.2	signalling path [IETF rtcweb-overview]: The communication channels used between entities participating in signaling to transfer signaling. There may be more entities in the signaling path than in the media path.

NOTE – Relates to the signalling plane call control interface (at MGC) in case of a WEBRTC H.248 gateway.

3.1.3	web browser [ITU-T H.351]: A software application capable of rendering HTML and XHTML documents.

3.1.4	WebRTC browser [IETF rtcweb-overview]: (also called a WebRTC User Agent or WebRTC UA) is something that conforms to both the protocol specification and the Javascript API defined 'above' (NOTE).

NOTE – See [IETF rtcweb-overview] concerning the referred to "Javascript API specification".

3.1.5	WebRTC endpoint [IETF rtcweb-overview]: is either a WebRTC browser or a WebRTC non-browser.  It conforms to the protocol specification.

3.1.6	WebRTC-compatible endpoint [IETF rtcweb-overview]: is an endpoint (NOTE) that is able to successfully communicate with a WebRTC endpoint, but may fail to meet some requirements of a WebRTC endpoint.  This may limit where in the network such an endpoint can be attached, or may limit the security guarantees that it offers to others.  It is not constrained by this specification; when it is mentioned at all, it is to note the implications on WebRTC-compatible endpoints of the requirements placed on WebRTC endpoints.

NOTE – The self-contained notion of 'endpoint' is consistent with [b-ITU-T H.Sup.13].

3.1.7	WebRTC gateway [IETF rtcweb-overview]: is a WebRTC-compatible endpoint that mediates media traffic to non-WebRTC entities.

3.1.8	WebRTC non-browser [IETF rtcweb-overview]: is something that conforms to the protocol specification, but does not claim to implement the Javascript API. This can also be called a "WebRTC device" or "WebRTC native application".

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	WEBRTC H.248 gateway: a decomposed gateway according H.248 (i.e., a MGC-MG tandem), using an H.248 profile with support of web-based real-time communication services.

NOTE 1 – The WEBRTC H.248 gateway could operate in WebRTC gateways mode and in WebRTC endpoint mode.

NOTE 2 – The particular (H.248) gateway type may be further qualified, - based on the application specific profile -, such as WEBRTC PSTN gateway, WEBRTC border gateway, WEBRTC IMS access gateway, etc.

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| API | Application Programming Interface |
| --- | --- |
| B2BTE | Back-to-Back TCP Endpoint |
| BFCP | Binary Floor Control Protocol |
| DTLS | Datagram Transport Layer Security |
| HTML |  |
| HTTP | Hyper-text transfer protocol |
| ICE | Interactive Connectivity Establishment |
| IMS | IP multimedia subsystem |
| IP | Internet protocol |
| IPv4 | Internet protocol version 4 |
| IPv6 | Internet protocol version 6 |
| LD | Local descriptor |
| MG | Media gateway |
| MGC | Media gateway controller |
| MSRP | Message Session Relay Protocol |
| MTI | Mandatory To Implement |
| NACK | (RTP/RTCP) Negative Acknowledgements |
| NAT | Network address translation |
| NAT-T | NAT Traversal |
| NGN | Next generation network |
| PPID | (SCTP) Payload Protocol Identifier |
| PSFB | (RTCP) Payload-Specific Feedback |
| PSTN | Public Switched Telephone Network |
| RD | Remote descriptor |
| RMCAT | RTP Media Congestion Avoidance Techniques (IETF WG) |
| RR | (RTCP) Receiver Report |
| RTCP | RTP Control Protocol |
| RTCWEB | Real-Time Communication in WEB-browsers (as work item in IETF) |
| RTP | Real-time Transport Protocol |
| RTPFB | (RTCP) ### |
| SCTP | Stream Control Transmission Protocol |
| SDES | (RTCP) #### |
| SDP | Session description protocol |
| SEP | (H.248) Stream endpoint |
| SEPP | (H.248) Stream endpoint pair |
| SIP | Session initiation protocol |
| SR | (RTCP) Sender Report |
| SSRC | Synchronization Source |
| SRTP | Secure RTP |
| STUN | Session Traversal Utilities for NAT |
| TCP | Transmission control protocol |
| TLS | Transport Layer Security |
| TURN | Traversal Using Relays around NAT |
| UDP | User datagram protocol |
| VP9 | (abbreviation unknown) |
| W3C | World Wide Web Consortium (http://www.w3.org/) |
| WEB | "WWW” (World Wide Web) |
| WEBRTC | Web Real-Time Communication = Real-Time Communication in WEB-browsers (as work item in IETF) |
| XHTML |  |
| XR | (RTCP) Extension Report |

[End Proposed Correction]

Change proposal #4:

[Begin Proposed Correction]

# 6	WEBRTC core technology overview

## 6.1	WEBRTC gateways

The WEBRTC communication model considers usage of WEBRTC gateways besides the native browser-to-browser scenarios. [b-IETF rtcweb-gateway] describes such gateways in general, whereas the specific instance of an H.248 WEBRTC gateway is in scope of this Recommendation.

## 6.2	Capabilities

The H.248 WEBRTC gateway is a peer to WEBRTC endpoints, given by IP host entities with "WEBRTC capable web browsers" as the application instance. WEBRTC endpoints are characterized by following mandatory (and optional) capabilities (according to the "RTCWEB protocol specification" [IETF rtcweb-overview]):

- WEBRTC as multimedia conversational communication service with application components audio, video and data (which covers again a bunch of "data services");

- Application data transport (see [IETF rtcweb-transports]):

  - audio, video: RTP/RTCP in specific capability sets (see [IETF rtp-usage])

  - data: SCTP/DTLS in specific capability sets

- Application data framing and securing (see [IETF rtcweb-security-arch], [IETF rtcweb-secur]):

  - audio, video: SRTP

  - data: (SCTP over) DTLS

- Application data formats:

  - audio codec types (see [IETF rtcweb-audio]): Opus [b-IETF RFC 6716] and G.711 PCM µ-/A-law [b-ITU-T G.711]

  - video codec types: H.264 [b-ITU-T H.264], VP8 [b-IETF RFC 6386] (and their successor technologies H.265 [b-ITU-T H.265] and VP9 probably in future)

  - real-time text conversation (based on [ITU-T T.140])

  - instant messaging (using MSRP session-mode)

  - image sharing (e.g., data transfer application with binary encoded data)

  - conference / floor control (based BFCP)

- WebRTC data channel services:

  - two data services are under consideration (see [IETF WebRTCDC]):

    - unreliable data  (non-critical information includes state information)

    - reliable data  (both real-time and non-realtime data)

- NAT traversal techniques for end-to-end IP media path connectivity

  - ICE for UDP-based media transport

  - ICE for alternative TCP-based media transport

  - Latching (in case of hosted NAT traversal support [b-IETF latching])

- Quality of service

- Performance monitoring

{Editor’s note (2014-07): the overview needs to be cross-checked again in future meetings).}

## 6.3	Transport of user plane WebRTC traffic

### 6.3.1	Objective

There is the basic assumption that a WebRTC endpoint could be located behind a NAT/FW device, and thus needs NAT/FW traversal in order to have end-to-end connectivity at IP network and transport layer. This objective results in mainly in two design aspects:

- usage of multiplexing methods in order to minimize the number of required IP transport connections;

- usage of TCP-based transport as a last resort when the preferred option of UDP-based transport is blocked due to NAT/FW behaviour.

Thus, there are two protocol stacks of consideration from H.248 WEBRTC gateway perspective.

### 6.3.2	Protocol stack for UDP-based transport

See Figure 1:

Figure 1 – User plane WebRTC protocol stack in case of UDP- and TCP-based transport

Notably, there is

- SCTP/DTLS used for the transport of text conversation according to [ITU-T T.140] (instead of the native RTP-based transport according to [IETF RFC 4103]

### 6.3.3	Protocol stack for TCP-based transport

{Editor’s note (2015-06): the four protocol stack options don't relate actually to the aimed solution. A revised figure is required, similar to Figure 2α, - but with a single DTLS connection only and SRTP over RFC 4571 directly (w/o additional DTLS layer). See update in Figure 2}

See also Figure 1.

Notably, there is

- DTLS-over-TCP used ("despite the fact of TLS-over-TCP as native transport security protocol"), and

- DTLS still used SRTP key exchange with SRTP-over-RFC 4571 transport.

Hence, there is a single DTLS connection again as in case of UDP-based transport. Whether the underlying DTLS session is of type resumable or not is not specified.

[End Proposed Correction]

Change proposal #5:

[Begin Proposed Correction]

# 7	H.248 WEBRTC gateways: example use cases

Some example use cases are described in this clause.

## 7.1	Point-to-point communication

All examples here are characterized by a two-party call (between A and B) and that party A always uses a WebRTC browser environment.

### 7.1.1	Use case #1: WebRTC-to-WebRTC interworking

See Figure 3.

Figure 3 – Use case #1: WebRTC-to-WebRTC interworking

Use case #1 relates to a browser-to-browser scenario: the WEBRTC H.248 gateway may be needed e.g. due to different NAT-T requirements in the two networks where the WebRTC endpoints reside, due to codec mismatch between both WebRTC endpoints, due to different QoS architecture of the networks where the WebRTC endpoints reside, etc.

### 7.1.2	Use case #2: WebRTC-to-NGN/IMS interworking

Figure 4 depicts an interworking scenario with NGN/IMS networks.

Figure 4 – Use case #2: WebRTC-to-NGN/IMS interworking

Use case #2 is fundamentally motivated by the fact that NGN/IMS IP user equipment is supposed not to be fully identical to WebRTC endpoints (in terms of capabilities as listed in clause 8).

### 7.1.3	Use case #3: WebRTC-to-PSTN/ISDN interworking

Figure 5 outlines an interworking scenario to existing PSTN/ISDN infrastructures.

Figure 5 – Use case #3: WebRTC-to-PSTN/ISDN interworking

Use case #3 provides again many variants, such as

- the down-negotiation of an initial multimedia call request to an audio-only call,

- the transcoding between different audio codecs,

- the support of PSTN/ISDN multimedia terminals (e.g. [ITU-T H.324], [ITU-T H.320]).

## 7.2	Multipoint communication

Multiparty calls are basically in scope of WEBRTC, leading to the usual realtime conferencing support from network side. For instance, WebRTC components audio and video may demand for "RTP mixer” functionality, as e.g. typically provided by media servers. Such a network scenario may therefore relate to H.248 profile types concerning media servers (aka media resource processors).

The usual connection model of media servers for multiparty call types is given by "back-to-back endpoint" structures, i.e., there are multiple interconnected WebRTC client instances as part of such an H.248 context in a media server.

[End Proposed Correction]

Change proposal #6:

[Begin Proposed Correction]

# 8	Functional requirements for H.248 WEBRTC gateways

There are general and use case specific functional requirements for H.248 WEBRTC gateways. WEBRTC gateway requirements could be classified in subsequent functional areas.

## 8.1	Requirements related to NAT traversal support

### 8.1.1	Media latching based NAT traversal support

R-8.1.1/1: The H.248 WEBRTC gateway is required to support media latching according to the H.248 IP NAPT traversal package [ITU-T H.248.37].

NOTE – Latching as hosted NAT traversal mechanism (see [b-IETF latching]) translates to [ITU-T H.248.37] when provided by a decomposed H.248 gateway.

R-8.1.1/2: The H.248 WEBRTC gateway is recommended to support the media latching associated address reporting capability according to the H.248 address reporting package [ITU-T H.248.37].

NOTE – This is an optional capability and not mandatory for the basic WebRTC call establishment. The MGC might use this capability as complementary information, e.g. as part of an overall service concerning the identification of NAT behavioural types.

### 8.1.2	ICE based NAT traversal support

IP transport protocols:

R-8.1.2/1: The H.248 WEBRTC gateway is required to support ICE for UDP connections, according to [ITU-T H.248.50].

R-8.1.2/2: The H.248 WEBRTC gateway is required to support ICE for TCP connections, according to [ITU-T H.248.50].

Following [ITU-T H.248.50] building blocks are behind the two L4 protocol type related requirements at minimum:

- support of package-independent procedures (clause 10/[ITU-T H.248.50]); and

- STUN support profile given by the three H.248 packages "MG Act-as STUN server", "Originate STUN continuity check" and "STUN consent freshness".

ICE mode:

R-8.1.2/3: The H.248 WEBRTC gateway is required to support basically ICE-full mode (see clause 3.4/[IETF rtcweb-transports]), according to [ITU-T H.248.50].

R-8.1.2/4: The H.248 WEBRTC gateway is required to support ICE-lite mode only (according to [ITU-T H.248.50]), dependent on the location of the H.248 WEBRTC gateway in the end-to-end path and/or the "NAT traversal" relevant network architecture.

Trickle ICE:

R-8.1.2/5: The H.248 WEBRTC gateway is required to support the ICE extensions "trickle ICE" (see [b-IETF trickle-ice]), according to [ITU-T H.248.50].

IPv4/IPv6 dual stack and multihomed fairness extension for ICE:

R-8.1.2/5: The H.248 WEBRTC gateway can optionally support the ICE extensions "IPv4/IPv6 dual stack fairness" (see [b-IETF ice-dualstack]), according to [ITU-T H.248.50].

NOTE – The optional capability is actually orthogonal and not mandatory for the basic WebRTC call establishment. Its rather a network operator policy concerning IP version preferences.

Additional support of TURN server functionality:

A WebRTC access domain could require additional TURN functionality on top of STUN support. There is then the basic assumption that the IP media path will still be routed via the H.248 WebRTC media gateway, which implies that the H.248 MG could provide an embedded TURN server function. There seems to be no need for an additional TURN server in the media path.

R-8.1.2/6: The H.248 WEBRTC gateway can optionally provide TURN support according to [ITU-T H.248.50].

NOTE – The specific TURN related capabilities are for further studies.

## 8.2	Requirements related to communication topologies

### 8.2.1	H.248 WEBRTC gateway for point-to-point interworking

R-8.2.1/1: The H.248 WEBRTC gateway is required to support an H.248 (IP, IP) or (IP, non-IP connection models (use cases #1, 2 and #3).

### 8.2.2	H.248 WEBRTC gateway as WEBRTC conferencing point

R-8.2.2/1: The H.248 WEBRTC gateway is required to support the conferencing extensions for RTP media as indicated in clause 5.1/[IETF rtp-usage].

## 8.3	Requirements related to bearer traffic multiplexing

R-8.3.1/1: The H.248 WEBRTC gateway is required to support at least one (UDP- or TCP-based) transport connection for a single, multiplexed WEBRTC multimedia call.

NOTE – This is the bearer configuration with a maximum degree of multiplexing. All audio and video traffic are aggregated by usage of RTP transport and RTP media multiplexing. All data traffic is aggregated via a single SCTP association which is encrypted via DTLS. All three "middle stack" traffic components based on SRTP/SRTCP, DTLS and STUN sharing again a single L4 transport connection.

## 8.4	Requirements related to RTP-to-RTP type interworking

### 8.4.1	RTCP control flow component

R-8.4.1/1: The H.248 WEBRTC gateway is required to always support an RTCP control flow in addition to the RTP media flow (see clause 4.1/[IETF rtp-usage]).

Support of RTCP (i.e., the handling and processing of specific RTCP packet types) is dependent on the RTP topology used for the H.248 WEBRTC gateway (see clause 8.4.4). The following requirements are applicable for the example of the RTP topology "Back-to-back RTP end system (B2BRE)":

R-8.4.1/2: Basic RTCP services: The H.248 WEBRTC gateway is required to support sending and receiving RTCP SR, RR, SDES, and BYE packet types.

R-8.4.1/3: RTP profile dependent RTCP services: The H.248 WEBRTC gateway is required to support sending and receiving RTCP RTPFB (Generic RTP Feedback) and PSFB (Payload-specific Feebdack) packet types.

R-8.4.1/4: Supplementary RTCP services: The H.248 WEBRTC gateway could optionally support sending and receiving RTCP XR (extended report) packet types with respect to performance monitoring of RTP traffic (see also [ITU-T H.248.87]).

### 8.4.2	RTP profiles

R-8.4.2/1: The H.248 WEBRTC gateway is required to support RTP profile "RTP/SAVPF" according to [IETF RFC 5124, IETF RFC 7007] (see clause 4.2/[IETF rtp-usage]).

### 8.4.3	RTP multiplexing

R-8.4.3/1: The H.248 WEBRTC gateway is required to support RTP media multiplexing ("SSRC multiplexing") (see clause 4.4/[IETF rtp-usage]).

NOTE 1 – The multiplexing mode will only be used when all participants agree. If not, unmultiplexed mode will be used. Thus, any MGC involved in the call control level end-to-end capability negotiations could actually downgrade to unmultiplexed mode.

R-8.4.3/2: The H.248 WEBRTC gateway is required to support RTP transport multiplexing ("RTP / RTCP transport multiplexing") (see clause 4.5/[IETF rtp-usage]).

NOTE 2 – Same comment as in NOTE 1: backward compatibility requires the support of unmultiplexed mode as well.

### 8.4.4	RTP topologies

NOTE – All required RTP topologies are already outlined by [ITU-T H.248.88].

R-8.4.4/1: Use case independent: the H.248 WEBRTC gateway is required to comply with the "RTP topology" behaviour according to [ITU-T H.248.88].

R-8.4.4/2: Use case #1: the H.248 WEBRTC gateway is required to support topology

- "RTP transparent forwarding",

- "RTP transport translator" or

- "RTP media translator",

dependent on the call-level end-to-end negotiation of the used "RTP configuration" for WebRTC.

R-8.4.4/3: Use case #2: The H.248 WEBRTC gateway is required to support, in addition to RTP topologies of use case #1, the toplogy

- "Back-to-back RTP end system (B2BRE)"

due to the "legacy usage of RTP" in the "NGN/IMS IP domain".

NOTE – All existing H.248 profiles for border gateways are not explicit on RTP topologies in detail. Thus, the B2BRE topology represents a default topology (due to the back-to-back IP host configuration of an RTP-RTP context).

R-8.4.4/4: Use case #3: The H.248 WEBRTC gateway is required to support the "RTP endsystem" (RTPE) topology (see clause 7.1/[ITU-T H.248.88]).

### 8.4.5	RTCP based services, report types and packet formats

R-8.4.5/1: The H.248 WEBRTC gateway is required to support reduced size RTCP according to [IETF RFC 5506] (see clause 4.6/[IETF rtp-usage]).

### 8.4.6	Resource allocation rules for RTP/RTCP

#### 8.4.6.1	Port allocation rules for RTP and RTCP

R-8.4.6.1/1: The H.248 WEBRTC gateway is required to support symmetric RTP/RTCP according to [IETF RFC 4961] (see clause 4.7/[IETF rtp-usage]), which relates to the H.248-controlled port allocation as defined by [ITU-T H.248.57].

#### 8.4.6.2	SSRC allocation rules for RTP

R-8.4.6.2/1: The H.248 WEBRTC gateway is required to support signalled RTP synchronisation source (SSRC) identifiers according to [IETF RFC 5576] (see clause 4.8/[IETF rtp-usage]).

#### 8.4.6.3	CNAME allocation rules for RTP/RTCP

R-8.4.6.2/1: The H.248 WEBRTC gateway is required to support unique RTCP CNAME value(s) (see clause 4.9/[IETF rtp-usage]).

NOTE – A single CNAME might be sufficient for a single H.248 MG entity.

R-8.4.6.2/2: The H.248 WEBRTC gateway can optionally be requested to report received RTCP CNAME value(s) from remote WebRTC clients (according to the RTCP Source Description package as defined by [ITU-T H.248.71].

## 8.5	Requirements related to TCP-based media transport

### 8.5.1	Requirements related to TCP connection establishment

The establishment control of the TCP connection segment between the remote WebRTC client and H.248 WebRTC gateway is subject of the overal NAT traversal framework given by ICE/STUN procedures as executed between both WebRTC endpoints. Resulting in following TCP specific requirements.

R-8.5.1/1: The H.248 WEBRTC gateway is required to support autonomous TCP connection establishment according to the implicit results of the ICE candidate selection process.

NOTE – Above requirements translates in active, passive or simultaneous TCP connection establishment procedures. Thus, the TCP-enabled H.248 SEP is not using [ITU-T H.248.84] and [ITU-T H.248.89] during the establishment phase of TCP.

### 8.5.2	Requirements related to connectivity checks of TCP connection candidates

R-8.5.2/1: The H.248 WEBRTC gateway is required to support correspondent STUN-based connectivity checks for TCP.

### 8.5.3	Requirements related to TCP connection release

#### 8.5.3.1	Requirements related to immediate TCP connection release in case of non-used ICE candidates

R-8.5.3.1/1: The H.248 WEBRTC gateway is required to support the immediate release of non-selected TCP connection candidates.

#### 8.5.3.2	Requirements related to regular TCP connection release at the end of the WebRTC call

R-8.5.3.2/1: The H.248 WEBRTC gateway is required to support the usual TCP connection release capabilities as outlined in [ITU-T H.248.89].

### 8.5.4	Requirements related to application level framing protocol

R-8.5.4/1: The H.248 WEBRTC gateway is required to support the L4+ framing scheme according to [IETF RFC 4571] in case of TCP-based WebRTC transport.

### 8.5.5	Requirements related to MG-internal interworking of TCP traffic

#### 8.5.5.1	Requirements related to TCP-to-UDP interworking

R-8.5.5.1/1: The H.248 WEBRTC gateway is required to provide a complete termination of the TCP, i.e., a (TCP)-connection-endpoint function.

#### 8.5.5.2	Requirements related to TCP-to-TCP interworking

[ITU-T H.248.84] and [ITU-T H.248.89] describe various modes of operation related to TCP-to-TCP interworking in H.248 media gateways.

R-8.5.5.2/1: The H.248 WEBRTC gateway is required to support basically a Back-to-Back TCP Endsystem (B2BTE) mode due to the "ICE-controlled" TCP connection segment towards remote WebRTC clients.

NOTE – Background: L4 connectivity issues during the active call phase could lead to ICE refresh procedures, which might impact L4 connection updates.

{Editor’s note (2015-06 meeting): we might add optional requirements, indicating that the MG might autonomously transition to a more efficient mode of interworking (lightweight TCP proxy?) …}

#### 8.5.5.3	Requirements related to MG-internal buffering of TCP data and TCP flow control

That requirements area is implementation specific and out of scope of this Recommendation (to be confirmed).

## 8.6	Requirements related to media-aware type of interworking

### 8.6.1	Requirements related to WEBRTC audio

#### 8.5.1.1	Requirements related to media format (audio)

R-8.5.1.1/1: The H.248 WEBRTC gateway is required to support audio transcoding between the WEBRTC MTI audio codec(s) and other audio codecs as typically used in NGNs.

#### 8.5.1.2	Requirements related to media adaptation and rate control (audio)

Requirements are subject of clause 7/[IETF rtp-usage], however, there are not yet any explicit capabilities identified for the first phase of WebRTC service deployment. Correspondent capabilities are currently developed by IETF working group RMCAT (RTP Media Congestion Avoidance Techniques), see e.g. requirements in [b-IETF rmcat-cc-requirements], which are planned to be integrated later on in WEBRTC.

### 8.5.2	Requirements related to WEBRTC video

#### 8.5.2.1	Requirements related to media format (video)

R-8.5.2.1/1: The H.248 WEBRTC gateway as IP-IP media gateway is not required to support video transcoding (because, if required at all, provided by centralized media servers).

R-8.5.2.1/2: The H.248 WEBRTC gateway as media server can optionally support video transcoding between the WEBRTC video codec(s) and other video codecs as typically used in NGNs.

NOTE – The optional tagging of this requirement is due to the fact that it is expected that potential scenarios for video transcoding might be fairly low and exceptional. It is rather expected that WebRTC endpoints will negotiate a common video codec, or fallback to a default codec, or even not use video.

#### 8.5.2.2	Requirements related to media adaptation and rate control (video)

Status: see clause 8.5.1.2.

### 8.5.3	Requirements related to WEBRTC data

#### 8.5.3.1	Requirements for MSRP-based instant messaging

R-8.5.3.1/1: The H.248 WEBRTC gateway is required to support transparent forwarding of MSRP protocol data units ("MSRP messages"), independent of the underlying (WebRTC specific or legacy) transport (protocol stack). This includes interworking between WebRTC clients or a WebRTC and non-WebRTC client.

R-8.5.3.1/2: The H.248 WEBRTC gateway is required to support the underlying transport relay function in interworking between "SCTP/DTLS/UDP" to "TCP" for MSRP traffic.

R-8.5.3.1/3: The H.248 WEBRTC gateway can optionally support a bearer level application gateway (B-ALG) at MSRP layer according to [ITU-T H.248.78]. Such a capability relates to a specific MSRP B2BUA function with the purpose of a L4+ level NAT traversal support.

#### 8.5.3.2	Requirements for BFCP-based conference control

R-8.5.3.2/1: The H.248 WEBRTC gateway is required to support transparent forwarding of BFCP protocol data units ("BFCP messages"), independent of the underlying (WebRTC specific or legacy) transport (protocol stack). This includes interworking between WebRTC clients or a WebRTC and non-WebRTC client.

R-8.5.3.2/2: The H.248 WEBRTC gateway is required to support the underlying transport relay function in interworking between "SCTP/DTLS/UDP" to "TCP" for BFCP traffic.

R-8.5.3.2/1: The H.248 WEBRTC gateway can optionally to support the underlying transport relay function in interworking between "SCTP/DTLS/UDP" to "UDP" for BFCP traffic (see [b- draft-ietf-bfcpbis-rfc4582bis]).

### 8.5.3.3	Requirements for binary-encoded WEBRTC data

R-8.5.3.3/1: The H.248 WEBRTC gateway is required to support transparent forwarding of binary data units (images), independent of the underlying (WebRTC specific or legacy) transport (protocol stack). This includes interworking between WebRTC clients or a WebRTC and non-WebRTC client.

R-8.5.3.1/2: The H.248 WEBRTC gateway is required to support the underlying transport relay function in interworking between "SCTP/DTLS/UDP" to an applicable transport e.g. "TCP" for binary traffic.

### 8.5.4	Requirements related to WEBRTC text

WebRTC text telephony (text conversation) is considered as "data traffic" (despite the fact of the usual RTP-based transport in non-WebRTC environments).

R-8.5.4/1: The H.248 WEBRTC gateway is required to support transparent forwarding of T.140 protocol data units (according to [ITU-T T.140]), independent of the underlying (WebRTC specific or legacy) transport (protocol stack). This includes interworking between WebRTC clients or a WebRTC and non-WebRTC client.

R-8.5.4/2: The H.248 WEBRTC gateway is required to support the underlying transport relay function in interworking between "SCTP/DTLS/UDP" to "RTP/UDP" for T.140 traffic.

## 8.6	Requirements related to bearer security

### 8.6.1	Requirements related to transport security

#### 8.6.1.1	Protocol stacks with DTLS

R-8.6.1.1/1: The H.248 WEBRTC gateway is required to support transport security for UDP using DTLS, according to [ITU-T H.248.93].

R-8.6.1.1/2: The H.248 WEBRTC gateway is required to support transport security for TCP using DTLS (according to [ITU-T H.248.93]) and RFC 4571 as interim framing protocol.

R-8.6.1.1/3: The H.248 WEBRTC gateway is required to use a shared DTLS connection for SCTP-encapsulated WebRTC data traffic, according to [ITU-T H.248.93, H.248.SCTP].

R-8.6.1.1/4: The H.248 WEBRTC gateway is required to use a shared DTLS connection for SRTP related key exchange procedures (for WebRTC audio and video traffic), according to [ITU-T H.248.93, H.248.77].

#### 8.6.1.2	DTLS interworking scenarios

Transport of data traffic in a native WEBRTC domain relates to the protocol layering of "data-over-SCTP/DTLS/UDP/IP", whereas transport of data traffic in the native Internet uses basically "data-over-TCP/IP". The H.248 WEBRTC gateway needs to address interworking between both network paradigms (see also use cases #1 and #2 in clause 7.1), leading to following requirements:

R-8.6.1.2/1: The H.248 WEBRTC gateway is required to support DTLS transparent forwarding for the interworking between two network domains with SCTP/DTLS/UDP based WEBRTC data transport. (relates to the use case according to clause 6.2.2.1/[ITU-T H.248.93]).

R-8.6.1.2/2: The H.248 WEBRTC gateway is required to support DTLS-to-non-DTLS interworking for the interworking between two network domains with SCTP/DTLS/UDP and (TLS)/TCP based WEBRTC data transport. (relates to the use case #1.2 according to clause 6.2.1/[ITU-T H.248.93]).

#### 8.6.1.3	TLS support

R-8.6.1.3/1: The H.248 WEBRTC gateway is required to support TLS termination in case of the DTLS interworking scenario according to R-8.6.1.2/2 and transport security usage in the TCP domain.

### 8.6.2	Requirements related to media security

#### 8.6.2.1	Encryption method

R-8.6.2.1/1: The H.248 WEBRTC gateway is required to support media security for RTP using SRTP, according to [b-IETF RFC 3711].

#### 8.6.2.2	Key exchange scheme for SRTP

R-8.6.2.2/1: The H.248 WEBRTC gateway is required to support the media-path coupled SRTP key exchange (using DTLS-SRTP [IETF RFC 5764], see [b-IETF RFC 5763]) according to requirement [R-8.6.1/2].

R-8.6.2.2/2: The H.248 WEBRTC gateway can optionally support the signalling-path coupled SRTP key exchange, i.e., the "SDES" (SDP security descriptions) related method according to [ITU-T H.248.77].

R-8.6.2.2/3: The H.248 WEBRTC gateway can optionally support multiple key exchange schemes for SRTP within a single H.248 profile. The H.248 WebRTC gateway is then required to indicate the applied SRTP key exchange scheme according to [ITU-T H.248.77].

## 8.7	Requirements related to WEBRTC emergency and priority services

### 8.7.1	Emergency services within WebRTC calls

From the IETF side, requirements for WEBRTC emergency services are so far contained in the (expired) draft [b-IETF rtcweb-ecrit]. No additional requirements for H.248 WEBRTC gateways are derived taking this initial document as baseline.

### 8.7.2	Priority services within WebRTC calls

{Editor’s note (2015-06 meeting): this subclause is a placeholder in order to clarify whether there are specific requirements (such MPS, ETS, IEPS, etc)) in context of H.248.81, H.Sup9 and/or H.Sup12. Contributions are solicited.}

## 8.8	Requirements related to QoS support

### 8.8.1	IP traffic control: QoS marking

R-8.8.1/1: The H.248 WEBRTC gateway is required to support DiffServ based QoS marking according to [ITU-T H.248.52] (see clause 12.1.3/[IETF rtp-usage]).

NOTE 1 – The H.248 WEBRTC MG provides the so-called "DS pre-marker" role (see Appendix IV/[ITU-T H.248.52]).

NOTE 2 – QoS marking takes effect at IP level, i.e., on the complete aggregate of all WebRTC traffic components. Any kind of application protocol individual QoS markings is not required for initial WebRTC service support.

### 8.8.2	IP traffic control: traffic policing – IP byterate policing

R-8.8.2/1: The H.248 WEBRTC gateway is required to support IP byterate policing according to [ITU-T H.248.53].

NOTE 1 – Traffic policing takes effect at IP level, i.e., on the complete aggregate of all WebRTC traffic components.

### 8.8.3	Improved transport robustness for RTP

See clause 6/[IETF rtp-usage].

#### 8.8.3.1	Negative acknowledgements

In general:

R-8.8.3.1/1: The H.248 WEBRTC gateway is recommended to support negative acknowledgements (NACKs) for RTP data packets [IETF RFC 4585] (see clause 6.1/[IETF rtp-usage]).

In particular:

The origination and termination of RTCP Feedback (FB) messages is subject of RTP sender and RTP receiver entities [IETF RFC 4585], thus tightly coupled to "RTP topologies" (see clause 8.4.8). Hence, an H.248 WEBRTC gateway, as located within an end-to-end RTP media path, is normally only partially involved in the processing of RTCP FB messages. The following high level behaviour is derived:

R-8.8.3.1/2: The H.248 WEBRTC gateway is recommended to support the forwarding of RTCP FB messages when using a RTP translator topology.

R-8.8.3.1/3: The H.248 WEBRTC gateway is recommended to originate and terminate RTCP FB messages when using a Back-to-back RTP end system topology.

#### 8.8.3.2	Forward Error Correction

In general:

R-8.8.3.2/1: The H.248 WEBRTC gateway can optionally support Forward Error Correction (FEC) for RTP [IETF RFC 5109] (see clause 6.2/[IETF rtp-usage]).

In particular:

Roughly, an RTP sender generates the usual RTP "media packets" plus RTP "FEC packets", which are evaluated again by the RTP receiver in case of lost RTP "media packets". And there is an underlying assumption of unmodified RTP packets along the RTP media path. Thus, FEC becomes useless in case of interim RTP entities which would modify RTP packets (to be confirmed). Leading to following high-level behaviour:

R-8.8.3.2/2: The H.248 WEBRTC gateway is required to support the transparent forwarding of RTP traffic when using a RTP translator topology, - i.e., when FEC is required, then only RTP transport translator topology would be allowed.

R-8.8.3.2/3: The H.248 WEBRTC gateway is recommended to originate and terminate RTP "FEC packets" when using a Back-to-back RTP end system topology.

## 8.9	Requirements related to bearer-path coupled congestion controls

These requirements are tightly coupled with the capability of "media adaptation and rate control" (see clause 8.5). Specific requirements are not yet settled from IETF side for the first release of this Recommendation.

## 8.10	Requirements related to performance monitoring

The requirements framework in this clause follows the [ITU-T H.248.87] "Guidelines on the use of ITU-T H.248 capabilities for performance monitoring in RTP networks".

Browser-based WEBRTC endpoints may likely support the local generation of measurements for performance metrics according to [b-IETF rtcweb-xr]. The bowser-embedded measurement point and the set of supported WebRTC's statistics are defined by [b-W3C webrtc-stats].

### 8.10.1	Requirements related to measurement point (MP)

Generally, the set of performance measurements typically differs between WEBRTC endpoints (such as a WEBRTC browser) and WEBRTC gateways, e.g., because the observation of application level performance metrics is primary subject of communication endpoints and normally questionable or even meaningless in case of "in-path equipment".

R-8.10.1/1: The H.248 WEBRTC gateway is recommended to execute measurements for transport level performance metrics (term definition see clause 3.2.6/[ITU-T H.248.87]).

R-8.10.1/2: The H.248 WEBRTC gateway can optionally execute measurements for application level performance metrics (definition see clause 3.2.1/[ITU-T H.248.87]), - under the condition of a valuable semantic of the concerned metric.

Example metrics:

- RTP based WEBRTC traffic:

- see Appendix III/[ITU-T H.248.87] for a general overview

- RTCP XR related metrics are recommended (e.g., RTCP XR metrics are used instead of H.248 nt / rtp package statistics)

- Non-RTP based WEBRTC traffic: e.g. transport level metrics related to

- DTLS (see [ITU-T H.248.93]),

- SCTP (see [b-ITU-T H.248.SCTP]),

- TLS (see [ITU-T H.248.90]) and

- TCP (see [ITU-T H.248.84, ITU-T H.248.90]).

The actual set of performance metrics for a particular H.248 "WEBRTC gateway" profile depends, as usual, on the specific purpose, such as

- usage metering (relates typically to the traffic volume on the application level);

- reporting of user experience related metrics (i.e., QoS/QoE related metrics);

- reporting of network condition related metrics (i.e., GoS related metrics);

- recording of successful, unsuccessful or specific policing actions; or

- validation of network capacity allocations (relates typically to the traffic volume on the lowest layer of transport capacity reservation)

The primary focus is most likely (c) in case of a H.248 WEBRTC gateway due to its positioning at the border of network domains (with typically different Grade of Service conditions).

### 8.10.2	Requirements related to collection point (CP)

The set of supported performance metrics, which should be measured by WebRTC endpoints (as peered by H.248 WEBRTC gateway) is still undefined at the release of this Recommendation. It is e.g. expected that [b-IETF rtcweb-xr] might be supported for the RTP based WEBRTC traffic components. Performance metrics for non-RTP WEBRTC traffic is still unclear.

When WebRTC endpoints support the measurement of RTCP XR based metrics, then it is expected that they report their measurements via RTCP along the WebRTC media path.

R-8.10.2/1: The H.248 WEBRTC gateway is required to collect remote measurement data via incoming RTCP reports.

### 8.10.3	Requirements related to reporting point (RP)

R-8.10.3/1: The H.248 WEBRTC gateway is required to support the reporting of RTCP XR related H.248 statistics according to the RTCP XR block reporting package [ITU-T H.248.48].

### 8.10.4	Requirements related to filtering point (FP)

Measurement data carried by RTCP XR reports could be filtered by H.248 WEBRTC gateways, see [ITU-T H.248.87]. However, the particular H.248 WEBRTC gateway filter behaviour is subject of operator policies, thus out of scope of this Recommendation.

### 8.10.5	Requirements related to loopback point (LP)

None.

## 8.11	Requirements related to SDP-based description, declaration and negotiation of WEBRTC media configurations

### 8.11.1	Requirements related to supported SDP elements – Overall media descriptions

The H.248 WEBRTC gateway is required to support SDP elements (for the purpose of media gateway control) according to Tables 1, 2 and 3:

Table 1 – Requirements related to supported SDP elements – Part I SDP "m="-line

| Requirement | SDP "m=" line element | Purpose |
| --- | --- | --- |
| Media type values (in case of media type aware interworking modes): |  |  |
| R-8.11.1.1/1: | media type = audio | WEBRTC audio component |
| R-8.11.1.1/2: | media type = video | WEBRTC video component |
| R-8.11.1.1/3: | media type = application | WEBRTC data components |
| Transport protocol (stack) values: |  |  |
| R-8.11.1.1/4: | proto = RTP/SAVPF | RTP profile for all RTP-based traffic components |
| R-8.11.1.1/5: | proto = UDP/DTLS/SCTP proto = TCP/DTLS/SCTP | WEBRTC data traffic components (NOTE 1) |
| Media format values (in case of media format aware interworking modes) (NOTE 2): a) audio formats (from WebRTC endpoint perspective, see [b-IETF rtcweb-audio]): |  |  |
| R-8.11.1.1/6: | OPUS | Internet audio code [b-IETF RFC 6716] |
| R-8.11.1.1/7: | PCMA / PCMU & CN | voice G.711 µ/A-law with optional comfort noise |
| R-8.11.1.1/8: | telephone-event (event codepoints 0 to 11) | audio/telephone-event media format as specified in [b-IETF RFC 4733] for DTMF |
| b) audio formats (optional, from NGN/IMS endpoint perspective, see use case #2): |  |  |
| R-8.11.1.1/9: | AMR-WB | Adaptive Multi-Rate Wideband [b-IETF RFC 4867] |
| R-8.11.1.1/10: | EVRC | Enhanced Variable Rate Codec [b-IETF RFC 4788] |
| c) video formats (for consideration, see [b-IETF rtcweb-video]): |  |  |
| R-8.11.1.1/11: | VP8 | video codec [b-IETF RFC 6386], [b-IETF rtp-vp8] |
| R-8.11.1.1/12: | H264 | video codec [b-IETF RFC 6184] |
| R-8.11.1.1/13: | VP9 | video codec [b-IETF codec-vp9] |
| R-8.11.1.1/14: | H265 | video codec [b-IETF rtp-h265] |
| d) text formats (for consideration): |  |  |
| R-8.11.1.1/15: | t140 | conversational text [b-IETF RFC 4103] |
| e) WebRTC data channel formats |  |  |
| R-8.11.1.1/16: | webrtc-datachannel | generic data channel, realized as SCTP stream over a DTLS session (NOTE 3). |
| NOTE 1 – SDP element defined by [b-IETF sdp-sctp]. There might be a subset of the protocol stack related name used in case of specific H.248 stream grouping models. NOTE 2 – Codec technologies are continuously improving, leading to a limited timeline of usage and their future replacement by evolved codecs. Thus, all listed media format specific types are not mandatory, but are listed in order to provide concrete use cases for the SDP-based parameterization of such codecs in H.248 WEBRTC gateways (see clause 8.11.2). Mandatory to implement (MTI) type of media formats are out of scope of this Recommendation, rather subject of concrete H.248 profile specifications. NOTE 3 – SDP "m="-line media format value is defined by [b-IETF sdp-sctp]. |  |  |

{Editor’s note (2014-11 meeting): above tables are still in progress, dependent on H.248.SCTP, H.248.STGROUP, H.248.RTPMUX}

Table 2 – Requirements related to supported SDP elements – Part II SDP "a="-line

| Requirement | SDP element | Purpose |
| --- | --- | --- |
| NAT traversal support for media / bearer path: |  |  |
| R-8.11.1.2/1: | a=ice-ufrag | ICE credentials for media level STUN authentication procedure |
| R-8.11.1.2/2: | a=ice-pwd |  |
| R-8.11.1.2/3: | a=candidate: … TCP … | in case of last resort of TCP-based media transport (instead of UDP)) |
| Security in the media / bearer path: |  |  |
| R-8.11.1.2/4: | a=fingerprint | Authentication procedure for WEBRTC DTLS sessions. |
| R-8.11.1.2/5: | a=crypto | in case of optional SDES-based SRTP |
| Traffic multiplexing in the media / bearer path: |  |  |
| R-8.11.1.2/6: | a=group:BUNDLE | General media grouping using "bundle" (NOTE 1). |
| R-8.11.1.2/7: | a=mid: |  |
| R-8.11.1.2/8: | a=rtcp-mux | for transport multiplexed RTP traffic [ITU-T H.248.57] |
| R-8.11.1.2/9: | a=rtcp | optional, if transport unmultiplexed mode for RTP and explicit RTCP transport address allocation [ITU-T H.248.57] |
| R-8.11.1.2/10: | a=ssrc: | Not required at H.248 interface (to be confirmed) |
| WEBRTC usage of RTP / RTCP: |  |  |
| R-8.11.1.2/11: | a=rtcp-rsize | Reduced-size RTCP [IETF RFC 5506] |
| R-8.11.1.2/12: | a=rtcp-fb | [IETF RFC 5104] (to be confirmed) |
| WEBRTC data traffic configuration: {Editor’s note (2014-11 meeting): below information is preliminary, not yet mature due to the ongoing discussion in the IETF (MMUSIC, DISPATCH) on SDP elements related to the configuration of WEBRTC data traffic (see draft-ietf-mmusic-data-channel-sdpneg, draft-ietf-mmusic-msrp-usage-data-channel, draft-ietf-clue-datachannel)} |  |  |
| R-8.11.1.2/13: | a=fmtp:webrtc-datachannel … | Usage of an SCTP stream as "WEBRTC data channel" (NOTE 2) |
| R-8.11.1.2/14: | a=sctp-port:… | The actual SCTP port value of the SCTP association carried over DTLS/UDP, which is used for "WEBRTC data channels" (NOTE 2) |
| R-8.11.1.2/15: | a) application-agnostic: a=dcmap:<SCTP Stream ID> | Assignment of a particual SCTP stream to a WEBRTC data traffic component (NOTE 3) without any indication of the subprotocol. |
| R-8.11.1.2/16: | a) application-aware a=dcmap:<SCTP Stream ID> subprotocol="…" | Assignment of a particual SCTP stream to a WEBRTC data traffic component (NOTE 3) with indication of the subprotocol. |
| R-8.11.1.2/17: | a=dcsa:… | application-specific information of a particular subprotocol asWEBRTC data channel (NOTE 4, NOTE 5). |
| NOTE 1 – SDP element defined by [b-IETF sdp-bundle]. NOTE 2 – SDP element defined by [b-IETF sdp-sctp]. NOTE 3 – SDP element defined by [IETF DCSDP]. NOTE 4 – SDP element defined by [IETF DCSDP]. NOTE 5 – See [b-IETF data-channel-msrp] concerning the SDP profiling for the example usage of a WebRTC data channel for application protocol "MSRP". |  |  |

Table 3 – Requirements related to supported SDP elements – Part III other SDP line s

| Requirement | SDP element | Purpose |
| --- | --- | --- |
| Transport capacity reservation and allocation – H.248 Stream admission control: |  |  |
| R-8.11.1.3/1: | "b="-line | see [IETF rtp-usage] {which bandwidth modifiers? RR, RS, AS, CT, …} |

{Editor’s note (2014-11 meeting): above tables are still in progress, dependent on H.248.SCTP, H.248.STGROUP, H.248.RTPMUX}

### 8.11.2	Requirements related to supported SDP elements – Codec configurations

Table 1 introduced the required media formats ("codecs"), which usually include a set of parameters concerning their configuration. These parameters are signalled via SDP from MGC to MG and affect the RTP packet encapsulation ("internal structure of the protocol data unit"), codec processing and RTCP handling, - dependent on media-aware / media-agnostic modes of operation of the H.248 WEBRTC gateway.

#### 8.11.2.1	SDP elements related the parameterization of audio codec "OPUS"

See clause 2.1/[b-IETF RFC 6716]:

The Opus codec includes a number of control parameters that can be changed dynamically during regular operation of the codec, without interrupting the audio stream from the encoder to the decoder. These parameters only affect the encoder since any impact they have on the bitstream is signaled in-band such that a decoder can decode any Opus stream without any out-of-band signaling. Any Opus implementation can add or modify these control parameters without affecting interoperability. The most important encoder control parameters in the reference encoder are listed below.

The H.248 WEBRTC gateway, when enabled for media aware interworking, would provide such an encoder and decoder function, hence is required to support SDP elements (for the purpose of media gateway control) according to Table 4. [b-IETF rtp-opus] defines the formal codec parameters (in clause 6.1) and their mapping on SDP elements (in clause 6.2).

Table 4 – SDP parameters for IANA registered media type "audio/opus"

| Requirement | Codec parameter | SDP parameter | M/O | Value range | Default | Comments and H.248 profile |
| --- | --- | --- | --- | --- | --- | --- |
| R-8.11.2.1/1: | rate ("sampling rate")) | a=rtpmap:… opus/48000/2 | M | 8000, 12000, 16000, 24000, 48000 | 48000 (?) | all |
| R-8.11.2.1/2: | maxplaybackrate | a=fmtp: (NOTE 1) | O |  |  | all (tbd) |
| R-8.11.2.1/3: | sprop-maxcapturerate | a=fmtp: (NOTE 1) | O |  | 48000 |  |
| R-8.11.2.1/4: | maxptime | a=maxptime | O | 3, 5, 10, 20, 40, 60, or an arbitrary multiple of an frame size rounded up to the next full integer value, up to a max. value of 120 |  | When "media transcoding" mode, then it is recommended to align these parameter values between the OPUS and non-OPUS RTP domain in order to minimize end-to-end latency and to avoid interworking complexity. |
| R-8.11.2.1/5: | ptime | a=ptime | O |  |  |  |
| R-8.11.2.1/6: | minptime | a=fmtp: (NOTE 1) | O |  |  |  |
| R-8.11.2.1/7: | maxaveragebitrate | a=fmtp: (NOTE 1) | O | Any positive integer is allowed, but values outside the range 6000 to 510000 SHOULD be ignored |  |  |
| R-8.11.2.1/8: | stereo | a=fmtp: (NOTE 1) | O | 0 (mono), 1 (stereo) | 0 (mono) |  |
| R-8.11.2.1/9: | sprop-stereo | a=fmtp: (NOTE 1) | O |  | 0 (mono) |  |
| R-8.11.2.1/10: | cbr | a=fmtp: (NOTE 1) | O | 1 (constant bitrate), 0 (variable bitrate) | 0 (vbr) |  |
| R-8.11.2.1/11: | useinbandfec | a=fmtp: (NOTE 1) | O | 0 (no FEC), 1 (FEC) | 0 (no FEC) |  |
| R-8.11.2.1/12: | usedtx | a=fmtp: (NOTE 1) | O | 0 (no DTX), 1 (DTX) | 0 (no DTX) |  |
| NOTE 1 – Media type string in the form of a semicolon-separated list of parameter=value pairs, as part of the "a=fmtp" SDP attribute. E.g., a=fmtp:101 maxplaybackrate=16000; sprop-maxcapturerate=16000; maxaveragebitrate=20000; stereo=1; useinbandfec=1; usedtx=0 |  |  |  |  |  |  |  |

{Editor’s note (2014-11 meeting): above table format may be subject of revisions. E.g., there might be alternative table structures (see Draft H.248.CodecSDPprofile)}

### 8.11.3	Requirements related to SDP Offer/Answer

WEBRTC does not mandate a specific call control signalling protocol. Thus, there are not any guidelines for SDP offer/answer from IETF side in case of SIP as WEBRTC call control protocol.

In case of SIP networks with H.248 WEBRTC gateways, handling of SDP offer/answer procedures between call control level and their representation at H.248 gateway control level is outlined by [ITU-T H.248.80], particular when the revised SDP offer/answer model is used.

## 8.12	Requirements related to H.248 signalling

There might be additional, H.248 specific requirements beyond the pure WebRTC service consideration.

### 8.12.1	Indication of H.248 "WebRTC" terminations

A WebRTC-enabled H.248 termination implies fundamentally that the MG needs to provide a WebRTC client function for the particular termination in a context. Such information might be relevant e.g. from MG resource management perspective.

R-8.12.1/1: The H.248 WEBRTC gateway is not required to support additional H.248 signalling capabilities in order to discriminate (in MGC to MG direction) between H.248 WebRTC and non-WebRTC terminations.

NOTE – Above requirement makes the assumption that the termination type indication could be indirectly derived, e.g., from the termination naming scheme or termination/stream specific H.248 descriptor contents.

[End Proposed Correction]

Change proposal #7:

[Begin Proposed Correction]

Bibliography

[b-ITU-T H.264]	Recommendation ITU-T H.264 (02/2014), Advanced video coding for generic audiovisual services.

[b-ITU-T H.265]	Recommendation ITU-T H.265 (04/2013), High efficiency video coding.

[b-ITU-T H.Sup.13]	Draft Supplement ITU-T H.Sup.13 (##/201#), Gateway Control Protocol: ITU-T H.248.x-series - Supplement on common H.248 terminology – Release 2.

[b-IETF RFC 4347]	IETF RFC 4347 (2006), Datagram Transport Layer Security.

[b-IETF RFC 4788]	IETF RFC 4788 (2007), Enhancements to RTP Payload Formats for EVRC Family Codecs.

[b-IETF RFC 4867]	IETF RFC 4867 (2007), RTP Payload Format and File Storage Format for the Adaptive Multi-Rate (AMR) and Adaptive Multi-Rate Wideband (AMR-WB) Audio Codecs.

[b-IETF RFC 5763]	IETF RFC 5763 (05/2010), Framework for Establishing a Secure Real-time Transport Protocol (SRTP) Security Context Using Datagram Transport Layer Security (DTLS).

[b-IETF RFC 6184]	IETF RFC 6184 (05/2011), RTP Payload Format for H.264 Video.

[b-IETF RFC 6386]	IETF RFC 6386 (2011), VP8 Data Format and Decoding Guide.

[b-IETF RFC 6716]	IETF RFC 6716 (2012), Definition of the Opus Audio Codec.

[b-IETF clue-datachannel]	IETF draft-ietf-clue-datachannel (2015), CLUE Protocol Data Channel.

[b-IETF RFC 7362]	IETF RFC 7362 (09/2014), Latching: Hosted NAT Traversal (HNT) for Media in Real-Time Communication.

[b-IETF rmcat-cc-requirements]	IETF draft-ietf-rmcat-cc-requirements (2014), Congestion Control Requirements for Interactive Real-Time Media.

[b-IETF rtcweb-gateway]	IETF draft- rtcweb-gateways (2015), WebRTC Gateways.

[b-IETF trickle-ice]	IETF draft-ietf-mmusic-trickle-ice (2014), Trickle ICE: Incremental Provisioning of Candidates for the Interactive Connectivity Establishment (ICE) Protocol.

[b-IETF ice-dualstack]	IETF draft-martinsen-mmusic-ice-dualstack-fairness (##/201#), ICE Multihomed and IPv4/IPv6 Dual Stack Fairness.

[b-IETF rtp-opus]	IETF draft-ietf-payload-rtp-opus (2014), RTP Payload Format for Opus Speech and Audio Codec.

[b-IETF rtp-vp8]	IETF draft-ietf-payload-vp8 (2014), RTP Payload Format for VP8 Video.

[b-IETF codec-vp9]	IETF draft-grange-vp9-bitstream (2013), A VP9 Bitstream Overview.

[b-IETF rtp-h265]	IETF draft-ietf-payload-rtp-h265 (2014), RTP Payload Format for High Efficiency Video Coding.

[b-IETF rtcweb-ecrit]	IETF draft-aboba-rtcweb-ecrit-01 (2013), Emergency Services Support in WebRTC.

[b-IETF rtcweb-audio]	IETF draft-ietf-rtcweb-audio (2014), WebRTC Audio Codec and Processing Requirements.

[b-IETF rtcweb-video]	IETF draft-ietf-rtcweb-video (2014), WebRTC Video Processing and Codec Requirements.

[b-IETF rtcweb-xr]	IETF draft-huang-xrblock-rtcweb-rtcp-xr-metrics (2014), Considerations for Selecting RTCP Extended Report (XR) Metrics for the RTCWEB Statistics API.

[b-IETF sdp-bundle]	IETF draft-ietf-mmusic-sdp-bundle-negotiation (2015), Negotiating Media Multiplexing Using the Session Description Protocol (SDP).

[b-IETF sdp-sctp]	IETF draft-ietf-mmusic-sctp-sdp (2015), Stream Control Transmission Protocol (SCTP)-Based Media Transport in the Session Description Protocol (SDP).

[b-IETF data-channel-msrp]	IETF draft-ietf-mmusic-msrp-usage-data-channel (2015), MSRP over SCTP/DTLS data channels.

[b-W3C webrtc-stats]	W3C draft webrtc-stats (2014), Identifiers for WebRTC's Statistics API. <http://dev.w3.org/2011/webrtc/editor/webrtc-stats.html>

[b-IETF tls-terms]	IETF draft-guballa-tls-terminology (2015), Terminology related to TLS and DTLS.
URL: https://tools.ietf.org/html/draft-guballa-tls-terminology-01

[End Proposed Correction]

____________________