| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4716 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | H.TPS-SIG: Security |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This contribution proposes text for clause 10/H.TPS-SIG on security.

2.	Discussion

Currently clause 10 on Security contains the following editor’s note as a place holder for normative text on security:

{Editor’s note: The intention of this section is to describe security considerations around the support of telepresence (in particular CLUE signalling messages). The IETF CLUE work assumes the use of a DTLS/SCTP/IP transport. Therefore for CLUE signalling in H.series based conferences to be equivalent in functionality a secure method of signalling H.245 messages is needed.}

Security for telepresence endpoints shouldn’t be markedly different from existing H.323 endpoints. The exception is the signalling of the CLUE protocol. The CLUE protocol in the IETF utilises an SCTP/DTLS data channel that provides a secure mechanism for the transport of CLUE messages.

Currently in H.TPS-SIG CLUE messages may either be carried via H.245 generic messages or via a DTLS/SCTP data channel transport. For H.245 transport the endpoint has the option of using a H.235 for secure signalling. For the DTLS/SCTP transport the endpoint must utilise DTLS.

As such the Contributors propose to describe this in clause 10 on security.

3.	Proposal

It is proposed to accept the following changes. The changes are marked up against TD 226R1/WP1 from the Geneva, February Q5/16 Rapporteurs meeting.

Draft new ITU-T H.TPS-SIG

Signalling for telepresence-enabled conferencing

…

2 References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[F.734]	Recommendation ITU-T F.734 (2014), Definitions, Requirements, and use cases for Telepresence Systems.

[H.420]	Recommendation ITU-T H.420 (2014), Telepresence System Architecture.

[H.TPS-AV]	Recommendation ITU-T H.yyy (date), Audio/Video Parameters for Telepresence systems.

[ITU-T H.323]	Recommendation ITU-T H.323 v7 (12/2009), Packet-based multimedia communications systems.

[ITU-T H.245]	Recommendation ITU-T H.245 v16 (05/2011), Control protocol for multimedia communication.

[ITU-T H.225.0]	Recommendation ITU-T H.225.0 v7 (12/2009), Call signalling protocols and media stream packetization for packet-based multimedia communication systems.

[ITU-T H.235.0]	Recommendation ITU-T H.235.0 (01/2014), H.323 security: Framework for security in ITU-T H-series (ITU-T H.323 and other ITU-T H.245-based) multimedia systems.

[ITU-T H.323]	Recommendation ITU-T H.323 v7 (12/2009), Packet-based multimedia communications systems.

[ITU-T H.460.1]	Recommendation ITU-T H.460.1 (03/2002), Guidelines for the use of the generic extensible framework.

[IETF CLUE DM]	IETF draft-ietf-clue-data-model-schema-04, An XML Schema for the CLUE data model.

[IETF CLUE FW]	IETF draft-ietf-clue-framework-20, Framework for Telepresence Multi-Streams.

[IETF CLUE SIG] 	IETF draft-ietf-clue-signaling-04, CLUE Signaling.

[IETF CLUE DC]		IETF draft-ietf-clue-datachannel-08, CLUE Protocol Data Channel.

[IETF CLUE PROT]		IETF draft-ietf-clue-protocol-03, CLUE protocol.

[IETF RTCWEB DC]		IETF draft-ietf-rtcweb-data-channel-12, WebRTC Data Channels.

[IETF DCEP]		IETF draft-ietf-rtcweb-data-protocol-09, WebRTC Data Channel Establishment Protocol.

…

10 Security

Endpoints supporting telepresence may use authentication and security in accordance to clause 10.1/[ITU-T H.323] which makes use of [ITU-T H.235.0] and associated documents. Endpoints requiring encrypted transport of CLUE messages shall either use the mechanisms described in H.235.0 to secure H.245 messages or use the DTLS/SCTP data transport method for transporting CLUE messages.

__________________________

____________________