| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4731 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.WEBRTC: Distributed ToIP endpoints in WebRTC |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes a living list item related to the discussion of WebRTC data component "text".

1	Introduction

There was an email based discussion concerning the interpretation of RFC 4103 during idle periods. See attached emails to this contribution.

2	Discussion

The subject as such is firstly independent of WebRTC because also applicable for other gateway supported interworking services related to T.140-over-RTP. For instance, the question applies as well for gateways with V.151 support, global text telephony or alternate speech-text services (see also AVD-4687).

However, we'd like to anchor this topic with H.248.WEBRTC due to the additional aspect of the non-native T.140 transport (T.140-over-SCTP), which raises a couple of side questions.

The detailed discussion is provided in below change proposal.

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4663, the input draft for this meeting.

Change proposal #1:

[Begin Proposed Correction]

### 8.5.4	Requirements related to WEBRTC text

WebRTC text telephony (text conversation) is considered as "data traffic" (despite the fact of the usual RTP-based transport in non-WebRTC environments).

R-8.5.4/1: The H.248 WEBRTC gateway is required to support transparent forwarding of T.140 protocol data units (according to [ITU-T T.140]), independent of the underlying (WebRTC specific or legacy) transport (protocol stack). This includes interworking between WebRTC clients or a WebRTC and non-WebRTC client.

{Editor's note (2015-06): see also ongoing discussions on this aspect (capture in Appendix L6.}

R-8.5.4/2: The H.248 WEBRTC gateway is required to support the underlying transport relay function in interworking between "SCTP/DTLS/UDP" to "RTP/UDP" for T.140 traffic.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

Appendix L6
Living List – Distributed Text-over-IP endpoints for WebRTC data 'text'

(This appendix does only contain working material.)

{Editor's note (2015-06): this item is driven by AVD-4732.}

## L6.1	Purpose

The protocol stack decisions for conversational text within WebRTC services is done by IETF WG RTCWEB, but the evaluation of correspondent interworking services (as in scope of (H.248) WebRTC gateways) raised a couple of questions along the usage of RFC 4103, besides protocol configuration aspects related to SCTP and RTP.

This living list item tries to capture the latest status of this discussion.

## L6.2	Problem statement

Figure L6.1 recalls again the reference architecture for the definition of the V.151 text relay mode ("Text-over-IP", ToIP) in case of end-to-end communication between IP user equipment:

Figure L6.1 – Starting point: legacy ToIP in all-IP networks

It has to be reminded that there are fundamental interactions between the application level framing protocol (= RTP) and the application protocol (= T.140), given by the network transport requirements for communication service "text conversation" (or realtime text).

Figure L6.2 outlines the two main options which were considered for text conversation embedded in WebRTC:

Figure L6.2 – peer-to-peer WebRTC: options for WebRTC realtime text transport

Option #1 represents just the legacy usage of text-over-RTP (i.e., according to the baseline specification [ITU-T V.151]). This approach looks firstly straightforward because

- WebRTC comprises already audio-over-RTP and video-over-RTP, i.e., text-over-RTP would be just the third application component within the WebRTC "RTP suite"; and

- text conversation is inherently of type "realtime" too as the two other conversational realtime components audio and video (thus, transport via the "Realtime Transport Protocol").

Well, option #1 wasn't selected by the IETF ("and the authors of this living list item are not familiar with the actual argumentation behind that decision"). The IETF did decide for option #2, i.e., considering text conversation just as one out of many possible WebRTC data applications. Hence, realtime text uses the WebRTC data channel based transport with its SCTP/DTLS/L4/IP based stack.

Figure L6.3 indicates the associated WebRTC gateway scenario, which is the exclusive scope in this Recommendation:

Figure L6.3 – WebRTC gateway scenario – protocol stack model in user plane

The required WebRTC gateway service for interworking realtime text between WebRTC and non-WebRTC network domains looks straightforward on a first glance:

The H.248 WebRTC media gateway must provide (see clause 8.5.4)
a) a "T.140 PDU transparent forwarding" service besides 
b) the protocol stack interworking of "SCTP/DTLS/L4" to "(S)RTP/UDP".

Whether "T.140 transparent forwarding" is actually possible needs to be evaluated in more detail. IETF RFC 4103 defines behaviour for RTP endsystems (i.e., the RTP source and RTP sink of a T.140 stream), which might need a few modifications (ffs).

More precisely (using the "RTP grouping taxonomy" terminology): there will be a distributed model in case of WebRTC gateways because (see also Figure L6.4) 
a) the T.140 entities "media capture / media renderer". "media source / media sink", "media encoder / media decoder" and "media packetizer / media depacketizer" are located at the remote endsystem (the WebRTC client), and 
b) the media transport level entities "SCTP stream source / SCTP stream sink" and "RTP source / RTP sink" represent MG local functions.

Figure L6.4 – WebRTC gateway scenario – Model of distributed "T.140-over-RTP endpoint"

Another issue is the correct configuration of the underlying protocol stack with respect to support of sufficient reliability for text transport. Figure L6.5 tries to summarize questions for clarification:

Figure L6.5 – WebRTC gateway scenario – Network engineering aspects

Figure L6.6 illustrates the complete, unidirectional media processing model in WebRTC to non-WebRTC direction. It underlines the geographical separation (and SCTP-based interruption) of the normally tighlty coupled functions of the "T.140 media endcoder" (S:3) and the "RTP media packetizer" (S:5).

Figure L6.6 – WebRTC gateway scenario – Functional processing stages of the distributed "T.140-over-RTP endpoint" in WebRTC client to non-WebRTC UE direction

[End Proposed Correction]

Backup material: Email discussion with subject "RFC 4103 ToIP: RTP source behaviour in "Idle Periods"?" from January 2015

- Discussion started with RFC 4103 interpretation (independent of WebRTC, independent of Global Text Telephony, independent of V.151 ToIP)

Hello Albrecht,

Yes, we have had discussions about the interpretation of 5.2 with other implementors as well.

The first paragraph in 5.2 is for the case when no redundancy is used. And the hint then is to transmit a packet with an empty T140block once 300 ms after all valid text has been transmitted. There is no need to start sending any empty T140block after a pause.

But I hope you use redundancy.

The second paragraph is for the case you use redundancy. Then the recommendation is that when no new text is available for transmission, old text is sent at the transmission interval until it has been sent the intended number of times ( recommendation is original plus two redundant transmissions ). The number of generations in the packets are kept the same all the time.

Other implementors have interpreted this as if you should start after an idle period (when you have redundancy negotiated) with just sending the new text without redundancy, then 300 ms later send the text as first generation redundant text, etc.

So I suggest that your implementation is robust enough to receive that pattern.

Transmission during idle periods for keeping connections alive is another issue and may be needed and solved in different ways.

Many solutions send the Unicode character BOM  = EFBBBF in UTF-8. at regular intervals.  It could also be possible to send empty T140blocks.

However RFC 6263 recommends RTCP multiplexed on the RTP port, and that is probably best for cases where such multiplexing is supported.

Please note that you should open the connection after call establishment with sending the BOM character.  This is important of many reasons. One is for opening NAT connections.

Schwarz, Albrecht (Albrecht) skrev den 2015-01-27 09:09:

> Hello Gunnar, Paul,

>

> we are struggling with the description of clause 5.2/RFC 4103:

>

> there might be actually two meaningful behaviours of the RTP source:

>

> O1: sending of a single empty T140block and the start and end of a Idle Period.

GH: No, never at the end of an idle period, and at the beginning only for cases when you do not use redundancy.

> O2: periodical sending of empty T140blocks during the whole Idle Period (e.g., in order to address media inactivity detection).

GH: Treat keep-alive as a separate action from text transmission. But yes, as discussed above, it is possible to use empty T140blocks for keep-alive, but recommended with RTCP multiplexing.

>

> Our interpretation of RFC 4103 is option O1, - also due to the fact that media inactivity detection could be based on the still ongoing RTCP control flow in O2.

>

> Would you confirm?

>

> BTW: V.151 does also not mention O2 (e.g., for RFC 4103 section between two ToIP gateways or a ToIP gateway and IP terminal).

>

> Thanks,

> Albrecht

Gunnar

>

> 5.2. Transmission before and after "Idle Periods"

>

>     When valid T.140 data has been sent and no new T.140 data is

>     available for transmission after the selected buffering time, an

>     empty T140block SHOULD be transmitted.  This situation is regarded as

>     the beginning of an idle period.  The procedure is recommended in

>     order to more rapidly detect potentially missing text before an idle

>     period.

>

>     An empty T140block contains no data.

>

>     When redundancy is used, transmission continues with a packet at

>     every transmission timer expiration and insertion of an empty

>     T.140block as primary, until the last non-empty T140block has been

>     transmitted, as primary and as redundant data, with all intended

>     generations of redundancy.  The last packet before an idle period

>     will contain only one non-empty T140block as redundant data, while

>     the remainder of the redundancy packet will contain empty T140blocks.

>     Any empty T140block sent as primary data MUST be included as

>     redundant T140blocks in subsequent packets, just as normal text

>     T140blocks would be, unless the empty T140block is too old to be

>     transmitted.  This is done so that sequence number inference for the

>     redundant T140blocks will be correct, as explained in Section 4.2.

>

>     After an idle period, the transmitter SHOULD set the M-bit to one in

>     the first packet with new text.

- Discussion moved then to T.140 usage in WebRTC …

It is implicit in all work of RTCWEB and WebRTC that it is only audio and video that use RTP, and everything else use data channels over SCTP.

There is not much said about real-time text, but there is one requirement in draft-ietf-rtcweb-data-channel, that even places the requirement to be solved by a reliable data channel:..."

3.2.  Use Cases for Reliable Data Channels

...

U-C 5:  Realtime text chat during an audio and/or video call with an 
           individual or with multiple people in a conference."

Gunnar

Paul E. Jones skrev den 2015-01-29 09:29:

Gunnar,

So I guess there is still are still conflicting notions.  The IETF drafts discuss text as an RTP type that can be used in RTC Web.  I guess it's the WebRTC work in W3C that is limiting things?  Or is it just the browser vendors?

Paul

------ Original Message ------

…

Sent: 1/29/2015 1:53:48 AM

Subject: Re: RFC 4103 ToIP: RTP source behaviour in "Idle Periods"?

Paul,

I meant two types of media: audio and video.
Even if we consider real-time text to be a media type with real-time requirements close to the ones of audio and video, the standardization groups working with the WebRTC concept are not willing to specify other media types using RTP.  
Gunnar

Paul E. Jones skrev den 2015-01-29 07:49:

Gunnar,

"In WebRTC, only two media using RTP are allowed"

Where is that stated?  I don't see such limitations with specs like draft-ietf-rtcweb-rtp-usage.  Is this just a current browser limitation?  While I've not been tinkering with the browser APIs, I can tell you all the work I've been doing in this space fully anticipates a multiplicity of media flows from endpoints.

So, if I understand what this dialog is about, there is a belief that the browser cannot support more than a single audio and video flow?  So a workaround is underway?

Paul

------ Original Message ------

…

Sent: 1/29/2015 12:41:36 AM

Subject: Re: RFC 4103 ToIP: RTP source behaviour in "Idle Periods"?

For me, the reason for interoperability needs for real-time text between RTP carried RFC 4103 and  WebRTC can be anything.
 
RFC 4103 is specified for real-time text in 3GPP IMS Multimedia Telephony, in plain SIP total conversation, in NG911 and NG112 emergency service specifications from EENA and NENA, and in ETSI accessible procurement standard. 
In WebRTC, only two media using RTP are allowed; video and audio, so another mechanism than RTP needs to be used for real-time text. The data channel is mentioned for real-time text in some RTCWEB documents but without details. It is well timed to specify these details now, and I assume that that is the goal of the discussion, including interoperability.

I agree that the unreliable and semi reliable data channels would require us to invent an RFC 4103-like protocol on top of the RTCWEB data channel to be able to detect and indicate loss. 

But will it not be the same problem with the reliable channel? If it gets into trouble, it will break the association, without telling the application where in the stream it left off, so when the association is reconnected, the receiving application will not be sure if received data is in sequence with what was received before the break.

Or is the hope that such SCTP association breakdowns will be so rare that we can just as well disconnect the call when it happened?
Gunnar

 --------------------------------------------------------------------------------------
Paul E. Jones skrev den 2015-01-29 01:57:

I still feel like I'm missing some context.

Is it that the LTE network are implementing RFC 4103 and you want to interwork with a WebRTC client outside the LTE network using T.140 over the data channel?  Or is it something else?

What standard or draft documents use of real-time text in the WebRTC data channel?

Paul

------ Original Message ------

…

Sent: 1/28/2015 5:36:07 PM

Subject: RE: RFC 4103 ToIP: RTP source behaviour in "Idle Periods"?

>If the desire is to associate the text with the data channel, then we should ask these questions. Personally, I'd go for the reliable transport. Delays only exist under congestion and congestion effects both reliable and unreliable transports.
[Raju]

Though the initial focus was not WebRTC data channel interworking with native T.140 endpoints, since the thread already discussed some details I will go ahead and comment on the above point.

I also think reliable SCTP data channel is more appropriate. The reason is, if we use unreliable data channels then the lost “text” in SCTP layer can’t be conveyed end to end while interworking with IMS RTP endpoint as RTP endpoint depends on seq# to detect loss and there won’t be any missing seq # in this case.

BR

Raju

# […]

Gunnar,

If the desire is to associate the text with the data channel, then we should ask these questions. Personally, I'd go for the reliable transport. Delays only exist under congestion and congestion effects both reliable and unreliable transports.

But this really isn't RFC 4103 any longer, but just T.140 over SCTP. That's reasonable to have, especially in WebRTC.

Perhaps I misinterpreted the thread. Are these flows with RFC 4103 starting as WebRTC traffic? I assumed so and wondered why there was a conversion to SCTP at some later point. If this is to provide interworking legacy SIP with WebRTC, then the conversion makes sense to me.

Paul

# From: "Gunnar Hellström" <gunnar.hellstrom@omnitor.se>
Sent: January 28, 2015 1:40:23 AM EST
…
Subject: Re: RFC 4103 ToIP: RTP source behaviour in "Idle Periods"?

My question was the other way around.

Since this is for WebRTC, then SCTP is mandatory for data transport.

Then I wonder what taste of SCTP shall we use?

There are the unreliable, semi-reliable and reliable.

When we have SCTP, it looks tempting to use it to provide some reliability. But we want to avoid delays and risks for long periods of blocked communication. So I tend to think that the reliable mode is not so attractive for this case since it can have blockings of up to 15 seconds in bad network conditions.

Therefore I tend to think that the semi-reliable SCTP with a maximum life time of about 5 seconds would be optimal for real-time text. But there are problems with the semi-reliable method that no reports about loss seem to be guaranteed. So, do we need to add our own sequence numbering on top of the SCTP transport in order to detect loss.

That is the kind of questions I have had so far. In what direction do others think?

So far, we have used WebSockets for real-time text in WebRTC, waiting for the data protocol discussions and specifications to settle, but I am eager to get to some conclusions and specifications done.

Gunnar

Paul E. Jones skrev den 2015-01-27 23:49:

Albrecht,

I'm also curious why folks wish to put the RTP-based RFC 4103 media inside SCTP.  Is it just to improve reliability?  I recall back when Gunnar and I were looking at how many levels of redundancy, it was determined that, outside of bursty packet loss, just one or two levels of redundancy provided a near 100% guarantee of delivery.

Is it that the networks on the other side of the GW are highly susceptible to bursty loss? If it's fairly random loss, I would just stick with RFC 4103 and not perform the conversion.

Paul

------ Original Message ------

# From: "Schwarz, Albrecht (Albrecht)"

…

Sent: 1/27/2015 5:07:32 PM

Subject: RE: RFC 4103 ToIP: RTP source behaviour in "Idle Periods"?

[Cc'd some ITU-T Q.3/16 and 3GPP CT4 colleagues due to H.248 WEBRTC  gateway topic ("which of course should be addressed also by draft-alvestrand-rtcweb-gateways"). There are a couple of questions  arround the WebRTC data channel interworking related to T.140 text  conversation, even in application agnostic mode ...]

Hello Gunnar,

pls find my replies below inline.

Regards,

Albrecht

-----Original Message-----

# From: Gunnar Hellström

…

Subject: Re: RFC 4103 ToIP: RTP source behaviour in "Idle Periods"?

Albert,

It is good that you are working on a WebRTC carried RTT.

I assume that you then want to use what is called "reliable" or "semi-reliable" transport on SCTP, and convert that to/from RTP with redundancy.

This should be doable, but you are right in that it is needed to know what to do in all situations with transmission of text, redundant text, recovery at loss, insertions in case of loss, keep-alive, breakdown of the SCTP association etc.

Are there any draft 3GPP specifications on this?

[[ABS] ] No, not yet addressed in 3GPP because part of the WebRTC data channel work which was postponed from 3GPP Rel-12 to Rel-13. We are just establishing the new work item description. Actually the RTT interworking topic could be explicitly added.

And H.248.x ?

[[ABS] ] Not yet any description in detail, but subject is already addressed, primarily by work item H.248.WEBRTC and also partially by H.248.STGROUP (note: contribution 676 "H.248.STGROUP: model discussion (1) for use case "alternate speech-text telephony" discusses a RTT use case, but without yet focusing on the T.140 level as discussed below.)

Regards

Gunnar

# From: Gunnar Hellström [mailto:gunnar.hellstrom@omnitor.se] 
Sent: Thursday, January 29, 2015 7:08 AM

Raju,
Already in current RFC 4103 implementations there are cases when the replacement character can only mean that loss MAY have happened.

[Raju] I see. Thanks for the clarification. This is good input for the T.240 to data channel interworking work.

You say: 
"Well, I am sure we need lot more discussions when such interworking work actually starts in relevant standards’ forums."
What did we start yesterday with the discussion on the behavior at the edges of idle periods? Was that and today's discussion not a discussion on interworking close to a relvant standards forum? Can it be continued right away or are you just planning some Work Item?

[Raju] Albrecht may answer this question in a more accurate way about contributions etc. The context of our initial discussion was NOT webrtc. For T.140 to data channel interworking, we need a draft similar to draft-ejzak-mmusic-msrp-usage-data-channel for T.140 and then it has to be adopted by 3gpp.

BR

Raju

Gunnar

Makaraju, Maridi Raju (Raju) skrev den 2015-01-29 12:44:

Gunnar,

Thanks for the discussion.

>But it may perhaps happen so seldomly that we can specify to insert a mark for >possible loss in the incoming text stream at the point where a reconnect was needed. I would vote for that approach and not complicate it further. The intention of RTT is conversational text between humans anyway.
[Raju] This crossed my mind. But again, such design not only complicates it a bit, but also I am not sure indicating “possible loss” is unambiguous enough indication as actual loss may or may not have happened. But, such design is not a deal breaker again.. so I am ok with it too.

Well, I am sure we need lot more discussions when such interworking work actually starts in relevant standards’ forums.

BR

Raju

# From: Gunnar Hellström [mailto:gunnar.hellstrom@omnitor.se] 
Sent: Thursday, January 29, 2015 5:25 AM
…
Subject: Re: RFC 4103 ToIP: RTP source behaviour in "Idle Periods"?

Makaraju, Maridi Raju (Raju) skrev den 2015-01-29 12:09:

> Or is the hope that such SCTP association breakdowns will be so rare that we can just as well disconnect the call when it happened?

[Raju] IMHO, this is one ‘reasonable’ design.

I am not so sure that it is reasonable to hang up if the sctp association is broken. I have seen developers discussions saying that they get frequent breakdowns of the data channel, and since the association holds all data channels, a heavy data transport causing choked channel may cause breakdown of also the lightweight real-time text channel. But it may perhaps happen so seldomly that we can specify to insert a mark for possible loss in the incoming text stream at the point where a reconnect was needed. I would vote for that approach and not complicate it further. The intention of RTT is conversational text between humans anyway.

Gunnar

BR

Raju

# From: Gunnar Hellström [mailto:gunnar.hellstrom@omnitor.se] 
Sent: Wednesday, January 28, 2015 11:42 PM
…
Subject: Re: RFC 4103 ToIP: RTP source behaviour in "Idle Periods"?

For me, the reason for interoperability needs for real-time text between RTP carried RFC 4103 and  WebRTC can be anything.
 
RFC 4103 is specified for real-time text in 3GPP IMS Multimedia Telephony, in plain SIP total conversation, in NG911 and NG112 emergency service specifications from EENA and NENA, and in ETSI accessible procurement standard. 
In WebRTC, only two media using RTP are allowed; video and audio, so another mechanism than RTP needs to be used for real-time text. The data channel is mentioned for real-time text in some RTCWEB documents but without details. It is well timed to specify these details now, and I assume that that is the goal of the discussion, including interoperability.

I agree that the unreliable and semi reliable data channels would require us to invent an RFC 4103-like protocol on top of the RTCWEB data channel to be able to detect and indicate loss. 

But will it not be the same problem with the reliable channel? If it gets into trouble, it will break the association, without telling the application where in the stream it left off, so when the association is reconnected, the receiving application will not be sure if received data is in sequence with what was received before the break.

Or is the hope that such SCTP association breakdowns will be so rare that we can just as well disconnect the call when it happened?
Gunnar

 --------------------------------------------------------------------------------------
Paul E. Jones skrev den 2015-01-29 01:57:

I still feel like I'm missing some context.

Is it that the LTE network are implementing RFC 4103 and you want to interwork with a WebRTC client outside the LTE network using T.140 over the data channel?  Or is it something else?

What standard or draft documents use of real-time text in the WebRTC data channel?

Paul

------ Original Message ------

# From: "Makaraju, Maridi Raju (Raju)" <Raju.Makaraju@alcatel-lucent.com>

…

Sent: 1/28/2015 5:36:07 PM

Subject: RE: RFC 4103 ToIP: RTP source behaviour in "Idle Periods"?

>If the desire is to associate the text with the data channel, then we should ask these >questions. Personally, I'd go for the reliable transport. Delays only exist under >congestion and congestion effects both reliable and unreliable transports.
[Raju]

Though the initial focus was not WebRTC data channel interworking with native T.140 endpoints, since the thread already discussed some details I will go ahead and comment on the above point.

I also think reliable SCTP data channel is more appropriate. The reason is, if we use unreliable data channels then the lost “text” in SCTP layer can’t be conveyed end to end while interworking with IMS RTP endpoint as RTP endpoint depends on seq# to detect loss and there won’t be any missing seq # in this case.

BR

Raju

# From: Paul E. Jones [mailto:paulej@packetizer.com] 
Sent: Wednesday, January 28, 2015 12:52 AM
To: Gunnar Hellström; Schwarz, Albrecht (Albrecht)
Cc: LANDAIS, BRUNO (BRUNO); HE, BING (BING); Stoetzer-Bradler, Juergen (Juergen); Makaraju, Maridi Raju (Raju); CALME, Jim (Jim)** CTR **; Christian Groves; Yangweiwei (Tommy); Arturo Martin De Nicolas (AC/EED); Kall, Jan (NSN - FI/Espoo); Belling, Thomas (NSN - DE/Munich); Nevenka Biondic; Peter Schmitt
Subject: Re: RFC 4103 ToIP: RTP source behaviour in "Idle Periods"?

Gunnar,

If the desire is to associate the text with the data channel, then we should ask these questions. Personally, I'd go for the reliable transport. Delays only exist under congestion and congestion effects both reliable and unreliable transports.

But this really isn't RFC 4103 any longer, but just T.140 over SCTP. That's reasonable to have, especially in WebRTC.

Perhaps I misinterpreted the thread. Are these flows with RFC 4103 starting as WebRTC traffic? I assumed so and wondered why there was a conversion to SCTP at some later point. If this is to provide interworking legacy SIP with WebRTC, then the conversion makes sense to me.

Paul

# From: "Gunnar Hellström" <gunnar.hellstrom@omnitor.se>
Sent: January 28, 2015 1:40:23 AM EST
…
Subject: Re: RFC 4103 ToIP: RTP source behaviour in "Idle Periods"?

My question was the other way around.

Since this is for WebRTC, then SCTP is mandatory for data transport. Then I wonder what taste of SCTP shall we use? There are the unreliable, semi-reliable and reliable.

When we have SCTP, it looks tempting to use it to provide some reliability. But we want to avoid delays and risks for long periods of blocked communication. So I tend to think that the reliable mode is not so attractive for this case since it can have blockings of up to 15 seconds in bad network conditions.

Therefore I tend to think that the semi-reliable SCTP with a maximum life time of about 5 seconds would be optimal for real-time text. But there are problems with the semi-reliable method that no reports about loss seem to be guaranteed. So, do we need to add our own sequence numbering on top of the SCTP transport in order to detect loss.

That is the kind of questions I have had so far. In what direction do others think? So far, we have used WebSockets for real-time text in WebRTC, waiting for the data protocol discussions and specifications to settle, but I am eager to get to some conclusions and specifications done.

Gunnar

Paul E. Jones skrev den 2015-01-27 23:49:

Albrecht,

I'm also curious why folks wish to put the RTP-based RFC 4103 media  inside SCTP.  Is it just to improve reliability?  I recall back when  Gunnar and I were looking at how many levels of redundancy, it was  determined that, outside of bursty packet loss, just one or two levels  of redundancy provided a near 100% guarantee of delivery.

Is it that the networks on the other side of the GW are highly  susceptible to bursty loss? If it's fairly random loss, I would just stick with RFC 4103 and not perform the conversion.

Paul

------ Original Message ------

..

Sent: 1/27/2015 5:07:32 PM

Subject: RE: RFC 4103 ToIP: RTP source behaviour in "Idle Periods"?

[Cc'd some ITU-T Q.3/16 and 3GPP CT4 colleagues due to H.248 WEBRTC  gateway topic ("which of course should be addressed also by  draft-alvestrand-rtcweb-gateways"). There are a couple of questions  arround the WebRTC data channel interworking related to T.140 text  conversation, even in application agnostic mode ...]

Hello Gunnar,

pls find my replies below inline.

Regards,

Albrecht

-----Original Message-----

# From: Gunnar Hellström

[mailto:gunnar.hellstrom@omnitor.se]

Sent: Dienstag, 27. Januar 2015 14:11

…

Subject: Re: RFC 4103 ToIP: RTP source behaviour in "Idle Periods"?

Albert,

It is good that you are working on a WebRTC carried RTT.

I assume that you then want to use what is called "reliable" or "semi-reliable" transport on SCTP, and convert that to/from RTP with  redundancy.

This should be doable, but you are right in that it is needed to know  what to do in all situations with transmission of text, redundant  text, recovery at loss, insertions in case of loss, keep-alive,  breakdown of the SCTP association etc.

Are there any draft 3GPP specifications on this?

[[ABS] ] No, not yet addressed in 3GPP because part of the WebRTC data channel work which was postponed from 3GPP Rel-12 to Rel-13. We  are just establishing the new work item description. Actually the RTT interworking topic could be explicitly added.

And H.248.x ?

[[ABS] ] Not yet any description in detail, but subject is already  addressed, primarily by work item H.248.WEBRTC and also partially by  H.248.STGROUP (note: contribution 676 "H.248.STGROUP: model  discussion (1) for use case "alternate speech-text telephony"  discusses a RTT use case, but without yet focusing on the T.140 level  as discussed below.)

Regards

Gunnar

Schwarz, Albrecht (Albrecht) skrev den 2015-01-27 12:14:

Thanks Gunnar for your insights!

("we are considering here

1st the 3GPP 26.114 real-time text mode which mandates redundancy; but  also 2nd H.248 WebRTC gateways (with "T.140/SCTP/DTLS/L4" to  "T.140/RTP/UDP" interworking), where we got to consider transparent  forwarding of T.140 protocol data units plus RFC 4103 behaviour in the  GW-embedded RTP endsytem" ...)

Best regards,

Albrecht

____________________