| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4733 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | H.248.WebRTC/H.248.SCTP: Autonomous SCTP stream reset |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This contribution proposes a solution for sending of MG autonomous SCTP stream resets in response to a SCTP stream reset request.

# 2. Discussion

Clause 9.6.5 “Data channel closure” contains an editor’s note regarding the WebRTC Data channel behaviour that a reset received on one SCTP stream results in the generation of an SCTP stream result on an outgoing stream with the same identity. The current text allows this behaviour through MGC controlled actions, i.e. the MGC receives a notification that a reset has been received and it then requests the MG to initiate an outgoing SCTP stream reset. However at the last Q3 meeting the experts expressed a desire for a mechanism that allows an MG to autonomously send a reset in response to receiving a reset.

The editor’s note gives three possible solutions:

- Updating the SCTP reset package to incorporate an additional value for outresp in the detreset event.

- An extension in H.248.WebRTC that extends the SCTP reset package with the above value. This keeps the procedures related to data channel which specifies that an endpoint should initiate an reset in response to an incoming reset.

- An embedded initreset signal that is triggered when the outresp event indicates to accept incoming resets.

Briefly analysing the solutions the following positives and negatives have been identified:

Solution 1 (Proposal Option B)

Positives:

- Minimal additional protocol updates, one parameter value.

Negatives:

- Introduces WebRTC functionality into H.248.SCTP.

- Usually H.248 signals are used to initiate outgoing protocol messages.

Solution 2 (Proposal Option C)

Positives:

- Keeps WebRTC datachannel specific procedures out of H.248.SCTP.

Negatives:

- An extra package

- Introduces WebRTC functionality into H.248.SCTP.

- Usually H.248 signals are used to initiate outgoing protocol messages.

Solution 3 (Proposal Option A)

Positives:

- It is a procedural approach rather than defining new package elements.

- It does not introduce WebRTC specific behaviour into SCTP packages.

- The embedded signal mechanism is an existing H.248 mechanism.

Negatives:

- This solution would however mean that the sctpid parameter in the sctpreset/initreset signal would have to be optional for use in an embedded SignalsDescriptor due to the fact that the SCTP Stream ID would be unknown by the MGC.

There doesn’t appear to be any significant positive or negative for any of the solutions. The Contributors have a slight preference for Solution 1 (Option B).

# 3 Proposal

It is proposed to accept one of the following options. The changes for H.248.WEBRTC are marked up against TD 220R1/WP1 from the Geneva, February Q5/16 Rapporteurs meeting. The changes for H.248.SCTP are marked up against TD 225r2/WP1 from the same meeting.

Option A

Draft new Recommendation ITU-T H.248.xx (ex H.248.WEBRTC)

Web-based real-time communication services – H.248 protocol support and profile guidelines

### …

### 9.6.5 Data channel closure

WebRTC data channels are closed according to the procedures in clause 6.7/[draft-ietf-rtcweb-data-channel-12]. The procedures use the mechanism in [IETF RFC 6525] to support the closure of an SCTP Stream. The “SCTP Re-configuration Stream Reset Package” (sctpreset) allows for the support of the RE-CONFIG outgoing SCTP stream reset. This package shall be implemented if the dcep package is used.

To initiate a channel closure the MGC shall use the “Initiating an outgoing SCTP stream reset” procedures defined in clause 10.6.2/[ITU-T H.248.SCTP].

The MGC shall also use the “Responding to an outgoing SCTP stream reset request” procedures in clause 10.6.3/[ITU-T H.248.SCTP] in order to respond to remote channel closure requests.

An MGC may decide whether it will respond (MGC controlled)to a channel closure request or will allow the MG to autonomously respond (MG autonomous).

In the MGC controlled mode in addition to the procedures in clause 10.6.3/[ITU-T H.248.SCTP] the MGC should initiate a channel closure for the indicated SCTP Streams via the procedure in clause 10.6.2/[ITU-T H.248.SCTP] if it hasn’t done so already.

In the MG autonomous mode to allow the MG to respond autonomously the MGC shall when setting the “Detect outgoing SCTP stream reset” (sctpreset/detreset) Event include the “Initiate Outgoing SCTP Stream Reset” (sctpreset/initreset) Signal as an embedded signal (see clause 7.1.9.8/[ITU-T H.248.1]). The SCTP StreamID signal parameter is not included. On detection of the sctpreset/detreset Event the MG will initiate an outgoing SCTP stream reset on the SCTP stream ID associated with the Event according to 10.6.2/[ITU-T H.248.SCTP].

### …

Draft new ITU-T Recommendation H.248.SCTP

Gateway control protocol: 
H.248 support for control of SCTP bearer connections

### …

### 10.2.2	Outgoing SCTP Stream Reset Result

|  | Event name: | Outgoing SCTP Stream Reset Result |
| --- | --- | --- |
|  | Event ID: | result (0x0002) |
|  | Description: | This Event indicates the result of an outgoing SCTP stream reset sent to the remote peer (i.e. by the “Initiate Outgoing SCTP Stream Reset” (sctpreset/initreset) Signal). |

#### 10.2.2.1	EventsDescriptor parameters

None.

#### 10.2.2.2	ObservedEventsDescriptor parameters

##### 10.2.2.2.1	SCTP StreamID

|  | Parameter name: | SCTP StreamID |
| --- | --- | --- |
|  | Parameter ID: | sctpid (0x0001) |
|  | Description: | This parameter indicates the SCTP StreamID/s contained in the Re-configuration response to the original outgoing SCTP stream reset request. |
|  | Type: | Sub-list of Integer |
|  | Optional: | Yes Note: This parameter is only optional if the signal has been set in an embedded SignalsDescriptor. The MG shall respond with error code 457 “Missing parameter in signal or event”. |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

…

### 10.6.2 Initiating an outgoing SCTP stream reset

To request the MG to send an outgoing stream reset on a particular SCTP Stream/s the MGC shall send the “Initiate Outgoing SCTP Stream Reset” (sctpreset/initreset) Signal with the applicable SCTP streamIDs in the sctpid parameter. If the sctpreset/initreset Signal is set as part of an embedded SignalsDescriptor it may omit the sctpid parameter in which case the MG determines the SCTP id to use. On reception of the Signal the MG shall send an outgoing stream reset request with the applicable streamIDs according to clause 5.1.2/[IETF RFC 6525].

In order to determine the result of the request the MGC should also set the “Outgoing SCTP Stream Reset Result” (sctpreset/result) Event on the MG. On reception of the re-configuration response the MG shall perform the procedures from clause 5.2.7/[IETF RFC 6525].

Option B

Draft new Recommendation ITU-T H.248.xx (ex H.248.WEBRTC)

Web-based real-time communication services – H.248 protocol support and profile guidelines

### …

### 9.6.5 Data channel closure

WebRTC data channels are closed according to the procedures in clause 6.7/[draft-ietf-rtcweb-data-channel-12]. The procedures use the mechanism in [IETF RFC 6525] to support the closure of an SCTP Stream. The “SCTP Re-configuration Stream Reset Package” (sctpreset) allows for the support of the RE-CONFIG outgoing SCTP stream reset. This package shall be implemented if the dcep package is used.

To initiate a channel closure the MGC shall use the “Initiating an outgoing SCTP stream reset” procedures defined in clause 10.6.2/[ITU-T H.248.SCTP].

The MGC shall also use the “Responding to an outgoing SCTP stream reset request” procedures in clause 10.6.3/[ITU-T H.248.SCTP] in order to respond to remote channel closure requests.

If the MGC requires that a MG respond to a received outgoing stream reset request by autonomously sending its own outgoing reset request the MGC shall set the sctpreset/initreset Signal parameter outresp to “accint”.

### …

Draft new ITU-T Recommendation H.248.SCTP

Gateway control protocol: 
H.248 support for control of SCTP bearer connections

### …

### 10.2.1	Detect outgoing SCTP stream reset

|  | Event name: | Detect outgoing SCTP stream reset |
| --- | --- | --- |
|  | Event ID: | detreset (0x0001) |
|  | Description: | This Event allows the MG to detect outgoing SCTP stream reset requests from remote peers. It allows the MGC to be notified of the reception of an outgoing SCTP stream reset and to respond to the request. Alternatively it allows the MG to autonomously respond to the outgoing SCTP stream reset by sending its own response.. |

#### 10.2.1.1	EventsDescriptor parameters

##### 10.2.1.1.1	Outgoing stream reset response

|  | Parameter name: | Outgoing stream reset response |
| --- | --- | --- |
|  | Parameter ID: | outresp (0x0001) |
|  | Description: | This parameter indicates to the MG whether it should respond autonomously to the reception of a valid outgoing SCTP stream reset from the remote peer by initiating its own outgoing SCTP stream reset. |
|  | Type: | Enumeration |
|  | Optional: | Yes |
|  | Possible values: | “accept”; The MG shall accept the outgoing SCTP stream reset request and apply the procedures from clause 5.2.2./[IETF RFC6525].. “accinit”; The MG shall perform the procedures related to “accept” and in addition shall send an outgoing SCTP stream reset request with the applicable streamIDs according to clause 5.1.2/[IETF RFC 6525].. “deny”; The MG shall deny the outgoing SCTP stream reset request from clause 5.2.1./[IETF RFC6525].. “mgc”; The MG shall wait for an “Outgoing SCTP Stream Reset Response” (sctpreset/resetresp) Signal a indicating the action to take before sending a re-configuration response.. |
|  | Default: | “accept” |

…

### 10.6.3 Responding to an outgoing SCTP stream reset request

To detect and respond to outgoing SCTP stream reset requests from the remote peer the MGC shall set the “Detect outgoing SCTP stream reset” (sctpreset/detreset) Event. If the MGC requires that the MG autonomously send re-configuration responses it shall set the outresp parameter to “accept” or “deny”. If the MGC requires that in addition to “accept” behaviour that the MG shall autonomously generate an outgoing SCTP stream reset request it shall set the outresp parameter to “accinit”. Otherwise, it shall set the outresp parameter to “mgc” and the MGC shall indicate via the “Outgoing SCTP Stream Reset Response” (sctpreset/resetresp) with the applicable SCTP Stream ids. As the re-configuration response parameter does not contain a list of SCTP stream ids but rather a "re-configuration response sequence number", the MG needs to map this list of SCTP stream ids (as received from the MGC) to the "re-configuration request sequence number", which was contained in the "outgoing SSN reset request parameter" it had previously received (and whose "reconfiguration request sequence number" it has stored).

If an outgoing SCTP stream reset request is sent as a result of setting “accinit” the MG shall use the SCTP stream id/s received in the outgoing stream reset request that generated the ObservedEvent.

NOTE – The MG depends on the list of SCTP stream ids in the sctpreset/resetresp signal being identical to the list of SCTP stream ids, which it had sent to the MGC in the sctpreset/detreset event's "sctpid" ObservedEvents Descriptor parameter.

Signal what action to take. On reception of the Signal the MG shall act according to the relevant parameter value in x.2.1.1.1.

Option C

Draft new Recommendation ITU-T H.248.xx (ex H.248.WEBRTC)

Web-based real-time communication services – H.248 protocol support and profile guidelines

### …

### 9.6.5 Data channel closure

WebRTC data channels are closed according to the procedures in clause 6.7/[draft-ietf-rtcweb-data-channel-12]. The procedures use the mechanism in [IETF RFC 6525] to support the closure of an SCTP Stream. The “SCTP Re-configuration Stream Reset Package” (sctpreset) allows for the support of the RE-CONFIG outgoing SCTP stream reset. This package shall be implemented if the dcep package is used.

To initiate a channel closure the MGC shall use the “Initiating an outgoing SCTP stream reset” procedures defined in clause 10.6.2/[ITU-T H.248.SCTP].

The MGC shall also use the “Responding to an outgoing SCTP stream reset request” procedures in clause 10.6.3/[ITU-T H.248.SCTP] in order to respond to remote channel closure requests.

If the MGC requires that a MG respond to a received outgoing stream reset request by autonomously sending its own outgoing reset request the MGC shall set the dcreset/initreset Signal parameter outresp to “accint”.

### …

x	WebRTC data channel reset package

|  | Package name: | WebRTC data channel reset package |
| --- | --- | --- |
|  | Package ID: | dcreset (0x????) |
|  | Description: | This package introduces an extension to the sctpreset package to allow an MG to autonomously respond to a received outgoing stream reset by sending an outgoing stream reset. |
|  | Version: | 1 |
|  | Extends: | sctpreset (0x0122) version 1 |

x.1	Properties

As per the sctpreset package in [ITU-T H.248.SCTP].

x.2	Events

As per the sctpreset package in [ITU-T H.248.SCTP] with an additional value added to the “Detect outgoing SCTP stream reset” (detreset) “Outgoing stream reset response” (outresp) parameter as per below:

|  | Possible values: | “accinit”; The MG shall perform the procedures related to “accept” and in addition shall send an outgoing SCTP stream reset request with the applicable streamIDs according to clause 5.1.2/[IETF RFC 6525].. |
| --- | --- | --- |

x.3	Signals

As per the sctpreset package in [ITU-T H.248.SCTP].

x.4	Statistics

As per the sctpreset package in [ITU-T H.248.SCTP].

x.5	Error Codes

As per the sctpreset package in [ITU-T H.248.SCTP].

x.6	Procedures

In addition to the procedures of clause 10.6.3/[ITU-T H.248 SCTP]:

If the MGC requires that in addition to “accept” behaviour that the MG shall autonomously generate an outgoing SCTP stream reset request it shall set the outresp parameter to “accinit”.

If an outgoing SCTP stream reset request is sent as a result of setting “accinit” the MG shall use the SCTP stream id/s received in the outgoing stream reset request that generated the ObservedEvent.

__________________________

____________________