| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4745 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Spranto |  |  |  |
| Title: | H.225.0 Time Synchronization Mechanism |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This proposal is to add support for Time Synchronization from H.323 endpoints to a registering Gatekeeper. It is proposed to add an optional field to the GatekeeperRequest (GRQ) and an optional response in the GatekeeperConfirm (GCF) for the purpose of time synchronization between the two devices. This time synchronization may be used for a variety of purposes including time display to the user, time dependent security mechanisms and call record purposes.

# Introduction

This contribution was originally proposed to the last study group meeting in Geneva as C.704 and was designed solely for security mechanisms. The experts felt there may be broader application for the contribution. At that meeting the experts commented.

This could be useful even outside of H.235 and experts felt that perhaps we should add these procedures directly to H.225.0.  The contributor was encouraged to re-submit this proposal to the next meeting with changes against H.225.0 to achieve the same result.  Alternatively, the contributor can provide more justification as to why it should be a stand-alone H.235-series Recommendation.

As proposed by the experts in Q2, the contributor respectfully requests to broaden the scope of the contribution and apply directly to the procedures of H.225.0. It is envisage that these changes would be effectual of H.225.0 (rev 8) currently under review.

# Discussion

The proposed changes are put forward for consideration.

# Conclusion

This proposal describes changes to native H.225.0 RAS signalling for the purpose of synchronising endpoint time against a gatekeeper during registration. Time synchronization can be used for updating simple time display on the endpoint, time sensitive H.235.x profiles especially for registration , Call record timing or for other general implementers purposes.

# Proposed H.225.0 Textual changes

The following textual changes are proposed for H.225.0

7.8.1 GatekeeperRequest (GRQ)

Note that one GRQ is sent per logical endpoint; thus, an MCU or a Gateway might send many.

The GRQ message includes the following:

…

supportsAssignedGK – Indicates whether the endpoint supports the assigned gatekeeper

mechanism.

assignedGatekeeper – This field indicates the endpoint's current assigned gatekeeper.

supportsTimeSync – Indicates whether the endpoint supports time synchronization.

7.8.2 GatekeeperConfirm (GCF)

The GCF message includes the following:

…

assignedGatekeeper – Assigned gatekeeper of the endpoint.

rehomingModel – Indicates the mechanism to be used by the endpoint for re-registering to the

assigned gatekeeper.

timeSynchronization – Where the endpoint supports time synchronization (supportsTimeSync field present in GRQ) then return the gatekeeper’s current UTC time.

# Proposed H.225.0 ASN.1 Changes

The following ASN.1 changes are proposed.

GatekeeperRequest ::= SEQUENCE --(GRQ)

{

...

supportsAssignedGK              BOOLEAN,

assignedGatekeeper              AlternateGK OPTIONAL,

supportsTimeSync                NULL OPTIONAL

}

GatekeeperConfirm ::= SEQUENCE --(GCF)

{

...

assignedGatekeeper              AlternateGK OPTIONAL,

rehomingModel                   RehomingModel OPTIONAL,

timeSyncronization              TimeStamp OPTIONAL

}

__________________