| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4760 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Ericsson |  |  |  |
| Title: | H.248.STGROUP Miscellaneous updates |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution from Ericsson proposes a number of updates in the H.248.STGROUP draft related to the aggregation and de-aggregation streams and concepts.

# Discussion

1.1 Text for summary clauses

The description of aggregation and de-aggregation concepts and streams in the AAP summary and  in the summary clause does not distinguish clearly between both concepts. The text will benefit from this distinction, as both concepts serve different purposes.

1.2 Description of de-aggregation stream

The example chosen to illustrate the de-aggregation stream concept seems more suitable for an aggregation in reverse direction, i.e. when the aggregation streams sources media to the component streams. The example is modified to make it more applicable for de-aggregation.

1.3 Use of properties in aggregation and de-aggregation stream

The current draft describes that in cases of aggregation the properties in local control, local and remote descriptors are to be set in the external streams, the component streams. In the case of de-aggregation, the general rule is also to set the properties in the external stream, i.e. the de-aggregation stream, but it was already foreseen that the internal streams, i.e. the component streams, may have also properties.

In the contributor’s view, the difference is due to the different nature of the de-aggregation and aggregation concepts. The former is defined as a sort of (de-)multiplexing of several streams over the same transport. There is a distinct layer separation between the de-aggregation and the component stream. The de-aggregation stream is in general not aware of upper layer information, e.g. CLUE information that could be transported in an SCTP stream. For this reason, properties that affect the upper layer and that are not relevant to the de-aggregation stream should be set in the component stream. The rest of the properties should be defined in the external stream and conflicts  between the external and the internal stream caused by double definition should be avoided, as for the aggregation case.

# Proposals

The contribution proposes text to tackle the issues described above as well as some corrections, editorials and other enhancements.

[Begin Correction]

AAP Summary

This Recommendation discusses how H.248 streams may be grouped according to a particular semantic and how they are aggregated or de-aggregated internally in the Context for the execution of H.248 control actions. It defines a H.248 package that allows an explicit grouping semantic to be assigned to a group of streams. It also introduces the concept and support for a new type of H.248 stream, the “aggregation stream”, the “de-aggregation stream” and the “component stream”, to be used only in combination with stream grouping. The “aggregation stream” allows H.248 to better support operations on streams which are grouped. The “de-aggregation stream”  allows H.248 to better support multiple media streams which share a common transport.

[End Correction]

[Begin Correction]

Summary

This Recommendation discusses how H.248 streams may be grouped according to a particular semantic and how they are aggregated or de-aggregated internally in the Context for the execution of H.248 control actions. It defines a H.248 package that allows an explicit grouping semantic to be assigned to a group of streams. It also introduces the concept and support for new types of H.248 stream, the “aggregation stream”, the “de-aggregation stream” and the “component stream”, to be used only in combination with stream grouping.

The ability to group streams and to provide an explicit semantic allows the media gateway controller to control the processing of multiple streams. The concept of aggregation  allows Signals, Events, Statistics and Topology to be applied to an aggregation stream with benefits including for example: the ability to report cumulative Statistics rather than multiple individual statistics or report a single Event rather multiple events on the component streams.  The concept of de-aggregation allows a better handle of multiple streams that share the same transport, for example with regards to the different time dynamics of the transport layer and the upper layer streams.

[End Correction]

2.2 Editorial

[Begin Correction]

# 1	Scope

As described by the H.248 connection model (clause 6/[ITU-T H.248.1) a termination sources and/or sinks one or more streams. In a multimedia conference, a termination can be multimedia and sources or sinks multiple media streams. The H.248 protocol uses the stream concept to model individual media flows. That is typically there is one media (and optional control flow) per H.248 stream. The characteristics of each stream are described through the use of the local control, local and remote descriptors. Each termination contains a group of streams. H.248 makes no assumptions as to the relationship between the streams on a termination other than indicating that streams exiting a termination shall be synchronised with each other (clause 7.1.4/ITU-T H.248.1).

Certain network technologies may require a media gateway (MG) to have knowledge of how two or more streams relate to each other beyond simply knowing that they shall be synchronised. Where there is a multiplicity of streams there may be a different relation between different sets of streams. For the purposes of this Recommendation the relation between streams is defined as the “grouping semantic”. Depending on the grouping semantic the MG may be required to apply certain processing to the media related to the streams. For example: one semantic may indicate that the media is to be duplicated on separate streams, another semantic may indicate media is only sent on one of two streams at a time. Some grouping semantics require that media flows from multiple streams are aggregated into a single flow.

H.248.1 does not support a means to describe grouping semantics. This Recommendation defines the “media grouping package” that allows a media gateway controller (MGC)  to group media from two or more H.248 streams according to a particular semantic and optionally to aggregate/de-aggregate the media from the grouped streams according to the semantic.

The grouping framework in [IETF RFC5888] can in principle be applied to any transport. It is the actual grouping stream semantics that imposes limitations. I.e., BUNDLE may only be applied to IP transports. As such, the “media grouping package” and the concept of a H.248 aggregation stream can be in principle applied to any transport. The majority of defined stream semantics focus on IP based transports. As such, this Recommendation describes the semantics with respect to IP. Whilst some semantics may be used for any type of bearer technology the use of the semantics for these bearer technologies are not covered in this Recommendation.

[End Correction]

[Begin Correction]

3.2.2	Aggregation stream: a stream that sources media to two or more component streams and/ or sinks media from two or more component streams which are related to each other with a certain grouping semantics. The aggregation stream exists only in the interior of the context whereas the component streams represent external streams. Within one context a stream may be defined as an aggregation stream in one termination and as a normal stream in another termination.

[End Correction]

[Begin Correction]

3.2.4	De-aggregation: The MG internal process of splitting a stream (called the de-aggregation stream) on entering the H.248 context in the MG into multiple component streams. The way component streams are generated from a de-aggregation stream is dependent of the grouping semantics. It may consist on stripping off  lower layer information, demultiplexing  upper layer media and distributing it to different streams. In the reverse direction it may consist of sending multiple component streams over the same de-aggregation stream.

3.2.5	De-aggregation stream: a stream acting as a media sink to two or more component streams and/ or acting as a media source to two or more component streams that are related to each other with a certain grouping semantics. The component streams exist on the interior of the context whereas a de-aggregation stream represents an external stream. Within one context, a stream may be defined as a component stream in one termination and as a normal stream in another termination.

3.2.6	Group: Streams that share the same stream semantic are part of the same group. A stream may be part of several groups. An aggregation stream always requires a stream group but a stream group may exist without an aggregation stream. Depending on the stream semantic the use of aggregation may be advantageous or even necessary to model the connectivity and the stream handling in the context. Primarily [IETF RFC 5888] is utilised to group the streams and to provide the semantics.

[End Correction]

[Begin Correction]

## 6.2	Stream multiplexing using transport address information

Typically, a single media stream is assigned to a single H.248 Stream endpoint by specifying a unique address/port combination in the Local Descriptor per H.248 Stream on a Termination along with the associated media properties. In general, to indicate the use of a multiplexed media stream where multiple streams are sent using the same transport address, the MGC requests the same address/port in the Local Descriptor for all H.248 Streams that are to make up the multiplexed media stream.

As such when the same transport address/port is used for several streams there is an implicit grouping semantic which effectively multiplexes several streams over the same transport.

For text-based H.248, SDP is used for each Local Descriptor. In order to request to the same address/port the following conditions must be must:

- The value of the c-line must be the same for each Local Descriptor

- The value of m-line [IETF RFC 4566] <port> field is the same for each Local Descriptor.

A MGC may fully specify the address/port information for multiple H.248 Streams in a single Termination in an Add.req command. However in many cases the MGC will wildcard “choose” this information, in order for the MG to provide the address/port. In the case where wildcarding is used the MGC can:

- request the address/port information for one of the H.248 Streams involved in the multiplex and then use the same address/port information in subsequent Mod.req/s for additional H.248 Streams, or

- request address/port information for all the H.248 Streams involved in the multiplex and then select the address/port from one of the H.248 Streams and then send a subsequent Modify.request to modify the address/port information. This has the disadvantage over option a) that some resources may have been allocated to the previous address/port and these allocations would have to be changed.

Figure 1 shows example syntax for a Termination (T1) utilising two RTP Streams and Termination (T2) utilising one RTP Stream for two H.248 Streams.

| Two H.248 Streams, Two RTP Streams | Two H.248 Streams, One RTP Stream |
| --- | --- |
| MEGACO/3 [123.123.123.4]:55555 Transaction = 10003 { Context = 1 { Add = T1 { Media { Stream = 1 { Local { v=0 c=IN IP4 124.124.124.222 m=audio 2222 RTP/AVP 4 } } Stream = 2 { Local { v=0 c=IN IP4 124.124.124.222 m=video 2223 RTP/AVP 34 } } } } } } | MEGACO/3 [123.123.123.4]:55555 Transaction = 10004 { Context = 1 { Add = T2 { Media { Stream = 1 { Local { v=0 c=IN IP4 124.124.124.222 m=audio 2222 RTP/AVP 4 } } Stream = 2 { Local { v=0 c=IN IP4 124.124.124.222 m=video 2222 RTP/AVP 34 } } } } } } |

Figure 1 – Example single and multiplexed RTP streams

On reception of an Add, Modify or Move request command with several Local Descriptors from the same Termination containing the same address/port information the MG shall treat the associated H.248 Streams as a multiplexed media stream. Multiplexing is applied according to the type or profile of the stream. For example, in the case that a media profile RTP/AVP is given RTP multiplexing is applied. Each media stream component of the overall multiplexed RTP Stream is coded according to the information Local Descriptor / H.248 Stream for that particular media.

RTP multiplexing is described by [IETF RTPMUX].

This multiplexed media stream may contain more than one media type.

Multiple multiplexed media streams are also possible on a Termination.

Individual components of the media multiplex can be modified by changing the Local and/or Remote Descriptors of the related H.248 Stream. If the address/port information of a H.248 Stream is changed to be different from the other components of the multiplexed media stream then the media defined by the H.248 Stream will no longer be part of the multiplex and it will be its own media stream.

Clause 8.6.3 describes several grouping semantics where the use of transport address for multiplexing may be used.

[End Correction]

[Begin Correction]

## 7.1	Scope of Signals / Events and Statistics

Signals, Events and Statistics may be set at either a Termination or Stream level.  Generally when set at a Termination level the scope of these elements applies to all of the H.248 Streams on the Termination. For example: the octets sent statistic would be the sum of octets sent by each H.248 Stream and by corollary each media stream.

When these elements are set at a Stream level the scope of the element is given by the StreamID. In the case of the octets sent statistic it would contain only the octets sent by the H.248 Stream given by the StreamID.

When applying grouping to media Streams the above behaviour should be considered. The grouped Streams may consitute a sub-set of the total H.248 Streams in a Termination. For example: StreamID(1) could be a single stream, StreamIDs(2&3) could be grouped media Streams. This leads to complications regarding the scope of how Signals, Events and Statistics are applied. For example: A Termination level octets sent Statistic would correctly give the number of octets sent by the Termination however there is no way to request an octets sent Statistic related to the grouped media streams. The MGC would have to request a statistic for StreamID(2) and another statistic for StreamID(3) and add them together.

This issue is also applicable to Signals and Events. This is no way to indicate that a Signal or Event is only applicable to those grouped streams

The Media Grouping package defined in this Recommendation introduces an aggregation stream concept that allows signals, events and statistics to be set against an aggregation stream. What effect these elements have is based on the semantics associated with the aggregation.

[End Correction]

[Begin Correction]

### 8.1.1	Group Semantics

|  | Property Name: | Group Semantics |
| --- | --- | --- |
|  | Property ID: | groupse (0x0001) |
|  | Description: | This property allows a MGC to indicate which media streams to group and for which purpose they are grouped. This allows the MG to allocate resources accordingly. For example, providing a common address/port in response to a wildcarded request from the MGC in case multiplexing is supported. In addition, the optional “Direction” element allows indicating whether the grouping is applicable in both directions or in one particular direction. When not included a default of “bothway” is assumed. Several sets of grouped streams are allowed per Termination. |
|  | Type: | Sub-list of String |
|  | Possible values: | Each element of the sub-list follows the group-attribute syntax in [IETF RCF 5888], here represented using type “Group”: Group = Semantics 1*WSP StreamIDList[1*WSP Direction] ; (NOTE 1) WSP = SP/HTAB StreamIDList = StreamID *(WSP StreamID) StreamID = UINT16 Semantics = token token-char = %x21 / %x23-27 / %x2A-2B / %x2D-2E / %x30-39/ %x41-5A / %x5E-7E token = 1*(token-char) Direction = Bothtoken / Sendtoken / Receivetoken Bothtoken = “B” Sendtoken = “S” Receivetoken = “R” (NOTE 2) NOTE 1 – The syntax functionally duplicates the Grouping attribute syntax from [IETF RFC 5888] with the optional addition of a direction parameter. NOTE 2 – This syntax definition allows for a Stream ID list containing one single value. This case is only allowed in a reply to an AuditCapabilities command, where the StreamId shall be set to 0. |
|  | Default: | None. |
|  | Defined in: | TerminationState |
|  | Characteristics: | Read/Write |

[End Correction]

[Begin Correction]

### 8.6.1	Support of media grouping

Usage and support of media grouping may be pre-agreed in a profile specification or may be discovered via the auditing capability.

An MGC may audit the Packages Descriptor to determine if a MG supports media grouping. If the response contains the “Media Grouping Package” (mgroup) it can assume that the MG support media grouping. The MGC may perform an H.248 AuditCapability request command on the groupse property to determine which semantics are supported by the MG. In this case, the MG shall respond with the supported semantic tags and the Stream ID set to 0.

An MGC controlling a MG that supports the Media Grouping package can instruct the MG to group two or more sent and/or received media streams. Each media stream (including the sent and received directions) is described in its own Stream Descriptor and all are included in the same Media Descriptor.

The MGC can indicate to the MG:

- Which sent media streams to group

- Which received media streams to group

- For which purpose the media streams are grouped.

The mgroup package defines three properties, which may be used in various combinations. Table 1 indicates valid combinations:

Table 1 – Profile guidelines: package property combinations

NOTE – The notion of "self-contained" means that the H.248 Media Descriptor contains only the groupse property, without property stragg or strdeagg.

The groupse property can be used in a self-contained manner, i.e., used in a particular H.248 Termination without the stream-level properties stragg nor strdeagg. The stream-level properties stragg and strdeagg are always used in conjunction with the groupse property, i.e., there are (grouping semantic specific) tandems of {groupse|stragg} or {groupse|strdeagg} in the description of a particular H.248 Termination.

In general the mgroup properties can be used together with the H.248 Reserved Group concept (see clause 7.1.7/[ITU-T H.248.1] as the ReserveGroup property applies to alternatives related to a particular H.248 stream. The mgroup package relates multiple H.248 streams. If ReserveGroup is used with the mgroup package properties, the MGC should ensure that any semantic is applicable to each of reserved groups on the applicable stream.

[End Correction]

[Begin Correction]

### 8.6.5	Usage of stream aggregation

In some grouping scenarios, multiple media streams may be aggregated when they enter the context into one media stream and, in the other direction, may result from the reverse operation of splitting one media stream into several media streams. Clause 6.2/[ITU-T H.248.1] describes the multiplexing of Terminations through the use of a multiplex termination. However, it does not support multiplexing of streams within a Termination.

The mgroup package defines the concept of an Aggregation Stream. By setting the stragg property on an aggregation stream indicating the component streams it allows stream aggregation to be modelled internally in the MG. Media is then aggregated to/from the component streams to the aggregation stream according to the applicable grouping semantics.

Figure 2 illustrates the concept.

Figure 2 – Aggregation Stream

In Figure 2 media streams labelled media 1, 2, and 3 received as described by the Stream descriptors Sid1, Sid2, Sid3 will be aggregated according to the groupse semantic for those streams. This aggregation will be sent via stream Sid4 to Termination Tb. It is then sent externally from the media gateway as described by the Stream descriptor for Stream Sid4 on Termination Tb.  The MG may apply any interworking/adaptation etc. functions that would normally apply when the same media stream is set on two Terminations. The aggregation stream may also remain isolated to one Termination in a Context.

The component streams are external streams and their sending and receiving behaviour is characterized by the local control, local and remote descriptor, like for a normal stream.

The media flow between the component streams and the aggregation stream is further regulated by the grouping semantics, which may for example indicate that the aggregation stream sources media to only one of the component streams at a certain time (FID semantics).

Thus, the behaviour of the aggregation stream is well determined by the grouping semantics and the media descriptors of the corresponding component streams. The local control, local, and remote descriptors are not needed in the aggregation stream. If set the values of the properties should be compatible with those defined to the component streams and in no case should override any value set in those streams.

When entering or leaving the context, the component streams have a send and receive relationship with the aggregation stream. Thus, the stream mode does not need to be set in the aggregation stream. If set, it will be ignored by the MG.

A MGC may apply signals, events, statistics and topology to the aggregation stream as per normal streams. The behaviour is discussed in more detail in the sub-clauses below.

[End Correction]

[Begin Correction]

#### 8.6.5.1	Signals and aggregation streams

A signal applied to the aggregation stream will have the effect of being played out on one or more component streams. Which component streams the signal is played out over depends on the grouping semantic associated with the aggregation. Signals may still be specified to be played on the individual component streams. In this case, the playout is isolated to that particular component stream.

Figure 3 illustrates an example scenario using the Flow Identification semantic.

Figure 3 – Signals on an aggregation stream with FID semantic

Example 1:

Termination Ta has four streams, including an aggregation stream. It has two audio streams with identities 1 and 2 that use the same codec. They are grouped with the FID (Flow Identification) semantics [IETF RFC 5888]. Thus, they carry identical content. One is a replica of the other, usually sent to a different destination address. The termination has in addition other media stream with identity 3, which is not part of the media group. Termination Ta has an aggregation stream with streams 1 and 2 as the components. Requesting an an/apv [ITU-T H.248.7] signal on stream 4 to play an audio announcement should result in the announcement being sent synchronously to streams 1 and 2.

Example 2:

Termination Ta has 4 streams, including an aggregation stream. It has two audio streams with identities 1 and 2 that use different codecs. They are grouped with the FID semantics [IETD RFC 5888].  At a time, only one of the streams carries data in each direction while the other one is muted in that direction. This is dependent on the codec which is chosen dynamically by the end-user equipment. Termination Ta has an aggregation stream 4 with streams 1 and 2 as the components. Requesting an an/apv [ITU-T H.248.7] signal on stream 4 to play an audio announcement should result in the announcement being sent in the stream (1 or 2) which carries media at that moment.

Example 3:

A termination Ta has one video stream and two audio streams with identities 1, 2 and 3 grouped with the SRF (Single Reservation Flow) semantics [IETF RFC 3524]. Thus, resource reservation using RSVP and policy enforcing is applied to the complete group. The MGC defines an aggregation stream 4. In order to send RSVP messages related to all grouped streams, the MGC should set the signals of the rsvp package [ITU-T H.248.65], e.g. rsvp/path on the aggregation stream 4.

NOTE – RSVP sessions are normally defined by the triplet destination address, protocol ID, destination port. For the identification of the session related to grouped media, see [IETF RFC 3524].

[End Correction]

[Begin Correction]

#### 8.6.5.3	Statistics and aggregation streams

When statistics are set against an aggregation stream, the statistics are collected as if they had been set against all the component streams. For example, if the rtp/pr “RTP packets received” statistic is set on an aggregation stream it will count the sum of RTP packets received from all the component streams.

When statistics are set at the termination level on a termination that has an aggregation stream the effect will be that it is only set against the aggregation stream and streams not involved in an aggregation.

Figure 6 – Statistics on an aggregation stream with ANAT semantic

Example:

Figure 6 shows an example where a Context has a termination Ta that handles three audio streams with identities 1, 2 and 3 grouped with the BUNDLE semantic. The termination has in addition another media stream with identity 4, not part of the media group. The MGC has defined an aggregation stream with identity 5. The MGC wishes to collect statistics on the data volumes sent and received in the bundled group at different levels (network, transport, application). It uses the Network Package os and or [ITU-T H.248.1], the IP Layer Octets Counts Statistics Package ipos, ipor, ipps and ippr [ITU-T H.248.61] and the RTP Application Data Package payloados and payloador [ITU-T H.248.58] statistics. It requests these statistics by including them in the statisticsDescriptor of the streamDescriptor of stream 5. The MG should collect statistics in each of the three component streams and sum up the values.

[End Correction]

[Begin Correction]

### 8.6.6	Usage of stream de-aggregation

In some grouping scenarios a single media stream on entering the context may be de-aggregated into multiple component media streams and, in the other direction, may result from the reverse operation of merging multiple component media streams into a single media stream.

The mgroup package defines the concept of a de-aggregation Stream. By setting the mgroup/strdeagg property on a H.248 Stream indicating the component streams it allows stream de-aggregation to be modelled internally in the MG. Media is then de-aggregated to/from the de-aggregation stream to the component streams according to the applicable grouping semantics.

The mgroup/strdeagg property may be set on multiple H.248 streams on a Termination. However the component H.248 StreamIDs shall only be assigned to one de-aggregation stream. If the MGC assigns a component H.248 StreamID to more than one mgroup/strdeagg property on a Termination then error code 489 “Invalid aggregation and/or de-aggregation” shall be returned by the MG.

Figure 10 illustrates the concept.

Figure 10 – De-aggregation stream

In figure 10 media stream labelled media 1 described by Stream Descriptor Sid1 will be de-aggregated into components 1a, 1b and 1c according to the groupse semantic for those streams. This de-aggregation will be sent via streams Sid2, Sid3 and Sid4 to Termination Tb. Streams Sid2, Sid3 and Sid4 are the component streams. They are then sent externally from the media gateway as described by the Stream descriptor for Streams Sid2, Sid3 and Sid4 on Termination Tb.  The MG may apply any interworking/adaptation etc. functions that would normally apply when the same media stream is set on two Terminations. The de-aggregation stream may also remain isolated to one Termination in a Context.

The de-aggregation stream is an external stream and its sending and receiving behaviour has to be characterized with the local control, local and remote descriptor, like with a normal stream.

The concept of de-aggregation has been introduced in order to allow multiple component streams to share the same transport. The de-aggregation stream represents the common transport while the component streams represent a particular user of the common transport, i.e. an instance of an upper layer. Thus, the de-aggregation and the component streams represent different stacked layers.

This is a difference to the concept of aggregation, where the separation between aggregation and component stream is not related to different stacked layers.

Due to this, in the case of de-aggregation, component streams may be further characterized by local control, local and remote descriptors, with properties to describe upper layers not described in the de-aggregation stream. However, properties related to the layers characterized by the de-aggregation stream should be avoided in the component streams. If set the values of these properties in the component streams should be compatible with those defined in the de-aggregation stream and in no case should override any value set in the de-aggregation stream.

When entering or leaving the context through the de-aggregation stream, the component streams have a send and receive relationship with the de-aggregation stream. Thus the stream mode does not need to be set in the component stream. If set, it will be ignored by the MG.

A MGC may apply Signals, Events, Statistics and Topology to the de-aggregation and component streams. The behaviour is discussed in more detail in the sub-clauses below.

[End Correction]

[Begin Correction]

#### 8.6.6.1	Signals and de-aggregation streams

A Signal applied to a de-aggregation stream will usually have the effect of being played out externally. Its effect is dependent on the semantic of the signal. A Signal applied to the internal direction will have the effect of playing on each of the component streams.

A Signal applied to a component stream will usually have the effect of being played out externally on the de-aggregation stream in the part of that the component relates to.

[End Correction]

__________________