| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 280 (WP 1/16) |
|  |  | English only Original: English |
| Question(s): | 3/16 | Geneva, 12 - 23 October 2015 |
| Source: | Editor H.248.SIPREC |  |
| Title: | H.248.SIPREC "Gateway control protocol: Profile guidelines for H.248 supported SIP-based Media Recording" (New): Input draft Ed. 0.7 |  |

1.	Introduction

This document contains the editor's input draft ed. 0.7 for draft new H.248.SIPREC "Gateway control protocol: Profile guidelines for H.248 supported SIP-based Media Recording" (New).

2.	Discussion

This document is a copy of temporary document TD-14 from the last WP1/16 Rapporteur meeting in Chengdu (China), 8 - 12 June 2015.

Changes versus Ed. 0.6 (2015-06):

- Ref. "H.248.RTPMUX" replaced by "H.248.95"

3.	Proposal

It is proposed to accept this document as the baseline text for the meeting.

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.SIPREC Ed. 0.1 | Initial draft (AVD-4610) to WP 1/16 Seoul (KR) meeting (2014-11) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1411_Seo/AVD-4610.zip |
| H.248.SIPREC Ed. 0.2 | Output (TD-22) from WP 1/16 Seoul (KR) meeting (2014-11) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1411_Seo/TD-22.zip |
| H.248.SIPREC Ed. 0.3 | Input (TD 219/WP1) to ITU-T SG16 Geneva meeting, Feb 2015 Location: http://www.itu.int/md/T13-SG16-150209-TD-WP1-0219/en |
| H.248.SIPREC Ed. 0.4 | Output (TD 219R1/WP1) from ITU-T SG16 Geneva meeting, Feb 2015 Location: http://www.itu.int/md/T13-SG16-150209-TD-WP1-0219/en |
| H.248.SIPREC Ed. 0.5 | Input (AVD-4667) to WP 1/16 Chengdu meeting (2015-06) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1506_Gui/AVD-4667.zip |
| H.248.SIPREC Ed. 0.6 | Output (TD-14) from WP 1/16 Chengdu meeting (2015-06) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1506_Che/TD-14.zip |
| H.248.SIPREC Ed. 0.7 | Input (TD 280/WP1) to ITU-T SG16 Geneva meeting, Oct 2015 Location: |
| H.248.SIPREC Ed. 0.8 | Output (TD ###/WP1) from ITU-T SG16 Geneva meeting, Oct 2015 Location: |
| H.248.SIPREC Ed. 0.9 |  |
| H.248.SIPREC Ed. 0.10 |  |

CONTENTS

Page

1 Scope 6 2 References 7 3 Definitions 7 3.1 Terms defined elsewhere 7 3.2 Terms defined in this Recommendation 8 4 Abbreviations and acronyms 8 5 Conventions 9 6 Overview to SIP-based media recording 10 6.1 IETF defined use cases for SIP networks 10 6.2 IETF defined architecture for SIP networks 10 7 H.248 supported SIP-based media recording 11 7.1 H.248 gateway models for SIP-based media recording 11 7.1.1 Network level models 11 7.1.2 H.248 context model with SRC-to-SRS interface 12 7.2 H.248 supported use cases for SIP-based media recording 14 8 Profile requirements for H.248 supported SIP-based media recording 15 8.1 Relevant requirements from RFC 6341 15 8.2 Requirements for H.248 gateways in general 15 8.3 Use case specific requirements for H.248 gateways 15 9 H.248 profile specification guidelines for SRC-capable H.248 gateways 15 9.1 Profile Identification 15 9.2 Summary 15 9.3 Gateway Control Protocol Version 16 9.4 Connection Model 16 9.5 Context Attributes 16 9.6 Terminations 16 9.7 Descriptors 16 9.7.1 TerminationState Descriptor 16 9.7.2 Stream Descriptor 16 9.7.3 Events Descriptor 16 9.7.4 EventBuffer Descriptor 16 9.7.5 Signals Descriptor 16 9.7.6 DigitMap Descriptor 16 9.7.7 Statistics Descriptor 16 9.7.8 ObservedEvents Descriptor 17 9.7.9 Topology Descriptor 17 9.7.10 Error Descriptor 17 9.8 Command API 17 9.8.1 Add 17 9.8.2 Modify 17 9.8.3 Subtract 17 9.8.4 Move 17 9.8.5 AuditValue 17 9.8.6 AuditCapabilities 17 9.8.7 Notify 17 9.8.8 ServiceChange 17 9.8.9 Manipulating and auditing context attributes 17 9.9 Generic command syntax and encoding 17 9.10 Transactions 17 9.11 Messages 17 9.12 Transport 18 9.13 Security 18 9.14 Packages 18 9.14.1 Mandatory Package 18 9.14.2 Optional Package 18 9.14.3 Package usage information 18 9.15 Mandatory support of SDP and ITU-T H.248.1 Annex C information elements 18 9.16 Optional support of SDP and ITU-T H.248.1 Annex C information elements 19 9.17 Procedures 19 Appendix I Use case specific capability sets – Two examples 20 I.1 Overview 20 Bibliography 21

List of Tables

Page

Table 1 – H.248 supported use cases for SIP-based media recording 14

List of Figures

Page

Figure 1 – [b-IETF RFC 6341 (Fig. 1)] – Relationship between a recording session and communication session 10 Figure 2 – [b-IETF RFC 7245 (Fig. 1)] – B2BUA acts as the Session Recording Client 11 Figure 3 – H.248 "B2BUA" gateway hosted Session Recording Client function 12 Figure 4 – H.248 context model with SRC-to-SRS interface (example) 13

Draft new ITU-T Recommendation H.248.SIPREC

Gateway control protocol: 
Profile guidelines for 
H.248 supported SIP-based Media Recording

AAP Summary

<to be provided before Consent>

Summary

<Mandatory material>

Keywords

<Optional>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# 1	Scope

The network service of SIP-based media recording is defined by the IETF, see [b-IETF RFC 6341]. A so-called SIP-based communication session is recorded concerning their IP signalling and IP media traffic components. The recording instance, the so-called Session Recording Client (SRC) is reporting the recorded information to a Session Recording Server (SRS). There are multiple network architectural solutions how the SIP-based media recording could be realized, see [b-IETF RFC 7245]. Usage of H.248 gateways is one option, and when, H.248 gateways could provide either SRC or SRS support (called SRC-capable H.248 gateway and SRS-capable H.248 gateway).

The purpose of this Recommendation is to provide guidelines for H.248 profile development for SIP-based media recording.

The scope of this Recommendation is

- limited on H.248 gateway hosted Session Recording Clients and

- limited on the media portion of the recorded session only.

This Recommendation provides

- a brief overview of SIP-based media recording from H.248 gateway perspective (clause 6);

- examples models for H.248 gateways and contexts (clause 7);

- use cases and requirements (clause 8); and

- profile guidelines (clause 9).

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.9]	Recommendation ITU-T H.248.1 (12/2009), Gateway control protocol: Advanced media server packages.

[ITU-T H.248.73]	Recommendation ITU-T H.248.73 (11/2010), Gateway control protocol: Packages for media server control markup language and ITU-T H.248 interworking.

[ITU-T H.248.88]	Recommendation ITU-T H.248.88 (01/2014), Gateway control protocol: RTP topology dependent RTCP handling by ITU-T H.248 media gateways with IP terminations.

[IETF RFC 3261]	IETF RFC 3261 (06/2002), SIP: Session Initiation Protocol.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	Communication Session (CS) [b-IETF RFC 6341]: A session created between two or more SIP User Agents (UAs) that is the subject of recording.

3.1.2	Recording-aware User Agent (UA) [b-IETF RFC 7245]: A SIP User Agent that is aware of SIP extensions associated with the CS.  Such extensions may be used to notify the recording-aware UA that a session is being recorded, or by a recording-aware UA to express preferences as to whether a recording should be started, paused, resumed, or stopped.

3.1.3	Recording Metadata [b-IETF RFC 7245]: The metadata describing the CS that is required by the SRS. This will include, for example, the identities of users that participate in the CS and dialog state. Typically, this metadata is archived with the Replicated Media at the SRS. The recording metadata is delivered in real-time to the SRS.

3.1.4	Recording Session (RS) [b-IETF RFC 6341]: The SIP session created between an SRC and SRS for the purpose of recording a Communication Session.

3.1.5	Recording-unaware User Agent (UA) [b-IETF RFC 7245]: A SIP User Agent that is unaware of SIP extensions associated with the CS. Such a recording- unaware UA will be notified that a session is being recorded or will express preferences as to whether a recording should be started, paused, resumed, or stopped via some other means that is out of scope for the SIP media recording architecture.

3.1.6	Replicated Media [b-IETF RFC 7245]: A copy of the media that is associated with the CS, was created by the SRC, and was sent to the SRS. It may contain all the media associated with the CS (e.g., audio and video) or just a subset (e.g., audio). Replicated Media is part of the Recording Session.

3.1.7	RTP session (clause 2.2.2/[b-IETF draft taxonomy]|[b-ITU-T H.248.95]): An RTP Session is an association among a group of Participants communicating with RTP.  It is a group communications channel which can potentially carry a number of RTP Streams. Within an RTP Session, every Participant can find meta-data and control information (over RTCP) about all the RTP Streams in the RTP Session.  The bandwidth of the RTCP control channel is shared between all Participants within an RTP Session.

NOTE – [b-ITU-T H.248.57], clause 3.1.3 provides following definition:

RTP session: An RTP session comprises a single RTP flow and an optional RTCP flow.

It could be concluded that the "H.248.57 RTP session" is synonym to RTP stream in general..

3.1.8	RTP stream (clause 2.1.10/[b-IETF rtp-taxonomy]|[b-ITU-T H.248.95]): A stream of RTP packets containing media data, source or redundant. The RTP stream is identified by an SSRC belonging to a particular RTP session.

3.1.9	Session Recording Client (SRC) [b-IETF RFC 6341]: A Session Recording Client (SRC) is a SIP User Agent (UA) that acts as the source of the recorded media, sending it to the SRS. An SRC is a logical function. Its capabilities may be implemented across one or more physical devices.  In practice, an SRC could be a personal device (such as a SIP phone), a SIP Media Gateway (SMG), a Session Border Controller (SBC), or a SIP Media Server (SMS) integrated with an Application Server (AS). This specification defines the term "SRC" such that all such SIP entities can be generically addressed under one definition. The SRC provides metadata to the SRS.

NOTE – The abbreviations for SIP media gateway and SIP media server were extended in order not to be confused with the correspondent H.248 entities.

3.1.10	Session Recording Server (SRS) [b-IETF RFC 6341]: A Session Recording Server (SRS) is a SIP User Agent (UA) that is a specialized media server or collector that acts as the sink of the recorded media. An SRS is typically implemented as a multi-port device that is capable of receiving media from multiple sources simultaneously. An SRS is the sink of the recorded session metadata.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	SRC-capable H.248 gateway: an ITU-T H.248 gateway with partial or complete support of the Session Recording Client (SRC) function.

NOTE – The notion of 'partial' because SIP-based call control is streactly speaken out of scope of the definition of H.248 gateways.

3.2.2	SRS-capable H.248 gateway: an ITU-T H.248 gateway with partial or complete support of the Session Recording Server (SRS) function.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| B2BUA | Back-to-Back User Agent |
| --- | --- |
| CS | Communication Session |
| IP | Internet Protocol |
| IPv4 | Internet Protocol Version 4 |
| IPv6 | Internet Protocol Version 6 |
| ISUP | ISDN (Integrated Services Digital Network) User Part |
| L3 | Layer three |
| L4 | Layer four |
| L4+ | Above layer four |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| PBX | Private Branch Exchange |
| PSTN | Public Switched Telephone Network |
| RTP | Real-time Transport Protocol |
| SBC | Session Border Controller |
| SDP | Session Description Protocol |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SIP | Session Initiation Protocol |
| SIPREC | (IETF working group) SIP-based Media Recording |
| SMG | SIP Media Gateway |
| SMS | SIP Media Server |
| SRC | Session Recording Client |
| SRS | Session Recording Server |
| UA | (SIP) User Agent |
| UDP | User Datagram Protocol |
| UE | User Equipment |

# 5	Conventions

This document provides a list of items, labelled as R-x/y, where x refers to the clause number and y a number within that clause. Such items use the following keywords with meanings as prescribed below:

The keywords "is required to" indicate a requirement which must be strictly followed and from which no deviation is permitted if conformance to this document is to be claimed.

The keywords "is prohibited from" indicate a requirement which must be strictly followed and from which no deviation is permitted if conformance to this document is to be claimed.

The keywords "is recommended" indicate a requirement which is recommended but which is not absolutely required.  Thus this requirement need not be present to claim conformance.

The keywords "can optionally" indicate an optional requirement which is permissible, without implying any sense of being recommended. This term is not intended to imply that the vendor’s implementation must provide the option and the feature can be optionally enabled by the network operator/service provider.  Rather, it means the vendor may optionally provide the feature and still claim conformance with the specification.

# 6	Overview to SIP-based media recording

## 6.1	IETF defined use cases for SIP networks

## 6.2	IETF defined architecture for SIP networks

Motivation, use cases and requirements for SIP-based media recording is described and defined by [b-IETF RFC 6341]. A recording session is derived from a communication session, which represents a SIP session between IP-based communication end systems (such as the user equipment A and B in Figure 1):

Figure 1 – [b-IETF RFC 6341 (Fig. 1)] – Relationship between a recording session and communication session

{Editor's note (2014-11): to do: clarify active and passive recording (RFC 6341).}

The content of the recording session is based on the traffic of the communication session, which covers signalling traffic and meta-data besides the actual media traffic. Thus, the recording session data represents roughly an augmented copy of the communication session. There are multiple network architectural options concerning the production of recording sessions, given by the location of the session recording client (SRC). For instance, a communication session where both endpoints are located in different private network domains that are interconnected via a public network domain: the SRC could be located in the SIP terminal, "SIP PBX" or "SIP SBC" in the private network space or by correspondent network elements in the public network domain.

Figure 2 – [b-IETF RFC 7245 (Fig. 1)] – B2BUA acts as the Session Recording Client

The two extreme solutions are

- the fully distributed recording function at the network edge by SRC capable user equipment and

- the network centralized recording function within the network core by SRC capable signalling/media plane B2BUAs (Figure 2).

The scope of this Recommendation addresses solutions only with SRC functionality integrated in H.248 gateways.

# 7	H.248 supported SIP-based media recording

## 7.1	H.248 gateway models for SIP-based media recording

### 7.1.1	Network level models

#### 7.1.1.1	Network scenarios with SRC-capable H.248 gateways

There are three basic recording models in case of SRC-capable H.248 gateways:

- H.248 gateway at the border between a SIP network and a non-SIP network;

- non-SIP network could be also IP based, such as a H.323 enterprise network;

- non-SIP network could be circuit-switched such as a PSTN ISUP network;

- H.248 IP-to-IP gateway located within or between SIP network(s); and

- H.248 media server.

The three scenarios are typically given by different H.248 profile specifications (such as typically known as trunking gateway (1), border gateway (2) and media server (3) in SIP-based NGNs). The scope of this Recommendation is on scenario 2 only (Figure 3). Profile guidelines for the other scenarios may be simply derived.

Figure 3 – H.248 "B2BUA" gateway hosted Session Recording Client function

It has to be noted that only the media data (3) component of a recording session could be in scope of this Recommendation. Thus, only the SRC function related to the H.248 media gateway is in scope of this Recommendation (see next sub-clause 7.1.2).

#### 7.1.1.2	Network scenarios with SRS-capable H.248 gateways

The session recording server (SRS) functionality is typically centralized in a network and usually a basic capability of so called media servers. A SRS-capable H.248 gateway represents therefore a decomposed media server using H.248 signalling. Such SRS-capable H.248 media servers are out of scope of (the first release of) this Recommendation, but it would be expected that the applied H.248 profile would base the recording behaviour on H.248 packages from [ITU-T H.248.9] and [ITU-T H.248.73].

### 7.1.2	H.248 context model with SRC-to-SRS interface

SIP-based media recording covers multi-party, multimedia communication services in its entirety. Figure 4 illustrates the H.248 context model for the example of a two-party, dual media (audio, video) communication service between parties A and B.

H.248 modeling of the communication session:

The two RTP streams (definition see clause 3.1.8) for audio and video are mapped on two H.248 streams S1 and S2, i.e., the communication session is given by the two dual-stream-per-termination H.248 terminations TA and TB. Each H.248 stream contains two H.248 flow components for RTP and RTCP packet traffic.

H.248 modeling of the recording session:

Only the media plane traffic component of the recording session is considered here. There are two options for modelling the SRC-to-SRS interface: either the media traffic of both traffic directions could be merged or should be forwarded separately to the SRS?

The merge option might be only feasible for media type 'audio' but not for other media types. Thus, the general case would be separated forwarding, as illustrated in Figure 4. There are then two further dual-stream-per-termination H.248 terminations TC and TD. However, there is only egress traffic on these H.248 terminations and an RTCP flow in reverse direction, from SRS-to-SRC could be excluded.

Figure 4 – H.248 context model with SRC-to-SRS interface (example)

It has to be noted that above H.248 context model does only indicate an example (out of multiple options) and does not define the SRC-to-SRS interface in general.

## 7.2	H.248 supported use cases for SIP-based media recording

Following use cases could be supported by H.248 gateways, of course under the condition of a H.248 gateway hosted SRC function (Table 1):

Table 1 – H.248 supported use cases for SIP-based media recording

| No. | Use cases according to [b-IETF RFC 6341] | Support by H.248 gateways? |
| --- | --- | --- |
| 1 | Full-time Recording: One Recording Session for each Communication Session | Yes |
| 2 | Selective Recording: Start a Recording Session when a Communication Session to be recorded is established | Yes |
| 3 | Start/Stop a Recording Session during a Communication Session | Yes |
| 4 | Persistent Recording: A single Recording Session captures one or more Communication Sessions | "Yes" {Editor's note (2014-11): It was thought that H.248 support of persistent records needed further analysis. E.g. a persistent recording session could be modelling by moving terminations in and out of a Context with terminations representing the SRS.} |
| 5 | Real-time Recording Controls | "Yes" {Editor's note (2014-11): It was agreed further analysis is needed regarding real-time recording controls is needed. Particularly as the use case means that the media must still be delivered between endpoint A and B but must be controlled from being sent to the recording server.} |
| 6 | IVR / Voice Portal Recording | "Yes" (To Be Clarified: location of SRC versus IVR/Voice Portal? …) |
| 7 | Enterprise Mobility Recording | Yes (conditional on network architecture (NOTE 1)) |
| 8 | Geographically distributed or centralized recording | Yes (conditional on network architecture (NOTE 1)) |
| 9 | Record complex call scenarios | Yes |
| 10 | High availability and continuous recording | Yes |
| 11 | Record multi-channel, multimedia session | Yes (conditional on concrete requirements concerning speech analytics) |
| 12 | Real-time media processing | Yes (conditional on concrete requirements concerning real-time analytics) |
| NOTE 1 – Topology condition: the communication session must be routed across the H.248 gateway. |  |  |

# 8	Profile requirements for H.248 supported SIP-based media recording

It should be noted again that only SRC-capable H.248 gateways (see clause 7.1.1.1) are in scope of (the first release of) this Recommendation.

## 8.1	Relevant requirements from RFC 6341

Clause 5 in [b-IETF RFC 6341] defines a set of requirements for SIP-based media recording. Only a sub-set of these requirements is applicable in case of H.248 gateway hosted SRC functionality.

{Editor's note (2014-11): FIXTHIS …}

## 8.2	Requirements for H.248 gateways in general

Clause 5 in [b-IETF RFC 6341] defines a set of requirements for SIP-based media recording. Only a sub-set of these requirements is applicable in case of H.248 gateway hosted SRC functionality.

{Editor's note (2014-11): FIXTHIS …}

## 8.3	Use case specific requirements for H.248 gateways

Typically following requirements needs to be supported in case of H.248 gateway hosted SRC functionality:

{Editor's note (2014-11): FIXTHIS, e.g.,
- H.248 stream topology
- H.248 announcements for the communication session parties (concerning recording)
- RTP topologies for communication session
- media anlytics capabilities for the communication session
- recording session: control of pause/resume (via StreamMode?)
- recording session: H.248 control between full-time and selective recording?
- recording session: RTCP handling?
- recording of TCP-based media? TCP proxy mode for communication session and TCP endsystem mode for recording session etc
…}

# 9	H.248 profile specification guidelines for SRC-capable H.248 gateways

This clause provides guidelines for H.248 profile specifications. The structure follows the profile template according to Appendix III/[ITU-T H.248.1].

The template elements that are not applicable in this Recommendation are indicated by "Subject to profile specification".

Any profile guidelines are primarily dependent on the concerned network configuration and use case. The guidelines in this clause are therefore basically conditional. Two exemplary use cases are considered (as described in Appendix I), termed as capability set CSA and CSB.

## 9.1	Profile Identification

Subject to profile specification.

## 9.2	Summary

Subject to profile specification.

## 9.3	Gateway Control Protocol Version

Subject to profile specification.

SIP-based media recording may require specific H.248 capabilities, which are protocol version dependent. For instance:

- Topology Descriptor; or

- Stream-level statistics.

## 9.4	Connection Model

| Maximum number of contexts: | Subject to profile specification. |
| --- | --- |
| Maximum number of terminations per context: | Subject to profile specification. Examples: IF CSA THEN "4 (2 + 2)". IF CSB THEN " ### ". |
| Allowed termination type combinations in a context: | Subject to profile specification. |

## 9.5	Context Attributes

Subject to profile specification.

## 9.6	Terminations

Subject to profile specification.

## 9.7	Descriptors

### 9.7.1	TerminationState Descriptor

Subject to profile specification.

### 9.7.2	Stream Descriptor

Subject to profile specification.

### 9.7.3	Events Descriptor

Subject to profile specification.

{Editor's note (2014-11): for further studies (capability dependent on "media analytics" requirements …}

### 9.7.4	EventBuffer Descriptor

Subject to profile specification.

### 9.7.5	Signals Descriptor

Subject to profile specification.

### 9.7.6	DigitMap Descriptor

Subject to profile specification.

### 9.7.7	Statistics Descriptor

IF CSA THEN FIXTHIS.

IF CSB THEN FIXTHIS.

### 9.7.8	ObservedEvents Descriptor

Subject to profile specification.

### 9.7.9	Topology Descriptor

{Editor's note (2014-11): for further studies …}

### 9.7.10	Error Descriptor

Subject to profile specification.

## 9.8	Command API

NOTE – It is assumed that an Error Descriptor may be returned in any command reply.

### 9.8.1	Add

Subject to profile specification.

### 9.8.2	Modify

Subject to profile specification.

### 9.8.3	Subtract

Subject to profile specification.

### 9.8.4	Move

Subject to profile specification.

### 9.8.5	AuditValue

Subject to profile specification.

### 9.8.6	AuditCapabilities

Subject to profile specification.

### 9.8.7	Notify

Subject to profile specification.

### 9.8.8	ServiceChange

Subject to profile specification.

### 9.8.9	Manipulating and auditing context attributes

Subject to profile specification.

## 9.9	Generic command syntax and encoding

Subject to profile specification.

## 9.10	Transactions

Subject to profile specification.

## 9.11	Messages

Subject to profile specification.

## 9.12	Transport

Subject to profile specification.

## 9.13	Security

{Editor's note (2014-11): … there might be dedicated security requirements for the SRC-to-SRS interface, such as encryption of recorded media ….}

## 9.14	Packages

{Editor's note (2014-11): the text in clause 9.14 is preliminary due to ongoing work on H.248.SEPLINK. The table entries are not yet consistent.}

### 9.14.1	Mandatory Package

Mandatory: specifies the packages that shall be supported in this profile.

| Mandatory packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | Package ID | Version | Termination Types Supported |
| FIXTHIS |  |  |  |
| FIXTHIS |  |  |  |

### 9.14.2	Optional Package

| Optional packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | Package ID | Version | Termination Types Supported |
| FIXTHIS |  |  |  |
| FIXTHIS |  |  |  |

### 9.14.3	Package usage information

The following is a non-exhaustive list of package usage indications.

#### 9.14.3.1	RTP topology package

FIXTHIS

#### 9.14.3.x	xxxx package

FIXTHIS

## 9.15	Mandatory support of SDP and ITU-T H.248.1 Annex C information elements

At least the following:

| Supported Annex C and SDP information elements: |  |  |
| --- | --- | --- |
| Information Element | Annex C Support | SDP Support |
| "m="-line <proto> | not supported (in this example) | FIXTHIS |
|  |  | FIXTHIS |

Rest: subject to profile specification.

## 9.16	Optional support of SDP and ITU-T H.248.1 Annex C information elements

At least the following:

| Supported Annex C and SDP information elements: |  |  |
| --- | --- | --- |
| Information Element | Annex C Support | SDP Support |
| FIXTHIS |  |  |
| FIXTHIS |  |  |

Rest: subject to profile specification.

## 9.17	Procedures

FIXTHIS

Appendix I

Use case specific capability sets – Two examples

(This appendix does not form an integral part of this Recommendation)

## I.1	Overview

Profile content is mainly use case dependent as outlined in clause 1. In order to demonstrate profile specification guidelines in clause 9, this Recommendation considers two exemplary use cases:

- Capability set 'A' (CSA):

  - H.248 MG type: an IP-to-IP media gateway,

  - use cases [b-IETF RFC 6341] in scope:

    - use case (1) "full-time recording" service,

  - communication session:

    - two party conversational service,

    - two media types (audio, video),

    - bidirectional RTP sessions  (definition see clause 3.1.7) with RTCP,

    - RTP multiplexing: none,

    - RTP topologies: B2BRE and RTP translator types

  - recording session "media component",

    - "stereo mode", i.e., separate RTP sessions (i.e., UDP transport connections) per media type and per communication party,

    - RTCP: only in SRC-to-SRS direction,

    - performance monitoring: H.248 statistics with regards to the IP layer traffic volume,

  - media analytics support: none.

- Capability set 'B' (CSB):

  - FIXTHIS …,

  - …

Bibliography

[b-ITU-T H.248.57]	Recommendation ITU-T H.248.57 (10/2014), Gateway control protocol: RTP control protocol package.

[b-ITU-T H.248.95]	Recommendation ITU-T H.248.95 (##/2016), Gateway control protocol: H.248 support for RTP multiplexing.

[b-IETF RFC 6341]	IETF RFC 6341 (08/2011), Use Cases and Requirements for SIP-Based Media Recording (SIPREC).

[b-IETF RFC 7245]	IETF RFC 7245 (05/2014), An Architecture for Media Recording Using the Session Initiation Protocol.

[b-IETF siprec-prot]	IETF draft-ietf-siprec-protocol (201#), Session Recording Protocol. 
<http://tools.ietf.org/wg/siprec/draft-ietf-siprec-protocol/>

[b-IETF siprec-meta]	IETF draft-ietf-siprec-metadata (201#), Session Initiation Protocol (SIP) Recording Metadata. 
<http://tools.ietf.org/wg/siprec/draft-ietf-siprec-metadata/>

[b-IETF siprec-flow]	IETF draft-ietf-siprec-callflows (201#), Session Initiation Protocol (SIP) Recording Call Flows. 
<http://tools.ietf.org/wg/siprec/draft-ietf-siprec-callflows/>

[b-IETF siprec-msrp]	IETF draft-yan-siprec-msrp-recording (201#), Overview for MSRP Recording based on SIPREC. 
<http://tools.ietf.org/id/draft-yan-siprec-msrp-recording-01.txt>

[b-IETF rtp-taxonomy]	IETF draft-ietf-avtext-rtp-grouping-taxonomy (XX/2015), A Taxonomy of Grouping Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources.

___________________

Because traffic components outside an H.248 context are out of scope of the H.248 gateway control protocol.