| Rapporteur Meeting of Question 21/16 |  |  |  | AVD-4779 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Changzhou, 26 - 29 September 2016 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ETRI |  |  |  |
| Title: | HSTP-DIS-UAV: Proposed to add an introductory text on sensor devices |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution contains a proposed introductory text on sensor devices to draft ITU-T Technical Paper HSTP-DIS-UAV “Use cases and service scenarios of disaster information service using unmanned aerial vehicles”.

Introduction

In the last ITU-T SG16 meeting Geneva, 23 May – 3 June 2016, it was recommended that an introductory text on sensor devices such as visible light camera and thermographic camera is provided to help people to understand relevant use cases. This contribution proposes the requested introductory text on sensor devices.

Proposal

- Proposes to add the new clause 6.1 Sensors for disaster information service - Proposes to add new acronyms to clause 4

# Start proposal

4	Abbreviations and acronyms

CCD		Charge-Coupled Device

CMOS		Complementary Metal–Oxide–Semiconductor

6.1 	Sensors for disaster information service

The following sensors can be used for disaster information service using unmanned aerial vehicles.

Visible light camera (visible-band camera): consumer digital cameras are used for capturing images in the visible spectrum, which is the portion of the electromagnetic spectrum that is visible to the human eye. A typical digital camera uses a CCD or CMOS image sensor with a colour filter array and an infrared cut-off filter.

Thermographic camera: a thermographic camera, which is also called a thermal imaging camera or infrared camera, captures thermal images using infrared radiation. The thermal images contain apparent temperature measurements for each pixel.

Multispectral camera: a multispectral camera captures image data at several spectral frequencies across the electromagnetic spectrum. One digital image is acquired for each small spectral band, which represents a wavelength range of the electromagnetic spectrum. Near infrared, middle infrared, and far infrared bands are often used with red, green, and blue bands.

Hyperspectral camera: a hyperspectral camera produces image data for hundreds of continuous bands across the electromagnetic spectrum. The spectral bands in hyperspectral imagery are much narrower (10-20 nm) than those in multispectral imagery. In hyperspectral imagery, the spectral bands cover a continuous spectral range.

LIDAR: LIDAR is a remote sensing technology that measures a distance by illuminating a target with a pulsed laser and analysing the reflected light. When a UAV with a LIDAR sensor flies over a specific area, three-dimensional information about the shape of the area and its surface characteristics is obtained. The information can be used to create a DEM.

SAR: TBD

InSAR: TBD

# End Proposal

__________________