| Rapporteur Meeting of Question 21/16 |  |  |  | AVD-4782 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Changzhou, 26 - 29 September 2016 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ETRI |  |  |  |
| Title: | HSTP-DIS-UAV: Proposed to add use cases and service scenarios for earthquake |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This document describes use cases and service scenarios for earthquake utilizing unmanned aerial vehicles. UAVs are expected to be applicable to various disaster situations such as drought, fire, flood, landslide, earthquake, and volcano, etc. Use cases are described in all disaster phases, which are preparedness phase (before disaster), response and relief phase (at and during disaster), recovery and reconstruction (after disaster).

- Introduction

This contribution proposes to add use cases and its service scenarios for earthquake disaster situation.

- Discussion

Section 7.3 describes a disaster information service using UAVs that can be used for detection of wildfire and industrial fire. It is necessary to provide the use cases for other disasters such as earthquake, flood, and volcano to represent various disaster situation. So this contribution proposes to add contents for earthquake to clause 7 of use cases and service scenarios.

- Proposal

- Proposes to add contents to clause 7.2 on use cases for earthquake

[Begin Proposal]

7.2	Use cases for earthquake

7.2.1	Preparedness phase

The disaster information service using UAVs can be used for detection of earthquake. Use cases are described below.

- It was found that there was unusual increase in land surface temperature called the thermal anomaly before some strong earthquakes. The thermal anomaly happens about 1-24 days before an earthquake, with the temperature increasing by 3-12℃ or more and disappears a few days later. Thus, the risk for earthquake can be estimated from measurements of the land surface temperature by a UAV carrying a multispectral camera.

- Damages to man-made structures and topological changes by earthquakes can be detected by a UAV with a visible light camera.

Computer vision technologies can be used for automatic detection of earthquake.

(1)	Service scenario with on-board earthquake detection

An on-board computer system of a UAV analyses sensor data and detects earthquake in this service scenario. This scenario requires considerable on-board computing power and battery capacity.

- A UAV or UAVs patrol designated regions.

- The on-board computer system acquires overlapping aerial images of the regions and analyses sensor data.

- If an earthquake is detected, the sensor data and related information is sent to an earthquake incident command centre via wireless communication.

(2)	Service scenario with server-based earthquake detection

A ground computing facility analyses sensor data and detects earthquake in this service scenario. An on-board computer system of a UAV performs limited processing like resampling to the acquired sensor data.

- A UAV or UAVs patrol designated regions.

- The on-board computer system acquires overlapping aerial images of the regions and processes sensor data.

- The processed data is sent to a ground computing facility via wireless communication.

- The ground computing facility analyses the received sensor data.

- If an earthquake is detected, the sensor data and related information is sent to an earthquake incident command centre.

7.2.2	Response and relief phase

UAVs can be used for guiding earthquake response and evacuation in the response and relief phase. Use-cases are described below.

- Still images and live motion pictures from UAVs can be used for guiding earthquake response and evacuation efforts.

- UAVs carrying speakers can be used for alarming people in danger zone.

- UAVs can deliver relief packages to people.

7.2.3 Recovery and reconstruction phase

Images from UAVs can be used for damage assessment.

- Visible photographs, multispectral photographs and LIDAR data can be used to assess the extent of the damage and to monitor the recovery of the earthquake area.

# End Proposal

__________________