| Rapporteur Meeting of Question 21/16 |  | AVD-4796 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | STUDY GROUP 16 |
|  |  | Original: English |
| Question(s): | 21/16 | Changzhou, 26 - 29 September 2016 |
| RAPPORTEUR MEETING DOCUMENT |  |  |
| Source: | Ministry of Industry and Information Technology (UESTC, SWJTU, CATR), ZTE, Huawei |  |
| Title: | F.DICNReqs: Proposal to update clauses 3, 6, 7, 8 and 9 |  |

Summary

This is the updated draft based on the output document of draft Recommendation ITU-T F.DICNReqs “Requirements of deployment for information centric networks", as TD 383 produced at the ITU-T SG16 meeting in Geneva, 23 May-3 June 2016.

This contribution proposed to update clauses 3, 6, 7, 8 and 9 to the draft F.DICNReqs

CONTENTS

Page

1 Scope 4 2 References 4 3 Definitions 4 3.1 Terms defined elsewhere 4 3.2 Terms Defined within this document 5 4 Abbreviations 5 5 Convention 5 6 Overview 6 7 Scenarios for DICN 6 7.1 DICN for single ICN instance over multiple NSP networks 7 7.2 DICN for multiple independent ICN instances on a Single NSP Network 8 7.3 DICN for enabling interoperation between two ICN instances 9 8 Requirements for DICN 10 8.1 General requirements 10 8.2 Requirements for DICN-enabled switch 10 8.3 Requirements for DICN-enabled controller 11 9 Framework for DICN 11 10 Security Consideration 13 Bibliography 13

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure 7-1 An overview of ICN deployment 7 Figure 7-2 A single ICN instance deployed over multiple NSP networks 8 Figure 7 -3 multiple independent ICN instances deployed on a single NSP network 9 Figure 7-4 – Interoperation between two ICN instances 10 Figure 9-1 Framework of DICN 12 - - - - -

Draft new Recommendation F.DICNReqs

Requirements of deployment for information centric networks

AAP Summary

[To be provided before Consent]

Summary

[To be provided before Consent]

Keywords

information centric networks, software defined networking, deployment, interoperation

# Scope

This Recommendation describes the scenarios and requirements of deployment for information centric networks (DICN). The DICN supports flexible method for deploying various ICN (Information Centric Networks) instances on a single NSP (Network Service Provider) network, deploying an ICN instance over multiple NSP networks, and interoperating between different ICN instances.

This Recommendation describes scenarios and common requirements of the deployment for ICN. It also presents the framework for DICN. This Recommendation covers the following:

- Scenarios of DICN

- Requirements of DICN and its key components

- Framework for DICN

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below.

# Definitions

## 3.1 Terms defined elsewhere

This document uses the following terms defined elsewhere:

3.1.1	future network (FN) [ITU-T Y.3001]:  A network able to provide services, capabilities, and facilities difficult to provide using existing network technologies. A future network is either:

a) A new component network or an enhanced version of an existing one, or

b) A heterogeneous collection of new component networks or of new and existing component networks that is operated as a single network.

3.1.2	name [ITU-T Y.2091]:  A name is the identifier of an entity (e.g., subscriber, network

element) that may be resolved/translated into address.

## 3.2 Terms Defined within this document

This document defines the following terms

3.2.1	 ICN Service Provider (ICNSP): The operator provides content delivery service to users using ICN related technologies.

3.2.2	ICN Instance: An ICN instance is implemented on top of NSP networks by ICNSP to provide specific content delivery service to users.

3.2.3	DICN-enabled switch:  switch which can support DICN related operations including forwarding packets belonging to different ICN instances, providing interfaces to ICN instances to manage the resources allocated to them, supporting name-based routing, in-network cache and content-based security.

3.2.4	DICN-enabled controller: controller which perform DICN related tasks including providing interfaces for ICNSP to deploy their own ICN instances in the control plane with full control of the physical resources, providing interfaces between different ICN instances to exchange control information between each other, and providing functions of routing manager, content manager, storage manager, security manager and interoperation manager.

# Abbreviations

| API | Application Programming Interface |
| --- | --- |
| CONET | Content Networking |
| DICN | Deployment for Information Centric Networks |
| FN | Future Network |
| ICN | Information Centric Networks |
| ICNSP | ICN Service Provider |
| NDN | Named Data Networking |
| NetInf | Network of Information |
| NSP | Network Service Provider |
| PURSUIT | Publish-Subscribe Internet Technology |
| SDN | Software Defined Networking |

# Convention

In this Recommendation:

- The expression "is required to" indicates a requirement which must be strictly followed and from which no deviation is permitted if conformance to this Recommendation is to be claimed.

- The expression "is recommended" indicates a requirement which is recommended but not absolutely required. Thus this requirement need not be present to claim conformance.

# Overview

In recent years, future networks (FNs) have attracted considerable attention from both academia and industry. One of FN’s objectives is data awareness which enables retrieving data regardless of their locations. To achieve this objective, the concept of information centric networks (ICN) has been proposed and various ICN architectures have been proposed [b-ITU-T Y.3033][b-Dannewitz] [b-Jacobson] [b-Sarela] [b- Seskar] [b- Detti]. The next major challenge is to convince network service providers (NSPs) around the globe to adopt the concept of ICN and deploy ICN-enabled networks.

To achieve this goal, many prototypes and testbeds have been developed. In general, these proof-of-concept systems can be implemented in two approaches: overlay and non-overlay. With the first approach, some special overlay nodes are deployed to deal with ICN protocol-related operations on top of an existing physical network. For example, overlay-based systems have been developed for named data networking (NDN) [b-Jacobson] and network of information (NetInf) [b-Dannewitz].

On the other hand, in the non-overlay approach, ICN functions are implemented in routers directly. For instance, the prototype of Publish-Subscribe Internet Technology (PURSUIT) [[b-Sarela] is developed based on the Click modular router [b- Kohler], which allows network designers to create their own routers based on general-purpose personal computer hardware. Besides that, some ICN projects, such as MobilityFirst [b- Seskar] and Content Networking (CONET) [b- Detti], developed their prototypes based on software defined networking (SDN), which provides programmability to networks.

Despite the importance of these prototypes, the deployment of ICN is still an open question. One of the obvious reasons is that there are too many ICN candidates, which have different design principles and various technical details in terms of naming, routing, and so on. Taking content access as an example, while most ICN architectures are based on the assumption that a user will initiate a request and expect to access the content object as soon as possible, the PURSUIT project is based on the publish-subscribe model, in which both publisher and subscriber register to the ICN, and the publisher can “PUSH” content whenever it is generated. Clearly, these two approaches can be applied by different types of applications to optimize their performance.

As each ICN architecture has its own pros and cons and is best fit to some particular application requirements, various ICN architectures will be adopted in the near further. On the other hand, different ICN service providers (ICNSPs) can also implement their own ICN instances to provide specific services for users. For instance, one ICNSP provides streaming service and another ICNSP provides instant message service.  Thus, various ICN instances will coexist on top of the same physical network.

Nevertheless, existing ICN prototypes only focus on the implementation of individual ICN architectures. Neither the coexistence of multiple ICN instances in the same physical network nor the requirement of interoperation between different ICN instances is considered.

This document focuses on the issue of deployment for information centric networks (DICN), including the scenarios, requirements and framework.

# Scenarios for DICN

A general scenario of DICN is shown in Figure 7-1.

The deployment of ICN is partitioned into four layers. The first (bottom) layer is the geographical coverage layer, which specifies the coverage of NSPs’ networks.

Above the geographical coverage layer, the DICN-enabled network infrastructure layer which consists of multiple DICN-enabled networks established by different NSPs. Each NSP network consists of two types of networking elements, i.e., DICN-enabled switch and DICN-enabled controller. For example, in Figure 7-1 there are two NSP networks (i.e., NSP1 and NSP2).

The layer over the DICN-enabled network infrastructure layer is the ICN instances layer. In this layer, multiple ICN instances coexist, each of which is owned by a particular operator (which is defined as an ICN service provider (ICNSP)). Different ICN instances may adopt different ICN architectures.

An ICN users layer is defined as the top layer. Each ICN instance has its own group of users. A user of one ICNSP may move to a geographical area where her/his ICNSP does not cover. For example, a user of ICNSP1 may move to a place covered only by ICNSP4.

Figure 7-1 An overview of ICN deployment

## 7.1	DICN for single ICN instance over multiple NSP networks

With the current Internet and cellular technologies, users are typically associated with particular NSPs (e.g., AT&T and Verizon in the United States, or China Mobile and China Unicom in China).

On the other hand, these users who are associated with different NSPs may access the same Internet-based services, such as video streaming and online social networking. Thus, the traffic will traverse multiple NSP networks. However, all these Internet-based services are deployed on NSPs’ networks and do not separate from each other, which may not be optimized to support the services.

As ICN instances are also implemented on top of NSP networks to provide content delivery service, one common implementation scenario for DICN is one ICN instance is implemented over multiple NSP networks.  To guarantee the service quality, one ICNSP can rent resources from multiple NSP networks to deploy its own ICN and to optimize the content delivery efficiency.

For instance, ICNSP3 in Figure 7-2 is built on the networks of both NSP1 and NSP2. A user associated with ICNSP3 need not to be aware of the underlay NSP networks. When the user moves from one location in NSP1 network to another location in NSP2 network, it gets continuous services from ICNSP3 without interruption.

To facilitate an ICN instance across multiple NSP networks, the DICN-enabled network infrastructure provides both the data channels and control channels to allow the interconnection between DICN-enabled NSP networks.

The data channel is used for data transmission. The control channel is used to exchange the control information, e.g., routing and interconnection information. With such information, ICNSP3 set up communications across multiple NSP networks and provides unified service to its users.

Figure 7-2 A single ICN instance deployed over multiple NSP networks

## 7.2	DICN for multiple independent ICN instances on a Single NSP Network

Since different ICN architectures have their own pros and cons, multiple ICN instances employing different ICN architectures can coexist on top of the same NSP network.

One NSP network supports multiple ICNSPs, each of them creates its own ICN instance to provide optimized services to its own users. For example, as shown in Figure 7-3, ICNSP1 may choose NDN to provide services with the aim to optimize video streaming, while ICNSP2 may adopt PURSUIT to provide “PUSH” services to efficiently facilitate online social networking.

To allow the creation of multiple ICN instances on a single NSP network, the DICN-enabled network infrastructure provides a set of well-defined interfaces so that ICNSPs can perform ICN-architecture-related operations (name-based routing, in-network caching, etc.). In addition, the DICN-enabled network infrastructure also provides the capability of distinguishing flows belonging to different ICN instances.

Figure 7 -3 multiple independent ICN instances deployed on a single NSP network

## 7.3	DICN for interoperation between ICN instances

In a typical future Internet scenario, users are associated with ICNSPs. -,  A user of one ICNSP may need to access services from another ICNSP, where the two ICNSPs adopt different ICN architecture as shown in Figure 7-4.

One scenario is that U1 in ICNSP1 needs a content object which is owned by S1in ICNSP2 as shown in Figure 7-4 (a). Another scenario is that a user of ICNSP1 moves to a geographical area that is not covered by the underlying NSP1 of ICNSP1 as shown in Figure 7-4 (b).

To support interoperation, the DICN-enabled network infrastructure supports two typical types of solutions. The first one is tunnelling, with which a user of one ICNSP can connect to its ICN instance via tunnels established in another ICN instance. The second approach is to perform protocol translation in a DICN-enabled switch, which can efficiently serve the guest users.

(a)                                                                           (b)

Figure 7-4  – Interoperation between two ICN instances

# Requirements for DICN

This clause addresses the requirements for DICN.

## 8.1	General requirements

GEN-01: The DICN framework is required to decouple the data plane and the control plane. It is required to have one or multiple DICN-enabled switches and controllers.

GEN-02: The DICN framework is required to virtualize the resources of DICN-enabled network infrastructure to facilitate the ICN implementation and coexistence. That is, the software and hardware resources in DICN-enabled switches and controllers are required to be virtualized and provided to ICN instances.

GEN-03: The DICN framework is required to define appropriate inter-ICNSP interfaces to facilitate interoperations between two ICN instances.

GEN-04: The DICN framework is recommended to provide basic general functions, including name-based routing algorithms, in-network cache management and security functions as modules and the interfaces between different function modules are recommended to be defined. Then a particular ICN instance is deployed by reusing a set of selected function modules.

## 8.2	Requirements for DICN-enabled switch

DS-01: A DICN-enabled switch is required to support general ICN-related operations and process received  packets according to the defined rules configured by control programs (i.e., ICN instances). The general ICN-related operations include name-based routing, in-network caching, and content-based security.

DS-02: A DICN-enabled switch is required to provide the following primitive functions: forwarding packets and caching traversed content.

DS-03: A DICN-enabled switch is required to support virtualization for the implementation and coexistence of multiple ICN instances in the data plane. That is, the forwarding table and storage of DICN-enabled switch is required to be allocated for different ICN instances to guarantee the required service quality. Packets belonging to different ICN instances can be delivered to the appropriate control program and forwarded to appropriate interfaces.

DS-03: A DICN-enabled switch is required to provide the following primitive functions: forwarding packets and caching traversed content.

DS-04: A DICN-enabled switch is required to provide interfaces for ICN instances to manipulate the allocated resources, including forwarding table and storage.

DS-05: A DICN-enabled switch is recommended to provide the general ICN-related functions as modules. Then, ICN instances can be deployed by reusing these modules.

DS-06: A DICN-enabled switch is recommended to support inter-operation, including re-encapsulating packet and sending request according to the rules configured by control programs.

## 8.3	Requirements for DICN-enabled controller

DC-01: A DICN-enabled controller is required to provide interfaces for ICNSP to deploy their own ICN instances in the control plane with full control of the physical resources.

DC-02: A DICN-enabled controller is required to support virtualization for the coexistence of multiple ICN instances

DC-03: A DICN-enabled controller is required to support the registration of ICNSP.

DC-04: A DICN-enabled controller is required to support the registration of DICN-enable switches.

DC-05: A DICN-enabled controller is required to provide interfaces between different ICN instances to exchange control information between each other.

DC-06: A DICN-enabled controller is recommended to provide the general ICN-related functions as modules, including routing manager, content manager, storage manager, security manager and interoperation manager.

DC-07: A DICN-enabled controller is recommended to provide interfaces between different function modules. Then a particular ICN instance is deployed by reusing a set of selected function modules.

# Framework for DICN

This clause addresses the capabilities of key components of DICN.

Figure 9-1 Framework of DICN

Figure 9-1 shows the details of the framework of DICN, which is composed of the DICN-enabled switch and DICN-enabled controller.

- DICN-Enabled Switch

The DICN-enabled switch, consisting of the hardware and software systems, is responsible for forwarding packets belonging to different ICN instances. To support various ICN-related operations, including local caching decision and name recognition, there are six hardware elements in the hardware system: switching element, forwarding table, storage, forwarding management logic, storage management logic, and interoperation logic. These hardware resources are shared by different ICN instances. Such hardware resources are virtualized into different slices, each of which is dedicated to a particular ICN instance.

The DICN-enabled switch provides interfaces, including forwarding table application programming interface (API), storage management API, and interoperation API, to ICN instances to manage the resources allocated to them. The forwarding table API is used to instruct the forwarding management logic to manipulate one or more forwarding tables for a particular ICN instance. The storage management API is responsible for managing local caching policy in storage management logic or caching content in the content storage under the control of the DICN-Enabled controller. The interoperation API is designed to guide the interoperation logic to encapsulate packets and/or perform protocol translation received from an incompatible user.

- DICN-Enabled Controller

The DICN-Enabled controller is responsible for managing virtualized physical network resources for each ICN instance. The DICN-enabled controller provides five modules for ICNSPs, as shown in Figure 9-1. The content manager is responsible for maintaining information about content sources and destinations. The routing manager is used to maintain the topology of this ICN instance and perform ICN-related routing algorithms. The storage manager is used to set the policies for managing physical content storage. The security manager is responsible for providing secure service. The interoperation manager is used to guide the interoperation with other ICN instances. Each ICNSP can deploy its own ICN instance according to the specification of a particular ICN architecture by selecting appropriate modules to instantiate.

# Security Consideration

This Recommendation has some security considerations similar to those pertinent to data aware networking and aligns with the security requirements specified in [b-ITU-T Y.3033]. Specifically, the following are required:

- Secure channel: the channels between controller and switch and that among controllers are required to be secured and also have good privacy protection.

Bibliography

[b-ITU-T Y.3001]	ITU-T Recommendation Y.3001 (2011), Future networks: Objectives and design goals

[b-ITU-T Y.3033]	ITU-T Recommendation Y. 3033 (2014), Framework of data aware networking for future networks

[b-Dannewitz]	Dannewitz, C. (2009), NetInf: An Information-Centric Design for the Future Internet, In Proc. 3rd GI/ITG KuVS Workshop on The Future Internet, May.

[b- Detti]	Detti A, Blefari Melazzi N, Salsano S, et al. (2011), CONET: a content centric inter-networking architecture. In Proc. ACM SIGCOMM workshop on Information-centric networking. ACM.

[b-Jacobson]	Jacobson, V., et al. (2009), Networking Named Content, CoNEXT, December.

[b- Kohler]	Kohler E, Morris R, Chen B, et al.(2000), The Click modular router. ACM Transactions on Computer Systems (TOCS), 18(3)

[b-Sarela]	Särelä, M., et al. (2008), RTFM: Publish/Subscribe Internetworking Architecture, ICT-Mobile Summit, June.

[b- Seskar]	Seskar I, Nagaraja K, Nelson S, et al.,(2011) Mobilityfirst future internet architecture project. In Proc. 7th Asian Internet Engineering Conference. ACM.

_________________